| Product Name | HR_C7000 |
| --- | --- |
| Document Description | User Guide |
| Classification | Internal disclosure |

# HR_C7000 Chip User Guide

| Fiction | Xie Guojun | date | 2016-9-13​ |
| --- | --- | --- | --- |
| Reviewer |  |  |  |
| approve |  |  |  |

Zhejiang Dahua Technology Co., Ltd.

Formatting requirements:

- When writing documents, please pay attention to the prompt information in “ () ” and other italic text. “XXX” means that this part can be customized by the author.

- If the author believes that the current chapter needs to be expanded, he or she may add sub-chapter and give it a title of his or her own.

- Please be sure to follow the style information in the document template.

- The table of contents must contain: Title 1 , Title 2 , Title 3 , and Title 4 Inclusion in the catalog is at the discretion of the author.

- Please pay attention to keeping the style of graphics and tables consistent (for example: the size of module boxes in different graphics is basically uniform, the basic color of lines and graphics is black, and the basic font size of text is consistent ) .

Document modification history

| Revisionist | Completion Date | Revisions | Version number |
| --- | --- | --- | --- |
| Xie Guojun | 2016-9-13​ | First Draft | V1.0 |
| Xie Guojun | 2017-2-14​ | FullMask Version modification, IO Package Description | V2.0 |
| Xie Guojun | 2017-2-22​ | Modify the peripheral interface description, SPI , UART , GPIO wait | V2.1 |
| Xie Guojun | 2017-3- 15 | FullMask Version, pinmap Figure modification | V2.2 |
| Xie Guojun | 2017-4- 17 | FullMask Version, Ram The space becomes 288K , Corresponding address space modification | V2.3 |
| Xie Guojun | 2017-6- 16 | pinmap Latest picture update | V2.4 |
| Xie Guojun | 2017-6-27​ | Pinmap Pin description bitmap error correction, U S B Manual register description added | V2.5 |
| Xie Guojun | 2017-7- 14 | Voice 216 Bit information acquisition corresponding address empty For details, see Section 10.4.2 | V2.6 |
| Xie Guojun | 2017-7- 19 | RF_RECV_DELAY Register description revision Modify, add AF Mode parameter setting instructions. | V2.7 |

## Table of contents

HR_C7000 Chip User Guide 	1

- Preface 	31

  - Overview 	31

  - Table Content Conventions 	31

  - Register Access Type Convention 	31

  - Numerical unit convention 	31

- Product Overview 	32

  - Application Scenario 	32

  - Architecture 	32

    - Overview 	32

    - Processor core 	33

    - DMR Standard 	33

    - simulation FM 	33

    - Modulation, demodulation and channel coding and decoding 	34

    - simulation IP 	34

    - Vocoder supports 	34

    - RF interface 	34

    - Peripheral interface 	34

    - External memory interface 	34

  - Start Mode 	34

- Hardware Features 	35

  - HR_C7000 Package 	35

  - HR_C7000 Pin distribution 	37

  - HR_C7000 Pin Assignment Table 	38

  - HR_C7000 Pin Description 	41

    - System pin 	41

    - NFC Pin 	42

    - JTAG pin 	42

    - I2C Pin 	42

    - UART Pin 	43

    - PWM Pin 	43

    - SPI Pin 	43

    - SDIO pin 	44

    - I8080 ( LCD ) pin 	45

    - GPIO pin 	45

    - AK2401 Pin 	46

    - ADC Pin 	47

    - DAC Pin 	48

    - Codec Pin 	48

    - Power and ground pin 	48

  - Electrical performance parameters 	50

    - Voltage parameter 	50

    - Power consumption parameter 	50

    - Temperature parameter 	50

    - IP Indicator Parameter 	51

- System 	52

  - Reset 	52

    - Overview 	52

    - Functional Description 	52

      - POR 	53

      - Soft reset 	54

    - Register Overview 	55

    - Register Description 	55

      - SYS_SOFT_RSTN 	55

  - Clock 	55

    - Overview 	55

    - Functional Description 	56

      - Clock interface 	57

      - Clock frequency 	57

    - Working Mode 	57

      - Initialization 	57

      - Reconfiguration Operation Flow 	57

      - PLL Configuration 	59

    - Register Overview 	60

    - Register Description 	60

      - CLK_MGR_REG0x04 	60

      - CLK_MGR_REG0x08 	61

      - CLK_MGR_REG0x0c 	62

      - CLK_MGR_REG0x10 	62

      - CLK_MGR_REG0x14 	62

      - CLK_MGR_REG0x18 	63

      - CLK_MGR_REG0x1c 	63

      - CLK_MGR_REG0x20 	63

      - CLK_MGR_REG0x24 	63

      - CLK_MGR_REG0x28 	64

      - CLK_MGR_REG0x2c 	64

  - Low power control 	65

  - System Control 	67

    - Overview 	67

    - Features 	67

    - Register Overview 	67

    - Register Description 	68

      - LCSFC_BAUDR 	68

      - RTC_REQ_HOLD 	68

      - QUAD_ENABLE 	68

      - IO_DIPLEX0 	68

      - IO_DIPLEX1 	70

      - IO_DIPLEX2 	72

      - IOMGR_REN_REG0 	74

      - IOMGR_REN_REG1 	76

      - IOMGR_REN_REG2 	78

      - IOMGR_IE_IE0 	81

      - IOMGR_IE_IE1 	83

      - IOMGR_IE_IE2 	85

  - Processor subsystem 	88

    - Overview 	88

    - Features 	88

    - Address mapping relationship 	89

    - Start 	89

    - JTAG Debugging 	90

  - Baseband Communication System 	91

    - Overview 	91

    - Features 	91

    - Interrupt vector 	92

    - Register Overview 	94

    - Register Description 	95

      - DAC_CONTROL 	95

      - ADC_CONTROL 	95

      - AUDIO_CONTROL 	96

      - AUDIO_BUFFER_CLR 	97

      - LINEOUT_CTRL 	97

      - CODEC_I2C_MUX 	97

      - WORK_MODE 	97

      - RF_MODE 	98

      - SYS_INTERP_LIST 	100

      - SYS_INTERP_MASK 	100

      - SYS_INTERP_CLEAR 	102

      - LAYER3_INTERP_LIST 	102

      - LAYER3_INTERP_MASK 	103

      - LAYER3_INTERP_CLEAR 	104

      - INTERP_CLEAR 	105

  - Interrupt System 	105

    - Overview 	105

    - Functional Description 	105

    - Interrupt source assignment 	106

    - Working method 	107

      - Initialization 	107

      - Interrupt status query 	107

    - Register Overview 	107

    - Register Description 	108

      - PIC_MODE 	108

      - PIC_PO 	108

      - PIC_MASK 	108

      - PIC_VECTOR 	109

      - PIC_COW1 	109

      - PIC_PRIOR0- 3 	109

      - PIC_PRIOR4- 7 	110

      - PIC_PRIOR8- 11 	110

      - PIC_PRIOR12- 15 	110

      - PIC_PRIOR16- 19 	110

      - PIC_PRIOR20- 23 	110

      - PIC_PRIOR24- 27 	111

      - PIC_PRIOR28- 31 	111

      - PIC_COW2 	111

      - PIC_SYNC 	111

      - PIC_FFLAG_L 	112

      - PIC_INT_RECORD_SEL 	112

      - PIC_INT_ST 	112

      - PIC_INT_ST_1 	112

      - PIC_INT_ST_CNT 	112

      - PIC_MODE_1 	113

      - PIC_PO_1 	113

      - PIC_MASK_1 	113

      - PIC_PRIOR32- 35 	113

      - PIC_PRIOR36- 39 	114

      - PIC_PRIOR40- 43 	114

      - PIC_PRIOR44- 47 	114

      - PIC_PRIOR48- 51 	114

      - PIC_PRIOR52- 55 	114

      - PIC_PRIOR56- 59 	115

      - PIC_PRIOR60- 63 	115

      - PIC_FFLAG_H 	115

  - Timer 	115

    - Overview 	115

    - Functional Description 	116

    - Working method 	116

    - Register Overview 	116

    - Register Description 	117

      - Timer1LoadCount 	117

      - Timer1CurrentValue 	117

      - Timer1ControlReg 	117

      - Timer1EOI 	117

      - Timer1IntStatust 	118

      - Timer2LoadCount 	118

      - Timer2CurrentValue 	118

      - Timer2ControlReg 	118

      - Timer2EOI 	118

      - Timer2IntStatust 	119

      - Timer3LoadCount 	119

      - Timer3CurrentValue 	119

      - Timer3ControlReg 	119

      - Timer3EOI 	119

      - Timer3IntStatust 	119

      - Timer4LoadCount 	120

      - Timer4CurrentValue 	120

      - Timer4ControlReg 	120

      - Timer4EOI 	120

      - Timer4IntStatust 	120

      - Timer5LoadCount 	121

      - Timer5CurrentValue 	121

      - Timer5ControlReg 	121

      - Timer5EOI 	121

      - Timer5IntStatust 	121

      - Timer6LoadCount 	121

      - Timer6CurrentValue 	122

      - Timer6ControlReg 	122

      - Timer6EOI 	122

      - Timer6IntStatust 	122

      - TimersIntStatus 	122

      - TimersEOI 	123

      - TimersRawIntStatus 	123

      - TIMERS_COMP_VERSION 	123

  - RTC 	123

    - Overview 	123

    - Function Description 	123

    - Working Mode 	123

      - RTC Initialization 	124

      - Interrupt Handling 	124

    - Register Overview 	124

    - Register Description 	125

      - RTC_VAL_S 	125

      - RTC_VAL_M 	125

      - RTC_VAL_H 	125

      - RTC_VAL_D_L 	125

      - RTC_VAL_D_H 	126

      - RTC_MATCH_S 	126

      - RTC_MATCH_M 	126

      - RTC_MATCH_H 	126

      - RTC_MATCH_D_L 	126

      - RTC_MATCH_D_H 	126

      - RTC_MATCH_VEN 	127

      - RTC_MATCH_VSTAT 	127

      - RTC_LOAD_S 	127

      - RTC_LOAD_M 	127

      - RTC_LOAD_H 	127

      - RTC_LOAD_D_L 	127

      - RTC_LOAD_D_H 	128

      - RTC_CCR 	128

      - RTC_LOAD_VSTAT 	128

      - RTC_RSTAT 	128

      - RTC_CLKDIV_NUMER_L 	128

      - RTC_CLKDIV_NUMER_H 	129

      - RTC_CLKDIV_NUMER_VLD 	129

      - RTC_SOFT_RST_N 	129

  - Watchdog 	129

    - Overview 	129

    - Functional Description 	129

    - Working Mode 	130

      - Enable watchdog 	130

      - Disable watchdog 	130

      - Normal working 	130

    - Register Overview 	131

    - Register Description 	131

      - WDG_LOCK 	131

      - WDG_OT_LOAD_H 	131

      - WDG_OT_LOAD_L 	131

      - WDG_RST_LOAD 	132

      - WDG_EN 	132

      - WDG_KNOCK 	132

- Peripheral devices 	132

  - I2C132​​	​

    - Overview 	132

    - Functional Description 	132

    - Clock Frequency Configuration 	133

    - Working method 	133

      - Initialization 	133

      - Data transmission and reception 	134

      - Deactivate I²C​​ Controller 	134

    - Register Overview 	135

    - Register Description 	136

      - IC_CON 	136

      - IC_TAR 	137

      - IC_DATA_CMD 	137

      - IC_FS_SCL_HCNT 	137

      - IC_FS_SCL_LCNT 	137

      - IC_INTR_STAT 	138

      - IC_INTR_MASK 	138

      - IC_RAW_INTR_STAT 	138

      - IC_RX_TL 	139

      - IC_TX_TL 	139

      - IC_CLR_INTR 	139

      - IC_CLR_RX_UNDER 	140

      - IC_CLR_RX_OVER 	140

      - IC_CLR_TX_OVER 	140

      - IC_CLR_RD_REQ 	140

      - IC_CLR_TX_ABRT 	140

      - IC_CLR_RX_DONE 	141

      - IC_CLR_ACTIVITY 	141

      - IC_CLR_STOP_DET 	141

      - IC_CLR_START_DET 	141

      - IC_ENABLE 	141

      - IC_STATUS 	142

      - IC_TXFLR 	142

      - IC_RXFLR 	142

      - IC_TX_ABRT_SOURCE 	143

      - IC_ENABLE_STATUS 	144

      - IC_START 	144

  - SPI 	144

    - Overview 	144

    - Functional Description 	144

      - Features 	144

      - Typical Applications 	145

      - Clock polarity and phase configurable 	145

    - Working method 	147

      - Transmission Timing 	147

      - Clock and Reset 	147

      - Operation process 	148

      - SPI module 4 148 transmission modes

    - Register Overview 	150

    - Register Description 	152

      - CTRLR0 	152

      - CTRLR1 (for master ） 	153

      - SSIENR 	153

      - SER (only for master ） 	153

      - BAUDR (for master ） 	153

      - TXFTLR 	154

      - RXFTLR 	154

      - TXFLR 	154

      - RXFLR 	154

      - SR 	154

      - IMR 	155

      - ISR 	155

      - RISR 	155

      - TXOICR 	155

      - RXOICR 	156

      - RXUICR 	156

      - ICR 	156

      - IDR 	156

      - SSI_COMP_VERSION 	156

      - DR 	156

  - I80 	157

    - Overview 	157

    - Functional Description 	157

    - Working method 	157

    - Register Overview 	157

    - Register Description 	158

      - INDEX 	158

      - DATA 	158

      - WCFG 	158

      - RCFG 	158

      - SCFG 	159

      - AC_MODE 	159

  - UART 	159

    - Overview 	159

    - Functional Description 	159

    - Working Mode 	160

    - Register Overview 	162

    - Register Description 	163

      - RBR 	163

      - THR 	164

      - DLL 	164

      - DLH 	164

      - IER 	164

      - IIR 	165

      - FCR 	166

      - LCR 	166

      - MCR 	167

      - LSR 	167

      - USR 	168

      - TFL 	169

      - RFL 	169

  - SDIO 	169

    - Overview 	169

    - Functional Description 	169

    - Working method 	170

      - Overall Process 	170

      - Power Management 	170

      - Clock Control 	170

      - Interrupt Control 	171

      - Status Detection 	171

      - Device Enumeration 	171

      - Instruction Operation 	173

      - Data transmission 	174

      - Auto-stop Operation 	178

      - Stop data transmission 	178

      - Erase Operation 	178

    - Register Overview 	179

    - Register Description 	180

      - CTRL ( control register ) 	180

      - PWREN ( Power-on enable register ) 	181

      - CLKDIV ( Clock Divider Register ) 	181

      - CLKSRC ( Clock Source Register ) 	181

      - CLKENA ( clock enable register ) 	182

      - TMOUT(Timeout Register ) 	182

      - CTYPE ( Card Type Register ) 	182

      - BLKSIZ ( block size register ) 	182

      - BYTCNT (Byte Count Register ) 	182

      - INTMASK (Interrupt Mask Register ) 	183

      - CMDARG ( command_argument Register ) 	183

      - CMD (Command Register ) 	183

      - RESP0 (Response Reply Register 0 ） 	184

      - RESP1 (Response Reply Register 1 ) 	185

      - RESP2 (Response Reply Register 2 ) 	185

      - RESP3 (Response Reply Register 3 ) 	185

      - MINTSTS (Mask Interrupt Register ) 	185

      - RINTSTS (Raw Interrupt Status Register ) 	186

      - STATUS (Status Register ) 	186

      - FIFOTH​​ Threshold Register ) 	187

      - CDETECT (Card Detection Register ) 	188

      - WRTPRT (Write Protection Register ) 	188

      - TCBCNT ( CIU Module transmission byte Number register ) 	188

      - TBBCNT ( Host and BIU_FIFO Interaction data byte Number register ) 	188

      - DEBNCE (Card Detection Debounce Register (Unit: host clock Number ) 	188

      - USRID (User ID Register ) 	189

      - VERID (soft core version number register ) 	189

      - HCON (Soft Core Configuration Register ) 	189

      - RST_n (Card reset register ) 	189

      - CardThrCtl (Read Threshold Enable Register ) 	189

      - Backend_power​​ Power Register ) 	190

      - ENABLE_SHIFT (Phase Adjustment Control Register ) 	190

      - DATA ( FIFO Read and write entry address ) 	190

  - USB 	190

    - Overview 	190

    - Functional Description 	190

    - Working method 	191

    - Register Overview 	193

    - Register Description 	196

      - GAHBCFG 	196

      - GUSBCFG 	197

      - GRSTCTL 	197

      - GINTSTS 	198

      - GINTMSK 	199

      - GRXSTSR/ GRXSTSP 	200

      - GRXFSIZ 	201

      - GNPTXFSIZ 	201

      - GGPIO 	201

      - GUID 	202

      - GSYNPSID 	202

      - GHWCFG1 	203

      - GHWCFG2 	203

      - GHWCFG3 	203

      - GHWCFG4 	204

      - GUSRSTS 	204

      - DIEPTXF1/3 	205

      - DCFG 	205

      - DCTL 	206

      - DSTS 	207

      - DIEPMSK 	207

      - DOEPMSK 	208

      - DAINT 	208

      - DAINTMSK 	209

      - DTHRCTL 	209

      - DIEPMPMSK 	209

      - DIEPCTL0 	210

      - DIEPINT0/1/3 	211

      - DIEPTSIZ0 	211

      - DIEPDMA0/1/3 	212

      - DTXFSTS0/1/3 	212

      - DOEPCTL0 	212

      - DOEPINT0/2/4 	213

      - DOEPTSIZ0 	214

      - DOEPDMA0/2/4 	214

      - DIEPCTL1/3 	214

      - DIEPTSIZ1/3 	216

      - DOEPCTL2/4 	216

      - DOEPTSIZ2/4 	217

      - PCGCCTL 	218

  - GPIO 	218

    - Overview 	218

    - Functional Description 	218

    - Working method 	218

    - Register Overview 	219

    - Register Description 	220

      - SWPORTA_DR 	220

      - SWPORTA_DDR 	220

      - INTEN 	221

      - INTMASK 	221

      - INTTYPE_LEVEL 	221

      - INT_POLARITY 	221

      - INTSTATUS 	221

      - RAW_INTSTATUS 	221

      - DEBOUNCE 	222

      - PORTA_EOI 	222

      - EXT_PORTA 	222

      - INTR_BOTH_EDGE 	222

  - PWM 	222

    - Overview 	222

    - Functional Description 	222

    - Working method 	223

      - Continuous Mode 	223

      - One Shot Mode 	224

    - Register Overview 	224

    - Register Description 	225

      - PWM0_CFG 	225

      - PWM0_RPT 	225

      - PWM0_PER 	226

      - PWM0_FP 	226

      - PWM0_STATUS 226	​

      - PWM1_CFG 	226

      - PWM1_RPT 	227

      - PWM1_PER 	227

      - PWM1_FP 	227

      - PWM1_STATUS 227	​​

      - PWM2_CFG 	228

      - PWM2_RPT 	228

      - PWM2_PER 	229

      - PWM2_FP 	229

      - PWM1_STATUS 229	​​

      - PWM_INTR_STA 	229

  - ADC Controller 	229

    - Overview 	229

    - Functional Description 	230

    - Working method 	230

      - Single scan processing flow 	230

      - Single scan software operation process : 	232

      - Continuous Scan Processing Flow 	233

      - Continuous Scanning Software Operation Process 	234

      - Burr Filtering Process 	235

      - Low Power Process 	235

    - Register Overview 	235

    - Register Description 	236

      - ADC_CTRL 	236

      - ADC_INTR_DELTA 	236

      - ADC_INTR 	236

      - ADC_INTR_STA 	237

      - ADC_SCAN_TIME 	238

      - ADC_START 	238

      - ADC_CTRL_STATE 	238

      - ADC_INTR_THRESHOLE 	239

      - ADC_CTRL_STOP 	239

      - ADC_CH_VLD 	240

      - ADC_PD_SEOC_TIME 	240

      - ADC_P2S_EN 	240

      - ADC_DATA_AB 	240

      - ADC_DATA_CD 	241

      - ADC_DATA_EF 	241

      - ADC_DATA_GH 	241

      - ADC_CTRL_VERID 	241

      - ADC_DIR_DAT_OP 	241

  - DAC Controller 	242

    - Overview 	242

    - Functional Description 	242

    - Working method 	242

    - Register Overview 	242

    - Register Description 	242

      - DAC_PD_CTRL 	242

      - DAC_PD_MODE_EN 	242

      - DAC_DATA_A 	243

      - DAC_DATA_B 	243

      - DAC_DATA_C 	243

      - DAC_CTRL_VERID 	243

- CODEC Application 	244

  - Overview 	244

  - Functional Description 	244

  - Reference Circuit 	245

    - MIC Enter 	245

    - LINEOUT Output 	245

  - Working method 	246

    - Working mode description 	246

      - Power-off mode 	247

      - Power-down mode 	247

      - Playback mode 	247

      - Sleep mode 	247

      - Analog record mode 	247

    - Working mode switch 	247

      - Power-down Mode switched to voice playback mode 	247

      - Power-down Mode switch to sleep Mode 	248

      - Sleep Mode to CODEC ADC Opening 	248

      - Sleep Mode to audio playback mode 	248

      - Switch to any mode power-down Mode 	249

    - Codec Workflow 	249

      - Audio Interface Mode Settings 	249

      - Audio Send ( Analog record ） 	249

      - Audio Receive ( Play back ） 	250

    - Soft mute function mute ） 	251

    - AGC 	251

    - Soft clipping DRC 	252

    - Wind Noise Filter 	252

  - Register 	253

    - Register Overview 	253

    - Register Description 	253

      - SR1 	253

      - SR2 	254

      - MR 	254

      - AICR_DAC 	254

      - AICR_ADC 	255

      - CR_MIC1 	255

      - CR_MIC2 	256

      - CR_DAC 	256

      - CR_ADC 	256

      - CR_MIX 	256

      - DR_MIX 	258

      - CR_VIC 	259

      - CR_CK 	259

      - FCR_DAC 	259

      - SFCCR_DAC 	259

      - SFFCR_DAC 	259

      - FCR_ADC 	260

      - CR_TIMER_h 	260

      - CR_TIMER_l 	260

      - ICR 	260

      - IMR 	261

      - IFR 	261

      - IMR2 	261

      - IFR2 	261

      - GCR_DACL 	262

      - GCR_DACR 	262

      - GCR_MIC1 	263

      - GCR_ADCL 	263

      - GCR_ADCR 	263

      - GCR_MIXDACL 	263

      - GCR_MIXDACR 	264

      - GCR_MIXADCL 	264

      - GCR_MIXADCR 	264

      - CR_DAC_AGC 	265

      - DR_DAC_AGC 	265

      - CR_ADC_AGC 	266

      - DR_ADC_AGC 	266

- Audio channel 	268

  - DMR Mode 	268

    - Overview 	268

    - Working method 	269

      - Digital Audio Transmission 	269

      - Digital Audio Reception 	269

      - Digital call recording function 	270

        - Digital Send Recording 	270

        - Digital receiving recording 	270

  - FM Mode 	270

    - Overview 	270

    - Working method 	270

      - Analog audio send 	270

        - The audio source is CODEC  ADC 	271

        - The audio source is TX RAM 	271

      - Analog audio receiver 	271

      - Analog call recording function 	271

        - Simulate sending recording 	271

        - Analog receiving recording 	271

      - Analog sidetone playback 	272

  - Prompt tone channel 	272

    - Overview 	272

    - Functional Description 	272

      - Query-based prompt tone writing 	272

      - 272 interrupt -based prompts

- RF interface 	272

  - DAC Send 	272

    - Overview 	272

    - Functional Description 	272

    - Working method 	273

      - Two-point modulation 	273

      - Single-point modulation 	274

  - ADC Receive 	274

    - Overview 	274

    - Functional Description 	274

    - Working Mode 	275

      - AF Receive 	275

      - IF receiver 	275

  - Digital Receiver 	276

    - Overview 	276

    - Functional Description 	276

    - Working method 	276

- DMR Physical Layer Applications 	277

  - Overview 	277

  - Register 	278

    - Register Overview 	278

    - Register Description 	279

      - WORK_MODE 	279

      - RF_MODE 	280

      - SIG_CENTER 	281

      - TX_IF_FREQ 	281

      - RF_CONTROL 	282

      - RF_MOD_BIAS_CTRL 	282

      - DEV_LIMITER 	282

      - RSSI_BIAS_VALUE 	282

      - THRESHOLD_VALUE 	282

      - PHASE_OUT 	283

      - SEND_DATA_SYNC_H 	283

      - SEND_DATA_SYNC_L 	283

      - SEND_RC_SYNC_H 	284

      - SEND_RC_SYNC_L 	284

      - RECV_MS_SYNC_H 	284

      - RECV_MS_SYNC_L 	284

      - RECV_BS_SYNC_H 	284

      - RECV_BS_SYNC_L 	285

      - RECV_TDMA1_SYNC_H 	285

      - RECV_TDMA1_SYNC_L 	285

      - RECV_TDMA2_SYNC_H 	285

      - RECV_TDMA2_SYNC_L 	286

      - RECV_RC_SYNC_H 	286

      - RECV_RC_SYNC_L 	286

      - RECV_INFO_COUNT 	286

      - RECVFIR_COEF1 	287

      - RECVFIR_COEF2 	287

      - RECVFIR_COEF3 	287

      - RECVFIR_COEF4 	287

      - RECVFIR_COEF5 	288

      - RECVFIR_COEF6 	288

      - RECVFIR_COEF7 	288

      - RECVFIR_COEF8 	288

      - RECVFIR_COEF9 	289

      - RECVFIR_COEF10 	289

      - RECVFIR_COEF11 	289

      - RECVFIR_COEF12 	289

      - RECVFIR_COEF13 	289

      - RECVFIR_COEF14 	290

      - RECVFIR_COEF15 	290

      - RECVFIR_COEF16 	290

      - RECVFIR_COEF17 	290

      - RX_IF_FREQ 	291

      - RF_RECV_DELAY 	291

      - MODEM_AK_CLK_CALB 	291

      - AK2401_ADC_STATE 	291

      - PHY_RECEIVE_STATE 	292

      - LAYER2_CONTROL 	292

      - LAYER2_TXRX_CTRL 	292

  - Interrupt vector 	293

  - Memory Map 	293

    - Physical layer transmission Ram 	294

    - Physical layer reception Ram 	294

  - Physical layer sends 	294

    - Overview 	294

    - Functional Description 	294

    - Working method 	295

  - Physical layer receiving 	296

    - Overview 	296

    - Functional Description 	296

    - Working method 	296

- DMR Link Layer Application 	297

  - Overview 	297

  - Register 	297

    - Register Overview 	297

    - Register Description 	298

      - WORK_MODE 	298

      - PI_CRC_MASK 	299

      - RS_H_MASK 	299

      - RS_H_INIT 	299

      - RS_T_MASK 	300

      - RS_T_INIT 	300

      - CSBK_CRC_MASK 	300

      - MBCH_CRC_MASK 	300

      - DATAH_CRC_MASK 	300

      - CRC8_INIT 	301

      - LOCAL_CC 	301

      - VOICE_EMB_CTRL 	301

      - SCRAMBLE_REG 	301

      - RX_TYPE_INFO 	302

      - RX_INFO 	303

      - LAYER2_CONTROL 	303

      - LAYER2_ SLOTON 	304

      - LAYER2_TXRX_CTRL 	305

      - LAYER2_SLOT_CNT 	306

      - LAYER2_MAX_CNT 	306

      - LAYER2_SLOT_UPDATE 	306

      - LAYER2_SEND_TYPE 	307

      - LAYER2_STATUS 	309

      - LAYER2_SLOT_PRE 	309

      - LAYER2_BS_MODE 	309

      - BS_SECOND_REG 	309

      - BS_STATUS 	310

      - BS_CACH 	310

  - Interrupt vector 	310

  - Memory Map 	312

    - Link layer transmission Ram 	312

    - Link layer reception Ram 	313

  - Link Layer Timeslot Management 	314

    - Overview 	314

    - Send time slot 	314

      - Send time slot active establishment mode 	314

      - Send time slot receive establishment mode 	315

      - Transmit time slot temporary establishment mode 	315

    - Send temporary time slot 	316

      - Temporary time slot active establishment mode 	316

      - Temporary time slot reception establishment mode 	316

      - Temporary timeslot transmission establishment mode 	317

    - Receive time slot 	317

      - Receive time slot active establishment mode 	317

      - Receive time slot passive establishment mode 	317

    - Receive time slot synchronization strategy 	318

      - Transmitting time slot and receiving time slot synchronization 	318

      - Transmit time slot and transmit temporary time slot synchronization 	319

      - Send temporary time slot and receive time slot synchronization 	322

  - The link layer sends 	324

    - Overview 	324

    - Frame Type Description 	324

      - Speech frame 	324

      - Data control frame 	325

      - RC Frame 	327

    - Framing Mode 	327

    - Frame Type Definition 	328

    - Sending Process 	329

  - Link layer receives 	330

    - Overview 	330

    - Functional Description 	330

    - Receiving Application 	330

- FM Application 	331

  - Overview 	331

  - Register 	331

    - Register Overview 	331

    - Register Description 	332

      - WORK_MODE 	332

      - FM_BANDWIDTH 	332

      - FM_DEV_COEF 	333

      - FM_SQL 	333

      - SUBVOICE_COEF 	333

      - FM_MOD 	333

      - CTCSS_ADDR 	334

      - USER_DEFINE_CTC 	334

      - CDCSS_ADDR 	335

      - SELCALL_TONE 	335

      - TONE_SEND_CTRL 	335

      - XTCSS_ADDR 	336

      - FM_FUNCTION_COEF1 	336

      - FM_FUNCTION_COEF2 	336

      - FM_FUNCTION_COEF3 	336

      - FM_FUNCTION_COEF4 	336

      - FM_FUNCTION_COEF5 	337

      - FM_FUNCTION_COEF6 	337

      - FM_FUNCTION_COEF7 	337

      - FM_FUNCTION_COEF8 	337

      - FM_TONE1 	337

      - FM_TONE2 	338

      - FM_TONE3 	338

      - FM_TONE4 	338

      - FM_PTT 	338

      - FM_DECODE_FLAG1 	338

      - FM_DECODE_FLAG2 	338

      - FM_ADDR_SW 	339

  - Interrupt vector 	339

  - Memory Map 	340

    - FM send Ram 	340

    - FM take over Ram 	340

  - SQL 	340

  - CTCSS 	340

    - Overview 	340

    - Functional Description 	340

      - CTCSS Send 	341

      - CTCSS Receive 	342

    - Working method 	344

      - CTCSS Send 	344

      - CTCSS Receive 	345

  - CDCSS 	345

    - Overview 	345

    - Functional Description 	345

    - Working method 	345

      - CDCSS Send 	345

      - CDCSS Receive 	346

  - DTMF 	346

    - Overview 	346

    - Functional Description 	346

    - Working method 	347

      - DTMF Send 	347

      - DTMF Receive 	348

  - SELCALL 	349

    - Overview 	349

    - Functional Description 	349

    - Working method 	349

      - SELCALL Send 	349

      - SELCALL Receive 	350

12 	Terms 	350

- Abbreviations 	350

- appendix A : Digital group call application 	351

  - Functional Description 	351

  - Group call sent 	352

  - Group call received 	353

- appendix B : Digital SMS Application 	355

  - Functional Description 	355

  - SMS Service Sequence 	356

  - SMS data frame structure 	357

    - Data frame header structure 	357

    - Data frame structure 	359

    - Response data suspension time 	360

    - Response data frame 	361

  - IP V 4 Structural Description 	362

    - IPv4 Packet Data 	363

    - IPv4 Response Packet Data 	364

  - Non-confirmation SMS 	365

    - Non-confirmed SMS sent 	365

    - Unconfirmed SMS Receive 	366

  - Confirm SMS 	368

    - Confirm SMS sent to 	368

    - Confirm SMS receipt 	369

- appendix C : Digital Signaling Application 	370

  - Functional Description 	370

  - Signaling sent 	370

  - Signaling Receiving 	371

- appendix D : Digital Encryption Applications 	372

  - Overview 	372

  - Voice Encryption 	373

    - Basic Encryption at Rest 	373

    - Dynamic Enhanced Encryption 	373

      - Encryption Parameter Definition 	374

        - Initial vector 	374

        - KeyID and ALGOID 	376

      - Encrypted parameter passing 	377

        - Initial Key Transfer and Encryption Process 	377

        - Initial Key Receiving and Decryption Process 	378

      - Encrypted Control Frame 	379

        - Voice frame header 	379

        - voice PI Frame rate 	380

        - voice F frame EMB 	380

  - Data Encryption 	380

    - SMS encryption timing 	380

    - SMS Encryption Parameter Transmission 	381

- appendix E ：number DCDM Application 	382

  - Overview 	382

  - Basic Principles 	382

    - State Machine Type 	382

      - Leader and slot unknown status 	382

      - Leader unknown status 	382

      - Leader and slot known status 	383

      - Leader Status 	383

    - Data Frame Definition 	383

      - Synchronous frame definition 	383

      - Color code definition 	383

      - Channel Timing CSBK 	383

        - CT_CSBK_Beacon 	384

        - CT_CSBK_Prop 	384

        - CT_CSBK_Term 	384

        - CT_CSBK_Req 	385

        - CT_CSBK_Resp 	385

    - DCDM State Flow 	385

      - Leader and slot unknown status 	385

      - Leader and Slot Unknown State Process 	386

      - Leader unknown state process 	387

      - Leader and Slot Known State Process 	388

      - Leader State Process 	389

      - Leader ID Conflict State Process 	390

      - CT_CSBK Information Assessment Status Process 	392

      - Send Correction CT_CSBK State Flow 	392

      - Accept leader status process 	393

      - Designate a new leader state process 	394

      - Scheduled Push Status Process 	395

  - Application Example 	397

    - Basic Unit 	397

      - Time slot unit 	397

      - Leader 397​

      - Generation 	397

      - Synchronous Period 	399

      - Identifier 	399

      - Channel Timing Opcode 	400

      - New Leader Logo 	401

    - CT_CSBK PDU 	401

      - CT_CSBK Structure 	401

      - CT_CSBK Field Definition 	401

    - Timer Definition 	402

      - CT_RHOT Description 	402

    - Minimize System Example 	405

      - Single machine startup 	405

      - Boot leader confirms 	406

      - Beacon message sends 	407

      - Business sending process 	408

      - Leader Offline 	409

    - HR_C7000 Application 	410

      - Synchronous field application 	410

      - Color Code Application 	411

      - Time Slot Management 	411

Figure Catalog

picture 1 HR_C7000 Single chip application scenario 	32

picture 2 Chip overall block diagram 	33

picture 3 HR_C7000 Chip package top view 	35

picture 4 HR_C7000 Chip package bottom view 	36

picture 5 HR_C7000 Chip package side view 	36

picture 6 HR_C7000 Pin distribution diagram 	38

picture 7 Reset overall block diagram 	53

picture 8 Reset overall block diagram 	53

picture 9 POR Module block diagram 	54

picture 10 POR Module timing diagram 	54

picture 11 Clock Distribution Diagram 	56

picture 12 Reconfiguration Flowchart 	59

picture 13 PLL Circuit diagram 	60

picture 14 HR_C7000 Functional module division 	66

picture 15 HR_C7000 Functional Power Control Distribution 	66

picture 16 HR_C7000 Low Power Workflow 	67

picture 17 CPU System overall block diagram 	88

picture 18 System startup flow chart 	90

picture 19 CK803 JTAG Debugging 	91

picture 20 Interrupt controller functional block diagram 	105

picture twenty one Watchdog shutdown timing diagram 	130

picture twenty two I²C​​ Bus data transfer process 	133

picture twenty three Master device SPI Typical connection method 	145

picture twenty four Slave device SPI Typical connection method 	145

picture 25 SCPH=0 Data transmission and sampling timing 	146

picture 26 SCPH=1 Data transmission and sampling timing 	147

picture 27 SPI Data Serial Timing 	147

picture 28 SPI Transmit & Receive Mode 	149

picture 29 SPI Transmit Only Mode 	149

picture 30 SPI Receive Only Mode 	150

picture 31 SPI EEPROM Read Transfer Mode 	150

picture 32 8080 Interface Timing 	157

picture 33 UART Frame format 	159

picture 34 Interrupt mode data transmission flow chart 	161

picture 35 Interrupt mode data reception flow chart 	162

picture 36 SD Card enumeration and initialization process 	172

picture 37 SD Memory data transfer state transition diagram 	175

picture 38 SDIO Device data transmission state transition diagram 	175

picture 39 CTRL OUT Transmission Timing 	191

picture 40 CTRL IN transmission timing 	192

picture 41 No data CTRL Transmission Timing 	192

picture 42 Single OUT Transaction Sequence 	192

picture 43 repeatedly OUT Transaction Sequence 	193

picture 44 Single IN Transaction Sequence 	193

picture 45 repeatedly IN Transaction Sequence 	193

picture 46 PWM Output Timing Diagram 	223

picture 47 ONE_SHOT Normal mode diagram 	223

picture 48 CONTINUOUS Mode diagram 	223

picture 49 CONTINUOUS Mode software operation diagram 	224

picture 50 ADC_INTR_THRESHOLE and ADC_INTR_DELTA Relationship diagram 	230

picture 51 ADC_CTRL Single Scan Software Operation Process 	232

picture 52 ADC_CTRL Continuous Scan Channel Polling Diagram 	233

picture 53 ADC_CTRL Continuous scanning software operation flow chart 	234

picture 54 ADC_CTRL Low power mode timing diagram 	235

picture 55 Codec Module function block diagram 	244

picture 56 Codec Mic Differential and Single-Ended Schematics 	245

picture 57 Codec Micbias Differential and Single-Ended Schematics 	245

picture 58 Codec PWM Output Schematic 	246

picture 59 Voice sending data flow diagram 	249

picture 60 Voice reception data flow diagram 	250

picture 61 CODEC Automatic Gain Control ( AGC ) System Block Diagram 	251

picture 62 Two-point modulation interface signal 	273

picture 63 Single-point modulation interface signal 	274

picture 64 AF Receive reference interface circuit 	275

picture 65 IF receiving reference interface circuit 	276

picture 66 HR_C7000 Digital interface diagram 	276

picture 67 Digital SPI Interface Timing Diagram 	277

picture 68 DMR Digital Baseband Physical Layer Structure 	277

picture 69 DMR Standard transmission frame structure 	295

picture 70 Physical layer parsing data sequence 	296

picture 71 Physical layer receiving state flow chart 	297

picture 72 DMR Digital Baseband Link Layer Structure 	297

picture 73 Sending time slot actively establishes timing diagram 	315

picture 74 Transmitting time slot receiving time slot establishment timing diagram 	315

picture 75 Transmitting time slot temporary time slot establishment timing diagram 	316

picture 76 Receiving time slot passive establishment timing diagram 	318

picture 77 Transmit time slot synchronization to receive time slot timing diagram 	319

picture 78 Timing diagram 	of receiving time slot synchronization to transmitting time slot 319

picture 79 Transmitting time slot synchronization to transmitting temporary time slot timing diagram 	322

picture 80 Timing diagram 	of receiving time slot synchronization to sending temporary time slot 324

picture 81 Voice frame with synchronous frame structure 	324

picture 82 Voice frame with EMB Data Structure 	324

picture 83 Data frame, control frame structure 	325

picture 84 Control Channel C_RC Frame structure (PDT) 	325

picture 85 independent RC frame structure 	327

picture 86 Time slot voice timing 	328

picture 87 Continuous Speech Timing 	328

picture 88 Single slot data timing 	328

picture 89 Dual-slot data timing 	328

picture 90 Link layer transmission timing diagram 1 	329

picture 91 Link layer transmission timing diagram 2 	329

picture 92 Link layer receiving state flow chart 	330

picture 93 CTCSS Sending status flow chart 	344

picture 94 DTMF Sending block diagram 	347

picture 95 DTMF Code element frequency correspondence 	347

picture 96 DTMF Sending timing diagram 	348

picture 97 SELCALL Sending sequence diagram 	349

picture 98 Voice Service Structure Diagram 	351

picture 99 Group call sending service sequence diagram 1 	352

picture 100 Group call sending service sequence diagram 2 	352

picture 101 Group call reception service sequence diagram 	354

picture 102 Group call receiving state flow chart 	354

picture 103 SMS service application flow chart 	355

picture 104 SMS Service Sequence 	356

picture 105 Non-confirmed SMS service structure diagram 	356

picture 106 Confirm SMS service structure diagram 	357

picture 107 Data frame header PDU Structure 	357

picture 108 Unconfirmed Rate 1/2 Data block structure diagram 	359

picture 109 Data frame tail PDU Structural Diagram 	360

picture 110 Response data frame PDU Structural diagram 	361

picture 111 Feedback data package error pattern update process 	362

picture 112 IPv4 Structural diagram 	363

picture 113 UDP Verify and calculate the definition of each field 	363

picture 114 Non-confirmed SMS sending service sequence diagram 	365

picture 115 Non-confirmed SMS receiving service sequence diagram 	366

picture 116 SMS Receiving Flowchart 	367

picture 117 Confirm SMS sending service sequence diagram 	368

picture 118 Confirm SMS reception service sequence diagram 	369

picture 119 CSBK Signaling Service Structure Diagram 	370

picture 120 CSBK Signaling service sending sequence diagram 	370

picture 121 CSBK Signaling service reception sequence diagram 	371

picture 122 CSBK Signaling service receiving flow chart 	372

picture 123 Static encryption process diagram 	373

picture 124 Dynamic encryption process diagram 	373

picture 125 PI Frame PDU Structural diagram 	374

picture 126 Initial Vector Generation Process 	375

picture 127 Initial vector generation EMB transmission mode 	376

picture 128 KeyID and algorithm ID voice F Frame Transfer Mode 	376

picture 129 Dynamic key transmission update diagram 	377

picture 130 Dynamic key parameter encryption transfer flow chart 	378

picture 131 Dynamic key parameter decryption transfer flow chart 	379

picture 132 Unconfirmed SMS encryption sequence diagram 	381

picture 133 Confirm SMS encryption timing diagram 	381

picture 134 Custom data frame header PDU Structural diagram 	381

picture 135 Channel timing CSBK Termination 	385

picture 136 Power on and channel change state flow chart 	386

picture 137 Leader and Slot Unknown State Flowchart 	387

picture 138 Leader unknown state flow chart 	388

picture 139 Leader and Slot Known State Flowchart 	389

picture 140 Leader State Flowchart 	390

picture 141 Leader ID conflict state flow chart 	391

picture 142 CT_CSBK Information Assessment State Flowchart 	392

picture 143 Send correction CT_CSBK State Flowchart 	393

picture 144 Accepting Leader State Flowchart 	394

picture 145 Designate New Leader State Flowchart 	395

picture 146 Scheduled push status flow chart 	396

picture 147 Coverage diagram 	of multiple radio devices in DCDM mode 398

picture 148 Channel timing diffusion timing diagram 	399

picture 149 CT_CSBK Structural Diagram 	401

picture 150 CT_RHOT Using Flowchart 	404

picture 151 Single machine startup flowchart 	405

picture 152 Startup leader confirmation flow chart 	407

picture 153 Beacon message sending flow chart 	408

picture 154 Business sending flow chart 	409

picture 155 Leader Offline Flowchart 	410

picture 156 True dual-slot network access status management flow chart 	412

## Table of Contents

surface 1 HR_C7000 Chip packaging parameter description table 	36

surface 2 HR_C7000 Pin Assignment Table 	38

surface 3 I/O Type Description 	41

surface 4 Pull up and down agreement 	41

surface 5 System pin 	41

surface 6 NFC pin 	42

surface 7 JTAG Pin 	42

surface 8 I2C Pin 	42

surface 9 UART Pin 	43

surface 10 PWM Pin 	43

surface 11 SPI Pin 	43

surface 12 SDIO Pin 	44

surface 13 I8080 Pin 	45

surface 14 GPIO Pin 	45

surface 15 AK2401 Pin 	46

surface 16 ADC Pin 	47

surface 17 DAC Pin 	48

surface 18 Codec Pin 	48

surface 19 Power ground pin 	48

surface 20 Voltage parameter 	50

surface twenty one Temperature parameter 	50

surface twenty two IP Indicator Parameter 	51

surface twenty three POR Design parameter table 	54

surface twenty four Clock interface signal 	57

surface 25 Clock frequency range 	57

surface 26 Address mapping relationship 	89

surface 27 Baseband operating mode list 	91

surface 28 Baseband Communication System Interrupt List 	92

surface 29 Baseband communication system sub-interrupt list 	92

surface 30 Interrupt Source List 	106

surface 31 Watchdog working status table 	130

surface 32 I²C​​ Interface signal 	133

surface 33 Send instruction register setting 	173

surface 34 SD Memory read operation instruction set 	176

surface 35 SD Memory write operation instruction setting 	177

surface 36 Auto-Stop Supported card types and transfer modes 	178

surface 37 Legend symbol definition 	191

surface 38 Codec Micbias Parameter Description 	245

surface 39 Codec PWM Output Parameter Description 	246

surface 40 Codec Working Mode Definition 	246

surface 41 Codec Wind noise filter parameters 	252

surface 42 HR_C7000 Baseband transmit control register configuration 	272

surface 43 HR_C7000 Baseband receive control register configuration 	274

surface 44 Physical layer interrupt type list 	293

surface 45 Physical layer system interrupt sub-interrupt type list 	293

surface 46 Physical layer transmission Ram Allocation 	294

surface 47 Physical Receive Ram Allocation 	294

surface 48 Physical layer transmission configuration table 	295

surface 49 Physical layer receiving configuration table 	296

surface 50 Link layer interrupt type list 	310

surface 51 Link layer system interrupt sub-interrupt type list 	311

surface 52 Link layer transmission Ram Assignment 	312

surface 53 Link layer reception Ram Assignment 	313

surface 54 Group call 72bit LC Information Form 	324

surface 55 Link layer transmission frame type list 	328

surface 56 FM System Interrupt Sub-interrupt Type List 	339

surface 57 FM Send Ram Allocation 	340

surface 58 FM Receive Ram Allocation 	340

surface 59 CTCSS List 341 of mapping relationships between sending addresses and standard sub-tone frequencies

surface 60 CTCSS Receiving address and signal correlation detection coefficient coef Mapping relationship list 	342

surface 61 Unacknowledged data frame header PDU Definition 	358

surface 62 Confirm data frame header PDU Definition 	358

surface 63 Data frame body byte number table 	359

surface 64 1/2 Rate non-acknowledged data block (R_1_2_DATA) PDUs​ Content 	359

surface 65 1/2 Rate non-acknowledged last data block (R_1_2_LDATA) PDUs​ Content 	359

surface 66 3/4 Rate with confirmation data block (R_3_4_DATA) PDUs​ Content 	360

surface 67 3/4 Rate Band Acknowledgement Last Data Block (R_3_4_LDATA) PDUs​ Content 	360

surface 68 Data frame tail PDU Definition 	361

surface 69 Response data frame header PDU Definition 	362

surface 70 IPv4 Packet Data 	363

surface 71 IPv4 Response Packet Data 	364

surface 72 Static Basic Encryption Configuration Table 	373

surface 73 Dynamic Enhanced Encryption Configuration Table 	374

surface 74 Voice frame header definition 	380

surface 75 voice PI Frame Definition 	380

surface 76 Privacy Data Header 	381

surface 77 Synchronization frame field definition 	383

surface 78 Generation ( Gen ) Definition 	397

surface 79 Synchronous Period ( SA ) Definition 	399

surface 80 WATID Definition 	399

surface 81 Identifier ( ID ) definition 	400

surface 82 DI Definition 	400

surface 83 CTO Definition 	400

surface 84 New Leader Defining 	401

surface 85 CT_CSBK PDU Defining 	401

surface 86 Timer constant definition 	402

surface 87 CT_RHOT Value distribution table 	402

## Register Table Directory

Register Table 1 Reset Register Overview (Base Address: 0x11000000 ) 	55

Register Table 2 Clock Management Module Register Overview (Base Address: 0x11000000 ) 	60

Register Table 3 System Control Register Overview (Base Address: 0x11000000 ) 	67

Register Table 4 Overview of Baseband System Control Registers (Base Address: 0x11000000 ) 	94

Register Table 5 PIC Register Overview (Base Address : 0x1700_0000 ) 	107

Register Table 6 Timer Register Overview (Base Address: 0x1400_0000 ) 	116

Register Table 7 RTC Register Overview (Base Address: 0x1408_0000 ) 	124

Register Table 8 WDT Register Overview (Base Address: 0x1401_0000 ) 	131

Register Table 9 I2C0 Register Overview (Base Address is 0x14060000 ) 	135

Register Table 10 I2C1 Register Overview (Base Address is 0x14070000 ) 	135

Register Table 11 SPI0 Register Overview (Base Address : 0x140A_0000 ) 	150

Register Table 12 SPI1 Register Overview (Base Address : 0x140B_0000 ) 	151

Register Table 13 SPI2 Register Overview (Base Address : 0x140E_0000 ) 	152

Register Table 14 I80 Register Overview (Base Address : 0x1200_0000 ) 	157

Register Table 15 UART0 Register Overview (Base Address: 0x1403_0000 ) 	162

Register Table 16 UART1 Register Overview (Base Address: 0x1404_0000 ) 	163

Register Table 17 UART2 Register Overview (Base Address: 0x1405_0000 ) 	163

Register Table 18 SDIO Register Overview (Base Address : 0x1500_0000 ) 	179

Register Table 19 SDIO Register Overview (Base Address : 0x1500_0000 ) 	193

Register Table 20 GPIOA Register Overview (Base Address : 0x1402_0000 ) 	219

Register Table twenty one GPIOB Register Overview (Base Address: 0x1410_0000 ) 	219

Register Table twenty two GPIOC Register Overview (Base Address: 0x1411_0000 ) 	220

Register Table twenty three PWM Register Overview (Base Address : 0x140C_0000 ) 	224

Register Table twenty four ADC Register Overview (Base Address: 0x140D_0000 ) 	235

Register Table 25 DAC Register Overview (Base Address : 0x140D_0000 ) 	242

Register Table 26 Codec Register Overview (Base Address : 0x1600_09C0 ) 	253

Register Table 27 Physical Layer Register Overview (Base Address : 0x1100_0000 ) 	278

Register Table 28 Link Layer Register Overview (Base Address : 0x1100_0000 ) 	297

Register Table 29 FM Register Overview (Base Address : 0x1100_0000 ) 	331

## Preface

#### Overview

This article introduces the features and logical structure of the HR_C7000 series chips, details the various functional blocks, operating modes, register definitions, etc., and describes the chip pin definitions and uses as well as the chip's performance parameters and package dimensions.

#### Table content conventions

| content | illustrate |
| --- | --- |
| - | Indicates that the table cell where the symbol is located has no content |

#### Register access type convention

| type | illustrate |
| --- | --- |
| RO | Read-only, not writable |
| WO | Write only |
| RW | Readable and writable |
| RC | Read Clear |
| WC | Read, write 1 Clear, write 0 Remain unchanged. |

#### Numerical unit conventions

| category | symbol | Corresponding values |
| --- | --- | --- |
| Data capacity (such as memory capacity ) | 1K | 1024 |
|  | 1M | 1,048,576 |
|  | 1G | 1,073,741,824 |
| Frequency, data rate, etc. | 1k | 1,000 |
|  | 1M | 1,000,000 |
|  | 1G | 1,000,000,000 |

## Product Overview

HR_C7000 The chip is a processing chip for professional digital intercom systems, mainly used in handheld digital intercom terminals , digital repeaters, and simplifies product design. It has a built -in high-performance digital baseband modulation and demodulation, MCU Processor, vocoder and AD/DA interface and voice codec module, further improving the integration of single chip. At the same time, the chip integrates DMR Standard digital intercom protocol and built-in MCU The integration of powerful and rich features and complex protocol supplements has greatly lowered the development threshold of digital intercom systems.

#### Application Scenario

HR_C7000 The chip is a SOC for digital walkie-talkie system products chip.

FLASH

SPI	UART

USB

SDIO

LineOut

HR_C7000

DMR/FM

Analog

MIC_IN

GPIO I2C	I80	PWM	SPI

RF TX/RX

Figure 1 HR_C7000 Single-chip application scenarios

#### Architecture

###### Overview

HR_C7000 It is a digital intercom system processing SOC Chip, except for the integrated CPU In addition, it also includes a variety of peripherals such as SPI , UART , I2C , GPIO , PWM , ADC , DAC , SDIO , USB , etc. It integrates a high-performance 4FSK modem, channel codec, protocol processor, and also includes analog modules such as RTC , POR , DAC , ADC and Codec .

CPU_SYS

ROM	RAM

2KB	288KB

LCSFC

IAHB

iahb

CK803S

CLK IN

SPI

MIC

SPK

DAC

ADC

PLL

HR_C7000

RxI	RxQ

16bit TxI	TxQ PCM

Reg	Moderm

Ctrl

AHB Master wrap

RAM 5.8KB

Data Buffer

JTAG

IRQ

RAM

32KB

System

ahb

16bit

PCM

AHB

8bit

RAMdata

32bit

Regdata

2:1

APB

USB

CTRL

SDIO

GPIO	I2C	UART	SPI	PWM

I2C

USB1.1

Timer

RTC

ADC

DAC

Watch dog

Timer

ADC CTRL

DAC CTRL

i8080

Codec

Icache

4KB

FLASH

Figure 2 Chip overall block diagram

###### Processor core

CPU Using Zhongtian 32 CK803S​ processor.

###### DMR standard

- ETSI compliant TS102 361 ( DMR ) Tier I/II/III Standard protocol design

- Support independent control of physical layer, data link layer and call control layer

- Support true dual-slot synchronization header detection

- Using TDMA Technology, supporting full-duplex, half-duplex voice, data communication and simultaneous interpretation of data and speech

- Support IP Data Services

- Support single-frequency and dual-frequency relay

- Support 4.8Kbps and 9.6Kbps Data transmission

- Support digital analog intelligent detection

- Supports relay voice and data functions

- Support voice encryption function

- Support true dual time slot function

###### Analog FM

- Support 12.5KHz/25KHz channel communication

- Support emphasis and de-emphasis

- Support CDCSS/CTCSS Sub-audio processing

- Support 2-tone/5-tone deal with

- Support DTMF deal with

- Support analog squelch function

###### Modulation, demodulation and channel coding and decoding

- High-performance 4FSK Modem

- Integrated protocol-defined channel codec

###### Simulated IP

- High-performance 10-bit Bit -width ADC

- High-performance 12-bit Bit-width DAC

- LDO , using 3.3V Power supply, single power supply design

- High-performance PLL

- High-performance Codec , supports differential or single-ended Mic Input and Line_out Output

###### Vocoder support

- Support HR_V3000 ( Hongrui AMBE+2) , SELP Vocoder (Tsinghua ) , AVDS Vocoder ( 712 ) etc. SPI

The vocoder of the interface also provides an interface for digital recording, playback and prompt tone input

- Support digital voice encryption

###### RF interface

- The transmitting RF interface adopts single-ended output, analog two-point modulation and digital AK2401 interface

- The receiving RF interface adopts differential input mode, supports baseband IQ , IF and AF , and supports AK2401 Digital Interface

- The bias and amplitude of the two-way signal can be adjusted independently

- Support user-configured GPIO Control RF channels

###### Peripheral Interface

- Support POR and external reset

- Integration 4 UARTs​ Interface, 3 of which Can be reused as GPIO

- Integration 3 I2C​ Interface, 2 of which Can be reused as GPIO , 1 RTC​ use

- Integration 1 I8080​ Interface, can be reused as GPIO

- Integration 4 SPI​ Interface, supports master-slave mode; NFC SPI Fixed for nor Flash Interface, supports QualSPI Flash , and the other 3 can be reused as GPIO

- Integration 3 PWM​ Interface, can be reused as GPIO

- Integrated JTAG Interface, can be reused as GPIO

- Integration 8 Channel ADC Interface, 10bit Bit width, can be reused as GPIO

- Integration 3 Channel DAC Interface, 12bit Bit width, can be reused as GPIO

- Integrated RTC , supports 32.768K crystals

- Integrated USB Interface, support USB1.1 standard

- Integrated SDIO Interface, supports single-line and 4 Line Mode

###### External Memory Interface

- support SPI 	Flash

#### Startup Mode

- Support serial port boot mode

- support SPI 	Flash start up

## Hardware Features

#### HR_C7000 Encapsulation

HR_C7000 Using STFBGA Package, package size 11mm x 11mm x1.27mm , the pin pitch is 0.65mm , and the total number of pins is 220 .

Figure 3 HR_C7000 Chip package top view

Figure 4 HR_C7000 Chip package bottom view

Figure 5 HR_C7000 Chip package side view

Table 1 HR_C7000 Chip packaging parameter description table

| Symbol | Dimension in mm |  |  |
| --- | --- | --- | --- |
|  | MIN | NOM | MAX |

| A |  |  | 1.27 |
| --- | --- | --- | --- |
| A1 | 0.16 | 0.21 | 0.26 |
| A2 | 0.91 | 0.96 | 1.01 |
| c | 0.22 | 0.26 | 0.30 |
| D | 10.90 | 11.0 | 11.10 |
| E | 10.90 | 11.0 | 11.10 |
| D1 |  | 10.10 |  |
| E1 |  | 10.10 |  |
| e |  | 0.65 |  |
| b | 0.25 | 0.30 | 0.35 |
| aaa | 0.15 |  |  |
| bbb | 0.10 |  |  |
| ddd | 0.08 |  |  |
| eee | 0.15 |  |  |
| fff | 0.08 |  |  |
| MD/ME | 16/16 |  |  |

#### HR_C7000 Pin distribution

HR_C7000 A total of 220 The pin distribution is shown in the following figure:

Figure 6 HR_C7000 Pin distribution diagram

#### HR_C7000 Pin Assignment Table

Table 2 HR_C7000 Pin Assignment Table

| Location | Pin Name | Location | Pin Name |
| --- | --- | --- | --- |
| A1 | DVSS | J1 | DAC_VOUT_A |
| A2 | ADC1_IN | J2 | DAC_VOUT_B |
| A3 | ADC3_IN | J3 | DAC_AVDD33 |
| A4 | ADC6_IN | J5 | DAC_VREFP |
| A5 | ADC7_IN | J6 | DVSS |
| A6 | JTG_TDI | J7 | DVSS |
| A7 | SPI0_MOSI | J8 | DVSS |
| A8 | SPI0_CSN_0 | J9 | DVSS |
| A9 | I2C_SCL_1 | J10 | DVSS |
| A10 | UART1_TXD | J11 | DVSS |

| A11 | PTB27 | J12 | LDO_AVSS |
| --- | --- | --- | --- |
| A12 | LCD_DB6 | J14 | PWM_2 |
| A13 | Codec_MiC_INP2 | J15 | DVSS |
| A14 | Codec_MiC_INP1 | J16 | UART2_TXD |
| A15 | Codec_MICBIAS | K1 | DAC_VOUT_MCUA |
| A16 | DVSS | K2 | DAC_VOUT_MCUB |
| B1 | ADC0_IN | K3 | DAC_AVSS33 |
| B2 | ADC_VREF_MCU | K5 | DAC_VREFN |
| B3 | ADC2_IN | K6 | DVSS |
| B4 | ADC5_IN | K7 | DVSS |
| B5 | JTG_TCK | K8 | DVSS |
| B6 | JTG_TDO | K9 | DVSS |
| B7 | SPI0_MISO | K10 | DVSS |
| B8 | SPI0_CSN_1 | K11 | DVSS |
| B9 | I2C_SDA_1 | K12 | LDO_DVDD12_0 |
| B10 | PTB29 | K14 | UART2_RXD |
| B11 | PTB26 | K15 | PTB13 |
| B12 | LCD_DB7 | K16 | PTB14 |
| B13 | Codec_MIC_INN2 | L1 | DAC_VOUT_MCUC |
| B14 | Codec_MIC_INN1 | L2 | DVSS |
| B15 | Codec_VCAP | L3 | ADC_AVDD33 |
| B16 | Codec_VREFP | L5 | DVSS |
| C1 | DVSS | L6 | DVSS |
| C2 | ADC_AVSS33_MCU | L7 | DVSS |
| C3 | ADC_AVDD33_MCU | L8 | DVSS |
| C4 | ADC4_IN | L9 | DVSS |
| C5 | JTG_TMS | L10 | DVSS |
| C6 | JTG_RST_N | L11 | DVSS |
| C7 | DVSS | L12 | DVSS |
| C8 | SPI0_SCLK | L14 | PTB15 |
| C9 | UART1_RXD | L15 | PTB16 |
| C10 | PTB28 | L16 | UART3_RXD |
| C11 | PTB25 | M1 | ADC_VINP_Q |
| C12 | PTB24 | M2 | ADC_VINM_Q |
| C13 | Codec_AVDD | M3 | ADC_VREF |
| C14 | Codec_LINOUT2_LP | M5 | LDO_DVDD12_3 |
| C15 | Codec_LINOUT1_LP | M6 | PLL_AVDD |
| C16 | DVDD25 | M7 | PLL_AVSS |
| D1 | RTC_CLK_IN | M8 | DVSS |
| D2 | RTC_CLK_OUT | M9 | DVSS |
| D3 | LDO_RTC_AVDD33 | M10 | DVSS |
| D14 | Codec_AVSS33 | M11 | DVDD33 |
| D15 | DVSS | M12 | DVDD33 |

| D16 | Codec_LINOUT_VSS | M14 | UART3_TXD |
| --- | --- | --- | --- |
| E1 | DVSS | M15 | PTB17 |
| E2 | DVSS | M16 | PWM_1 |
| E3 | LDO_RTC_AVSS33 | N1 | ADC_VINCM_Q |
| E5 | RTC_AVSS | N2 | ADC_VINP_I |
| E6 | DVSS | N3 | ADC_AVSS33 |
| E7 | DVSS | N14 | NFC_CSN |
| E8 | LDO_DVDD12_1 | N15 | NFC_SCLK |
| E9 | LDO_AVDD33_1 | N16 | NFC_MOSI |
| E10 | LDO_AVSS | P1 | ADC_VINCM_I |
| E11 | DVSS | P2 | ADC_VINM_I |
| E12 | DVDD33 | P3 | LDO_AVDD33_3 |
| E14 | DVSS | P4 | LDO_AVSS |
| E15 | LCD_DB5 | P5 | PTB20 |
| E16 | LCD_DB4 | P6 | PTB23 |
| F1 | USB_DP | P7 | PTB3 |
| F2 | USB_DM | P8 | PTB6 |
| F3 | RTC_DVDD12 | P9 | I2C_SDA_0 |
| F5 | DVSS | P10 | UART0_TXD |
| F6 | DVSS | P11 | SPI1_MISO |
| F7 | DVSS | P12 | BS_INTER_IN |
| F8 | DVSS | P13 | PTB8 |
| F9 | DVSS | P14 | PTB11 |
| F10 | DVSS | P15 | NFC_MISO |
| F11 | DVSS | P16 | NFC_HOLD |
| F12 | DVDD33 | R1 | ADC_AVDD33 |
| F14 | LCD_DB3 | R2 | POR_EN |
| F15 | LCD_DB2 | R3 | NRESET |
| F16 | LCD_DB1 | R4 | PTB19 |
| G1 | DVSS | R5 | PTB21 |
| G2 | DVSS | R6 | CLK_OUT |
| G3 | USB_AVDD33 | R7 | PTB2 |
| G5 | DVSS | R8 | PTB5 |
| G6 | DVSS | R9 | I2C_SCL_0 |
| G7 | DVSS | R10 | SPI1_CSN_0 |
| G8 | DVSS | R11 | SPI1_MOSI |
| G9 | DVSS | R12 | BS_INTER_OUT |
| G10 | DVSS | R13 | PTB7 |
| G11 | DVSS | R14 | PTB10 |
| G12 | DVSS | R15 | PTB12 |
| G14 | LCD_DB0 | R16 | NFC_WP |
| G15 | LCD_RD | T1 | DVSS |
| G16 | LCD_WR | T2 | OSC_CLK_IN |

| H1 | PR_G | T3 | TEST_MODE |
| --- | --- | --- | --- |
| H2 | PR_V | T4 | PTB18 |
| H3 | DVSS | T5 | PTB22 |
| H5 | DVSS | T6 | PTB0 |
| H6 | DVSS | T7 | PTB1 |
| H7 | DVSS | T8 | PTB4 |
| H8 | DVSS | T9 | UART0_RXD |
| H9 | DVSS | T10 | SPI1_CSN_1 |
| H10 | DVSS | T11 | SPI1_SCLK |
| H11 | DVSS | T12 | TIME_SLOT_R_INTER |
| H12 | LDO_AVDD33_0 | T13 | TIME_SLOT_T_INTER |
| H14 | LCD_RS | T14 | PTB9 |
| H15 | LCD_CS | T15 | PWM_0 |
| H16 | LCD_NRESET | T16 | DVSS |

#### HR_C7000 Pin Description

The pin IO type description is shown in the following table.

Table 3 I/O Type Description

| I/O type | describe |
| --- | --- |
| I | Input signal |
| O | Output signal |
| I/O | Bidirectional input / output signal |
| P | power supply |
| G | land |

Pin IO The pull-up and pull-down instructions are shown in the following table.

Table 4 Pull-up and pull-down agreements

| Pull up and down | describe |
| --- | --- |
| U | Pull-up |
| D | drop down |
| - | No pull-up or pull-down |

IO Multiplexing control and IO For pull-up and pull-down control registers, refer to Section 4.4 System Control IO Related control registers.

###### System pins

Table 5 System pins

| HR_C7000 PIN | Pin Name | type | Drive (mA) | up and down pull | Voltage (V) | describe |
| --- | --- | --- | --- | --- | --- | --- |
| T2 | OSC_CLK_IN | I | - | - | 3.3 | Clock input |
| R6 | CLK_OUT | I/O | 4 | - | 3.3 | Function 0: GPIO PTC0 Function 1: PLL clock frequency division output; Function 2: RTC test clock output |
| R3 | NRESET | I | - | U | 3.3 | External reset input signal |
| R2 | POR_EN | I | - | U | 3.3 | POR enabled, |

|  |  |  |  |  |  | 1 : Enable POR function; 0 : Disable POR function |
| --- | --- | --- | --- | --- | --- | --- |
| T3 | TEST_MODE | I | - | D | 3.3 | Test mode selection: 1: Enter Test mode; 0 : Chip normal working mode |
| D1 | RTC_CLK_IN | I | - | - | 3.3 | RTC clock input |
| D2 | RTC_CLK_OUT | O | - | - | 3.3 | RTC clock feedback output |
| F1 | USB_DP | I/O | - | - | 3.3 | USB differential data |
| F2 | USB_DM | I/O | - | - | 3.3 | USB differential data |

###### NFC Pins

Table 6 NFC Pins

| HR_C7000 PIN | Pin Name | type | Drive (mA) | up and down pull | Voltage (V) | describe |
| --- | --- | --- | --- | --- | --- | --- |
| N15 | NFC_SCLK | O | 16 | U | 3.3 | NFC SPI interface clock, supports up to 96MHz . |
| N14 | NFC_CSN | O | 8 | U | 3.3 | Nor​ Flash SPI interface chip select signal, default Main Mode |
| N16 | NFC_MOSI | I/O | 8 | U | 3.3 | Serial SPI data output; QualSPI data IO0 |
| P15 | NFC_MISO | I/O | 8 | U | 3.3 | Serial SPI data input; QualSPI data IO1 |
| R16 | NFC_WP | I/O | 8 | U | 3.3 | WP control signal of serial SPI ; QualSPI data IO2 |
| P16 | NFC_HOLD | I/O | 8 | U | 3.3 | Hold control signal of serial SPI ; QualSPI digital According to IO3 |

###### JTAG Pins

Table 7 JTAG Pins

| HR_C7000 PIN | Pin Name | type | Drive (mA) | up and down pull | Voltage (V) | describe |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C5 | JTG_TMS | I/O | 4 | U | 3.3 | Function 0: JTAG mode select input Function 1: GPIO PTA0 , general purpose input and output |  |
| B5 | JTG_TCK |  |  |  |  |  | Function 0: JTAG clock input |
|  |  | I/O | 8 | U | 3.3 |  |  |
|  |  |  |  |  |  | Function 1: GPIO PTA1 , general purpose input and output |  |
| A6 | JTG_TDI | I/O | 4 | D | 3.3 | Function 0: JTAG data input Function 1: GPIO PTA2 , general purpose input and output |  |
| B6 | JTG_TDO | I/O | 4 | D | 3.3 | Function 0: JTAG data output Function 1: GPIO PTA3 , general purpose input and output |  |
| C6 | JTG_RST_N | I/O | 4 | U | 3.3 | Function 0: JTAG reset input Function 1: GPIO PTA4 , general purpose input and output |  |

###### I2C Pins

Table 8 I2C Pins

| HR_C7000 PIN | Pin Name | type | Drive (mA) | up and down pull | Voltage (V) | describe |
| --- | --- | --- | --- | --- | --- | --- |
| R9 | I2C_SCL_0 | I/O | 4 | U | 3.3 | Function 0: I2C_SCL_0, I2C0 clock Function 1: GPIO PTA5 , general purpose input and output |
| P9 | I2C_SDA_0 | I/O | 4 | U | 3.3 | Function 0: I2C_SDA_0, I2C0 data/address Function 1: GPIO PTA6 , general purpose input and output |
| A9 | I2C_SCL_1 | I/O | 4 | U | 3.3 | Function 0: I2C_SCL_1, I2C1 clock Function 1: GPIO PTA7 , general purpose input and output |
| B9 | I2C_SDA_1 | I/O | 4 | U | 3.3 | Function 0: I2C_SDA_1, I2C1 data/address Function 1: GPIO PTA8 , general purpose input and output |

###### UART Pins

Table 9 UART Pins

| HR_C7000 PIN | Pin Name | type | Drive (mA) | up and down pull | Voltage (V) | describe |
| --- | --- | --- | --- | --- | --- | --- |
| T9 | UART0_RXD | I | 4 | U | 3.3 | Serial port 0 data input, can be used for debugging and boot upgrade class |
| P10 | UART0_TXD | O | 4 | U | 3.3 | Serial port 0 data output can be used for debugging and boot upgrade class |
| C9 | UART1_RXD | I/O | 4 | U | 3.3 | Function 0: UART1_RXD , serial port 1 data input Function 1: GPIO PTA9 , general purpose input and output |
| A10 | UART1_TXD | I/O | 4 | U | 3.3 | Function 0: UART1_TXD , serial port 1 data output Function 1: GPIO PTA10 , general purpose input and output |
| K14 | UART2_RXD | I/O | 4 | U | 3.3 | Function 0: UART1_RXD , serial port 2 data input Function 1: GPIO PTA11 , general purpose input and output |
| J16 | UART2_TXD | I/O | 4 | U | 3.3 | Function 0: UART1_TXD , serial port 2 data output Function 1: GPIO PTA12 , general purpose input and output |
| L16 | UART3_RXD | I/O | 4 | U | 3.3 | Function 0: UART1_RXD , serial port 3 data input Function 1: GPIO PTA13, general input and output Function 2 : Baseband RF transmission start interrupt output |
| M14 | UART3_TXD | I/O | 4 | U | 3.3 | Function 0: UART1_TXD , serial port 3 data output Function 1: GPIO PTA14, general input and output Function 2 : Baseband RF transmission end interrupt output |

###### PWM Pins

Table 10 PWM Pins

| HR_C7000 PIN | Pin Name |  |  |  | superior |  |  |  | describe |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | kind type | Drive (mA) |  |  |  |  |  |  |
|  |  |  |  | Down pull |  |  | Voltage (V) |  |  |
| T15 | PWM_0 | I/O | 4 | U |  |  | 3.3 |  | Function 0: PWM_0 , pulse width modulation 0 output Function 1: GPIO PTA15 , general purpose input and output |
| M16 | PWM_1 | I/O | 4 | U |  |  | 3.3 |  | Function 0: PWM_1 , pulse width modulation 1 output Function 1: GPIO PTA16 , general purpose input and output |
| J14 | PWM_2 | I/O | 4 | D |  |  | 3.3 |  | Function 0: PWM_2 , pulse width modulation 2 output Function 1: GPIO PTA17 , general purpose input and output |

###### SPI Pins

Table 11 SPI Pins

| HR_C7000 PIN | Pin Name | type | Drive (mA) | up and down pull | Voltage (V) | describe |
| --- | --- | --- | --- | --- | --- | --- |
| A8 | SPI0_CSN_0 | I/O | 4 | U | 3.3 | Function 0: SPI0 chip select 0 output, SPI0 master mode Function 1: SPI0 chip select input, SPI0 slave mode Function 2: GPIO PTA18 , general purpose input and output |
| B8 | SPI0_CSN_1 | I/O | 4 | U | 3.3 | Function 0: SPI0 chip select 1 output, SPI0 master mode Function 1: GPIO PTA19 , general purpose input and output |
| C8 | SPI0_SCLK | I/O | 8 | D | 3.3 | Function 0: SPI0 clock output, SPI0 master mode Function 1: SPI0 clock input, SPI0 slave mode Function 2: GPIO PTA20 , general purpose input and output |
| A7 | SPI0_MOSI | I/O | 4 | U | 3.3 | Function 0: SPI0 data output, SPI0 master mode Function 1: SPI0 data input, SPI0 slave mode Function 2: GPIO PTA21 , general purpose input and output |
| B7 | SPI0_MISO | I/O | 4 | D | 3.3 | Function 0: SPI0 data input, SPI0 master mode Function 1: SPI0 data output, SPI0 slave mode Function 2: GPIO PTA22 , general purpose input and output |
| R10 | SPI1_CSN_0 | I/O | 4 | U | 3.3 | Function 0: SPI1 chip select 0 output, SPI1 master mode Function 1: SPI1 chip select input, SPI1 slave mode Function 2: GPIO PTA23, general input and output Function 3: SDIO data, SDIO_DAT3 |

| T10 | SPI1_CSN_1 | I/O | 4 | U | 3.3 | Function 0: SPI1 chip select 1 output, SPI1 master mode Function 1: GPIO PTA24, general input and output Function 2 : Baseband system interrupt output |
| --- | --- | --- | --- | --- | --- | --- |
| T11 | SPI1_SCLK | I/O | 8 | D | 3.3 | Function 0: SPI1 clock output, SPI1 master mode Function 1: SPI1 clock input, SPI1 slave mode Function 2: GPI1 PTA25, general purpose input and output Function 3: SDIO clock output, SDIO_CLK |
| R11 | SPI1_MOSI | I/O | 4 | U | 3.3 | Function 0: SPI1 data output, SPI1 master mode Function 1: SPI1 data input, SPI1 slave mode Function 2: GPIO PTA26, general purpose input and output Function 3: SDIO command, SDIO_CMD |
| P11 | SPI1_MISO | I/O | 4 | U | 3.3 | Function 0: SPI1 data input, SPI1 master mode Function 1: SPI1 data output, SPI1 slave mode Function 2: GPIO PTA27, general purpose input and output Function 3: SDIO data, SDIO_DAT0 |
| H15 | LCD_CS | I/O | 4 | U | 3.3 | Function 0: SPI2 chip select output, SPI2 master mode Function 1: SPI2 chip select input, SPI2 slave mode Function 2: GPIO PTC3, general input and output Function 3: I8080 CS , LCD_CS |
| H14 | LCD_RS | I/O | 8 | U | 3.3 | Function 0: SPI2 clock output, SPI2 master mode Function 1: SPI2 clock input, SPI2 slave mode Function 2: GPIO PTC4, general purpose input and output Function 3: I8080 RS , LCD_RS |
| G14 | LCD_DB0 | I/O | 4 | D | 3.3 | Function 0: SPI2 data input, SPI2 master mode Function 1: SPI2 data output, SPI2 slave mode Function 2: GPIO PTC7, general purpose input and output Function 3: I8080 data, LCD_DB0 |
| F16 | LCD_DB1 | I/O | 4 | D | 3.3 | Function 0: SPI2 data output, SPI2 master mode Function 1: SPI2 data input, SPI2 slave mode Function 2: GPIO PTAC8, general input and output Function 3: I8080 data, LCD_DB1 |

###### SDIO Pins

HR_C7000	管脚名称 PIN

类	驱动	上

型	(mA)	下拉

电压（V）	描述

功能 0：SDIO 时钟输出，SDIO_CLK

T11

SPI1_SCLK

I/O	8	D

3.3

R11

SPI1_MOSI

I/O	4	U

3.3

P11

SPI1_MISO

I/O	4	U

3.3

R12

BS_INTER_OUT

I/O	4	U

3.3

P12

BS_INTER_IN

I/O	4	U

3.3

R10

SPI1_CSN_0

I/O	4	U

3.3

T12

TIME_SLOT_R_IN

TER

I/O

4	D

3.3

功能 1：SPI1 时钟输出，SPI1 主模式

功能 2：SPI1 时钟输入，SPI1 从模式功能 3：GPI1 PTA25，通用输入输出功能 0：SDIO 命令，SDIO_CMD

功能 1：SPI1 数据输出，SPI1 主模式功能 2：SPI1 数据输入，SPI1 从模式功能 3：GPIO PTA26，通用输入输出功能 0：SDIO 数据，SDIO_DAT0

功能 1：SPI1 数据输入，SPI1 主模式功能 2：SPI1 数据输出，SPI1 从模式功能 3：GPIO PTA27，通用输入输出功能 0: SDIO 数据，SDIO_DAT1

功能 1：基带 BS 模式中断输出

功能 2：GPIO PTA29 通用输入输出功能 0：SDIO 数据，SDIO_DAT2

功能 1：基带 BS 模式中断输入

功能 2：GPIO PTA28，通用输入输出功能 0：SDIO 数据，SDIO_DAT3

功能 1：SPI1 片选 0 输出，SPI1 主模式功能 2：SPI1 片选输入，SPI1 从模式 功能 3：GPIO PTA23，通用输入输出

功能 0：SDIO 卡检测，SDIO_CARD_DETECT功能 1：基带 30ms 接收时隙中断输出

功能 2：GPIO PTA30，通用输入输出

Table 12 SDIO Pins

| T13 | TIME_SLOT_T_IN TER | I/O | 4 | D | 3.3 | Function 0: SDIO write protection, SDIO_CARD_CWPR Function 1 : Baseband 30ms send time slot interrupt output Function 2: GPIO PTA31, general input and output Function 3: POR reset output |
| --- | --- | --- | --- | --- | --- | --- |

###### I8080 ( LCD ) pins

Table 13 I8080 Pins

| HR_C7000 PIN | Pin Name |  |  | type |  |  | Drive (mA) |  | up and down pull |  | Voltage (V) | describe |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H16 | LCD_NRESET |  |  | I/O |  |  | 4 |  | U |  | 3.3 | GPIO PTC2 , general purpose input and output |
| H15 | LCD_CS |  |  | I/O |  |  | 4 |  | U |  | 3.3 | Function 0: I8080 CS , LCD_CS Function 1: SPI2 chip select output, SPI2 master mode Function 2: SPI2 chip select input, SPI2 slave mode Function 3: GPIO PTC3 , general purpose input and output |
| H14 | LCD_RS |  |  | I/O |  |  | 8 |  | U |  | 3.3 | Function 0: I8080 RS , LCD_RS Function 1: SPI2 clock output, SPI2 master mode Function 2: SPI2 clock input, SPI2 slave mode Function 3: GPIO PTC4 , general purpose input and output |
| G16 | LCD_WR |  |  | I/O |  |  | 4 |  | U |  | 3.3 | Function 0: I8080 WR , LCD_WR Function 1: GPIO PTC5 , general purpose input and output |
| G15 | LCD_RD |  |  | I/O |  |  | 4 |  | U |  | 3.3 | Function 0: I8080 RD , LCD_RD Function 1: GPIO PTC6 , general purpose input and output |
| G14 | LCD_DB0 |  |  | I/O |  |  | 4 |  | D |  | 3.3 | Function 0: I8080 data, LCD_DB0 Function 1: SPI2 data input, SPI2 master mode Function 2: SPI2 data output, SPI2 slave mode Function 3: GPIO PTC7 , general purpose input and output |
| F16 | LCD_DB1 |  |  |  |  |  | 4 |  | D |  | 3.3 | Function 0: I8080 data, LCD_DB1 Function 1: SPI2 data output, SPI2 master mode Function 2: SPI2 data input, SPI2 slave mode Function 3: GPIO PTAC8 , general purpose input and output |
|  |  |  |  | I/O |  |  |  |  |  |  |  |  |
| F15 |  |  |  |  |  |  |  |  |  |  |  | Function 0: I8080 data, LCD_DB2 Function 1: GPIO PTAC9 , general purpose input and output |
|  |  | LCD_DB2 |  | I/O |  |  | 4 |  | D |  | 3.3 |  |
| F14 |  |  |  |  |  |  |  |  |  |  |  | Function 0: I8080 data, LCD_DB3 Function 1: GPIO PTAC10 , general purpose input and output |
|  | LCD_DB3 |  |  |  | I/O |  | 4 |  |  | U | 3.3 |  |
| E16 | LCD_DB4 |  |  | I/O |  |  | 4 |  | U |  | 3.3 | Function 0: I8080 data, LCD_DB4 Function 1: GPIO PTAC11 , general purpose input and output |
| E15 | LCD_DB5 |  |  | I/O |  |  | 4 |  | U |  | 3.3 | Function 0: I8080 data, LCD_DB5 Function 1: GPIO PTAC12 , general purpose input and output |
| A12 | LCD_DB6 |  |  | I/O |  |  | 4 |  | U |  | 3.3 | Function 0: I8080 data, LCD_DB6 Function 1: GPIO PTAC13 , general purpose input and output |
| B12 | LCD_DB7 |  |  | I/O |  |  | 4 |  | D |  | 3.3 | Function 0: I8080 data, LCD_DB7 Function 1: GPIO PTAC14 , general purpose input and output |

###### GPIO Pins

Table 14 GPIO Pins

| HR_C7000 PIN | Pin Name | type | Drive (mA) | up and down pull | Voltage (V) | describe |
| --- | --- | --- | --- | --- | --- | --- |
| T6 | PTB0 | I/O | 4 | U | 3.3 | GPIO PTB0 , general purpose input and output |
| T7 | PTB1 | I/O | 4 | U | 3.3 | GPIO PTB1 , general purpose input and output |
| R7 | PTB2 | I/O | 4 | U | 3.3 | Function 0: GPIO PTB2 , general purpose input and output Function 1: AK2401, AK_DAC_FS |

| P7 | PTB3 |  | I/O |  |  | 4 |  | U | 3.3 |  | Function 0: GPIO PTB3 , general purpose input and output Function 1: AK2401, AK_DAC_SCLK |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T8 | PTB4 |  | I/O |  |  | 4 |  | D | 3.3 |  | Function 0: GPIO PTB4 , general purpose input and output Function 1: AK2401, AK_DAC_SDI |  |  |
| R8 | PTB5 |  | I/O |  |  | 4 |  | D | 3.3 |  | Function 0: GPIO PTB5 , general purpose input and output Function 1: AK2401, AK_ADC_FS |  |  |
| P8 | PTB6 |  | I/O |  |  | 4 |  | D | 3.3 |  | Function 0: GPIO PTB6 , general purpose input and output Function 1: AK2401, AK_ADC_SCLK |  |  |
| R13 | PTB7 |  | I/O |  |  | 4 |  | U | 3.3 |  | Function 0: GPIO PTB7 , general purpose input and output Function 1: AK2401, AK_ADC_SDO |  |  |
| P13 | PTB8 |  | I/O |  |  | 4 |  | U | 3.3 |  | GPIO PTB8 , general purpose input and output |  |  |
| T14 | PTB9 |  | I/O |  |  | 4 |  | U | 3.3 |  | GPIO PTB9 , general purpose input and output |  |  |
| R14 | PTB10 |  | I/O |  |  | 4 |  | U | 3.3 |  | GPIO PTB10 , general purpose input and output |  |  |
| P14 | PTB11 |  | I/O |  |  | 4 |  | D | 3.3 |  | GPIO PTB11 , general purpose input and output |  |  |
| R15 | PTB12 |  | I/O |  |  | 4 |  | D | 3.3 |  | GPIO PTB12 , general purpose input and output |  |  |
| K15 | PTB13 |  | I/O |  |  | 4 |  | U | 3.3 |  | GPIO PTB13 , general purpose input and output |  |  |
| K16 | PTB14 |  | I/O |  |  | 4 |  | U | 3.3 |  | GPIO PTB14 , general purpose input and output |  |  |
| L14 | PTB15 |  | I/O |  |  | 4 |  | U | 3.3 |  | GPIO PTB15 , general purpose input and output |  |  |
| L15 | PTB16 |  | I/O |  |  | 4 |  | U | 3.3 |  | GPIO PTB16 , general purpose input and output |  |  |
| M15 | PTB17 |  | I/O |  |  | 4 |  | D | 3.3 |  | GPIO PTB17 , general purpose input and output |  |  |
| T4 | PTB18 |  |  |  |  |  |  | D |  |  | GPIO PTB18 , general purpose input and output |  |  |
|  |  |  | I/O |  |  |  | 4 |  | 3.3 |  |  |  |  |
| R4 | PTB19 |  |  |  |  | 4 |  | U |  |  |  |  |  |
|  |  |  |  | I/O |  |  |  |  | 3.3 |  | GPIO PTB19 , general purpose input and output |  |  |
| P5 | PTB20 |  |  |  |  | 4 |  | U | 3.3 |  |  |  |  |
|  |  |  | I/O |  |  |  |  |  |  |  | GPIO PTB20 , general purpose input and output |  |  |
| R5 | PTB21 |  | I/O |  |  | 4 |  | U | 3.3 |  | GPIO PTB21 , general purpose input and output |  |  |
| T5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PTB22 |  |  | I/O |  | 4 |  | U | 3.3 |  |  | GPIO PTB22 , general purpose input and output |  |
| P6 | PTB23 |  | I/O |  |  | 4 |  | D | 3.3 |  | GPIO PTB23 , general purpose input and output |  |  |
| C12 | PTB24 |  | I/O |  |  | 4 |  | D | 3.3 |  | GPIO PTB24 , general purpose input and output |  |  |
| C11 | PTB25 |  | I/O |  |  | 4 |  | U | 3.3 |  | GPIO PTB25 , general purpose input and output |  |  |
| B11 | PTB26 |  | I/O |  |  | 4 |  | U | 3.3 |  | GPIO PTB26 , general purpose input and output |  |  |
| A11 | PTB27 |  | I/O |  |  | 4 |  | U | 3.3 |  | GPIO PTB27 , general purpose input and output |  |  |
| C10 | PTB28 |  | I/O |  |  | 4 |  | U | 3.3 |  | GPIO PTB28 , general purpose input and output |  |  |
| B10 | PTB29 |  | I/O |  |  | 4 |  | D | 3.3 |  | GPIO PTB29 , general purpose input and output |  |  |

###### AK2401 Pins

Table 15 AK2401 Pins

| HR_C7000 PIN | Pin Name | type | Drive (mA) | up and down pull | Voltage (V) | describe |
| --- | --- | --- | --- | --- | --- | --- |
| R7 | PTB2 | I/O | 4 | U | 3.3 | Function 0: GPIO PTB2 , general purpose input and output Function 1: AK2401, AK_DAC_FS |

| P7 | PTB3 | I/O | 4 | U | 3.3 | Function 0: GPIO PTB3 , general purpose input and output Function 1: AK2401, AK_DAC_SCLK |
| --- | --- | --- | --- | --- | --- | --- |
| T8 | PTB4 | I/O | 4 | D | 3.3 | Function 0: GPIO PTB4 , general purpose input and output Function 1: AK2401, AK_DAC_SDI |
| R8 | PTB5 | I/O | 4 | D | 3.3 | Function 0: GPIO PTB5 , general purpose input and output Function 1: AK2401, AK_ADC_FS |
| P8 | PTB6 | I/O | 4 | D | 3.3 | Function 0: GPIO PTB6 , general purpose input and output Function 1: AK2401, AK_ADC_SCLK |
| R13 | PTB7 | I/O | 4 | U | 3.3 | Function 0: GPIO PTB7 , general purpose input and output Function 1: AK2401, AK_ADC_SDO |

###### ADC Pins

Table 16 ADC Pins

| HR_C7000 PIN | Pin Name | type | Drive (mA) | up and down pull | Voltage (V) |  | describe |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1 | ADC0_IN | I/O | 4 | D | 3.3 |  | Function 0: CPU ADC channel 0 input Function 1: GPIO PTAC15 , general purpose input and output |  |
| A2 | ADC1_IN | I/O | 4 | D | 3.3 |  | Function 0: CPU ADC channel 1 input Function 1: GPIO PTAC16 , general purpose input and output |  |
| B3 | ADC2_IN | I/O | 4 | D | 3.3 |  | Function 0: CPU ADC channel 2 input Function 1: GPIO PTAC17 , general purpose input and output |  |
| A3 | ADC3_IN | I/O | 4 | D | 3.3 |  | Function 0: CPU ADC channel 3 input Function 1: GPIO PTAC18 , general purpose input and output |  |
| C4 | ADC4_IN | I/O | 4 | D | 3.3 |  | Function 0: CPU ADC channel 4 input Function 1: GPIO PTAC19 , general purpose input and output |  |
| B4 | ADC5_IN | I/O | 4 | D | 3.3 |  | Function 0: CPU ADC channel 5 input Function 1: GPIO PTAC20 , general purpose input and output |  |
| A4 | ADC6_IN | I/O | 4 | D |  |  | Function 0: CPU ADC channel 6 input Function 1: GPIO PTAC21 , general purpose input and output |  |
|  |  |  |  |  | 3.3 |  |  |  |
| A5 | ADC7_IN | I/O | 4 | D | 3.3 |  | Function 0: CPU ADC channel 7 input Function 1: GPIO PTAC22 , general purpose input and output |  |
| N2 | ADC_VINP_I | I | - | - | 3.3 |  | Baseband ADC Route I : In differential mode, the common-mode voltage of the I -way input is provided externally when EXTCM=1 ; when EXTCM=0 When, the pin is floating; |  |
|  |  |  |  |  |  |  |  | In single-ended mode, this pin is connected to the analog ground; In pseudo-differential mode, this pin is the same as the VINM input. |
| P1 | ADC_VINM_I | I | - | - | 3.3 |  | Baseband ADC Route I : In differential input mode, it is the differential negative terminal of I input ; In pseudo differential mode, the input voltage is fixed at 3.3V . |  |
| P2 | ADC_VINCM_I | I | - | - | 3.3 |  | Baseband ADC Route I : In differential input mode, it is the differential positive terminal of I input ; In differential and single-ended modes, it is the signal input terminal |  |
| M1 | ADC_VINP_Q | I | - | - | 3.3 |  | Baseband ADC Q Road: In differential mode, the Q -channel input common-mode voltage is provided externally when EXTCM=1; when EXTCM=0 , the pin is floating. In single-ended mode, this pin is connected to the analog ground; In pseudo-differential mode, this pin is the same as the VINM input. |  |
| M2 | ADC_VINM_Q | I | - | - | 3.3 |  | Baseband ADC Q Road: In differential input mode, it is the differential negative terminal of Q input ; In pseudo differential mode, the input voltage is fixed at 3.3V . |  |
| N1 | ADC_VINCM_Q | I | - | - | 3.3 |  | Baseband ADC Q Road: In differential input mode, the differential positive input of Q channel end; |  |

|  |  |  |  |  |  | In differential and single-ended modes, it is the signal input terminal |
| --- | --- | --- | --- | --- | --- | --- |

###### DAC Pins

Table 17 DAC Pins

| HR_C7000 PIN | Pin Name | type | Drive (mA) | up and down pull | Voltage (V) | describe |
| --- | --- | --- | --- | --- | --- | --- |
| K1 | DAC_VOUT_MCUA | I/O | 4 | U | 3.3 | Function 0: CPU DAC A- channel output Function 1: GPIO PTAC23 , general purpose input and output |
| K2 | DAC_VOUT_MCUB | I/O | 4 | U | 3.3 | Function 0: CPU DAC B -channel output Function 1: GPIO PTAC24 , general purpose input and output |
| L1 | DAC_VOUT_MCUC | I/O | 4 | U | 3.3 | Function 0: CPU DAC C -channel output Function 1: GPIO PTAC25 , general purpose input and output |
| J1 | DAC_VOUT_A | O | - | - | 3.3 | Baseband DAC I- channel output |
| J2 | DAC_VOUT_B | O | - | - | 3.3 | Baseband DAC Q -channel output |

###### Codec Pins

Table 18 Codec Pins

| HR_C7000 PIN | Pin Name |  | type | Drive (mA) |  | up and down pull | Voltage (V) |  | describe |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A14 | Codec_MICBIAS Codec_MiC_INP1 |  | I | - |  | - |  |  | MIC1 differential input positive terminal, or Linein1 Single-ended Input |  |
|  |  |  |  |  |  |  | - |  |  |  |
| B14 | Codec_MiC_INN1 |  |  | - |  | - | - |  |  |  |
|  |  |  | I |  |  |  |  |  | MIC1 Differential input negative terminal |  |
| A13 | Codec_MiC_INP2 |  | I | - |  | - | - |  | MIC2 differential input positive terminal, or Linein2 Single-ended Input |  |
| B13 | Codec_MiC_INN2 |  | I |  |  | - | - |  | MIC2 Differential input negative terminal |  |
|  |  |  |  |  | - |  |  |  |  |  |
| A15 |  |  |  | - |  | - | - |  | Codec Mic bias voltage |  |
|  |  |  | I |  |  |  |  |  |  |  |
| C15 | Codec_LINOUT1_ LP |  | O | - |  | - | 3.3 |  | Codec LineOut1 output, PWM signal |  |
| C14 | Codec_LINOUT2_ LP |  | O | - |  | - | 3.3 |  | Codec LineOut2 output, PWM signal |  |

###### Power and ground pins

Table 19 Power ground pin

| HR_C7000 PIN | Pin Name | type | Voltage (V) | describe |
| --- | --- | --- | --- | --- |
| B2 | ADC_VREF_MCU | P | 3.3 | Reference voltage for ADC of MCU peripheral interface |
| C3 | ADC_AVDD33_MCU | P | 3.3 | MCU peripheral interface ADC supply voltage |
| C2 | ADC_AVSS33_MCU | G | - | Analog ground of MCU peripheral interface ADC |
| E3 | LDO_RTC_AVSS33 | G | - | Analog ground for the RTC module. |
| D3 | LDO_RTC_AVDD33 | P | 3.3 | the RTC module. The RTC needs to be powered separately. |
| E5 | RTC_AVSS | G | - | Analog ground of the RTC module |
| F3 | RTC_DVDD12 | P | 1.2 | RTC circuit has built-in LDO core1.2 voltage output |
| G3 | USB_AVDD33 | P | 3.3 | USB power supply |
| H2 | PR_V | G | - | reserve |

| H1 | PR_G | G | - | reserve |
| --- | --- | --- | --- | --- |
| K5 | DAC_VREFN | G | - | Baseband DAC and MCU The negative terminal of the reference voltage shared by DAC , External connection point analog ground |
| J5 | DAC_VREFP | P | 3.3 | Baseband DAC and MCU The positive reference voltage terminal shared by DAC , External connection point for analog 3.3V |
| J3 | DAC_AVDD33 | P | 3.3 | Baseband DAC and MCU Analog 3.3V power supply shared by DAC |
| K3 | DAC_AVSS33 | G | - | Baseband DAC and MCU DAC shared analog ground |
| M3 | ADC_VREF | P | - | The analog reference output voltage of the baseband ADC can be configured to modulate the output voltage by using bg_trim and buffer_trim references. Output voltage, the default value is 500mV . |
| N3 | ADC_AVSS33 | G | - | Analog ground of baseband ADC |
| R1 | ADC_AVDD33 | P | 3.3 | Analog 3.3V power supply for baseband ADC |
| L3 | ADC_AVDD33 | P | 3.3 | Analog 3.3V power supply for baseband ADC |
| M6 | PLL_AVDD_1 | P | 3.3 | PLL analog 3.3V power supply |
| M7 | PLL_AVSS_0 | G | - | PLL analog ground |
| P3 | LDO_AVDD33_3 | P | 3.3 | The internal LDO supply voltage is typically 3.3V . Internal core voltage conversion |
| M5 | LDO_DVDD12_3 | P | 1.2 | LDO output 1.2 voltage reference |
| P4 | LDO_AVSS | G | - | LDO analog ground |
| H12 | LDO_AVDD33_0 | P | 3.3 | The internal LDO supply voltage is typically 3.3V . Internal core voltage conversion |
| K12 | LDO_DVDD12_0 | P | 1.2 | LDO output 1.2 voltage reference |
| J12 | LDO_AVSS | G | - | LDO analog ground |
| D16 | Codec_LINOUT_VSS | G | - | LineOut1 and LineOut2 are two IO addresses |
| C16 | DVDD25 | P | 2.5 | LineOut1 and LineOut2 are two IO power supplies, voltage 2.5V , provided by Codec_VREFP |
| B16 | Codec_VREFP | P | 2.5 | Codec outputs 2.5V reference voltage |
| C13 | Codec_AVDD | P | 3.3 | Codec analog power supply 3.3V |
| D14 | Codec_AVSS33 | G | - | Codec analog ground |
| B15 | Codec_VCAP | P | 3.3 | Codec bandgap voltage |
| E9 | LDO_AVDD33_1 | P | 3.3 | The internal LDO supply voltage is typically 3.3V . Internal core voltage conversion |
| E8 | LDO_DVDD12_1 | P | 1.2 | LDO output 1.2 voltage reference |
| E10 | LDO_AVSS | G | - | LDO analog ground |
| E12, F12, M11, M12 | DVDD33 | P | 3.3 | Digital power supply 3.3V |
| A1, A16, C1, C7, D15, E1, E2, E6, E7, E11, E14, F5, F6, F7, F8, F9, F10, F11, G1, G2, G5, G6, G7, G8, G9, G10, G11 , G12, H3, H5, H6, H7, H8, H9, H10, H11, J6, J7, J8, J9, J10, J11, J15, K6, K7, K8, K9, K10, K11, L2, L3, L5, L6, L7, L8, L9, L10, L11, L12, M8.M9 ,M10,T1,T16 | DVSS | G | - | Digitally |

#### Electrical performance parameters

###### Voltage parameters

Table 20 Voltage parameters

|  | name | describe | MIN | TYP | MAX | unit |
| --- | --- | --- | --- | --- | --- | --- |
| Digital Voltage | DVDD33 | Digital power supply | 2.97 | 3.3 | 3.63 | V |
| Codec power supply | DVDD25 | Codec LineOut digital PWM​ IO power supply electricity | 2.35 | 2.5 | 2.65 | V |
|  | Codec_VREFP | Codec reference voltage output | 2.35 | 2.5 | 2.65 | V |
|  | Codec_MICBIAS | MICBIAS_V=0 |  | 2.08V |  | V |
|  |  | MICBIAS_V=1 |  | 1.66 |  | V |
|  | Codec_AVDD | Codec analog power supply | 2.97 | 3.3 | 3.63 | V |
|  | Codec_VCAP | Codec bandgap output voltage |  | 2 |  | V |
| MCU peripheral ADC Voltage | ADC_AVDD33_MCU | MCU ADC power supply | 2.97 | 3.3 | 3.63 | V |
|  | ADC_VREF_MCU | MCU ADC reference voltage | 0.5* AVDD33 |  | 0.9* AVDD33 | V |
| RTC power supply | LDO_RTC_AVDD33 | RTC independent LDO power supply | 2.0 | 3.3 | 3.63 | V |
|  | RTC_DVDD12 | RTC independent LDO core power supply | 1.08 | 1.2 | 1.32 | V |
| USB powered | USB_AVDD33 | USB powered | 2.8 | 3.3 | 3.63 | V |
| DAC power supply | DAC_AVDD33 | DAC analog power supply | 2.97 | 3.3 | 3.63 | V |
|  | DAC_VREFP | DAC reference voltage input | 2.97 | 3.3 | 3.63 | V |
| PHY ADC powered by | ADC_AVDD33 | Baseband ADC analog power supply | 3.04 | 3.3 | 3.6 | V |
|  | ADC_VREF | Reference voltage output for baseband ADC | 0.485 | 0.5 | 0.515 | V |
| PLL power supply | PLL_AVDD_1 | PLL analog power supply | 2.97 | 3.3 | 3.63 | V |
| LDO power supply | LDO_AVDD33_1 | LDO 1Analog power supply | 2.97 | 3.3 | 3.63 | V |
|  | LDO_AVDD33_2 | LDO 2Analog power supply | 2.97 | 3.3 | 3.63 | V |
|  | LDO_AVDD33_3 | LDO 3Analog power supply | 2.97 | 3.3 | 3.63 | V |
|  | LDO_DVDD12_1 | LDO 1Conversion voltage output | 1.08 | 1.2 | 1.32 | V |
|  | LDO_DVDD12_1 | LDO 2Conversion voltage output | 1.08 | 1.2 | 1.32 | V |
|  | LDO_DVDD12_1 | LDO 3Conversion voltage output | 1.08 | 1.2 | 1.32 | V |
| Reference Ground | DVSS | Digitally |  | 0 |  | V |
|  | Codec_LINOUT_VSS | Codec LineOutIO analog ground |  | 0 |  | V |
|  | Codec_AVSS33 | Codec analog ground |  | 0 |  | V |
|  | ADC_AVSS33_MCU | MCU ADC analog ground |  | 0 |  | V |
|  | LDO_RTC_AVSS33 | RTL comes with LDO analog ground |  | 0 |  | V |
|  | RTC_AVSS | RTC analog ground |  | 0 |  | V |
|  | USB_AVSS33 | USB analog ground |  | 0 |  | V |
|  | DAC_AVSS33 | DAC analog ground |  | 0 |  | V |
|  | DAC_VREFN | DAC negative reference |  | 0 |  | V |
|  | ADC_AVSS33 | ADC analog ground |  | 0 |  | V |
|  | PLL_AVSS_0 | PLL analog ground |  | 0 |  | V |
|  | LDO_AVSS | LDO analog ground |  | 0 |  | V |
| Input and output voltage thresholds | VIH | Input high voltage | 2.0 |  | 3.63 | V |
|  | VIL | Input low voltage | - 0.3 |  | 0.8 | V |
|  | VOL | Output low voltage |  |  | 0.4 | V |
|  | VOH | Output high voltage | 2.4 |  |  | V |

###### Power consumption parameters

###### Temperature parameters

Table 21 Temperature parameters

| name | describe | MIN | TYP | MAX | unit |
| --- | --- | --- | --- | --- | --- |
| Oprating temp |  | - 40 | 25 | 85 | °C |
| Junction temp |  | - 40 | 25 | 100 | °C |
| Solder temp | The device can be reflowed three times at 260°C and maintain normal mechanical and electrical properties ( the high temperature section time is about 260°C for 10s); Manual soldering is recommended to be done at high temperature for as short a time as possible. Repair conditions 340°C for 5-10 seconds, |  | 260 |  | °C |

###### IP Indicator parameters

Table 22 IP Indicator parameters

| Classification | parameter | describe | MIN | TYP | MAX | unit |
| --- | --- | --- | --- | --- | --- | --- |
| DAC | Resistive load | Anolog output |  | 1.5 |  | kOhm |
|  | Capacitve load | Anolog output |  |  | 30 | pF |
|  | Voltage swing | Anolog output | VREFN |  | VREFP | V |
|  | RES |  |  | 12 |  | bit |
|  | DNL |  |  | ±1 |  | LSB |
|  | INL |  |  | ±1.5 |  | LSB |
|  | SNR |  |  |  |  |  |
|  | THD |  |  |  |  |  |
| PHY ADC | VINdiff | Differential input voltage (VINP- VINM) |  |  | 0.9 | Vpp |
|  | VINse | Single-ended or pseudo-differential input voltage, VINM=com mode of VINP |  |  | 0.45 | Vpp |
|  | VINcm- dc | VINCM common mode voltage (DC mode ) | 0.5 |  | 1.5 | V |
|  | VINcm- ac | VINCM common mode voltage (AC mode ) |  | 0 |  | V |
|  | Vnol | AVDD Dyamic Noise |  |  | 50 | mVpp |
|  | RES |  |  | 10 |  | bit |
|  | Cin | VINP/VINM input capacitor |  | 2 |  | pF |
|  | Cref | VREF reference voltage capacitor |  | 100 |  | nF |
|  | DNL |  |  | ±1 |  | LSB |
|  | INL |  |  | ±2 |  | LSB |
|  | Gain Error | Internal reference, before BGTRIM correction | - 4 |  | +4 | % |
|  | Gain Error | Internal reference, after BGTRIM correction | - 1 |  | +1 | % |
|  | Gain Error | External reference | - 1 |  | +1 | % |
|  | OS | Input offset error | - 5 |  | +5 | mV |
|  | LAT | Analog input to digital output latency |  | 12 |  | cycles |
|  | WT | Wake up time |  | 120 |  | cycles |
|  | BTime | Buffer start-up time |  | 15 | 20 | uS |
|  | Total_Stime | Complete ADC start up time |  | WT+ BTimte |  | uS |
|  | SNR |  |  | 56 |  | dB |
|  | ENOB |  |  | 9 |  | bit |
| MCU ADC | Input Range | 8-channel single-ended input | 0.01 VREF |  | 0.99 VREF | V |
|  | Input capacitance |  |  | 1 |  | pF |
|  | RES |  |  |  | 10 | bit |
|  | DNL |  | - 0.2852 |  | 0.3599 | LSB |
|  | INL |  | - 0.7344 |  | 0.9160 | LSB |
|  | Offset Error |  |  | 0.5064 |  | %FS |
|  | Gain Error |  |  | - 0.0432 |  | %FS |
|  | Data Latency |  |  | 10 |  | cycle |
|  | SINAD | Fin = 1.03K |  | 57.91 |  | dB |
|  | SFDR | Fin = 1.03K |  | 63.81 |  | dB |
|  | THD +3HD | Fin = 1.03K |  | - 64.64 |  | dB |
|  | ENOB | Fin = 1.03K |  | 9.33 |  | bit |
| USB | VOL | 1.5K pull-up resistor on DP to USB_AVDD33 |  |  | 0.3 | V |
|  | VOH | 15K pull-down reisitor on DP/DM to AGND | 2.8 |  |  | V |
|  | Vdiff | ¦V(DP)-V(DM)¦ | 0.2 |  |  | V |
|  | Vcm | USB_AVDD33 > Vcm+0.8 | 0.8 |  | 2.5 | V |
|  | Vse- l | Single-ended Receiving low level input voltage |  |  | 0.8 | V |
|  | Vse- h | Single-ended Receiving high level input voltage | 2.0 |  |  | V |
|  | Fs | Sampling frequency | 8 | 16 | 192 | KHz |
|  | Mclk |  |  | twenty four |  | MHz |

| Codec | Duty cycle | Mclk | 0.45 | 0.5 | 0.55 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WT | From power-down to playback mode |  | 181 | 493 | ms |
|  |  | From power-down to record mode |  | 124 | 230 | ms |
|  | Micbias out current |  |  |  | 4 | mA |
|  | Micbias out noise | A-weighted |  | 20 | 40 | uVrms |
|  | Micbias capacitor | Decoupling capacitor Cmic | 0.75 | 1 | 1.25 | nF |
|  | Input level | ADC full scale, Gain GID=0dB, GIM=20dB | 0.189 | 0.212 | 0.239 | Vpp |
|  | SNR | ADC , A-weighted, 1KHz sine wave, full scale and gain 0dB | 85 | 90 |  | dB |
|  | THD | ADC, 1KHz sine wave -1dB and gain 0dB |  | - 80 | - 70 | dB |
|  | Dynamic range | ADC, A-weighted, 1KHz sine wave, full scale and gain 0dB | 85 | 90 |  | dB |
|  | PSRR | ADC, 100mVpp 1KHz sinewave is applied to AVD, Gain GID=0dB, GIM=20dB |  | 90 |  | dB |
|  | Input resisitance | ADC, GIM=0dB, differential config | 132 | 160 | 200 | KOhm |
|  |  | ADC, GIM=20dB, differential | 20 | 26 | 30 | KOhm |
|  |  | ADC, GIM=0dB, single- ended | 92 | 115 | 138 | KOhm |
|  |  | ADC, GIM=20dB, single- ended | 19 | twenty four | 29 | KOhm |
|  | Input capacitance | ADC, Cbyline |  | 1 |  | uF |
|  |  | ADC, includes 10uf for ESD, bongding and package pins cap |  |  | 25 | pF |
|  | SNR | PWM out, A-weighted, 1KHz sine wave, full scale and gain 0dB | 100 | 105 |  | dB |
|  | THD | PWM out, 1KHz sine wave -1dB and gain 0dB, Fs <=16KHz |  | - 88 | - 80 | dB |
|  | THD+N | PWM out, 1KHz sine wave -1dB and gain 0dB, Fs <=16KHz |  | - 87 | - 80 | dB |
|  | Dynamic Range | PWM out, 1KHz sine wave, full scale -60dB and volume=0dB | 100 | 105 |  | dB |
|  | Gain range | PWM out, Godl 6-bit programmable range | - 31 |  | 32 | dB |
|  | Gain step | PWM out, |  | 1 |  | dB |
|  | PWM frequency | MCLK =24MHz |  | 750 |  | kHz |
|  | Modulation rate |  | 25 |  | 75 | % |
| LDO | I max | Load current |  |  | 100 | mA |
|  | Vtrip | Max ripple on Vout, Vin = 3.3V |  |  | 100 | mVpp |
|  | Cout | External capacitor |  |  | 4.7 | uF |

## system

#### Reset

###### Overview

The reset management module manages the reset of the entire chip and each functional module in a unified manner, including:

- Internal integrated POR Module

- Power-on reset management and control

- Individual soft reset control of functional modules

- The reset signal is synchronized to the corresponding clock domain of each module

###### Functional Description

HR_C7000 The chip has a built-in power-on automatic reset module, which is located at the chip pin POR_EN When the high level is valid, the internal automatic multiplexing module works and the chip is globally reset. At the same time, the external reset pin NRESET can also play a reset role.

When the chip pin POR_EN When it is at a low level, the internal automatic reset circuit does not work, and the global reset of the chip is performed by the external reset pin NRESET. Completed. To ensure that the external reset is valid, NRESET is required The low level duration is more than 400ns .

i_por_en

por_rst_n i_ext_rst_n

1 0

1

rst_src_deglitch

i_deglitch_rst_n

global_rst_n

i_osc_clk

watch_dog_rst_n

software_rst_n

rst_deglitch

Figure 7 Reset overall block diagram

In addition to external pins or internal POR Except for the global hardware reset performed by the module, watch_dog Exceptions can also trigger a global reset of the system. In addition, all soft reset registers are set within the system, which can also actively reset each module of the system (except the register configuration module).

In addition to the overall chip reset control, internal module functions can also be reset through their own soft reset, which is divided into register soft reset and function module request soft reset.

software_c

odec_rst_n

software_

cpu_rst_n

software_

adc_rst_n

software_m

odem_rst_n

global_rst_n

rst_modem

rst_adc

rst_cpu

rst_codec

rst_soc

software_soc_rst_n

Figure 8 Reset overall block diagram

      - POR

POR ( Powre On Reset ) module detects the power-on VDD voltage to generate a reset signal. When the POR_EN pin When the signal is high, the internal POR Valid, otherwise invalid, through the external reset pin RST_N_IN To achieve overall chip reset. The specific module block diagram and timing diagram are as follows:

Figure 9 POR Module Block Diagram

VDD

VDET

VC

VRESET

TD

Figure 10 POR module timing chart 23 POR Design parameter table

| PARAMETER | SYMBOL | DESIGN RECOMMANDATONS |  |  | UNITS |
| --- | --- | --- | --- | --- | --- |
|  |  | MIN | TYP | MAX |  |
| Power Supply | VDD | 1.08 | 1.2 | 1.32 | V |
| Total Current | ICC | 1.95 | 2.7 | 20.3 | uA |
| Time delay | TD | 35 | 50 | 80 | mS |
| Detect Voltage | VDET | 0.65 | 0.785 | 0.9 | V |

Soft reset

The chip supports soft reset of each module, CPU A soft reset signal can be generated by configuring the soft reset register. Using a pure software approach , writing 0 to the soft reset register indicates a valid reset, automatically releasing the soft reset register value to 1 .

###### Register Overview

Register Table 1 Reset Register Overview (Base Address: 0x11000000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x00 | SYS_SOFT_RSTN | Soft reset register control | 55 |

###### Register Description

      - SYS_SOFT_RSTN

Offset address: 0x00 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:9] | - | reserved |  |  |
| [8] | RW | cpu_soft_rstn | 0x1 | CPU Soft reset control, low active |
| [7] | RW | sys_soft_rstn | 0x1 | System global soft reset control, low effective |
| [6] | RW | adc_ctrl_soft_rstn | 0x1 | Built-in CPU IP ADC soft reset control, low effective |
| [5] | RW | adc_soft_rstn | 0x1 | Built-in baseband IP ADC soft reset control, low effective |
| [4] | RW | codec_soft_rstn | 0x1 | Built-in IP CODEC soft reset control, low effective |
| [3] | RW | audio_soft_rstn | 0x1 | Audio interface module soft reset control, low effective |
| [2] | RW | fm_soft_rstn | 0x1 | FM module soft reset control, low effective |
| [1] | RW | phy_soft_rstn | 0x1 | Physical layer module soft reset control, low effective |
| [0] | RW | protocol_soft_rstn | 0x1 | Protocol layer module soft reset control, low effective |

#### clock

###### Overview

The clock management module provides unified management of chip clock input, clock generation, and control, including:

- Clock input management and control

- Clock division and control

- Generates the working clock of each module

o_dac_clk

i_apll_clk

117.9648MHz

dac_clk_div i_osc_clk

0

1

apll_clk

apll_clk_gt

o_apll_clk_out

i_osc_clk

0

1

i_apll_clk_out_en_reg | i_test_pll

clk_32khz

MODEM_TOP

i_scan_mode

clk_192khz

modem_clk_div

9.8304MHz

o_modem_clk_fmtx

0 o_modem_clk

1

FMTX

i_dft_clk

i_modem_clk_fmtx_en_reg

o_modem_clk_fmrx

gate

i_modem_clk_fmrx_en_reg

o_modem_clk_tx

FMRX

SYSTX

i_test_codec | mclk_sel

i_scan_mode

o_mclk

i_osc_clk

clk_mgr_aud

512KHz

mclk_div

1

0

24MHz

0

1

i_clk_512k_en_reg

o_lrck

i_modem_clk_tx_en_reg

gate

i_modem_clk_rx_en_reg

gate

i_mc_clk_en_reg

o_modem_clk_rx

SYSRX

i_dft_clk

o_mc_clk

o_modem_clk o_mclk

o_ahb_clk

AUDIO_TOP

8KHz

i_clk_8k_en_reg

o_bclk

i_scan_mode

clk_mgr_soc

/16

24MHz

mclk_div

i_usb_speed

o_usb_wpc_clk

usb_clk_div

usb_clk

FOUTVCO

768MHz

i_bpll_clk_vco

384MHz

0

i_dft_clk 1

i_scan_mode usb_phy_clk

/2

bpll_clk_vco_div2_gt

gate

i_usb_clk_en_reg

0

48MHz

1

o_usb_phy_clk48

~i_usb_suspend

/N

N=1~49

0 o_usb_phy_clk

i_dft_clk 1

USB

o_ahb_clk

i_usb_hclk_en_reg

o_usb_hclk

BPLL

FOUTPOSTDIV

bpll_clk_vco_div2

gate

384MHz

bpll_clk_vco_div2_gt1

sdio_clk

96MHz

i_sdio_clk_in_en_reg

gate

/4

bpll_clk_gt

o_bpll_clk_out

o_sdio_cclk_in 48MHz

o_sdio_cclk_in_drv

sdio_clk_mgr

o_sdio_cclk_in_sample

o_ahb_clk	o_sdio_hclk

i_sdio_hclk_en_reg

gate

192MHz

i_bpll_clk

i_bpll_clk_out_en_reg |

i_bclk_work_sel

i_test_pll

cpu_clk_tmp

o_cpu_clk

i_osc_clk 1

0

i_bclk_work_sel

o_ahb_clk

o_ahb_gated_clk

i_hclk_en_reg

i_bclk_work_sel

apb_clk_div

apb_clk_tmp

o_apb_clk

i_osc_clk

0

1

osc_clk_gt

test_adc

adc_ctrl_clk

o_apb_gated_clk

i_pclk_en_reg

adc_clk_en_reg

i_osc_clk

0 o_adc_ctrl_clk

1

CPU_ADC

i_scan_mode

test_dac

osc_clk_gt1

dac_ctrl_clk

i_dft_clk

0

1

o_adc_ctrl_clk_dft

dac_clk_en_reg

o_dac_ctrl_clk

i_osc_clk

0

1

gate

gate

gate

gate

gate

gate

gate

gate

gate

gate

gate

CPU_DAC

/20

ADC Control

/10

occ

/n(1/2/4)

occ

SOC/CPU

/20

SDIO

/8

/8

FOUTVCO

/N N=1~49

APLL

FOUTPOSTDIV

div

frac_div

CODEC

/12

frac_div

frac_div

/6

modem_clk_gen_en

PHY_DAC

/16

PHY_ADC

ADC INTERFACE

i_scan_mode

i_test_adc

o_adc_clk_dft

0

i_dft_clk  1

clk_mgr_modem

o_adc_clk

i_osc_clk

0

1

i_test_dac

i_osc_clk

i_aclk_work_sel

ahb_clk_div

0

i_osc_clk 1

ahb_clk_tmp

occ

/n(1/2)

Figure 11 Clock Distribution Block Diagram

###### Functional Description

The clock unit has the following features:

- External crystal oscillator provides 24MHz clock work;

- Internal Integration 2 PLLs​

APLL Used to generate Modem Source clock

BPLL Used to generate system clock, such as CPU Clock, AHB/APB Bus clock

- Support real-time switching of the clock during operation

##### Clock interface

The clock interface signals are shown in the following table:

Table 24 Clock interface signal

| signal name | direction | describe |
| --- | --- | --- |
| OSC_ CLK_IN | I | External crystal oscillator clock input, requires input Vpp Up to 2.8V above |
| CLK_OUT | O | PLL Frequency-divided clock output, PLLA and PLLB Frequency division output optional |

##### Clock frequency

Table 25 Clock frequency range

| name | describe | MIN | TYP | MAX | unit |
| --- | --- | --- | --- | --- | --- |
| OSC_CLK_IN | Main clock crystal | - - | twenty four | - - | MHz |
| adc_clk | Baseband ADC Work clock | - - | 117.9648 | - - | MHz |
| dac_clk | Baseband DAC Work clock | 0.0384 | 0.6144 | 1 | MHz |
| mdem_clk | Modem Baseband working clock | - - | 9.8304 | - - | MHz |
| mdem_clk_fm_tx | Modem Baseband FM Send Clock | - - | 9.8304 | - - | MHz |
| mdem_clk_fm_rx | Modem Baseband FM Receive clock | - - | 9.8304 | - - | MHz |
| mdem_clk_tx | Modem Baseband transmit clock | - - | 9.8304 | - - | MHz |
| mdem_clk_rx | Modem Baseband receive clock | - - | 9.8304 | - - | MHz |
| mc_clk | Codec MC Interface clock | - - | 9.8304 | - - | MHz |
| mclk | Codec working master clock | - - | twenty four | - - | MHz |
| bpll_clk_vco | BPLL VCO Output clock | - - | - - | 768 | MHz |
| usb_wpc_clk | USB Work clock | 6 | 48 | - - | MHz |
| usb_phy_clk | USB Work clock | 6 | 48 | - - | MHz |
| sdio_cclk_in | sdio Work clock | 40 | 48 | 48 | MHz |
| cpu_clk | cpu Core operating clock | twenty four | 120 | 192 | MHz |
| ahb_clk | AHB Bus clock | twenty four | 120 | 192 | MHz |
| apb_clk | APB Bus clock | twenty four | 60 | 96 | MHz |
| adc_ctrl_clk | MCU Built-in ADC Work clock |  |  | 2.2 | MHz |
| dac_ctrl_clk | MCU Comes with DAC Work clock |  |  | 1 | MHz |

###### How it works

##### initialization

After the system is powered on, the CPU and bus are both operating at 24M Crystal oscillator clock.

##### Reconfiguration Operation Process

The reconfiguration process is the process of changing the clock configuration after the system starts normally under the default clock configuration.

The reconfiguration action sequence is to first prepare the parameters to be configured, write them to the corresponding registers, and then enable reconfiguration, after which the reconfiguration action will be automatically completed.

If you want the system to work with the default PLL Under clock configuration, only 0x11000000 is needed bit[1] and bit[0] are written as 1 That's it,

Each gated clock is disabled by default. If you need to enable the gate, you can configure it online in real time.

the PLL at any time Please operate PLL first. Configuration register, then write 1 to 0x11000000 bit[1] and bit[0] Then, when reconfiguring, the PLL Will temporarily POWER DOWN , CPU The bus clock will automatically switch to the crystal oscillator clock and the PLL lock After that, the CPU and bus clock configuration register 0x11000000 Set bit[3] and bit[2] to 0 to switch back to PLL clock operation.

Refer to the figure below for the reconfiguration usage and the entire reconfiguration process. The reference steps are as follows: Power-on clock configuration workflow:

1. Power on.

2. CK​ From bootrom start up;

3. Start clock configuration;

4. The default configuration of the clock management module after reset can generate the operating frequency under the mainstream working scenario, but the gating of each clock is closed and needs to be turned on by the CPU through register configuration. The switching clock can be performed online in real time.

5. At any time, only the configuration register selects PLL Output clock, working clock configuration will be effective, the system will work on osc clock by default after startup

6. If you prefer the default configuration, just configure register CLK_MGR_REG0x04 bit 0 and bit1 1​ Indicates that PLL configuration is enabled. If you want to customize the configuration, you need to pass CK Operate the corresponding PLL one by one Frequency division and corresponding working clock frequency division coefficient register, and then configure register CLK_MGR_REG0x04 bit 0 and bit1 1​ Indicates that PLL is turned on Configuration, then PLL Enter POWER DOWN status ( regardless of PLL Is the current POWER DOWN , the action will be executed )

7. Automatic PLL inside the chip Configuration process, in which the PLL Execute POWER DOWN The time is about 2us ;

8. By reading register CLK_MGR_REG0x04 Bit31 , judge the two PLL Are all locked flags required? Due to the register read and write time requirements, PLL is required Lock mark continuous judgment 10 More than times can be identified PLL Locked, or require to wait 100us before reading the register to determine whether it is locked;

9. PLL​ After locking, the system operating clock starts to switch from the crystal oscillator clock to the PLL Output clock, configuration register

CLK_MGR_REG0x04 Bit3​ and Bit2 is 0 ;

10. Wait for 	the clock to be ready OK Flags, read registers CLK_MGR_REG0x04​ Bit30 It can be high. Due to the requirements of register read and write time, it is required clk_rdy flag continuous judgment More than 10 times can be recognized;

11. Clock ready​	 After that, the system performs a global soft reset operation ;

12. To reconfigure the clock frequency, re- execute	 3 — 11 .

Y

default_cfg

N

register config

时钟重配置启动

aclk_re_cfg_reg = 1

bclk_re_cfg_reg = 1

Y

clk_re_cfg

N

CPU 启动

系统全局软复位

CPU从bootrom启动

System startup

| Automatic clock reconfiguration |  |
| --- | --- |
| PLL lock | detected |

| Clock Switching aclk_work_sel=0 bclk_work_sel=0 |  |
| --- | --- |
| clk_rdy | detected |

Figure 12 Reconfiguration Flowchart

Gray filled boxes are operations that require software. register_config Indicates CPU Prepare various configuration parameters. The clock management module detects clock reconfiguration requests by checking the rising edge of clk_cfg_rdy .

The clock management module has only one reset source - power-on reset. Once the clock reconfiguration parameters are written to the CPU Write operation will be temporarily stored in the register inside the clock management module, and will not be affected by PLL A reset caused by a loss of lock will clear it.

      - PLL Configuration

The chip integrates 2 PLL , APLL​ and BPLL , each PLL Using independent configuration registers, the APLL

Working in fractional mode, BPLL Works in integer mode.

PLL Frequency calculation method:

- DSMPD=1 , integer mode: FOUTVCO=FREF/REFDIV*FBDIV

FOUTPOSTDIV=FOUTVCO/POSTDIV1/POSTDIV2

- DSMPD=0 , decimal mode: FOUTVCO=FRED/REFDIV*(FBDIV+FRAC/(2^24)) FOUTPOSTDIV=FOUTVCO/POSTDIV1/POSTDIV2

Figure 13 PLL Circuit Schematic

###### Register Overview

Register Table 2 Clock Management Module Register Overview (Base Address: 0x11000000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x04 | CLK_MGR_REG0x04 | PLL Reconfiguration Control Register |  |
| 0x08 | CLK_MGR_REG0x08 | APLL Divider Configuration Register |  |
| 0x0c | CLK_MGR_REG0x0c | APLL Frequency Divider Configuration Register |  |
| 0x10 | CLK_MGR_REG0x10 | BPLL frequency division configuration register |  |
| 0x14 | CLK_MGR_REG0x14 | BPLL frequency division configuration register |  |
| 0x18 | CLK_MGR_REG0x18 | Baseband related clock division coefficient register |  |
| 0x1c | CLK_MGR_REG0x1c | FM 32K , 192K frequency division coefficient register |  |
| 0x20 | CLK_MGR_REG0x20 | Code I2S interface clock frequency division coefficient register |  |
| 0x24 | CLK_MGR_REG0x24 | Bus-related clock division factor register |  |
| 0x28 | CLK_MGR_REG0x28 | SDIO , USB clock division coefficient register |  |
| 0x2c | CLK_MGR_REG0x2c | Gating clock control switch register |  |

###### Register Description

      - CLK_MGR_REG0x04

Offset address: 0x04 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RO | pll_ld | 0x0 | Overall PLL lock flag (apll_ld & bpll_ld) |
| [30] | RO | clk_rdy | 0x0 | Clock switch ready flag |
| [29:4] | - | reserved |  |  |
| [3] | RW | bclk_work_sel | 0x1 | SOC system clock control signal, the default value after reset is 1 . 0 : SOC system works in PLL clock; 1 : The SOC system works on the crystal oscillator clock. |
| [2] | RW | aclk_work_sel | 0x1 | Baseband system clock control signal, the default value is 1 after reset . 0 : The baseband system works on the PLL clock; 1 : The baseband system operates on the crystal oscillator clock. |
| [1] | RW | bclk_re_cfg_reg | 0x0 | BPLL clock reconfiguration information is ready flag, which is CPU , high valid |
| [0] | RW | aclk_re_cfg_reg | 0x0 | APLL clock reconfiguration information is ready flag, which is CPU , high valid |

      - CLK_MGR_REG0x08

Offset address: 0x08 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RO | apll_ld | 0x0 | APLL Clock output lock flag ( after the system is up Must be 1 ) |
| [30] | RO | apll_pd | 0x0 | APLL POWER DOWN sign: 1 : power down ; 0 : non- power down . |
| [29] | RW | apll_dsmpd_reg | 0x0 | Power down Delta-Sigma Modulator : 0 : DSM is active ; 1 : DSM is powered down . |
| [28] | RW | apll_dacpd_reg | 0x0 | Power down noise canceling DAC in FRAC mode : 0 : DAC is active (default mode) ; 1 : DAC is not active (test mode only) . |
| [27] | RW | apll_bypass_reg | 0x0 | APLL FOUTPOSTDIV bypassed to FREF , high efficient |
| [26] | RW | apll_foutvcopd_reg | 0x1 | APLL post VCO output shut down enable, high effective |
| [25] | RW | apll_foutpostdivpd_reg | 0x0 | APLL output frequency division control |
| [twenty four] | RW | apll_fout4phasepd_reg | 0x1 | 4phase Clock generator shutdown enable (4phase output Output can also be closed by apll_foutpostdivpd ) , high effective |
| [23:18] | RW | apll_refdiv_reg | 0x1 | APLL reference clock division factor (1~63) |
| [17:6] | RW | apll_fbdiv_reg | 0x3a | APLL feedback div value(16~2400 in integer mode, 20~240 in fraction mode) |

| [5:3] | RW | apll_postdiv1_reg | 0x6 | APLL post div1(1~7) |
| --- | --- | --- | --- | --- |
| [2:0] | RW | apll_postdiv2_reg | 0x2 | APLL post div2(1~7) |

      - CLK_MGR_REG0x0c

Offset address: 0x0c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:0] | RW | apll_frac | 0xfb7e91 | APLL fractional division value |

      - CLK_MGR_REG0x10

Offset address: 0x10 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RO | bpll_ld | 0x0 | BPLL Clock output lock flag ( after the system is up Must be 1 ) |
| [30] | RO | bpll_pd | 0x0 | BPLL POWER DOWN sign: 1 : power down ; 0 : non- power down . |
| [29] | RW | bpll_dsmpd_reg | 0x1 | Power down Delta-Sigma Modulator : 0 : DSM is active ; 1 : DSM is powered down . |
| [28] | RW | bpll_dacpd_reg | 0x1 | Power down noise canceling DAC in FRAC mode : 0 : DAC is active (default mode) ; 1 : DAC is not active (test mode only) . |
| [27] | RW | bpll_bypass_reg | 0x0 | BPLL FOUTPOSTDIV bypassed to FREF , high efficient |
| [26] | RW | bpll_foutvcopd_reg | 0x0 | BPLL Post vco output close enable, high effective |
| [25] | RW | bpll_foutpostdivpd_reg | 0x0 | BPLL output frequency division control |
| [twenty four] | RW | bpll_fout4phasepd_reg | 0x1 | 4phase Clock generator shutdown enable (4phase output Output can also be closed by bpll_foutpostdivpd ) , high effective |
| [23:18] | RW | bpll_refdiv_reg | 0x2 | BPLL reference clock division factor (1~63) |
| [17:6] | RW | bpll_fbdiv_reg | 0x28 | BPLL feedback div value(16~2400 in integer mode, 20~240 in fraction mode) |
| [5:3] | RW | bpll_postdiv1_reg | 0x2 | BPLL post div1(1~7) |
| [2:0] | RW | bpll_postdiv2_reg | 0x2 | BPLL post div2(1~7) |

      - CLK_MGR_REG0x14

Offset address: 0x14 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:0] | RW | bpll_frac | 0x0 | BPLL fractional division value |

      - CLK_MGR_REG0x18

Offset address: 0x18 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | RW | dac_clk_div_num | 0x10 | IP DAC working clock frequency division coefficient setting |
| [23:16] | RW | bclk_out_div_num | 0xa | BPLL pin output clock frequency division coefficient setting |
| [15:8] | RW | aclk_out_div_num | 0xc | APLL pin output clock frequency division coefficient setting |
| [7:0] | RW | modem_clk_div_num | 0xc | Baseband working system clock division coefficient setting |

      - CLK_MGR_REG0x1c

Offset address: 0x1c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:27] | RW | clk_32k_div_denom | 0x5 | 32K decimal frequency coefficient——denominator |
| [26:16] | RW | clk_32k_div_numer | 0x600 | 32K fractional frequency coefficient——numerator |
| [15:11] | RW | clk_192k_div_denom | 0x5 | 192K decimal frequency coefficient——denominator |
| [10:0] | RW | clk_192k_div_numer | 0x100 | 192K fractional frequency coefficient——numerator |

      - CLK_MGR_REG0x20

Offset address: 0x20 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RW | mclk_sel_reg | 0x1 | Codec MCLK Clock selection: 0 : PLL output clock divided by frequency; 1 : Crystal oscillator clock. |
| [30:26] | RW | mclk_div_num | 0x9 | Codec MCLK requires 24MHz clock division coefficient, mclk_div_num+1 corresponds to frequency division, and the clock source is the BPLL VCO output clock divided by 2 . |
| [25] | - | reserved |  |  |
| [24:13] | RW | clk_8k_div_num | 0xbb8 | 8K frequency division coefficient |
| [12:9] | RW | clk_512k_div_denom | 0x8 | 512K decimal frequency coefficient——denominator |
| [8:0] | RW | clk_512k_div_numer | 0x177 | 512K fractional frequency coefficient——numerator |

      - CLK_MGR_REG0x24

Offset address: 0x24 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:16] | RW | cpu_dac_clk_div_num | 0x18 | CPU DAC working clock frequency division coefficient setting, 0 means no frequency division, dac_clk_div_num+1 corresponds to the frequency division multiple number |
| [15:12] | - | reserved |  |  |
| [11:10] | RW | sdio_clk_dly_reg_drv | 0x2 | SDIO cclk_in_drv clock delay Delay = n/4*cclk_in . For example, if cclk_in = 20ns and n = 1 , delay = 5ns , which means a 90 ° phase shift. |
| [9:8] | RW | sdio_clk_dly_reg_sample​ | 0x0 | SDIO cclk_in_sample clock delay Delay = n/4*cclk_in . For example, if cclk_in = 20ns and n = 1 , delay = 5ns , which means a 90 ° phase shift. |
| [7:4] | RW | adc_clk_div_num | 0xa | CPU ADC working clock frequency division coefficient setting, 0 means No frequency division, adc_clk_div_num+1 corresponds to the frequency division multiple |
| [3:1] | RW | apb_clk_div_num | 0x1 | AHB and APB clock frequency relationship configuration register: 0 , AHB:APB=1:1 ; 1 , AHB:APB=1:2 ; 2 , AHB:APB=1:4 ; |
| [0] | RW | cpu_clk_ratio_reg | 0x0 | AHB and CPU clock frequency relationship configuration register: 0 , AHB:CPU=1:1 ; 1 , AHB:CPU=1:2 ; |

      - CLK_MGR_REG0x28

Offset address: 0x28 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:21] | - | reserved |  |  |
| [20:16] | RW | sdio_clk_div_num | 0x2 | 2 times the SDIO clock (default 80MHz , maximum 96MHz ) division coefficient, sdio_clk_div_num+1 corresponds to the division frequency, the clock source is The BPLL VCO output clock is divided by 2 . |
| [15:5] | - | reserved |  |  |
| [4:0] | RW | usb_clk_div_num | 0x4 | 48MHz clock division factor required by USB is usb_clk_div_num+1 corresponds to frequency division, and the clock source is the BPLL VCO output clock divided by 2 . |

      - CLK_MGR_REG0x2c

Offset address: 0x2c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [twenty three] | RW | dac_clk_en | 0x0 | CPU DAC clock gating enable, high effective |
| [twenty two] | RW | adc_clk_en | 0x0 | CPU low-speed ADC clock gating enable, high effective |

| [twenty one] | RW | hclk_en | 0x0 | Controllable hclk clock gating of peripherals on the AHB bus Enable, high effective |
| --- | --- | --- | --- | --- |
| [20] | RW | pclk_en | 0x0 | Controllable PCLK clock gating of peripherals on the APB bus Enable, high effective |
| [19] | RW | sdio_hclk_en | 0x0 | sdio hclk input clock gating enable, high effective |
| [18] | RW | sdio_clk_en | 0x0 | sdio clock gating enable, high effective |
| [17] | RW | usb_hclk_en | 0x0 | USB hclk input clock gating enable, high effective |
| [16] | RW | usb_clk_en | 0x0 | USB clock gating enable, high effective |
| [15:11] | - | reserved |  |  |
| [10] | RW | mc_clk_en | 0x0 | Codec MC interface configuration clock enable, high effective |
| [9] | RW | modem_clk_rx_en | 0x0 | Baseband receive clock gating enable, high effective |
| [8] | RW | modem_clk_tx_en | 0x0 | Baseband transmit clock gating enable, high effective |
| [7] | RW | modem_clk_fmrx_en | 0x0 | FM receive clock gating enable, high effective |
| [6] | RW | modem_clk_fmtx_en | 0x0 | FM transmit clock gating enable, high effective |
| [5] | - | reserved |  |  |
| [4] | - | reserved |  |  |
| [3] | RW | clk_512k_en_reg | 0x0 | 512K clock gating enable, high effective |
| [2] | RW | clk_8k_en_reg | 0x0 | 8K clock gating enable, high effective |
| [1] | RW | bclk_out_en_reg | 0x0 | BPLL pin output clock gating enable, high effective |
| [0] | RW | aclk_out_en_reg | 0x0 | APLL pin output clock gating enable, high valid |

#### Low power control

基带模块

SOC模块

DH4570

HR_C7000 The power consumption inside the chip is mainly distributed in the functional modules shown in the following figure.

| Modem Tx |  |  | ADCx2 | Reggrop |
| --- | --- | --- | --- | --- |
|  |  |  | Modem Rx |  |
| DACx2 | Codec |  |  |  |
| PLLA |  |  |  |  |

| PLLB | MCU Core |  |  |  |
| --- | --- | --- | --- | --- |
|  |  |  |  | Timer0 |
|  | MCU AHB/APB |  |  | Interrupt |
|  | Memory | Timer1-5​ | I8080 | UART0 |
|  | I2C0,1 | UART1,2,3 | PWM0,1,2 | DAC |
|  | USB | SDIO | SPI0-2​ | Watchdog |
|  | ADC | RTC |  |  |

Figure 14 HR_C7000 Functional module division

of HR_C7000 are divided into two parts: baseband module and SOC Module, where the baseband module includes a digital modulation and demodulation part ( Modem Tx and Modem Rx , and related registers Register op ) ; the analog part includes two ADCs and two DACs , as well as an audio Codec and a PLL module ( PLLA ) .

SOC Module by PLLB The module generates clocks. The main functional blocks include the MCU Core unit, MCU bus ( AHB/APB ) and the peripherals of the bus.

4

5

When the above modules are in standby mode, clock gating can be performed on some modules according to the functional requirements. According to the modules in different color areas in the above figure, they can be processed in different control modes. Specifically, they can be divided into the following 5 types: Functional class.

| Modem Tx |  |
| --- | --- |
| DACx2 | Codec |

| ADCx2 | Reggrop |
| --- | --- |
| Modem Rx |  |

|  | PLLA | PLLB |  |
| --- | --- | --- | --- |
| MCU Core |  |  |  |
| MCU AHB/APB |  |  |  |

| Memory | Timer1-5 |  |
| --- | --- | --- |
| I2C0,1 | UART1,2,3 | PWM0,1,2 |
| USB | SDIO | SPI0-2 |
| ADC | RTC | I8080 |

Timer0

中断

UART0

DAC

Watchdog

1

2

3

Figure 15 HR_C7000 Functional power control distribution

In standby mode, 2 and 4 The included modules need to be turned on to ensure normal signal and interrupt access. 1 The included modules can operate at low clock frequencies ( MCUCore and AHB Clock and APB The clocks all work on the crystal oscillator clock ) . In the IF receiving solution, only one ADC needs to be enabled among the modules in 4 , and the other ADC can be disabled.

During the normal voice reception process, according to the corresponding prompt interruption of reception, switch 1 Included Codec DAC​ After receiving, the codec is turned off again and the device returns to standby mode.

During the voice transmission process, according to the prompt interrupt corresponding to the start of transmission, the baseband DAC is turned on and Codec ADC​ Function: The traditional single-point modulation scheme only needs to open one DAC . After sending, close the Codec again. and DAC ; return to standby mode.

During data transmission and reception, the baseband DAC is turned on according to the interruption of transmission and reception. Functions and Codecs DAC​

The function completes the baseband transmission and reception and the data transmission and reception prompt tone play; after the transmission and reception are completed, the baseband DAC and the Codec DAC are turned off and the device returns to the standby state.

The specific process is shown in the figure below.

1. Complete all module initialization

2, module 4, module 1, module 2 are all turned on;

3, module 5, module 3 all closed

发送结束中断

All modules in module 5 are turned on except the Codec DAC module.

The devices used in module 3 are turned on, and the others are turned off.

接收结束中断

Module 4 ADCx2 off

The DAC of the Codec in block 5 is turned on, and he is turned off;

Sleep

接收中断

Tx Work

Rx Work

模

其模开

发送按键中断

or 发送指令

Rx&Tx

Work

接收结束中断

接收中断

发送结束中断

发送按键中断

Or 发送指令

The peripherals used in block 3 are enabled, and the others are disabled.

Module 5 is all turned on;

The peripheral modules used in module 3 are turned on, and the others are turned off.

Figure 16 HR_C7000 Low-power workflow

#### System Control

###### Overview

System control mainly includes LCSFC SPI SCLK Clock division, LCSFC Four-wire control, RTC Control and pin multiplexing control.

###### Features

- LCSFC SPI SCLK Clock division setting

- LCSFC Working Configuration

- RTC Output clamp configuration

- Pin multiplexing

###### Register Overview

Register Table 3 System Control Register Overview (Base Address: 0x11000000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x30 | LCSFC_BAUDR | LCSFC SPI SCLK clock division |  |
| 0x58 | RTC_REQ_HOLD | RTC Output clamp hold register control |  |
| 0x5c | QUAD_ENABLE | LCSFC SPI four-wire control register |  |
| 0x34 | IO_DIPLEX0 | IO Multiplex Select Register 0 |  |
| 0x38 | IO_DIPLEX1 | IO Multiplex select register 1 |  |
| 0x3c | IO_DIPLEX2 | IO Multiplex select register 2 |  |
| 0x40 | IOMGR_REN_REG0 | IO Pull-up and pull-down enable register 0 |  |
| 0x44 | IOMGR_REN_REG1 | IO Pull-up and pull-down enable register 1 |  |
| 0x48 | IOMGR_REN_REG2 | IO Pull-up and pull-down enable register 2 |  |
| 0x4c | IOMGR_IE_IE0 | IO Input Enable Register 0 |  |
| 0x50 | IOMGR_IE_IE1 | IO Input Enable Register 1 |  |

| 0x54 | IOMGR_IE_IE2 | IO Input Enable Register 2 |  |
| --- | --- | --- | --- |

###### Register Description

      - LCSFC_BAUDR

Offset address: 0x30 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | lcsfc_baudr | 0x4 | LCSFC SPI SCLK clock frequency setting, default value 4​ |

      - RTC_REQ_HOLD

Offset address: 0x58 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | - | reserved |  |  |
| [0] | RW | reg_rtc_hold | 0x1 | RTC output clamp hold register control |

      - QUAD_ENABLE

Offset address: 0x5c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | RW | sys_sample_delay | 0x0 | Four-wire data line input sampling delay setting, range 0~8 |
| [23:1] | - | reserved |  |  |
| [0] | RW | quad_enable | 0x0 | LCSFC SPI four-wire control enable, high effective |

      - IO_DIPLEX0

Offset address: 0x34 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RW | ptb7_sel | 0x1 | PTB7 multiplexing selection 1 : GPIO1_PTB7 0 : AK_ADC_SDO |
| [30:20] | - | reserved |  |  |
| [19] | RW | pwm2_sel | 0x1 | PWM_2 multiplexing selection 1 : GPIO_PTA17 0 : PWM_2 |
| [18] | RW | pwm1_sel | 0x1 | PWM_1 multiplexing selection 1 : GPIO_PTA16 0 : PWM_1 |
| [17] | RW | pwm0_sel | 0x1 | PWM_0 multiplexing selection 1 : GPIO_PTA15 0 : PWM_0 |

| [16:15] | RW | uart3_txd_sel | 0x3 | UART3_TXD multiplexing selection 3 : GPIO_PTA14 2 : Reserved 1 : Baseband rf_rx_inter 0 : UART3_TXD |
| --- | --- | --- | --- | --- |
| [14:13] | RW | uart3_rxd_sel | 0x3 | UART3_RXD multiplexing selection 3 : GPIO_PTA13 2 : Reserved 1 : Baseband rf_tx_inter 0 : UART3_RXD |
| [12] | RW | uart2_txd_sel | 0x1 | UART2_TXD multiplexing selection 1 : GPIO_PTA12 0 : UART2_TXD |
| [11] | RW | uart2_rxd_sel | 0x1 | UART2_RXD multiplexing selection 1 : GPIO_PTA11 0 : UART2_RXD |
| [10] | RW | uart1_txd_sel | 0x1 | UART1_TXD multiplexing selection 1 : GPIO_PTA10 0 : UART1_TXD |
| [9] | RW | uart1_rxd_sel | 0x1 | UART1_RXD multiplexing selection 1 : GPIO_PTA9 0 : UART1_RXD |
| [8] | RW | i2c1_sda_sel | 0x1 | I2C_SDA_1 Multiplexing Selection 1 : GPIO_PTA8 0 : I2C_SDA_1 |
| [7] | RW | i2c1_scl_sel | 0x1 | I2C_SCL_1 multiplexing selection 1 : GPIO_PTA7 0 : I2C_SCL_1 |
| [6] | RW | i2c0_sda_sel | 0x1 | I2C_SDA_0 Multiplexing Selection 1 : GPIO_PTA6 0 : I2C_SDA_0 |
| [5] | RW | i2c0_scl_sel | 0x1 | I2C_SCL_0 multiplexing selection 1 : GPIO_PTA5 0 : I2C_SCL_0 |
| [4] | RW | jtg_rstn_sel | 0x0 | JTG_RST_N multiplexing selection 1 : GPIO_PTA4 0 : JTG_RST_N |
| [3] | RW | jtg_tdo_sel | 0x0 | JTG_TDO multiplexing selection 1 : GPIO_PTA3 0 : JTG_TDO |
| [2] | RW | jtg_tdi_sel | 0x0 | JTG_TDI multiplexing selection 1 : GPIO_PTA2 0 : JTG_TDI |
| [1] | RW | jtg_tck_sel | 0x0 | JTG_TCK multiplexing selection |

|  |  |  |  | 1 : GPIO_PTA1 0 : JTG_TCK |
| --- | --- | --- | --- | --- |
| [0] | RW | jtg_tms_sel | 0x0 | JTG_TMS multiplexing selection 1 : GPIO_PTA0 0 : JTG_TMS |

      - IO_DIPLEX1

Offset address: 0x38 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RW | ptb6_sel | 0x1 | PTB6 multiplexing selection 1 : GPIO1_PTB6 0 : AK_ADC_SCLK |
| [30] | RW | ptb5_sel | 0x1 | PTB4 multiplexing selection 1 : GPIO1_PTB5 0 : AK_ADC_FS |
| [29] | RW | ptb4_sel | 0x1 | PTB4 multiplexing selection 1 : GPIO1_PTB4 0 : AK_DAC_SDI |
| [28] | RW | ptb3_sel | 0x1 | PTB3 multiplexing selection 1 : GPIO1_PTB3 0 : AK_DAC_SCLK |
| [27] | RW | ptb2_sel | 0x1 | PTB2 multiplexing selection 1 : GPIO1_PTB2 0 : AK_DAC_FS |
| [26:25] | RW | time_slot_tx_sel | 0x3 | TIME_SLOT_T_INTER multiplexing selection 3 : GPIO_PTA31 2 : SDIO sdio_card_write_prt 1 : Reserved 0 : TIME_SLOT_T_INTER |
| [24:23] | RW | time_slot_rx_sel | 0x3 | TIME_SLOT_R_INTER multiplexing selection 3 : GPIO_PTA30 2 : SDIO sdio_card_detect_n 1 : Reserved 0 : TIME_SLOT_R_INTER |
| [22:21] | RW | bs_inter_out_sel | 0x3 | BS_INTER_OUT multiplexing selection 3 : GPIO_PTA29 2 : SDIO sdio_cdata[1] 1 : Reserved 0 : BS_INTER_OUT |
| [20:19] | RW | bs_inter_in_sel | 0x3 | BS_INTER_IN multiplexing selection 3 : GPIO_PTA28 2 : SDIO sdio_cdata[2] |

|  |  |  |  | 1 : POR_RST_N 0 : BS_INTER_IN |
| --- | --- | --- | --- | --- |
| [18:17] | RW | spi1_miso_sel | 0x3 | SPI1_MISO multiplexing selection 3 : GPIO_PTA27 2 : SDIO sdio_cdata[0] 1 : Slave SPI1_MISO 0 : Master SPI1_MISO |
| [16:15] | RW | spi1_mosi_sel | 0x3 | SPI1_MOSI multiplexing selection 3 : GPIO_PTA26 2 : SDIO sdio_ccmd 1 : Slave SPI1_MOSI 0 : Master SPI1_MOSI |
| [14:13] | RW | spi1_sclk_sel | 0x3 | SPI1_SCLK multiplexing selection 3 : GPIO_PTA25 2 : SDIO sdio_cclk_out 1 : Slave SPI1_SCLK 0 : Master SPI1_SCLK |
| [12:11] | RW | spi1_csn1_sel | 0x3 | SPI1_CSN_1 multiplexing selection 3 : GPIO_PTA24 2 : Reserved 1 : Baseband sys_inter 0 : Master SPI1_CSN_1 |
| [10:9] | RW | spi1_csn0_sel | 0x3 | SPI1_CSN_0 multiplexing selection 3 : GPIO_PTA23 2 : SDIO sdio_cdata[3] 1 : Slave SPI1_CSN_0 0 : Master SPI1_CSN_0 |
| [8:7] | RW | spi0_miso_sel | 0x3 | SPI0_MISO multiplexing selection 3 : GPIO_PTA22 2 : Reserved 1 : Slave SPI0_MISO 0 : Master SPI0_MISO |
| [6:5] | RW | spi0_mosi_sel | 0x3 | SPI0_MOSI multiplexing selection 3 : GPIO_PTA21 2 : Reserved 1 : Slave SPI0_MOSI 0 : Master SPI0_MOSI |
| [4:3] | RW | spi0_sclk_sel | 0x3 | SPI0_SCLK multiplexing selection 3 : GPIO_PTA20 2 : Reserved 1 : Slave SPI0_SCLK 0 : Master SPI0_SCLK |
| [2] | RW | spi0_csn1_sel | 0x1 | SPI0_CSN_1 multiplexing selection 1 : GPIO_PTA19 |

|  |  |  |  | 0 : SPI0_CSN_1 |
| --- | --- | --- | --- | --- |
| [1:0] | RW | spi0_csn0_sel | 0x3 | SPI0_CSN_0 multiplexing selection 3 : GPIO_PTA18 2 : Reserved 1 : Slave SPI0_CSN_0 0 : Master SPI0_CSN_0 |

      - IO_DIPLEX2

Offset address: 0x3c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:30] | - | reserved |  |  |
| [29] | RW | dac_vout_mcuc_sel | 0x1 | DAC_VOUT_MCUC multiplexing selection 1 : GPIO_PTC25 0 : DAC_VOUT_MCUC |
| [28] | RW | dac_vout_mcub_sel | 0x1 | DAC_VOUT_MCUB multiplexing selection 1 : GPIO_PTC24 0 : DAC_VOUT_MCUB |
| [27] | RW | dac_vout_mcua_sel | 0x1 | DAC_VOUT_MCUA multiplexing selection 1 : GPIO_PTC23 0 : DAC_VOUT_MCUA |
| [26] | RW | adc7_in_sel | 0x1 | ADC7_IN multiplexing selection 1 : GPIO_PTC22 0 : ADC7_IN |
| [25] | RW | adc6_in_sel | 0x1 | ADC6_IN multiplexing selection 1 : GPIO_PTC21 0 : ADC6_IN |
| [twenty four] | RW | adc5_in_sel | 0x1 | ADC5_IN multiplexing selection 1 : GPIO_PTC20 0 : ADC5_IN |
| [twenty three] | RW | adc4_in_sel | 0x1 | ADC4_IN multiplexing selection 1 : GPIO_PTC19 0 : ADC4_IN |
| [twenty two] | RW | adc3_in_sel | 0x1 | ADC3_IN multiplexing selection 1 : GPIO_PTC18 0 : ADC3_IN |
| [twenty one] | RW | adc2_in_sel | 0x1 | ADC2_IN multiplexing selection 1 : GPIO_PTC17 0 : ADC2_IN |
| [20] | RW | adc1_in_sel | 0x1 | ADC1_IN multiplexing selection 1 : GPIO_PTC16 0 : ADC1_IN |
| [19] | RW | adc0_in_sel | 0x1 | ADC0_IN multiplexing selection |

|  |  |  |  | 1 : GPIO_PTC15 0 : ADC0_IN |
| --- | --- | --- | --- | --- |
| [18] | RW | lcd_db7_sel | 0x1 | LCD_DB7 multiplexing selection 1 : GPIO_PTC24 0 : LCD_DB7 |
| [17] | RW | lcd_db6_sel | 0x1 | LCD_DB6 multiplexing selection 1 : GPIO_PTC13 0 : LCD_DB6 |
| [16] | RW | lcd_db5_sel | 0x1 | LCD_DB5 multiplexing selection 1 : GPIO_PTC12 0 : LCD_DB5 |
| [15] | RW | lcd_db4_sel | 0x1 | LCD_DB4 multiplexing selection 1 : GPIO_PTC11 0 : LCD_DB4 |
| [14] | RW | lcd_db3_sel | 0x1 | LCD_DB3 multiplexing selection 1 : GPIO_PTC10 0 : LCD_DB3 |
| [13] | RW | lcd_db2_sel | 0x1 | LCD_DB2 multiplexing selection 1 : GPIO_PTC9 0 : LCD_DB2 |
| [12:11] | RW | lcd_db1_sel | 0x3 | LCD_DB1 multiplexing selection 3 : GPIO_PTC8 2 : Slave SPI2_MOSI 1 : Master SPI2_MOSI 0 : LCD_DB1 |
| [10:9] | RW | lcd_db0_sel | 0x3 | LCD_DB0 multiplexing selection 3 : GPIO_PTC7 2 : Slave SPI2_MISO 1 : Master SPI2_MISO 0 : LCD_DB0 |
| [8] | RW | lcd_rd_sel | 0x1 | LCD_RD multiplexing selection 1 : GPIO_PTC6 0 : LCD_RD |
| [7] | RW | lcd_wr_sel | 0x1 | LCD_WR multiplexing selection 1 : GPIO_PTC5 0 : LCD_WR |
| [6:5] | RW | lcd_rs_sel | 0x3 | LCD_RS multiplexing selection 3 : GPIO_PTC4 2 : Slave SPI2_SCLK 1 : Master SPI2_SCLK 0 : LCD_RS |
| [4:3] | RW | lcd_cs_sel | 0x3 | LCD_CS multiplexing selection 3 : GPIO_PTC3 2 : Slave SPI2_CS |

|  |  |  |  | 1 : Master SPI2_CS 0 : LCD_CS |
| --- | --- | --- | --- | --- |
| [2] | - | reserved |  |  |
| [1:0] | RW | clk_out_sel | 0x3 | CLK_OUT multiplexing selection 3 : GPIO_PTC0 2 : RTC_CLK 1 : PLLB CLK_OUT 0 : PLLA CLK_OUT |

      - IOMGR_REN_REG0

Offset address: 0x40 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RW | time_slot_rx_ren | 0x1 | TIME_SLOT_R_INTER pull-down enable 1 : enable off 0 : Enable valid |
| [30] | RW | time_slot_tx_ren | 0x1 | TIME_SLOT_T_INTER pull-down enable 1 : enable off 0 : Enable valid |
| [29] | RW | bs_inter_out_ren | 0x1 | BS_INTER_OUT pull-up enable 1 : enable off 0 : Enable valid |
| [28] | RW | bs_inter_in_ren | 0x1 | BS_INTER_IN pull-up enable 1 : Enable or disable 0 : Enable valid |
| [27] | RW | spi1_miso_ren | 0x1 | SPI1_MISO pull-up enable 1 : enable off 0 : Enable valid |
| [26] | RW | spi1_mosi_ren | 0x1 | SPI1_MOSI pull-up enable 1 : enable or disable 0 : Enable valid |
| [25] | RW | spi1_sclk_ren | 0x1 | SPI1_SCLK pull-down enable 1 : enable off 0 : Enable valid |
| [twenty four] | RW | spi1_csn1_ren | 0x1 | SPI1_CSN_1 pull-up enable 1 : enable off 0 : Enable valid |
| [twenty three] | RW | spi1_csn0_ren | 0x1 | SPI1_CSN_0 pull-up enable 1 : enable or disable 0 : Enable valid |
| [twenty two] | RW | spi0_miso_ren | 0x1 | SPI0_MISO pull-down enable 1 : enable off 0 : Enable valid |

| [twenty one] | RW | spi0_mosi_ren | 0x1 | SPI0_MOSI pull-up enable 1 : enable or disable 0 : Enable valid |
| --- | --- | --- | --- | --- |
| [20] | RW | spi0_sclk_ren | 0x1 | SPI0_SCLK pull-down enable 1 : enable off 0 : Enable valid |
| [19] | RW | spi0_csn1_ren | 0x1 | SPI0_CSN_1 pull-up enable 1 : enable off 0 : Enable valid |
| [18] | RW | spi0_csn0_ren | 0x1 | SPI0_CSN_0 pull-up enable 1 : enable or disable 0 : Enable valid |
| [17] | RW | pwm2_ren | 0x1 | PWM_2 pull-down enable 1 : enable off 0 : Enable valid |
| [16] | RW | pwm1_ren | 0x1 | PWM_1 pull-up enable 1 : enable off 0 : Enable valid |
| [15] | RW | pwm0_ren | 0x1 | PWM_0 pull-up enable 1 : enable or disable 0 : Enable valid |
| [14] | RW | uart3_txd_ren | 0x1 | UART3_TXD pull-up enable 1 : enable or disable 0 : Enable valid |
| [13] | RW | uart3_rxd_ren | 0x1 | UART3_RXD pull-up enable 1 : enable or disable 0 : Enable valid |
| [12] | RW | uart2_txd_ren | 0x1 | UART2_TXD pull-up enable 1 : enable or disable 0 : Enable valid |
| [11] | RW | uart2_rxd_ren | 0x1 | UART2_RXD pull-up enable 1 : enable or disable 0 : Enable valid |
| [10] | RW | uart1_txd_ren | 0x1 | UART1_TXD pull-up enable 1 : enable or disable 0 : Enable valid |
| [9] | RW | uart1_rxd_ren | 0x1 | UART1_RXD pull-up enable 1 : enable or disable 0 : Enable valid |
| [8] | RW | i2c1_sda_ren | 0x1 | I2C_SDA_1 pull-up enable 1 : enable off 0 : Enable valid |
| [7] | RW | i2c1_scl_ren | 0x1 | I2C_SCL_1 pull-up enable |

|  |  |  |  | 1 : Enable off 1 : Enable valid |
| --- | --- | --- | --- | --- |
| [6] | RW | i2c0_sda_ren | 0x1 | I2C_SDA_0 pull-up enable 1 : enable off 2 : Enable valid |
| [5] | RW | i2c0_scl_ren | 0x1 | I2C_SCL_0 pull-up enable 1 : enable off 0 : Enable valid |
| [4] | RW | jtg_rstn_ren | 0x1 | JTG_RST_N pull-up enable 1 : enable off 0 : Enable valid |
| [3] | RW | jtg_tdo_ren | 0x1 | JTG_TDO pull-down enable 1 : enable off 0 : Enable valid |
| [2] | RW | jtg_tdi_ren | 0x1 | JTG_TDI pull-down enable 1 : enable off 0 : Enable valid |
| [1] | RW | jtg_tck_ren | 0x1 | JTG_TCK pull-up enable 1 : enable off 0 : Enable valid |
| [0] | RW | jtg_tms_ren | 0x1 | JTG_TMS pull-up enable 1 : enable off 0 : Enable valid |

      - IOMGR_REN_REG1

Offset address: 0x44 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RW | uart0_txd_ren | 0x1 | UART0_TXD pull-up enable 1 : enable or disable 0 : Enable valid |
| [30] | RW | uart0_rxd_ren | 0x1 | UART0_RXD pull-up enable 1 : enable or disable 0 : Enable valid |
| [29] | RW | nfc_miso_ren | 0x1 | NFC_MISO pull-up enable 1 : Enable off 0 : Enable valid |
| [28] | RW | nfc_mosi_ren | 0x1 | NFC_MOSI pull-up enable 1 : enable off 0 : Enable valid |
| [27] | RW | nfc_sclk_ren | 0x1 | NFC_SCLK pull-up enable 1 : Enable or disable 0 : Enable valid |

| [26] | RW | nfc_csn_ren | 0x1 | NFC_CSN pull-up enable 1 : enable off 0 : Enable valid |
| --- | --- | --- | --- | --- |
| [25] | RW | nfc_wp_ren | 0x1 | NFC_WP pull-up enable 1 : enable off 0 : Enable valid |
| [twenty four] | RW | nfc_hold_ren | 0x1 | NFC_HOLD pull-up enable 1 : Enable off 0 : Enable valid |
| [twenty three] | RW | dac_vout_mcuc_ren | 0x1 | DAC_VOUT_MCUC Pull-up enable 1 : Enable off 0 : Enable valid |
| [twenty two] | RW | dac_vout_mcub_ren | 0x1 | DAC_VOUT_MCUB Pull-up enable 1 : Enable off 0 : Enable valid |
| [twenty one] | RW | dac_vout_mcua_ren | 0x1 | DAC_VOUT_MCUA Pull-up enable 1 : Enable off 0 : Enable valid |
| [20] | RW | adc7_in_ren | 0x1 | ADC7_IN Pull-down enable 1 : Enable off 0 : Enable valid |
| [19] | RW | adc6_in_ren | 0x1 | ADC6_IN Pull-down enable 1 : Enable off 0 : Enable valid |
| [18] | RW | adc5_in_ren | 0x1 | ADC5_IN pull-down enable 1 : enable off 0 : Enable valid |
| [17] | RW | adc4_in_ren | 0x1 | ADC4_IN pull-down enable 1 : enable off 0 : Enable valid |
| [16] | RW | adc3_in_ren | 0x1 | ADC3_IN pull-down enable 1 : enable off 0 : Enable valid |
| [15] | RW | adc2_in_ren | 0x1 | ADC2_IN pull-down enable 1 : enable off 0 : Enable valid |
| [14] | RW | adc1_in_ren | 0x1 | ADC1_IN pull-down enable 1 : enable off 0 : Enable valid |
| [13] | RW | adc0_in_ren | 0x1 | ADC0_IN pull-down enable 1 : enable off 0 : Enable valid |
| [12] | RW | lcd_db7_ren | 0x1 | LCD_DB7 pull-down enable |

|  |  |  |  | 1 : Enable off 0 : Enable valid |
| --- | --- | --- | --- | --- |
| [11] | RW | lcd_db6_ren | 0x1 | LCD_DB6 Pull-up enable 1 : Enable off 0 : Enable valid |
| [10] | RW | lcd_db5_ren | 0x1 | LCD_DB5 Pull-up enable 1 : Enable off 0 : Enable valid |
| [9] | RW | lcd_db4_ren | 0x1 | LCD_DB4 Pull-up enable 1 : Enable off 0 : Enable valid |
| [8] | RW | lcd_db3_ren | 0x1 | LCD_DB3 Pull-up enable 1 : Enable off 0 : Enable valid |
| [7] | RW | lcd_db2_ren | 0x1 | LCD_DB2 Pull-down enable 1 : Enable off 0 : Enable valid |
| [6] | RW | lcd_db1_ren | 0x1 | LCD_DB1 Pull-down enable 1 : Enable off 0 : Enable valid |
| [5] | RW | lcd_db0_ren | 0x1 | LCD_DB0 Pull-down enable 1 : Enable off 0 : Enable valid |
| [4] | RW | lcd_rd_ren | 0x1 | LCD_RD pull-up enable 1 : Enable off 0 : Enable valid |
| [3] | RW | lcd_wr_ren | 0x1 | LCD_WR Pull-up enable 1 : Enable off 0 : Enable valid |
| [2] | RW | lcd_rs_ren | 0x1 | LCD_RS pull-up enable 1 : enable off 0 : Enable valid |
| [1] | RW | lcd_cs_ren | 0x1 | LCD_CS pull-up enable 1 : Enable or disable 0 : Enable valid |
| [0] | RW | lcd_nreset_ren | 0x1 | LCD_NRESET pull-up enable 1 : enable off 0 : Enable valid |

      - IOMGR_REN_REG2

Offset address: 0x48 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |

| [31] | - | reserved |  |  |
| --- | --- | --- | --- | --- |
| [30] | RW | clk_out_ren | 0x1 | CLK_OUT Pull-down enable 1 : Enable off 0 : Enable valid |
| [29] | RW | ptb29_ren | 0x1 | PTB29 pull-down enable 1 : enable off 0 : Enable valid |
| [28] | RW | ptb28_ren | 0x1 | PTB28 pull-up enable 1 : enable off 0 : Enable valid |
| [27] | RW | ptb27_ren | 0x1 | PTB27 pull-up enable 1 : enable off 0 : Enable valid |
| [26] | RW | ptb26_ren | 0x1 | PTB26 pull-up enable 1 : enable off 0 : Enable valid |
| [25] | RW | ptb25_ren | 0x1 | PTB25 pull-up enable 1 : enable off 0 : Enable valid |
| [twenty four] | RW | ptb24_ren | 0x1 | PTB24 pull-down enable 1 : enable off 0 : Enable valid |
| [twenty three] | RW | ptb23_ren | 0x1 | PTB23 pull-down enable 1 : enable off 0 : Enable valid |
| [twenty two] | RW | ptb22_ren | 0x1 | PTB22 pull-up enable 1 : enable off 0 : Enable valid |
| [twenty one] | RW | ptb21_ren | 0x1 | PTB21 pull-up enable 1 : enable off 0 : Enable valid |
| [20] | RW | ptb20_ren | 0x1 | PTB20 pull-up enable 1 : enable off 0 : Enable valid |
| [19] | RW | ptb19_ren | 0x1 | PTB19 pull-up enable 1 : enable off 0 : Enable valid |
| [18] | RW | ptb18_ren | 0x1 | PTB18 pull-down enable 1 : enable off 0 : Enable valid |
| [17] | RW | ptb17_ren | 0x1 | PTB17 pull-down enable 1 : enable off 0 : Enable valid |

| [16] | RW | ptb16_ren | 0x1 | PTB16 pull-up enable 1 : enable off 0 : Enable valid |
| --- | --- | --- | --- | --- |
| [15] | RW | ptb15_ren | 0x1 | PTB15 pull-up enable 1 : enable off 0 : Enable valid |
| [14] | RW | ptb14_ren | 0x1 | PTB14 pull-up enable 1 : enable off 0 : Enable valid |
| [13] | RW | ptb13_ren | 0x1 | PTB13 pull-up enable 1 : enable off 0 : Enable valid |
| [12] | RW | ptb12_ren | 0x1 | PTB12 pull-down enable 1 : enable off 0 : Enable valid |
| [11] | RW | ptb11_ren | 0x1 | PTB11 pull-down enable 1 : enable off 0 : Enable valid |
| [10] | RW | ptb10_ren | 0x1 | PTB10 pull-up enable 1 : enable off 0 : Enable valid |
| [9] | RW | ptb9_ren | 0x1 | PTB9 pull-up enable 1 : enable off 0 : Enable valid |
| [8] | RW | ptb8_ren | 0x1 | PTB8 pull-up enable 1 : enable off 0 : Enable valid |
| [7] | RW | ptb7_ren | 0x1 | PTB7 pull-up enable 1 : enable off 0 : Enable valid |
| [6] | RW | ptb6_ren | 0x1 | PTB6 pull-down enable 1 : enable off 0 : Enable valid |
| [5] | RW | ptb5_ren | 0x1 | PTB5 pull-down enable 1 : enable off 0 : Enable valid |
| [4] | RW | ptb4_ren | 0x1 | PTB4 pull-down enable 1 : enable off 0 : Enable valid |
| [3] | RW | ptb3_ren | 0x1 | PTB3 pull-up enable 1 : enable off 0 : Enable valid |
| [2] | RW | ptb2_ren | 0x1 | PTB2 pull-up enable |

|  |  |  |  | 1 : Enable off 0 : Enable valid |
| --- | --- | --- | --- | --- |
| [1] | RW | ptb1_ren | 0x1 | PTB1 pull-up enable 1 : enable off 0 : Enable valid |
| [0] | RW | ptb0_ren | 0x1 | PTB0 pull-up enable 1 : enable off 0 : Enable valid |

      - IOMGR_IE_IE0

Offset address: 0x4c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RW | time_slot_rx_ie | 0x1 | TIME_SLOT_R_INTER input enable 1 : Enable valid 0 : Enable off |
| [30] | RW | time_slot_tx_ie | 0x1 | TIME_SLOT_T_INTER input enable 1 : Enable valid 0 : Enable off |
| [29] | RW | bs_inter_out_ie | 0x1 | BS_INTER_OUT input enable 1 : Enable valid 0 : Enable off |
| [28] | RW | bs_inter_in_ie | 0x1 | BS_INTER_IN input enable 1 : Enable valid 0 : Enable off |
| [27] | RW | spi1_miso_ie | 0x1 | SPI1_MISO input enable 1 : Enable valid 0 : Enable off |
| [26] | RW | spi1_mosi_ie | 0x1 | SPI1_MOSI input enable 1 : enable valid 0 : Enable off |
| [25] | RW | spi1_sclk_ie | 0x1 | SPI1_SCLK input enable 1 : Enable valid 0 : Enable off |
| [twenty four] | RW | spi1_csn1_ie | 0x1 | SPI1_CSN_1 input enable 1 : enable valid 0 : Enable off |
| [twenty three] | RW | spi1_csn0_ie | 0x1 | SPI1_CSN_0 input enable 1 : enable valid 0 : Enable off |
| [twenty two] | RW | spi0_miso_ie | 0x1 | SPI0_MISO input enable 1 : Enable valid 0 : Enable off |

| [twenty one] | RW | spi0_mosi_ie | 0x1 | SPI0_MOSI input enable 1 : Enable valid 0 : Enable off |
| --- | --- | --- | --- | --- |
| [20] | RW | spi0_sclk_ie | 0x1 | SPI0_SCLK input enable 1 : Enable valid 0 : Enable off |
| [19] | RW | spi0_csn1_ie | 0x1 | SPI0_CSN_1 input enable 1 : enable valid 0 : Enable off |
| [18] | RW | spi0_csn0_ie | 0x1 | SPI0_CSN_0 input enable 1 : Enable valid 0 : Enable off |
| [17] | RW | pwm2_ie | 0x1 | PWM_2 input enable 1 : Enable valid 0 : Enable off |
| [16] | RW | pwm1_ie | 0x1 | PWM_1 input enable 1 : Enable valid 0 : Enable off |
| [15] | RW | pwm0_ie | 0x1 | PWM_0 input enable 1 : Enable valid 0 : Enable off |
| [14] | RW | uart3_txd_ie | 0x1 | UART3_TXD input enable 1 : Enable valid 0 : Enable off |
| [13] | RW | uart3_rxd_ie | 0x1 | UART3_RXD input enable 1 : Enable valid 0 : Enable off |
| [12] | RW | uart2_txd_ie | 0x1 | UART2_TXD input enable 1 : Enable valid 0 : Enable off |
| [11] | RW | uart2_rxd_ie | 0x1 | UART2_RXD input enable 1 : Enable valid 0 : Enable off |
| [10] | RW | uart1_txd_ie | 0x1 | UART1_TXD input enable 1 : Enable valid 0 : Enable off |
| [9] | RW | uart1_rxd_ie | 0x1 | UART1_RXD input enable 1 : Enable valid 0 : Enable off |
| [8] | RW | i2c1_sda_ie | 0x1 | I2C_SDA_1 input enable 1 : enable valid 0 : Enable off |
| [7] | RW | i2c1_scl_ie | 0x1 | I2C_SCL_1 input enable |

|  |  |  |  | 1 : Enable valid 0 : Enable off |
| --- | --- | --- | --- | --- |
| [6] | RW | i2c0_sda_ie | 0x1 | I2C_SDA_0 input enable 1 : Enable valid 0 : Enable off |
| [5] | RW | i2c0_scl_ie | 0x1 | I2C_SCL_0 input enable 1 : Enable valid 0 : Enable off |
| [4] | RW | jtg_rstn_ie | 0x1 | JTG_RST_N input enable 1 : enable valid 0 : Enable off |
| [3] | RW | jtg_tdo_ie | 0x1 | JTG_TDO input enable 1 : Enable valid 0 : Enable off |
| [2] | RW | jtg_tdi_ie | 0x1 | JTG_TDI input enable 1 : Enable valid 0 : Enable off |
| [1] | RW | jtg_tck_ie | 0x1 | JTG_TCK input enable 1 : Enable valid 0 : Enable off |
| [0] | RW | jtg_tms_ie | 0x1 | JTG_TMS input enable 1 : Enable valid 0 : Enable off |

      - IOMGR_IE_IE1

Offset address: 0x50 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RW | uart0_txd_ie | 0x1 | UART0_TXD input enable 1 : Enable valid 0 : Enable off |
| [30] | RW | uart0_rxd_ie | 0x1 | UART0_RXD input enable 1 : Enable valid 0 : Enable off |
| [29] | RW | nfc_miso_ie | 0x1 | NFC_MISO input enable 1 : Enable valid 0 : Enable off |
| [28] | RW | nfc_mosi_ie | 0x1 | NFC_MOSI input enable 1 : Enable valid 0 : Enable off |
| [27] | RW | nfc_sclk_ie | 0x1 | NFC_SCLK input enable 1 : Enable valid 0 : Enable off |

| [26] | RW | nfc_csn_ie | 0x1 | NFC_CSN input enable 1 : Enable valid 0 : Enable off |
| --- | --- | --- | --- | --- |
| [25] | RW | nfc_wp_ie | 0x1 | NFC_WP input enable 1 : Enable valid 0 : Enable off |
| [twenty four] | RW | nfc_hold_ie | 0x1 | NFC_HOLD input enable 1 : Enable valid 0 : Enable off |
| [twenty three] | RW | dac_vout_mcuc_ie | 0x1 | DAC_VOUT_MCUC input enable 1 : Enable valid 0 : Enable off |
| [twenty two] | RW | dac_vout_mcub_ie | 0x1 | DAC_VOUT_MCUB input enable 1 : Enable valid 0 : Enable off |
| [twenty one] | RW | dac_vout_mcua_ie | 0x1 | DAC_VOUT_MCUA input enable 1 : Enable valid 0 : Enable off |
| [20] | RW | adc7_in_ie | 0x1 | ADC7_IN input enable 1 : Enable valid 0 : Enable off |
| [19] | RW | adc6_in_ie | 0x1 | ADC6_IN input enable 1 : Enable valid 0 : Enable off |
| [18] | RW | adc5_in_ie | 0x1 | ADC5_IN input enable 1 : Enable valid 0 : Enable off |
| [17] | RW | adc4_in_ie | 0x1 | ADC4_IN input enable 1 : Enable valid 0 : Enable off |
| [16] | RW | adc3_in_ie | 0x1 | ADC3_IN input enable 1 : Enable valid 0 : Enable off |
| [15] | RW | adc2_in_ie | 0x1 | ADC2_IN input enable 1 : Enable valid 0 : Enable off |
| [14] | RW | adc1_in_ie | 0x1 | ADC1_IN input enable 1 : Enable valid 0 : Enable off |
| [13] | RW | adc0_in_ie | 0x1 | ADC0_IN input enable 1 : Enable valid 0 : Enable off |
| [12] | RW | lcd_db7_ie | 0x1 | LCD_DB7 input enable |

|  |  |  |  | 1 : Enable valid 0 : Enable off |
| --- | --- | --- | --- | --- |
| [11] | RW | lcd_db6_ie | 0x1 | LCD_DB6 input enable 1 : Enable valid 0 : Enable off |
| [10] | RW | lcd_db5_ie | 0x1 | LCD_DB5 input enable 1 : Enable valid 0 : Enable off |
| [9] | RW | lcd_db4_ie | 0x1 | LCD_DB4 input enable 1 : Enable valid 0 : Enable off |
| [8] | RW | lcd_db3_ie | 0x1 | LCD_DB3 input enable 1 : Enable valid 0 : Enable off |
| [7] | RW | lcd_db2_ie | 0x1 | LCD_DB2 input enable 1 : Enable valid 0 : Enable off |
| [6] | RW | lcd_db1_ie | 0x1 | LCD_DB1 input enable 1 : Enable valid 0 : Enable off |
| [5] | RW | lcd_db0_ie | 0x1 | LCD_DB0 input enable 1 : Enable valid 0 : Enable off |
| [4] | RW | lcd_rd_ie | 0x1 | LCD_RD input enable 1 : Enable valid 0 : Enable off |
| [3] | RW | lcd_wr_ie | 0x1 | LCD_WR input enable 1 : Enable valid 0 : Enable off |
| [2] | RW | lcd_rs_ie | 0x1 | LCD_RS input enable 1 : Enable valid 0 : Enable off |
| [1] | RW | lcd_cs_ie | 0x1 | LCD_CS input enable 1 : Enable valid 0 : Enable off |
| [0] | RW | lcd_nreset_ie | 0x1 | LCD_NRESET input enable 1 : Enable valid 0 : Enable off |

      - IOMGR_IE_IE2

Offset address: 0x54 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |

| [31] | - | reserved |  |  |
| --- | --- | --- | --- | --- |
| [30] | RW | clk_out_ie | 0x1 | CLK_OUT input enable 1 : Enable valid 0 : Enable off |
| [29] | RW | ptb29_ie | 0x1 | PTB29 input enable 1 : enable valid 0 : Enable off |
| [28] | RW | ptb28_ie | 0x1 | PTB28 input enable 1 : enable valid 0 : Enable off |
| [27] | RW | ptb27_ie | 0x1 | PTB27 input enable 1 : enable valid 0 : Enable off |
| [26] | RW | ptb26_ie | 0x1 | PTB26 input enable 1 : enable valid 0 : Enable off |
| [25] | RW | ptb25_ie | 0x1 | PTB25 input enable 1 : enable valid 0 : Enable off |
| [twenty four] | RW | ptb24_ie | 0x1 | PTB24 input enable 1 : enable valid 0 : Enable off |
| [twenty three] | RW | ptb23_ie | 0x1 | PTB23 input enable 1 : enable valid 0 : Enable off |
| [twenty two] | RW | ptb22_ie | 0x1 | PTB22 input enable 1 : enable valid 0 : Enable off |
| [twenty one] | RW | ptb21_ie | 0x1 | PTB21 input enable 1 : enable valid 0 : Enable off |
| [20] | RW | ptb20_ie | 0x1 | PTB20 input enable 1 : enable valid 0 : Enable off |
| [19] | RW | ptb19_ie | 0x1 | PTB19 input enable 1 : enable valid 0 : Enable off |
| [18] | RW | ptb18_ie | 0x1 | PTB18 input enable 1 : enable valid 0 : Enable off |
| [17] | RW | ptb17_ie | 0x1 | PTB17 input enable 1 : enable valid 0 : Enable off |

| [16] | RW | ptb16_ie | 0x1 | PTB16 input enable 1 : enable valid 0 : Enable off |
| --- | --- | --- | --- | --- |
| [15] | RW | ptb15_ie | 0x1 | PTB15 input enable 1 : enable valid 0 : Enable off |
| [14] | RW | ptb14_ie | 0x1 | PTB14 input enable 1 : enable valid 0 : Enable off |
| [13] | RW | ptb13_ie | 0x1 | PTB13 input enable 1 : enable valid 0 : Enable off |
| [12] | RW | ptb12_ie | 0x1 | PTB12 input enable 1 : enable valid 0 : Enable off |
| [11] | RW | ptb11_ie | 0x1 | PTB11 input enable 1 : enable valid 0 : Enable off |
| [10] | RW | ptb10_ie | 0x1 | PTB10 input enable 1 : enable valid 0 : Enable off |
| [9] | RW | ptb9_ie | 0x1 | PTB9 input enable 1 : enable valid 0 : Enable off |
| [8] | RW | ptb8_ie | 0x1 | PTB8 input enable 1 : enable valid 0 : Enable off |
| [7] | RW | ptb7_ie | 0x1 | PTB7 input enable 1 : enable valid 0 : Enable off |
| [6] | RW | ptb6_ie | 0x1 | PTB6 input enable 1 : enable valid 0 : Enable off |
| [5] | RW | ptb5_ie | 0x1 | PTB5 input enable 1 : enable valid 0 : Enable off |
| [4] | RW | ptb4_ie | 0x1 | PTB4 input enable 1 : enable valid 0 : Enable off |
| [3] | RW | ptb3_ie | 0x1 | PTB3 input enable 1 : enable valid 0 : Enable off |
| [2] | RW | ptb2_ie | 0x1 | PTB2 input enable |

|  |  |  |  | 1 : Enable valid 0 : Enable off |
| --- | --- | --- | --- | --- |
| [1] | RW | ptb1_ie | 0x1 | PTB1 input enable 1 : enable valid 0 : Enable off |
| [0] | RW | ptb0_ie | 0x1 | PTB0 input enable 1 : enable valid 0 : Enable off |

#### Processor subsystem

###### Overview

HR_C7000 processor uses Zhongtian CK803S , 32 -bit address and data path, AHB interface, and a maximum operating frequency of 192M . It is a 32-bit control-oriented High-performance embedded CPU Core, with low cost, low power consumption and high code density . Using 16/32 Bit mixed encoding instruction system, with streamlined and efficient 3 The system also integrates on-chip RAM for CPU use, with an SRAM capacity of 288KB . The overall framework is shown below:

| SPI Flash |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| JTAG | CPU_SYS ROM RAM 2KB 288KB LCSFC IAHB iahb |  |  |  |  |  |
|  |  | CK803S | Icache 4KB |  |  |  |

SYS_AHB

Figure 17 CPU System overall block diagram

###### Features

- Reduced Instruction Set Processor Architecture ( RISC )

- 32 bit data, 16/32 Bit-mixed encoding instructions

- 16 32​ bit general register

- Integrated 4KB Instruction Cache

- 3 Stage pipeline

- Launch in sequence, execute in sequence, retire in sequence

- Configurable multi-bus interface

- Supports multiple processor clock and system clock ratios

###### Address mapping relationship

HR_C7000 The address mapping relationship is as follows:

Table 26 Address mapping relationship

| Starting address | End address | Function | size | illustrate |
| --- | --- | --- | --- | --- |
| 0x0000_0000 | 0x0000_FFFF | Boot ROM storage space | 2KB |  |
| 0x0001_0000 | 0x0005_7FFF | IRAM storage space | 288KB |  |
| 0x0300_0000 | 0x03FF_FFFF | LCSFC Address Space | 16MB | Flash runtime |
| 0x1100_0000 | 0x1100_FFFF | Modem | 64KB |  |
| 0x1200_0000 | 0x1200_FFFF | i8080 | 64KB |  |
| 0x1300_0000 | 0x1303_FFFF | USB | 256KB |  |
| 0x1400_0000 | 0x1400_FFFF | TIMER | 64KB |  |
| 0x1401_0000 | 0x1401_FFFF | WDG | 64KB |  |
| 0x1402_0000 | 0x1402_FFFF | GPIOA | 64KB |  |
| 0x1403_0000 | 0x1403_FFFF | UART0 | 64KB |  |
| 0x1404_0000 | 0x1404_FFFF | UART1 | 64KB |  |
| 0x1405_0000 | 0x1405_FFFF | UART2 | 64KB |  |
| 0x1406_0000 | 0x1406_FFFF | I2C0 | 64KB |  |
| 0x1407_0000 | 0x1407_FFFF | I2C1 | 64KB |  |
| 0x1408_0000 | 0x1408_FFFF | I2C2 | 64KB | Internal RTC dedicated |
| 0x1409_0000 | 0x1409_FFFF | UART3 | 64KB |  |
| 0x140A_0000 | 0x140A_FFFF | SPI Master 0 | 64KB |  |
| 0x140B_0000 | 0x140B_FFFF | SPI Master 1 | 64KB |  |
| 0x140C_0000 | 0x140C_FFFF | PWM | 64KB |  |
| 0x140D_0000 | 0x140D_FFFF | ADC | 64KB |  |
| 0x140E_0000 | 0x140E_FFFF | SPI2 | 64KB |  |
| 0x140F_0000 | 0x140F_FFFF | DAC | 64KB |  |
| 0x1410_0000 | 0x1410_FFFF | GPIOB | 64KB |  |
| 0x1411_0000 | 0x1411_FFFF | GPIOC | 64KB |  |
| 0x1412_0000 | 0x1412_FFFF | SPI Slave 0 | 64KB |  |
| 0x1413_0000 | 0x1413_FFFF | SPI Slave 1 | 64KB |  |
| 0x1414_0000 | 0x1414_FFFF | SPI Slave 2 | 64KB |  |
| 0x1415_0000 | 0x1415_FFFF | SPI Master 3 | 64KB | EFUSE only |
| 0x1500_0000 | 0x1500_FFFF | SDIO | 64KB |  |
| 0x1600_0000 | 0x1600_FFFF | Modem_Buffer | 64KB |  |
| 0x1700_0000 | 0x1700_FFFF | PIC | 64KB |  |
| 0x1800_0000 | 0x1800_7FFF | SRAM | 32KB | SRAM on SAHB |

###### start up

In normal working mode, the CPU Supports two system startup methods:

- Serial port debugging mode: After reset, CPU Determine whether there is a program loaded from the serial port. If so, load the program into the memory. Otherwise, wait until the timeout and enter the Flash boot mode.

- Flash Startup: Users only need to burn the pre-set program into Flash After reset and startup program enters Flash Mode, CPU Verify Flash The effectiveness of the program . After the valid program in Instructions will be fetched from Flash through LCSFC for execution.

The system startup flow chart is shown below:

是

下载命

令？

读取跳转地

址(0x04)

否

跳转命令？

否

是

退出命

令？

执行写命令，并依据执行情况应答

调试模式，等待命令

跳转 执行程序

跳转 执行程序

执行下载命令，并依据执行情况应答

执行读命令，并依据执行情况应答

是

是否有主机回应？

发送串口信息

初始化硬件

是

是

否

是

写命令？

是

否

是

检查Flash0地址是否为

DH_FLAG

否

否

读命令？

否

上电复位

Figure 18 System startup flow chart

###### JTAG debug

CK803S CPU JTAG support Debugging. CK803S JTAG The main features of the debug interface are as follows:

- Uses standard JTAG Protocol debugging

- Non-invasive CPU acquisition state

- Support soft breakpoints, 8 Hardware interrupt

- Multiple memory breakpoints can be set

- Checking and Setting the CPU Register value

- Checking and changing memory values

- Instructions can be executed in single or multi-step

- Quick Downloader

- Available on CPU Enter debug mode after reset or in normal user mode

CK803S Debugging is accomplished through the collaboration of debugging software, a debugging agent, a debugger, and a debugging interface. The following figure shows the location of the debugging interface within the debugging environment. The debugging software and debugging agent are connected via a network , the debugging agent is connected to the debugger via USB , and the debugger is connected to the CPU 's debugging interface via JTAG .

Figure 19 CK803 JTAG debug

#### Baseband communication system

###### Overview

The baseband communication system mainly involves baseband working mode, RF Mode and settings of simulation related modules.

###### Features

- DMR Physical layer mode

- DMR Layer 2 direct mode

- DMR Layer 2 relay mode

- DMR Layer 3 direct mode

- DMR Layer 3 relay mode

- FM model

- DMR Layer 2 DCDM model

- DMR/FM Digital and analog simultaneous detection mode

- Support multiple RF Interface Mode

- Analog modules DAC , ADC , Code Control Settings

Table 27 Baseband operating mode list

| sequence | Working Mode | Register Name | Register configuration value | describe |
| --- | --- | --- | --- | --- |

| Number |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | DMR Physical layer mode | WORK_MODE | 0x22 | Time slot mode |
|  |  |  | 0x02 | Continuous mode |
| 2 | DMR Layer 2 direct mode | WORK_MODE | 0x6a or 0x6b |  |
| 3 | DMR Layer 2 relay mode | WORK_MODE | 0x6e | Time Slot 2 |
|  |  |  | 0x6f | Time Slot 1 |
| 4 | DMR Layer 3 direct mode | WORK_MODE | 0x73 |  |
| 5 | DMR Layer 3 relay mode | WORK_MODE | 0x76 | Time Slot 2 |
|  |  |  | 0x77 | Time Slot 1 |
| 6 | FM model | WORK_MODE | 0x80 |  |
| 7 | DMR Layer 2 DCDM model | WORK_MODE | 0x6a | TDMA2 Time Slot |
|  |  |  | 0x6b | TDMA1 Time Slot |
| 8 | DMR/FM Digital and analog simultaneous detection mode | WORK_MODE | 0x16a or 0x16b | Layer 2 digital and analog simultaneous detection |

###### Interrupt vector

The following table defines the interrupt vector list of the baseband communication system.

Table 28 Baseband communication system interrupt list

| Serial number | name | type | describe |
| --- | --- | --- | --- |
| 1 | RF_TX_INTERP | Level trigger, high effective | RF switching interrupt, RF Sending start interrupt, through INTERP_CLEAR Register corresponding bit Bit write 1 Clear |
| 2 | RF_RX_INTERP | Level trigger, high effective | RF switching interrupt, RF Receive start interrupt, through INTERP_CLEAR Register corresponding bit Bit write 1 Clear |
| 3 | SYS_INTERP | Level trigger, high effective | Baseband system interrupt, sub-interrupt see the table below, through SYS_INTERP_CLEAR Write 1 to the corresponding bit of the register Clear |
| 4 | TIME_SLOT_TX_INTERP | Level trigger, high effective | Baseband digital 30ms Send time slot interrupt, through INTERP_CLEAR Register corresponding bit Bit write 1 Clear |
| 5 | TIME_SLOT_RX_INTERP | Level trigger, high effective | Baseband digital 30ms Receive time slot interrupt, through INTERP_CLEAR Register corresponding bit Bit write 1 Clear |
| 6 | PCM_WR_INTERP | Level trigger, high effective | Codec Audio data stream write interrupt, through INTERP_CLEAR Register corresponding bit Bit write 1 Clear |
| 7 | PCM_RD_INTERP | Level trigger, high effective | Codec Audio data stream reading interrupt, through INTERP_CLEAR Register corresponding bit Bit write 1 Clear |

The following table defines the system sub-interrupt vector list of the baseband system interrupt ( SYS_INTERP) .

Table 29 Baseband communication system sub-interrupt list

| Serial number | name | type | describe |
| --- | --- | --- | --- |
| 1 | DLL_RDY_INTERP_TX | Level trigger, high effective | Layer 2 sending processing interruption: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; |

|  |  |  | Via SYS_INTERP_CLEAR Register corresponding bit Bit Write 1 Clear. |
| --- | --- | --- | --- |
| 2 | DLL_RDY_INTERP_RX | Level trigger, high effective | Layer 2 receive processing interruption: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Via SYS_INTERP_CLEAR Register corresponding bit Bit Write 1 Clear. |
| 3 | DLL_LATELC_RX_INTERP | Level trigger, high effective | Layer 2 voice access interruption: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Through SYS_INTERP_CLEA Register corresponding bit Bit write 1 Clear. |
| 4 | DLL_FRAME_RX_INTERP | Level trigger, high effective | Interruption of receiving various types of frame information on Layer 2: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Through SYS_INTERP_CLEA Register corresponding bit Bit write 1 Clear. |
| 5 | DLL_SHORTLC_RX_INTERP | Level trigger, high effective | Second floor shortlc Information reception interruption: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Through SYS_INTERP_CLEA Register corresponding bit Bit write 1 Clear. |
| 6 | DLL_TX_SLOT_ABNORMAL | Level trigger, high effective | Abnormal interruption of the Layer 2 sending timeslot: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Through SYS_INTERP_CLEA Register corresponding bit Bit write 1 Clear. |
| 7 | DLL_RX_SLOT_ABNORMAL | Level trigger, high effective | Abnormal interruption of the Layer 2 receiving timeslot: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; corresponding bit by writing 1 to the SYS_INTERP_CLEA register . |
| 8 | PHY_RECV_ABNORMAL | Level trigger, high effective | The physical layer receives an abnormal time slot boundary and prompts an interruption: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Through SYS_INTERP_CLEA Register corresponding bit Bit write 1 Clear. |
| 9 | PHY_RDY_INTERP | Level trigger, high effective | Physical layer processing interrupt: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Via SYS_INTERP_CLEAR Register corresponding bit Bit Write 1 Clear. |
| 10 | PHY_RX_INTERP | Level trigger, high effective | Physical layer reception deframing interruption: Via SYS_INTERP_MASK Corresponding bit of the register |

|  |  |  | bit masking; Via SYS_INTERP_CLEAR Register corresponding bit Bit Write 1 Clear. |
| --- | --- | --- | --- |
| 11 | FM_TX_INTERP | Level trigger, high effective | FM Send data processing interrupt: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Via SYS_INTERP_CLEAR Register corresponding bit Bit Write 1 Clear. |
| 12 | FM_RX_INTERP | Level trigger, high effective | FM Receive data processing interrupt: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Via SYS_INTERP_CLEAR Register corresponding bit Bit Write 1 Clear. |
| 13 | FM_SIG_RX_INTERP | Level trigger, high effective | Simulate function receive detect interrupt: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Through SYS_INTERP_CLEA Register corresponding bit Bit write 1 Clear. |
| 14 | RDY_NEXT_SHORT_LC | Level trigger, high effective | The repeater sends CACH shortlc​ Information ready for interrupt via SYS_INTERP_MASK The corresponding bit mask of the register ; Through SYS_INTERP_CLEA Register corresponding bit Bit write 1 Clear. |

###### Register Overview

Register Table 4 Overview of Baseband System Control Registers (Base Address: 0x11000000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x70 | DAC_CONTROL | Baseband DAC Control registers |  |
| 0x74 | ADC_CONTROL | Baseband ADC Control registers |  |
| 0x80 | AUDIO_CONTROL | Baseband audio path control register |  |
| 0x84 | AUDIO_BUFFER_CLR | Baseband audio path buffer clear control register |  |
| 0x88 | LINEOUT_CTRL | Codec Lineout Output Control Register |  |
| 0x8c | CODEC_I2C_MUX | CodecI2C Control registers |  |
| 0x100 | WORK_MODE | Baseband working mode setting |  |
| 0x104 | RF_MODE | Baseband RF interface mode setting |  |
| 0x398 | SYS_INTERP_LIST | System interrupt vector list |  |
| 0x39c | SYS_INTERP_MASK | System interrupt vector mask |  |
| 0x3a0 | SYS_INTERP_CLEAR | System interrupt vector clear |  |
| 0x3a4 | LAYER3_INTERP_LIST | Three-level interrupt vector list |  |
| 0x3a8 | LAYER3_INTERP_MASK | Three-level interrupt vector mask |  |

| 0x3ac | LAYER3_INTERP_CLEAR | Three-level interrupt vector clear |  |
| --- | --- | --- | --- |
| 0x3b0 | INTERP_CLEAR | 30ms Interrupt, rf Interrupt, pcm Interrupt clear |  |

###### Register Description

      - DAC_CONTROL

Offset address: 0x70 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RW | dac_data_ctrl | 0x1 | DAC data mode of baseband modulation output : 0 : Complement code mode; 1 : Offset code mode. |
| [30:6] | - | reserved |  |  |
| [5] | RW | pwda | 0x1 | A- channel DAC power control, 0 : power-on ; 1 : power- down |
| [4] | RW | pwdb | 0x1 | B -channel DAC power control, 0 : power-on ; 1 : power- down |
| [3] | - | reserved |  |  |
| [2] | RW | lpmodea | 0x1 | A- channel DAC low power mode, 0 : Turn off low power mode; 1 : Turn on low power mode |
| [1] | RW | lpmodeb | 0x1 | B -channel DAC low power mode, 0 : Turn off low power mode; 1 : Turn on low power mode |
| [0] | - | reserved |  |  |

      - ADC_CONTROL

Offset address: 0x74 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:30] | RO | adc_testout | 0x0 | Test output status |
| [29:28] | - | reserved |  |  |
| [27:24] | RW | adc_testin | 0x0 | Test input control |
| [23:15] | - | reserved |  |  |
| [14] | RW | adc_data_ctrl | 0x1 | ADC data mode: 0 : Complement code mode; 1 : Offset code mode. |
| [13:10] | RW | buffer_trim | 0x0 |  |
| [9] | RW | intv_mode | 0x0 |  |
| [8] | RW | enadc0 | 0x0 | I- channel ADC function is enabled. ADC The I channel output data is valid after enadc0 is high and lasts for the wake-up time (usually 120 working clock cycles). enadc0 is set to 0 , ADC I channel output data will |

|  |  |  |  | The last status value is maintained. |
| --- | --- | --- | --- | --- |
| [7] | RW | enadc1 | 0x0 | Q channel ADC function is enabled. The ADC Q channel output data is valid after enadc1 is high and lasts for the wake-up time (usually 120 working clock cycles). When enadc1 is set to 0 , the ADC Q channel output data will The last status value is maintained. |
| [6] | RW | adc_enref | 0x0 | Reference level enable. 0 : off ; 1 : on |
| [5:2] | RW | adc_bgtrim | 0x0 | band-gap Trim configuration value: When using the internal reference ( enref=1 ), it is used to calibrate the level difference between the bandgap voltage and VREF . The default value of Bgtrim is ' 0XXX '. Usually 8 calibrations can adjust the difference between the bang-gap and VREF from +-4% to +-1% ( bgtrim value ' 1000 ' to ' 1111 ') . |
| [1] | RW | adc_extcm | 0x0 | Common-mode voltage control: 1 : Common mode voltage is externally input VCM0/VCM1 Sure 0 : Common mode voltage uses internal reference, ( VINP0+VINM0)/2 , (VINP1+VINM1)/2 |
| [0] | RW | adc_semode | 0x1 | ADC input mode is optional: 0 : VINP/VINM is differential input; 1 : VINP/VINM are single-ended inputs, where VINP is connected to the signal input source and VINM is connected in two ways : In AF mode, VINM and VCM are connected to a fixed level ( AF bias); in IF mode, VINM and VCM to GND together . |

      - AUDIO_CONTROL

Offset address: 0x80 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RO | ring_full_two | 0x0 | The prompt tone PCM data is written to the storage empty and full flag bit. When the bit is 1 , it means that the fifo is full and stops sending data to the fifo until the bit is 0. Only then can data be sent. |
| [30:8] | - | reserved |  |  |
| [7] | RW | ring_play_ctrl | 0x0 | Select the prompt sound path, high effective |
| [6] | RW | fm_play_ctrl | 0x0 | Select FM playback channel, high effective |
| [5] | RW | ahb_rd_voice_en | 0x0 | AHB read audio enable is turned on, high effective; |
| [4] | RW | ahb_wr_voice_en | 0x0 | ahb write audio start flag, high effective; |
| [3] | RW | fm_sidetone_en | 0x0 | Analog side tone playback is enabled, high effective; |
| [2] | RW | sel_i2s | 0x0 | I2S interface enabled, used in baseband test mode |
| [1] | RW | i2s_slave_mode | 0x1 | Audio I2S interface master and slave mode settings, 0 master mode, 1 Used in slave mode and baseband test mode |

| [0] | RW | tx_voice_source | 0x0 | Voice frame sending (receiving also needs to be configured) audio source selection: 0 : Indicates that the voice data comes from a Codec or vocoder (Apply with conventional handheld); 1 : Indicates that the voice data comes from the RAM space corresponding to the MCU ( applicable to the second-layer BS sending mode) |
| --- | --- | --- | --- | --- |

      - AUDIO_BUFFER_CLR

Offset address: 0x84 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [30:2] | - | reserved |  |  |
| [1] | WC | clr_rd_ram_cpu | 0x0 | Clear the data in the vocoder decoding buffer . Enable, when this bit is 1 , clear the value in the buffer , and then the system automatically returns this bit to 0 |
| [0] | WC | clr_wr_ram_cpu | 0x0 | Clear the data in the vocoder encoding buffer Enable, when this bit is 1 , clear the value in the buffer . Then the system automatically replies that the bit is 0 |

      - LINEOUT_CTRL

Offset address: 0x88 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RO | standby_lo | 0x1 | Standby of the mercury DAC |
| [30:2] | - | reserved |  |  |
| [1] | RW | line1out_en | 0x0 | Codec Line1out output enable, high effective |
| [0] | RW | line2out_en | 0x0 | Codec Line2out output enable, high effective |

      - CODEC_I2C_MUX

Offset address: 0x8c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:8] | - | reserved |  |  |
| [7] | RW | codec_si2c_reg | 0x0 | Codec IP register configuration interface selection: 0 : MC interface; 1 : I2C interface. |
| [6:0] | RW | codec_i2c_address_reg | 0x0 | Codec IP I2C interface device address |

      - WORK_MODE

Offset address: 0x100 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:9] | - | reserved |  |  |
| [8] | RW | digital_analog_en | 0x0 | The digital and analog simultaneous detection switch is required to be turned on in the default configuration digital mode. After receiving a digital signal or an analog signal, it switches to the digital mode or the analog mode accordingly. 1 : Indicates that the simultaneous digital and analog reception control is enabled; 0 : Indicates that only digital or analog reception is enabled. According to Bit7 setting. |
| [7] | RW | modulator_mode | 0x0 | Working mode settings: 0 : Indicates DMR digital mode; 1 : Indicates FM analog mode. |
| [6] | RW | is_tier1_mode | 0x1 | 0 : indicates Tier I mode; 1 : Indicates Tier II mode. |
| [5] | RW | is_continuemode | 0x1 | 0 : Indicates Continue mode; 1 : Indicates TimeSlot mode. |
| [4:3] | RW | layermode | 0x1 | Hierarchy mode settings: 0 : indicates physical layer mode; 1 : indicates Layer 2 mode; 2 : Indicates three-layer mode. |
| [2] | RW | is_repeater | 0x0 | 0 : indicates non-relay mode; 1 : Indicates relay mode. |
| [1] | RW | is_aligned | 0x1 | 0 : indicates offset mode (offset in non-relay mode indicates single-frequency mode); 1 : Indicates alignment mode. |
| [0] | RW | work_slot_sel | 0x1 | The Layer 3 non-relay mode must be set to 1 ; 0 : indicates time slot 1 ; 1 : Indicates time slot 2 . |

      - RF_MODE

Offset address: 0x104 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RW | reserved | 0x0 | In the physical layer receiving test mode, it is required to be configured as 1 . Normal mode set to 0 |
| [30] | RW | reserved | 0x0 |  |
| [29] | RW | reserved | 0x0 | In the physical layer receiving test mode, it is required to be configured as 1 . Normal mode set to 0 |
| [28:26] | - | reserved |  |  |
| [25] | RW | phase_avg_en | 0x0 | AF mode average calculation switch, high effective |
| [twenty four] | RW | af_mode | 0x0 | AF mode settings : 0 : AF signal reception disabled; 1 : AF signal reception enabled. |

| [23:16] | RW | af_recv_ctrl | 0x0 | AF input signal amplitude gain control. Signal amplitude = input signal amplitude * af_recv_ctrl/16 |
| --- | --- | --- | --- | --- |
| [15] | RW | rf_recviq_mode | 0x0 | PHY demodulation reception and actual RF reception ( confirmed by the actual RF reception solution): 0 : means unchanged; 1 : Indicates the opposite. |
| [14:13] | RW | rf_recv_mode | 0x0 | RF receiving mode: 0x0 : indicates intermediate frequency receiving mode; 0x1 : indicates intermediate frequency IQ receiving mode; 0x2 : Indicates baseband IQ reception mode. |
| [12] | RW | Adjustsyncmax | 0x1 | Control and adjust the synchronization head detection threshold enable: 0 : indicates a fixed synchronization detection threshold setting value; 1 : Indicates automatic dynamic synchronization detection threshold. Note: Required to be set to 0 when cc_opt of the LOCAL_CC register is set to 1 . |
| [11] | RW | use_dual_phy | 0x0 | Turn on the two receiver channel control switches in time slots : 0 : Disable dual receiving channels; 1 : Enable dual receiving channels. (Applicable to relay chip receiving mode and DCDM mode) |
| [10] | RW | two_point_ctr | 0x0 | Two-point modulation test enable: 0 : means off; 1 : Indicates output of 40Hz sine wave. |
| [9] | RW | up_ctr | 0x0 | smooth rise and fall of signal strength at timeslot boundaries : 0 : means off; 1 : Enabled. |
| [8] | RW | iq_just_debug | 0x0 | IQ two-way balance debugging test enable: 0 : means unchanged; 1 : Indicates adding an offset value. |
| [7] | RW | rf_transiq_mode | 0x1 | PHY modulation transmission and actual RF transmission (confirmed by the actual RF transmission scheme): 0 : means unchanged; 1 : Indicates the opposite. |
| [6:5] | RW | rf_trans_mode | 0x3 | RF transmission mode: 0x0 : indicates intermediate frequency transmission mode; 0x1 : indicates intermediate frequency IQ transmission mode; 0x2 : indicates baseband IQ transmission mode; 0x3 : Indicates two-point modulation transmission mode. |
| [4] | RW | ak2401_mode | 0x0 | AK2401 mode setting: 0 : AK2401 mode off; 1 : AK2401 mode is on. |
| [3:0] | RW | ak_adc_shift | 0x0 | AK2401 mode input ADC signal shift control Control register |

      - SYS_INTERP_LIST

Offset address: 0x398 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:18] | - | reserved |  |  |
| [17] | RO | rdy_next_short_lc | 0x0 | for the repeater to send shortlc information in CACH Interrupt |
| [16] | RO | phy_recv_slot_abnorma l | 0x0 | In the physical layer mode, the receiving time slot boundary abnormality prompt is in progress Break |
| [15] | RO | fm_tx_interp | 0x0 | Simulate data sending interruption |
| [14] | RO | fm_rx_interp | 0x0 | Simulates receive data interruption |
| [13] | RO | fm_sig_rx_interp | 0x0 | Simulate function receive detect interrupt |
| [12] | RO | phy_rdy_interp | 0x0 | Early interruption of time slot in physical layer mode |
| [11] | RO | phy_rx_interp | 0x0 | Physical layer data reception interruption for bit error rate testing |
| [10] | RO | ccl_rx_interp | 0x0 | Layer 3 receiving status interrupt |
| [9] | RO | ccl_tx_interp | 0x0 | Layer 3 sending status interruption |
| [8] | RO | gps_slot_abnormal | 0x0 | Abnormal interruption at the simulcast slot boundary |
| [7] | RO | gps_lost_interp | 0x0 | Simulcast GPS pulse loss interruption |
| [6] | RO | dll_rx_slot_abnormal | 0x0 | Abnormal interruption of Layer 2 receiving timeslot |
| [5] | RO | dll_tx_slot_abnormal | 0x0 | Abnormal interruption of Layer 2 sending timeslot |
| [4] | RO | dll_shortlc_rx_interp | 0x0 | Layer 2 shortlc information reception interruption |
| [3] | RO | dll_frame_rx_interp | 0x0 | Interruption of receiving various types of layer 2 frame information |
| [2] | RO | dll_latelc_rx_interp | 0x0 | Layer 2 voice access interruption |
| [1] | RO | dll_rdy_interp_rx | 0x0 | Layer 2 receive processing interruption |
| [0] | RO | dll_rdy_interp_tx | 0x0 | Layer 2 send processing interruption |

      - SYS_INTERP_MASK

Offset address: 0x39c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:18] | - | reserved |  |  |
| [17] | RO | rdy_next_short_lc_mas k | 0x1 | the shortlc information preparation interrupt sent by the repeater in CACH : 0 : shield interrupt; 1 : Enable interrupt. |
| [16] | RW | phy_recv_slot_abnorma l_mask | 0x1 | In the physical layer mode, the abnormality of the receiving time slot boundary prompts interruption : 0 : shield interrupt; 1 : Enable interrupt. |
| [15] | RW | fm_tx_interp_mask | 0x1 | Whether to shield the interruption of analog data transmission: |

|  |  |  |  | 0 : shield interrupt; 1 : Enable interrupt. |
| --- | --- | --- | --- | --- |
| [14] | RW | fm_rx_interp_mask | 0x1 | Whether to shield the interruption of analog receiving data: 0 : shield interrupt; 1 : Enable interrupt. |
| [13] | RW | fm_sig_rx_interp_mask | 0x1 | Whether the analog function reception detection interrupt is masked: 0 : shield interrupt; 1 : Enable interrupt. |
| [12] | RW | phy_rdy_interp_mask | 0x1 | Whether to shield the time slot early interruption in physical layer mode: 0 : shield interrupt; 1 : Enable interrupt. |
| [11] | RW | phy_rx_interp_mask | 0x1 | Whether the physical layer data reception interrupt is masked: 0 : shield interrupt; 1 : Enable interrupt. |
| [10] | RW | ccl_rx_interp_mask | 0x1 | Whether the third-layer receiving status interrupt is shielded: 0 : shield interrupt; 1 : Enable interrupt. |
| [9] | RW | ccl_tx_interp_mask | 0x1 | Whether the third layer sending status interrupt is shielded: 0 : shield interrupt; 1 : Enable interrupt. |
| [8] | RW | gps_slot_abnormal_mas k | 0x1 | Whether to shield abnormal interruption at simulcast time slot boundary: 0 : shield interrupt; 1 : Enable interrupt. |
| [7] | RW | gps_lost_interp_mask | 0x1 | the simulcast GPS pulse loss interruption: 0 : shield interrupt; 1 : Enable interrupt. |
| [6] | RW | dll_rx_slot_abnormal_mask​ | 0x1 | Whether to shield abnormal interruption of Layer 2 receiving timeslot: 0 : shield interrupt; 1 : Enable interrupt. |
| [5] | RW | dll_tx_slot_abnormal_mask​ | 0x1 | Whether to shield abnormal interruption of Layer 2 sending timeslot: 0 : shield interrupt; 1 : Enable interrupt. |
| [4] | RW | dll_shortlc_rx_interp_m ask | 0x1 | the Layer 2 shortlc information reception interrupt is masked: 0 : shield interrupt; 1 : Enable interrupt. |
| [3] | RW | dll_frame_rx_interp_mask​ | 0x1 | Whether to shield the interruption of receiving various types of frame information on the second layer: 0 : shield interrupt; 1 : Enable interrupt. |
| [2] | RW | dll_latelc_rx_interp_mask​ | 0x1 | Whether to block the access interrupt after the second layer voice: 0 : shield interrupt; 1 : Enable interrupt. |
| [1] | RW | dll_rdy_interp_rx_mask | 0x1 | Whether the second layer receive processing interrupt is shielded: 0 : shield interrupt; |

|  |  |  |  | 1 : Enable interrupt. |
| --- | --- | --- | --- | --- |
| [0] | RW | dll_rdy_interp_tx_mask | 0x1 | Whether the second layer sending processing interrupt is shielded: 0 : shield interrupt; 1 : Enable interrupt. |

      - SYS_INTERP_CLEAR

Offset address: 0x3a0 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:17] | - | reserved |  |  |
| [16:0] | RW | clear_sys_interp | 0x0 | Write 1 to automatically clear the system interrupt vector list |

      - LAYER3_INTERP_LIST

Offset address: 0x3a4 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RO | ccl_latelc_rx_inter p | 0x0 | Layer 3 voice access interruption |
| [30] | RO | ccl_frame_rx_inter p | 0x0 | Interruption of receiving various types of frame information at layer 3 |
| [29] | RO | ccl_shortlc_rx_inte rp | 0x0 | Layer 3 shortlc information reception interruption |
| [28] | RO | ccl_rx_voice_abno rmal | 0x0 | Abnormal interruption of Layer 3 voice reception |
| [27] | RO | ccl_rx_data_ok | 0x0 | Layer 3 SMS reception CRC32 check OK interrupt |
| [26] | RO | ccl_rx_data_error | 0x0 | Layer 3 SMS receiving CRC32 check error interrupt |
| [25] | RO | ccl_rx_data_abnor mal | 0x0 | Abnormal interruption of receiving layer 3 unconfirmed SMS |
| [24:15] | - | reserved |  |  |
| [14] | RO | tx_complete_data_ confirmed | 0x0 | The third-layer SMS service has completed sending the last frame and needs to confirm the waiting feedback interrupt |
| [13] | RO | tx_complete_data | 0x0 | The third-layer SMS service has completed sending the last frame. No confirmation is required and waiting for feedback. Break |
| [12] | RO | tx_data_finish | 0x0 | Layer 3 SMS service sending is interrupted |
| [11] | RO | tx_data_allretry_st art | 0x0 | All third-layer SMS messages are resent and interrupted |
| [10] | RO | tx_data_retry_start | 0x0 | Partial resending of Layer 3 SMS started interrupted |
| [9] | RO | tx_data_start | 0x0 | Layer 3 SMS sending starts to be interrupted |
| [8] | RO | tx_v_embf_update | 0x0 | EMB area update of the F frame sent by Layer 3 voice is interrupted |
| [7] | RO | tx_v_key_update | 0x0 | Layer 3 voice transmission key update interruption |
| [6] | RO | tx_v_emb_update | 0x0 | Layer 3 voice transmission EMB area update interruption |

| [5] | RO | tx_voice_finish | 0x0 | Layer 3 voice transmission end interrupt |
| --- | --- | --- | --- | --- |
| [4] | RO | tx_voice_start | 0x0 | Layer 3 voice transmission begins to be interrupted |
| [3] | RO | tx_oacsu_req | 0x0 | Layer 3 OACSU request call interruption |
| [2] | RO | tx_bs_overtime | 0x0 | Layer 3 BS activation timeout interrupt |
| [1] | RO | tx_oacsu_overtime | 0x0 | Layer 3 OACSU call request timeout interrupt |
| [0] | RO | tx_denied | 0x0 | Layer 3 send request rejection interrupt |

      - LAYER3_INTERP_MASK

Offset address: 0x3a8 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RW | ccl_latelc_rx_inter p_mask | 0x1 | Whether to block the access interrupt after Layer 3 voice: 0 : shield interrupt; 1 : Enable interrupt. |
| [30] | RW | ccl_frame_rx_inter p_mask | 0x1 | Whether to shield the interruption of receiving various types of frame information at Layer 3: 0 : shield interrupt; 1 : Enable interrupt. |
| [29] | RW | ccl_shortlc_rx_inte rp_mask | 0x1 | the Layer 3 shortlc information reception interrupt is masked: 0 : shield interrupt; 1 : Enable interrupt. |
| [28] | RW | ccl_rx_voice_abno rmal_mask | 0x1 | Whether to shield abnormal interruption of Layer 3 voice reception: 0 : shield interrupt; 1 : Enable interrupt. |
| [27] | RW | ccl_rx_data_ok_m ask | 0x1 | of receiving the CRC32 check OK for the third-layer SMS message blocked? 0 : shield interrupt; 1 : Enable interrupt. |
| [26] | RW | ccl_rx_data_error_mask​ | 0x1 | CRC32 check Error when receiving Layer 3 SMS message disabled? 0 : shield interrupt; 1 : Enable interrupt. |
| [25] | RW | ccl_rx_data_abnor mal_mask | 0x1 | Whether to shield abnormal interruption of receiving layer 3 non-confirmed SMS: 0 : shield interrupt; 1 : Enable interrupt. |
| [24:15] | - | reserved |  |  |
| [14] | RW | tx_complete_data_confirmed_mask | 0x1 | After the last frame of the Layer 3 SMS service is sent, you need to confirm whether the waiting feedback interrupt is disabled: 0 : shield interrupt; 1 : Enable interrupt. |
| [13] | RW | tx_complete_data_mask​ | 0x1 | The Layer 3 SMS service has completed sending the last frame, and there is no need to confirm whether the waiting feedback interrupt is blocked: 0 : shield interrupt; 1 : Enable interrupt. |
| [12] | RW | tx_data_finish_mas k | 0x1 | Whether to block the interruption of the third-layer SMS service sending end: 0 : shield interrupt; |

|  |  |  |  | 1 : Enable interrupt. |
| --- | --- | --- | --- | --- |
| [11] | RW | tx_data_allretry_start_mask | 0x1 | Whether to block the interruption of resending all the third-layer SMS messages: 0 : shield interrupt; 1 : Enable interrupt. |
| [10] | RW | tx_data_retry_start _mask | 0x1 | Whether to block the interruption of resending part of the third-layer SMS message: 0 : shield interrupt; 1 : Enable interrupt. |
| [9] | RW | tx_data_start_mask | 0x1 | Whether to block the interruption of the start of the third-layer SMS sending: 0 : shield interrupt; 1 : Enable interrupt. |
| [8] | RW | tx_v_embf_update _mask | 0x1 | EMB area update interruption of F frames sent by Layer 3 voice : 0 : shield interrupt; 1 : Enable interrupt. |
| [7] | RW | tx_v_key_update_mask​ | 0x1 | Whether to block the key update interruption of Layer 3 voice transmission: 0 : shield interrupt; 1 : Enable interrupt. |
| [6] | RW | tx_v_emb_update_mask​ | 0x1 | EMB area update interruption when sending Layer 3 voice : 0 : shield interrupt; 1 : Enable interrupt. |
| [5] | RW | tx_voice_finish_m ask | 0x1 | Whether to shield the interruption of the third-layer voice sending end: 0 : shield interrupt; 1 : Enable interrupt. |
| [4] | RW | tx_voice_start_mas k | 0x1 | Whether to block the interruption of the start of the third-layer voice transmission: 0 : shield interrupt; 1 : Enable interrupt. |
| [3] | RW | tx_oacsu_req_mas k | 0x1 | the Layer 3 OACSU call interruption request: 0 : shield interrupt; 1 : Enable interrupt. |
| [2] | RW | tx_bs_overtime_m ask | 0x1 | the Layer 3 BS activation timeout interrupt: 0 : shield interrupt; 1 : Enable interrupt. |
| [1] | RW | tx_oacsu_overtime _mask | 0x1 | the Layer 3 OACSU call request timeout interrupt: 0 : shield interrupt; 1 : Enable interrupt. |
| [0] | RW | tx_denied_mask | 0x1 | Whether to block the Layer 3 send request rejection interrupt: 0 : shield interrupt; 1 : Enable interrupt. |

      - LAYER3_INTERP_CLEAR

Offset address: 0x3ac Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | clear_layer3_interp | 0x0 | Write 1 to automatically clear the three-level interrupt vector list |

      - INTERP_CLEAR

Offset address: 0x3b0 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:6] | - | reserved |  |  |
| [5] | WC | clear_pcm_rd_int | 0x0 | Write 1 to automatically clear the PCM data read interrupt |
| [4] | WC | clear_pcm_wr_int | 0x0 | Writing 1 automatically clears the PCM data write interrupt |
| [3] | WC | clear_rf_rx_inter | 0x0 | Writing 1 automatically clears the RF reception start interrupt |
| [2] | WC | clear_rf_tx_inter | 0x0 | Write 1 to automatically clear the RF transmission start interrupt |
| [1] | WC | clear_time_rxslot_inte r | 0x0 | Write 1 to automatically clear the 30ms receive time slot interrupt |
| [0] | WC | clear_time_txslot_inte r | 0x0 | Write 1 to automatically clear the 30ms transmit time slot interrupt |

#### Interrupt system

###### Overview

The interrupt controller is a module used to manage internal / external abnormal events, interrupt the normal execution of the CPU program, and make the CPU enter the specified interrupt service function.

###### Functional Description

The interrupt controller has the following features:

- Support 64 interrupt sources;

- Support high level, low level trigger, support rising edge, falling edge trigger;

- Support configurable interrupt source priority;

- Support interrupt source masking;

The functional block diagram of the interrupt controller is shown in the figure:

IRQ[63:0]

Interrupt Request

PIC

Interrupt priority determination

INT_n

普通中断服务

快速中断服务

CPU

FINT_n

中断向量

VEC

Figure 20 Interrupt Controller Functional Block Diagram

###### Interrupt source assignment

Table 30 Interrupt source list

| Serial number | Interrupt Source | Default interrupt vector | Interrupt trigger mode |
| --- | --- | --- | --- |
| 0 | reserve | 32 |  |
| 1 | TIMER_1 | 33 | Level trigger, high effective |
| 2 | TIMER_2 | 34 | Level trigger, high effective |
| 3 | TIMER_3 | 35 | Level trigger, high effective |
| 4 | TIMER_4 | 36 | Level trigger, high effective |
| 5 | TIMER_5 | 37 | Level trigger, high effective |
| 6 | TIMER_6 | 38 | Level trigger, high effective |
| 7 | I2C1 | 39 | Level trigger, high effective |
| 8 | I2C0 | 40 | Level trigger, high effective |
| 9 | I2C2 | 41 | Level trigger, high effective |
| 10 | UART1 | 42 | Level trigger, high effective |
| 11 | UART2 | 43 | Level trigger, high effective |
| 12 | UART0 | 44 | Level trigger, high effective |
| 13 | UART3 | 45 | Level trigger, high effective |
| 14 | GPIOB | 46 | Level trigger, high effective |
| 15 | GPIOA | 47 | Level trigger, high effective |
| 16 | SPI0 | 48 | Level trigger, high effective |
| 17 | GPIOC | 49 | Level trigger, high effective |
| 18 | PWM | 50 | Level trigger, high effective |
| 19 | RTC | 51 | Level trigger, high effective |
| 20 | SDIO | 52 | Level trigger, high effective |
| twenty one | USB | 53 | Level trigger, high effective |
| twenty two | SPI1 | 54 | Level trigger, high effective |
| twenty three | SPI2 | 55 | Level trigger, high effective |
| twenty four | LCSFC | 56 | Level trigger, high effective |
| 25 | DAC | 57 | Level trigger, high effective |
| 26 | ADC | 58 | Level trigger, high effective |
| 27 | PCM_rd_data_interp | 59 | Level trigger, high effective |
| 28 | PCM_wr_data_interp | 60 | Level trigger, high effective |
| 29 | System_inter | 61 | Level trigger, high effective |
| 30 | Time_slot_inter_tx | 62 | Level trigger, high effective |
| 31 | Time_slot_inter_rx | 63 | Level trigger, high effective |
| 32 | Rf_tx_inter | 64 | Level trigger, high effective |
| 33 | Rf_rx_inter | 65 | Level trigger, high effective |
| 34 | SlaveSPI0 | 66 | Level trigger, high effective |
| 35 | SlaveSPI1 | 67 | Level trigger, high effective |
| 36 | SlaveSPI2 | 68 | Level trigger, high effective |
| 37 | Codec_interp | 69 | Level trigger, high effective |

###### How it works

##### initialization

The interrupt controller blocks all interrupt requests by default. Before use, you need to configure the interrupt controller registers. The steps are as follows:

- Configure the interrupt trigger mode:

By configuring PIC_MODE The trigger mode of the interrupt source can be set;

- Configure interrupt polarity:

By configuring PIC_PO The trigger polarity of the interrupt source can be set;

- Configure interrupt source priority, fast interrupt, etc.

  - Configuring PIC_PRIOR0-3 ~ PIC_PRIOR60-63 , indicating the initial value of each interrupt source priority counter. A smaller initial value indicates a higher priority. If not configured, the default priority is used;

  - Configure PIC_FFLAG_L , PIC_FFLAG_H You can configure any interrupt source as a fast interrupt. By default, all interrupts are normal interrupts. The C7000 only supports normal interrupts and does not support fast interrupts.

- Clear interrupt status (optional )

Configure PIC_INT_ST , PIC_INT_ST_1 Equal to 0xffffffff , clear the interrupt record registers PIC_INT_ST , PIC_INT_ST_1 , PIC_INT_ST_CNT .

- Enable interrupt detection (important )

Configure PIC_MASK , PIC_MASK_1 Corresponding bit 0 , enable interrupt source interrupt request detection function ;

##### Interrupt status query

By querying PIC_INT_ST , you can know which interrupt sources have issued interrupt requests. Each interrupt source can only record an interrupt request once. Writing 1 to the corresponding bit can clear the status.

###### Register Overview

Register Table 5 PIC Register Overview ( Base Address: 0x1700_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x00 | PIC_MODE | Low 32 Bit interrupt source trigger mode selection |  |
| 0x04 | PIC_PO | Low 32 Trigger polarity selection of bit interrupt source |  |
| 0x08 | PIC_MASK | Low 32 Is the interrupt source masked? |  |
| 0x0c | PIC_VECTOR | Interrupt vector base address |  |
| 0x10 | PIC_COW1 | End of interruption |  |
| 0x14 | PIC_PRIOR0-3​ | Interrupt sources 0-3 Corresponding counter initial value |  |
| 0x18 | PIC_PRIOR4-7​ | Interrupt sources 4-7 Corresponding counter initial value |  |
| 0x1c | PIC_PRIOR8- 11 | Interrupt sources 8-11 Corresponding counter initial value |  |
| 0x20 | PIC_PRIOR12- 15 | Interrupt sources 12-15 Corresponding counter initial value |  |
| 0x24 | PIC_PRIOR16- 19 | Interrupt sources 16-19 Corresponding counter initial value |  |
| 0x28 | PIC_PRIOR20- 23 | Interrupt sources 20-23 Corresponding counter initial value |  |
| 0x2c | PIC_PRIOR24- 27 | Interrupt sources 24-27 Corresponding counter initial value |  |
| 0x30 | PIC_PRIOR28-31​ | Interrupt sources 28-31 Corresponding counter initial value |  |
| 0x34 | PIC_COW2 | Control registers |  |
| 0x38 | PIC_SYNC | Asynchronous Process Control Register |  |
| 0x3c | PIC_FFLAG_L | Low 32 Bit fast interrupt setting |  |

| 0x40 | PIC_RECORD_SEL | Select an interrupt source to record the number of interrupts |  |
| --- | --- | --- | --- |
| 0x44 | PIC_INT_ST | 0-31 interrupt request records |  |
| 0x48 | PIC_INT_ST_1 | 32-63 Interrupt request logging |  |
| 0x4c | PIC_INT_CNT | Interrupt source interrupt count |  |
| 0x60 | PIC_MODE_1 | Height 32 Bit interrupt source trigger mode selection |  |
| 0x64 | PIC_PO_1 | Height 32 Trigger polarity selection of bit interrupt source |  |
| 0x68 | PIC_MASK_1 | Height 32 Is the interrupt source masked? |  |
| 0x6c | PIC_PRIOR32-35​ | Interrupt sources 32-35 Corresponding counter initial value |  |
| 0x70 | PIC_PRIOR36- 39 | Interrupt sources 36-39 Corresponding counter initial value |  |
| 0x74 | PIC_PRIOR40- 43 | Interrupt sources 40-43 Corresponding counter initial value |  |
| 0x78 | PIC_PRIOR44- 47 | Interrupt sources 44-47 Corresponding counter initial value |  |
| 0x7c | PIC_PRIOR48-51​ | Interrupt sources 48-51 Corresponding counter initial value |  |
| 0x80 | PIC_PRIOR52-55​ | Interrupt sources 52-55 Corresponding counter initial value |  |
| 0x84 | PIC_PRIOR56-59​ | Interrupt sources 56-59 Corresponding counter initial value |  |
| 0x88 | PIC_PRIOR60- 63 | Interrupt sources 60-63 Corresponding counter initial value |  |
| 0x8c | PIC_FFLAG_H | Height 32 Bit fast interrupt setting |  |

###### Register Description

      - PIC_MODE

Offset address: 0x00 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | R/W | edge_level | 0xffffffff | Each bit controls the triggering mode of the corresponding interrupt source, IRQ0 corresponds to bit 0 , IRQ31 corresponds to bit 31. 1 : edge triggered; 0 : Level trigger. |

      - PIC_PO

Offset address: 0x04 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | R/W | pol | 0xffffffff | Each bit controls the trigger polarity of the corresponding interrupt source, IRQ0 corresponds to bit 0 , IRQ31 corresponds to bit 31 . 1 : rising edge trigger / high level trigger; 0 : Falling edge trigger / low level trigger. |

      - PIC_MASK

Offset address: 0x08 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | mask | 0xffffffff | Each bit controls whether the corresponding interrupt source is shielded, IRQ0 corresponds to bit 0 , |

|  |  |  |  | IRQ31 corresponds to bit 31. 1 : interrupt mask; 0 : Interrupt enabled. |
| --- | --- | --- | --- | --- |

      - PIC_VECTOR

Offset address: 0x0c Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:7] | - | reserved | - |  |
| [6:0] | RW | vector0 | 0x20 | The interrupt vector corresponding to IRQ0 has a value range of 32 ~ 112. vector0 is the base address of the interrupt vector table, and the following IRQx VEC​ = x + vector0 ; where x is the interrupt source port number. |

      - PIC_COW1

Offset address: 0x10 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:4] | - | reserved | - |  |
| [3] | WO | feoi | 0x0 | When the CPU finishes processing the fast interrupt, it writes 1 to this bit . After obtaining FEOI , the interrupt controller clears the FISR register and pulls the FINT (active low) request line high. 1 ： CPU interrupt service ends; 0 ： CPU Still processing outage service. |
| [2] | WO | eoi | 0x0 | When the CPU finishes processing the interrupt, it writes 1 to this bit . After receiving the eoi signal, the interrupt controller will clear the ISR register and pull the INT (active low) request line high. 1 ： CPU interrupt service ends; 0 ： CPU Still processing outage service. |
| [1:0] | - | reserved | - |  |

      - PIC_PRIOR0-3​

Offset address: 0x14 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | RW | prior_irq3 | 0x3 | the interrupt source 3 priority counter, the smaller the value, the higher the priority |
| [23:16] | RW | prior_irq2 | 0x2 | The initial value of the priority counter of interrupt source 2 , the smaller the value, the higher the priority |
| [15:8] | RW | prior_irq1 | 0x1 | the interrupt source 1 priority counter, the smaller the value, the higher the priority |
| [7:0] | RW | prior_irq0 | 0x0 | the interrupt source 0 priority counter, the smaller the value, the higher the priority |

      - PIC_PRIOR4-7​

Offset address: 0x18 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | RW | prior_irq7 | 0x7 | The initial value of the priority counter of interrupt source 7 , the smaller the value, the higher the priority |
| [23:16] | RW | prior_irq6 | 0x6 | interrupt source 6 priority counter, smaller value, higher priority |
| [15:8] | RW | prior_irq5 | 0x5 | interrupt source 5 priority counter, smaller value, higher priority |
| [7:0] | RW | prior_irq4 | 0x4 | interrupt source 4 priority counter, smaller value, higher priority |

      - PIC_PRIOR8- 11

Offset address: 0x1C Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | RW | prior_irq11 | 0xb | The initial value of the priority counter of interrupt source 11 , the smaller the value, the higher the priority |
| [23:16] | RW | prior_irq10 | 0xa | the interrupt source 10 priority counter, the smaller the value, the higher the priority |
| [15:8] | RW | prior_irq9 | 0x9 | interrupt source 9 priority counter, smaller value, higher priority |
| [7:0] | RW | prior_irq8 | 0x8 | interrupt source 8 priority counter, smaller value, higher priority |

      - PIC_PRIOR12- 15

Offset address: 0x20 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | RW | prior_irq15 | 0xf | The initial value of the priority counter of interrupt source 15 , the smaller the value, the higher the priority |
| [23:16] | RW | prior_irq14 | 0xe | The initial value of the priority counter of interrupt source 14 , the smaller the value, the higher the priority |
| [15:8] | RW | prior_irq13 | 0xd | The initial value of the priority counter of interrupt source 13 , the smaller the value, the higher the priority |
| [7:0] | RW | prior_irq12 | 0xc | the interrupt source 12 priority counter, the smaller the value, the higher the priority |

      - PIC_PRIOR16- 19

Offset address: 0x24 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | RW | prior_irq19 | 0x13 | The initial value of the priority counter of interrupt source 19 , the smaller the value, the higher the priority |
| [23:16] | RW | prior_irq18 | 0x12 | The initial value of the priority counter of interrupt source 18 , the smaller the value, the higher the priority |
| [15:8] | RW | prior_irq17 | 0x11 | The initial value of the priority counter of interrupt source 17 , the smaller the value, the higher the priority |
| [7:0] | RW | prior_irq16 | 0x10 | the interrupt source 16 priority counter, the smaller the value, the higher the priority |

      - PIC_PRIOR20- 23

Offset address: 0x28 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |

| [31:24] | RW | prior_irq23 | 0x17 | The initial value of the priority counter of interrupt source 23 , the smaller the value, the higher the priority |
| --- | --- | --- | --- | --- |
| [23:16] | RW | prior_irq22 | 0x16 | The initial value of the priority counter of interrupt source 22 , the smaller the value, the higher the priority |
| [15:8] | RW | prior_irq21 | 0x15 | The initial value of the priority counter of interrupt source 21 , the smaller the value, the higher the priority |
| [7:0] | RW | prior_irq20 | 0x14 | The initial value of the priority counter of interrupt source 20 , the smaller the value, the higher the priority |

      - PIC_PRIOR24- 27

Offset address: 0x2C Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | RW | prior_irq27 | 0x1b | The initial value of the priority counter of interrupt source 27 , the smaller the value, the higher the priority |
| [23:16] | RW | prior_irq26 | 0x1a | The initial value of the priority counter of interrupt source 26 , the smaller the value, the higher the priority |
| [15:8] | RW | prior_irq25 | 0x19 | The initial value of the priority counter of interrupt source 25 , the smaller the value, the higher the priority |
| [7:0] | RW | prior_irq24 | 0x18 | the interrupt source 24 priority counter, the smaller the value, the higher the priority |

      - PIC_PRIOR28-31​

Offset address: 0x30 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | RW | prior_irq31 | 0x1f | The initial value of the priority counter of interrupt source 31 , the smaller the value, the higher the priority |
| [23:16] | RW | prior_irq30 | 0x1e | the interrupt source 30 priority counter, the smaller the value, the higher the priority |
| [15:8] | RW | prior_irq29 | 0x1d | The initial value of the priority counter of interrupt source 29 , the smaller the value, the higher the priority |
| [7:0] | RW | prior_irq28 | 0x1c | The initial value of the priority counter of interrupt source 28 , the smaller the value, the higher the priority |

      - PIC_COW2

Offset address: 0x34 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:2] | - | reserved | - |  |
| [1] | RW | avec | 0x1 | 1 ： CPU Use VEC sent by interrupt controller ; 0 ： CPU Use automatic vector number 0x 28 , or 0x 2c . |
| [0] | RW | priority | 0x0 | 1 ：Use user-defined interrupt source priority settings, that is, use Prior_irq[x] priority setting, a smaller initial value indicates a higher priority. 0 : Use the default priority setting IRQ0 > IRQ1 > …… > IRQ5 > IRQ6 > … > IRQ63 . |

      - PIC_SYNC

Offset address: 0x38 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |

| [31:8] | - | reserved | - |  |
| --- | --- | --- | --- | --- |
| [7:0] | RW | min_interval | 0x0 | In asynchronous processing, the level duration after the first pulse signal is related to the frequency difference between the two clock domains. To prevent the two pulses from being too close, Causes both pulses to be lost after asynchronous processing |

      - PIC_FFLAG_L

Offset address: 0x3c Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | fflag_l | 0x0 | 1 : fast interrupt, 0 : normal interrupt. The priority of fast interrupt is higher than that of normal interrupt. Fast interrupts are counted. The priority is determined by the value of the device, and the smaller the initial value, the higher the priority. |

      - PIC_INT_RECORD_SEL

Offset address: 0x40 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:7] | - | reserved | - |  |
| [6:0] | RW | irq_record_sel | 0x0 | Select an interrupt source and record the number of interrupts (after asynchronous and mask processing) Processing back-end interrupt request) PIC_INT_ST_CNT . |

      - PIC_INT_ST

Offset address: 0x44 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RWC | int_state | 0x0 | Records interrupt requests from the lower 32 interrupt sources. Writing 1 to the corresponding bit clears the record and the value of PIC_INT_ST_CNT . |

      - PIC_INT_ST_1

Offset address: 0x48 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RWC | int_state | 0x0 | Records the interrupt requests of the upper 32 interrupt sources. Writing 1 to the corresponding bit clears the record and the value of PIC_INT_ST_CNT . |

      - PIC_INT_ST_CNT

Offset address: 0x48 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | int_cnt | 0x0 | Record the interrupt source selected by PIC_INT_RECORD_SEL The number of interrupt requests. |

      - PIC_MODE_1

Offset address: 0x60 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | edge_level | 0xffffffff | Each bit controls the triggering mode of the corresponding interrupt source, IRQ32 corresponds to bit 0 , and IRQ63 corresponds to bit 31 . 1 : Edge triggered; 0 : Level trigger. |

      - PIC_PO_1

Offset address: 0x64 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | pol | 0xffffffff | Each bit controls the trigger polarity of the corresponding interrupt source, IRQ32 corresponds to bit 0 , and IRQ63 corresponds to bit 31 . 1 : Rising edge trigger / high level trigger; 0 : Falling edge trigger / low level trigger. |

      - PIC_MASK_1

Offset address: 0x68 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | mask | 0xffffffff | Each bit controls whether the corresponding interrupt source is masked, IRQ32 corresponds to bit 0 , and IRQ63 corresponds to bit 31 . 1 : Interrupt mask; 0 : Interrupt open. |

      - PIC_PRIOR32-35​

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | RW | prior_irq35 | 0x27 | 中断源35优先级计数器初始值，值小优先级高 |
| [23:16] | RW | prior_irq34 | 0x26 | 中断源34优先级计数器初始值，值小优先级高 |
| [15:8] | RW | prior_irq33 | 0x25 | 中断源33优先级计数器初始值，值小优先级高 |
| [7:0] | RW | prior_irq32 | 0x24 | 中断源32优先级计数器初始值，值小优先级高 |

Offset address: 0x6c Reset mode: H /S

      - PIC_PRIOR36- 39

Offset address: 0x70 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | RW | prior_irq39 | 0x27 | The initial value of the priority counter of interrupt source 39 , the smaller the value, the higher the priority |
| [23:16] | RW | prior_irq38 | 0x26 | The initial value of the priority counter of interrupt source 38 , the smaller the value, the higher the priority |
| [15:8] | RW | prior_irq37 | 0x25 | The initial value of the priority counter of interrupt source 37 , the smaller the value, the higher the priority |
| [7:0] | RW | prior_irq36 | 0x24 | interrupt source 36 priority counter, smaller value, higher priority |

      - PIC_PRIOR40- 43

Offset address: 0x74 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | RW | prior_irq43 | 0x2b | The initial value of the priority counter of interrupt source 43 , the smaller the value, the higher the priority |
| [23:16] | RW | prior_irq42 | 0x2a | the interrupt source 42 priority counter, the smaller the value, the higher the priority |
| [15:8] | RW | prior_irq41 | 0x29 | The initial value of the priority counter of interrupt source 41 , the smaller the value, the higher the priority |
| [7:0] | RW | prior_irq40 | 0x28 | The initial value of the priority counter of interrupt source 40 , the smaller the value, the higher the priority |

      - PIC_PRIOR44- 47

Offset address: 0x78 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | RW | prior_irq47 | 0x2f | The initial value of the priority counter of interrupt source 47 , the smaller the value, the higher the priority |
| [23:16] | RW | prior_irq46 | 0x2e | The initial value of the priority counter of interrupt source 46 , the smaller the value, the higher the priority |
| [15:8] | RW | prior_irq45 | 0x2d | The initial value of the priority counter of interrupt source 45 , the smaller the value, the higher the priority |
| [7:0] | RW | prior_irq44 | 0x2c | The initial value of the priority counter of interrupt source 44 , the smaller the value, the higher the priority |

      - PIC_PRIOR48-51​

Offset address: 0x7c Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | RW | prior_irq51 | 0x33 | the interrupt source 51 priority counter, the smaller the value, the higher the priority |
| [23:16] | RW | prior_irq50 | 0x32 | The initial value of the priority counter of interrupt source 50 , the smaller the value, the higher the priority |
| [15:8] | RW | prior_irq49 | 0x31 | The initial value of the priority counter of interrupt source 49 , the smaller the value, the higher the priority |
| [7:0] | RW | prior_irq48 | 0x30 | the interrupt source 48 priority counter, the smaller the value, the higher the priority |

      - PIC_PRIOR52-55​

Offset address: 0x80

Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | RW | prior_irq55 | 0x37 | The initial value of the priority counter of interrupt source 55 , the smaller the value, the higher the priority |
| [23:16] | RW | prior_irq54 | 0x36 | The initial value of the priority counter of interrupt source 54 , the smaller the value, the higher the priority |
| [15:8] | RW | prior_irq53 | 0x35 | The initial value of the priority counter of interrupt source 53 , the smaller the value, the higher the priority |
| [7:0] | RW | prior_irq52 | 0x34 | the interrupt source 52 priority counter, the smaller the value, the higher the priority |

      - PIC_PRIOR56-59​

Offset address: 0x84 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | RW | prior_irq59 | 0x3b | The initial value of the priority counter of interrupt source 59 , the smaller the value, the higher the priority |
| [23:16] | RW | prior_irq58 | 0x3a | The initial value of the priority counter of interrupt source 58 , the smaller the value, the higher the priority |
| [15:8] | RW | prior_irq57 | 0x39 | The initial value of the priority counter of interrupt source 57 , the smaller the value, the higher the priority |
| [7:0] | RW | prior_irq56 | 0x38 | the interrupt source 56 priority counter, the smaller the value, the higher the priority |

      - PIC_PRIOR60- 63

Offset address: 0x88 Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | RW | prior_irq63 | 0x3f | the interrupt source 63 priority counter, the smaller the value, the higher the priority |
| [23:16] | RW | prior_irq62 | 0x3e | the interrupt source 62 priority counter, the smaller the value, the higher the priority |
| [15:8] | RW | prior_irq61 | 0x3d | The initial value of the priority counter of interrupt source 61 , the smaller the value, the higher the priority |
| [7:0] | RW | prior_irq60 | 0x3c | the interrupt source 60 priority counter, the smaller the value, the higher the priority |

      - PIC_FFLAG_H

Offset address: 0x8c Reset mode: H /S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | fflag_h | 0x0 | 1 : fast interrupt, 0 : normal interrupt. The priority is higher than that of all normal interrupts, and the priority among fast interrupts is determined by the counter value. |

#### Timer

###### Overview

HR_C7000 Contains 6 independent 32 -bit timers ( Timer1~6 ) to realize timing and counting functions. It can be used by the operating system as the system clock or by application programs for timing and counting.

###### Functional Description

The timer has the following features:

- Timer1~6 can be used individually and each generates a timer interrupt;

- Supports two working modes: free running and user-defined counting;

- Supports querying the current count values of four timers.

###### How it works

The timer implements 4 The timer counts down from a fixed value.

The count value becomes 0 An interrupt will occur.

The initial value of each timer is loaded from the corresponding load count register ( TimerNLoadCount ). The following two events will trigger the loading of the initial value from the TimerNLoadCount register:

- The timer is enabled after being reset or disabled.

- The timer counts to 0

All status registers and interrupt registers can be accessed at any time.

###### Register Overview

Register Table 6 Timer Register Overview (Base Address: 0x1400_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x00 | Timer1LoadCount | Timer 1 Load register |  |
| 0x04 | Timer1CurrentValue | Timer 1 Current value register |  |
| 0x08 | Timer1ControlReg | Timer 1 Control registers |  |
| 0x0c | Timer1EOI | Timer 1 Interrupt Clear Register |  |
| 0x10 | Timer1IntStatus | Timer 1 Interrupt Status Register |  |
| 0x14 | Timer2LoadCount | Timer 2 Load register |  |
| 0x18 | Timer2CurrentValue | Timer 2 Current value register |  |
| 0x1c | Timer2ControlReg | Timer 2 Control registers |  |
| 0x20 | Timer2EOI | Timer 2 Interrupt Clear Register |  |
| 0x24 | Timer2IntStatus | Timer 2 Interrupt Status Register |  |
| 0x28 | Timer3LoadCount | Timer 3 Load register |  |
| 0x2c | Timer3CurrentValue | Timer 3 Current value register |  |
| 0x30 | Timer3ControlReg | Timer 3 Control registers |  |
| 0x34 | Timer3EOI | Timer 3 Interrupt Clear Register |  |
| 0x38 | Timer3IntStatus | Timer 3 Interrupt Status Register |  |
| 0x3c | Timer4LoadCount | Timer 4 Load register |  |
| 0x40 | Timer4CurrentValue | Timer 4 Current value register |  |
| 0x44 | Timer4ControlReg | Timer 4 Control registers |  |
| 0x48 | Timer4EOI | Timer 4 Interrupt Clear Register |  |
| 0x4c | Timer4IntStatus | Timer 4 Interrupt Status Register |  |
| 0x3c | Timer5LoadCount | Timer 5 Load register |  |
| 0x40 | Timer5CurrentValue | Timer 5 Current value register |  |
| 0x44 | Timer5ControlReg | Timer 5 Control registers |  |

| 0x48 | Timer5EOI | Timer 5 Interrupt Clear Register |  |
| --- | --- | --- | --- |
| 0x4c | Timer5IntStatus | Timer 5 Interrupt Status Register |  |
| 0x3c | Timer6LoadCount | Timer 6 Load register |  |
| 0x40 | Timer6CurrentValue | Timer 6 Current value register |  |
| 0x44 | Timer6ControlReg | Timer 6 Control registers |  |
| 0x48 | Timer6EOI | Timer 6 Interrupt Clear Register |  |
| 0x4c | Timer6IntStatus | Timer 6 Interrupt Status Register |  |
| 0xa0 | TimersIntStatus | Interrupt status of all timers |  |
| 0xa4 | TimersEOI | Clear interrupts from all registers |  |
| 0xa8 | TimersRawIntStatus | Unmasked interrupt status of all timers |  |
| 0xac | TIMERS_COMP_VERSION | Timer version number |  |

###### Register Description

      - Timer1LoadCount

Offset address: 0x0 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | Timer1LoadCount | 0x0 | Timer 1 load register |

      - Timer1CurrentValue

Offset address: 0x4 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | Timer1LoadValue | 0x0 | Timer 1 current value register |

      - Timer1ControlReg

Offset address: 0x8 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:3] | - | Reserved | - | reserve |
| [2] | RW | TimerInterruptMas k | 0x0 | Timer 1 interrupt mask |
| [1] | RW | TimerMode | 0x0 | Timer 1 Mode |
| [0] | RW | TimerEnable | 0x0 | Timer 1 enable |

      - Timer1EOI

Offset address: 0xc Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | - | Reserved | - | reserve |

| [0] | RC | EOI | 0x0 | Reading returns 0 and clears the Timer 1 interrupt |
| --- | --- | --- | --- | --- |

      - Timer1IntStatust

Offset address: 0x10 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | - | Reserved | - | reserve |
| [0] | RO | IntStatus | 0x0 | Timer 1 interrupt status |

      - Timer2LoadCount

Offset address: 0x14 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | Timer2LoadCount | 0x0 | Timer 2 load register |

      - Timer2CurrentValue

Offset address: 0x18 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | Timer2LoadValue | 0x0 | Timer 2 current value register |

      - Timer2ControlReg

Offset address: 0x1c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:3] | - | Reserved | - | reserve |
| [2] | RW | TimerInterruptMas k | 0x0 | Timer 2 interrupt mask |
| [1] | RW | TimerMode | 0x0 | Timer 2 Mode |
| [0] | RW | TimerEnable | 0x0 | Timer 2 enable |

      - Timer2EOI

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | - | Reserved | - | 保留 |
| [0] | RC | EOI | 0x0 | 读返回0，并清除定时器1的中断 |

Offset address: 0x20 Reset mode: H/S

      - Timer2IntStatust

Offset address: 0x24 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | - | Reserved | - | reserve |
| [0] | RO | IntStatus | 0x0 | Timer 2 interrupt status |

      - Timer3LoadCount

Offset address: 0x28 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | Timer1LoadCount | 0x0 | Timer 3 load register |

      - Timer3CurrentValue

Offset address: 0x2c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | Timer1LoadValue | 0x0 | Timer 3 current value register |

      - Timer3ControlReg

Offset address: 0x30 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:3] | - | Reserved | - | reserve |
| [2] | RW | TimerInterruptMas k | 0x0 | Timer 3 interrupt mask |
| [1] | RW | TimerMode | 0x0 | Timer 3 Mode |
| [0] | RW | TimerEnable | 0x0 | Timer 3 enable |

      - Timer3EOI

Offset address: 0x34 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | - | Reserved | - | reserve |
| [0] | RC | EOI | 0x0 | Reading returns 0 and clears the Timer 3 interrupt |

      - Timer3IntStatust

Offset address: 0x38 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | - | Reserved | - | reserve |
| [0] | RO | IntStatus | 0x0 | Timer 3 interrupt status |

      - Timer4LoadCount

Offset address: 0x3c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | Timer4LoadCount | 0x0 | Timer 4 load register |

      - Timer4CurrentValue

Offset address: 0x40 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | Timer4LoadValue | 0x0 | Timer 4 current value register |

      - Timer4ControlReg

Offset address: 0x44 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:3] | - | Reserved | - | reserve |
| [2] | RW | TimerInterruptMas k | 0x0 | Timer 4 interrupt mask |
| [1] | RW | TimerMode | 0x0 | Timer 4 mode |
| [0] | RW | TimerEnable | 0x0 | Timer 4 enable |

      - Timer4EOI

Offset address: 0x48 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | - | Reserved | - | reserve |
| [0] | RC | EOI | 0x0 | Reading returns 0 and clears the Timer 4 interrupt |

      - Timer4IntStatust

Offset address: 0x4c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | - | Reserved | - | reserve |
| [0] | RO | IntStatus | 0x0 | Timer 4 interrupt status |

      - Timer5LoadCount

Offset address: 0x0 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | Timer5LoadCount | 0x0 | Timer 5 load register |

      - Timer5CurrentValue

Offset address: 0x4 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | Timer5LoadValue | 0x0 | Timer 5 current value register |

      - Timer5ControlReg

Offset address: 0x8 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:3] | - | Reserved | - | reserve |
| [2] | RW | TimerInterruptMas k | 0x0 | Timer 5 interrupt mask |
| [1] | RW | TimerMode | 0x0 | Timer 5 Mode |
| [0] | RW | TimerEnable | 0x0 | Timer 5 enable |

      - Timer5EOI

Offset address: 0xc Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | - | Reserved | - | reserve |
| [0] | RC | EOI | 0x0 | Reading returns 0 and clears the Timer 5 interrupt |

      - Timer5IntStatust

Offset address: 0x10 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | - | Reserved | - | reserve |
| [0] | RO | IntStatus | 0x0 | Timer 5 interrupt status |

      - Timer6LoadCount

Offset address: 0x0

Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | Timer6LoadCount | 0x0 | Timer 6 load register |

      - Timer6CurrentValue

Offset address: 0x4 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | Timer6LoadValue | 0x0 | Timer 6 current value register |

      - Timer6ControlReg

Offset address: 0x8 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:3] | - | Reserved | - | reserve |
| [2] | RW | TimerInterruptMas k | 0x0 | Timer 6 interrupt mask |
| [1] | RW | TimerMode | 0x0 | Timer 6 mode |
| [0] | RW | TimerEnable | 0x0 | Timer 6 enable |

      - Timer6EOI

Offset address: 0xc Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | - | Reserved | - | reserve |
| [0] | RC | EOI | 0x0 | Reading returns 0 and clears the Timer 6 interrupt |

      - Timer6IntStatust

Offset address: 0x10 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | - | Reserved | - | reserve |
| [0] | RO | IntStatus | 0x0 | Timer 6 interrupt status |

      - TimersIntStatus

Offset address: 0xa0 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | TimersIntStatus | 0x0 | Interrupt status of all timers |

      - TimersEOI

Offset address: 0xa4 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RC | TimersEOI | 0x0 | Reading returns 0 and clears all timers. Break |

      - TimersRawIntStatus

Offset address: 0xa8 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | TimersRawIntStatus s | 0x0 | All timer unmasked interrupt status |

      - TIMERS_COMP_VERSION

Offset address: 0xac Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | TIMERS_COMP_ VERSION | 0x3230322a | Module version |

### RTC

###### Overview

HR_C7000 includes a real-time clock module that is responsible for system timing, can save the current world time, and can be set to generate an interrupt signal at a future time point.

###### Functional Description

RTC The main features are as follows:

- RTC Internally, a 6- bit wide second and minute register, a 5- bit wide hour register, a 16- bit wide day register, and a 1Hz second register are used. The clock counts, and when it reaches 60 The hour and minute registers are incremented by one, and the second register is reset to zero. And so on. The day register has 16 bits and can count 65,526 days, which is equivalent to 179 years.

- The interrupt generation time is configurable, and whether the interrupt is generated can be controlled.

- RTC The initial time is configurable.

- Can control RTC Start or stop.

- RTC after the chip is powered off Still works, use an external coin cell battery to power it.

###### How it works

HR_C7000 Medium RTC Mounted on internal I 2 C On, through I 2 C The bus operates. The timing clock is provided externally

32.768KHz crystal.

      - RTC initialization

RTC When powered on for the first time, the system needs to set the RTC Initialize RTC The initialization process is as follows:

- Pull rtc_rst_n low to reset the RTC I2C​​​ Interface module.

- Wait 60ns ( 3 apb​ clock cycle ) .

- Configuring RTC_SOFT_RST_N Register is 0 , perform soft reset. ( Device address 7′b1110000 )

- Wait 25ms ( counter at 128Hz When operating under the clock of the asynchronous reset, the synchronous removal of the asynchronous reset requires 2 3.44 ms ) .

- Configuring RTC_SOFT_RST_N Register is 1 to cancel the soft reset.

- Configure RTC_MATCH_S , RTC_MATCH_M , RTC_MATCH_H , RTC_MATCH_D_L , RTC_MATCH_D_H in sequence to set the RTC The time when the interrupt is generated, configure RTC_MATCH_VEN[0] to 1 Update the interrupt generation time (this bit will be automatically cleared ) , configure RTC_MATCH_VSTAT[0] to 1 Indicates that the current interrupt generation time has been configured. If you need to reconfigure the interrupt generation time, first configure this bit to 0 , and then configure this bit to 1 after configuring the interrupt generation time .

- Configure RTC_LOAD_S in sequence 、 RTC_LOAD_M , RTC_LOAD_H , RTC_LOAD_D_L , RTC_LOAD_D_H , set RTC Initial time, configure RTC_CCR[2] to 1 Update the current time (this bit will be automatically cleared ) , configure RTC_LOAD_VSTAT[0] to 1 Indicates that the initial time has been set. If you need to reconfigure the initial time, first configure this bit to 0 , and then configure this bit to 1 after configuring the initial time .

- Configure RTC_CCR[0] to 1 to enable interrupt.

- Configure RTC_CCR[1] to 1 to start the timer.

- RTC At 1HZ The counting clock frequency is from RTC_LOAD_S , RTC_LOAD_M , RTC_LOAD_H , RTC_LOAD_D_L , RTC_LOAD_D_H When the count value reaches the value in RTC_MATCH_S , RTC_MATCH_M , RTC_MATCH_H , RTC_MATCH_D_L , RTC_MATCH_D_H , it will decide whether to generate an interrupt based on the setting of RTC_CCR[0] .

      - Interrupt handling

The system receives RTC After the interrupt is issued, it means that the timing time is up and the user can perform the corresponding custom operation.

RTC The counter keeps counting. The interrupt processing process is as follows:

- Read RTC_RSTAT , clear RTC The interrupt status.

- If you need to set the timing time, first turn off the interrupt enable, configure RTC_MATCH_VSTAT[0]=0 , and send the following signals to RTC_MATCH_S , RTC_MATCH_M , RTC_MATCH_H , RTC_MATCH_D_L , RTC_MATCH_D_H

Setting a new RTC Compare values, configure RTC_MATCH_VEN[0]=1 , RTC_MATCH_VSTAT[0]=1 Then enable the interrupt .

###### Register Overview

Register Table 7 RTC Register Overview (Base Address: 0x1408_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x00 | RTC_VAL_S | RTC_VAL_S Overall parameter configuration |  |
| 0x01 | RTC_VAL_M | RTC_VAL_M Overall parameter configuration |  |
| 0x02 | RTC_VAL_H | RTC_VAL_H Overall parameter configuration |  |
| 0x03 | RTC_VAL_D_L | RTC_VAL_D_L Overall parameter configuration |  |
| 0x04 | RTC_VAL_D_H | RTC_VAL_D_H Overall parameter configuration |  |
| 0x05 | RTC_MATCH_S | RTC_MATCH_S Overall parameter configuration |  |
| 0x06 | RTC_MATCH_M | RTC_MATCH_M Overall parameter configuration |  |
| 0x07 | RTC_MATCH_H | RTC_MATCH_H Overall parameter configuration |  |

| 0x08 | RTC_MATCH_D_L | RTC_MATCH_D_L Overall parameter configuration |  |
| --- | --- | --- | --- |
| 0x09 | RTC_MATCH_D_H | RTC_MATCH_D_H Overall parameter configuration |  |
| 0x0a | RTC_MATCH_VEN | RTC_MATCH_VEN Overall parameter configuration |  |
| 0x0b | RTC_MATCH_VSTAT | RTC_MATCH_VSTAT Overall parameter configuration |  |
| 0x0c | RTC_LOAD_S | RTC_LOAD_S Overall parameter configuration |  |
| 0x0d | RTC_LOAD_M | RTC_LOAD_M Overall parameter configuration |  |
| 0x0e | RTC_LOAD_H | RTC_LOAD_H Overall parameter configuration |  |
| 0x0f | RTC_LOAD_D_L | RTC_LOAD_D_L Overall parameter configuration |  |
| 0x10 | RTC_LOAD_D_H | RTC_LOAD_D_H Overall parameter configuration |  |
| 0x11 | RTC_CCR | RTC_CCR Overall parameter configuration |  |
| 0x12 | RTC_LOAD_VSTAT | RTC_LOAD_VSTAT Overall parameter configuration |  |
| 0x13 | RTC_RSTAT | RTC_RSTAT Number Configuration |  |
| 0x14 | RTC_CLKDIV_NUMER_L | RTC_CLKDIV_NUMER_L Number Configuration |  |
| 0x15 | RTC_CLKDIV_NUMER_H | RTC_CLKDIV_NUMER_H Number Configuration |  |
| 0x16 | RTC_CLKDIV_NUMER_VLD | RTC_CLKDIV_NUMER_VLD Number Configuration |  |
| 0x17 | RTC_SOFT_RST_N | RTC_SOFT_RST_N Number Configuration |  |

###### Register Description

      - RTC_VAL_S

RTC_VAL_S Overall parameter configuration offset address: 0x0000

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:6] | - | reserved | - | reserve |
| [5:0] | RO | rtc_val_s | 0x00 | Record the seconds of the current time |

      - RTC_VAL_M

RTC_VAL_M Overall parameter configuration offset address: 0x0001

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:6] | - | reserved | - | reserve |
| [5:0] | RO | rtc_val_m | 0x00 | Record the current time |

      - RTC_VAL_H

RTC_VAL_H Overall parameter configuration offset address: 0x0002

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:5] | - | reserved | - | reserve |
| [4:0] | RO | rtc_val_h | 0x00 | Record the hour of the current time |

      - RTC_VAL_D_L

RTC_VAL_D_L Overall parameter configuration offset address: 0x0003

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:0] | RO | rtc_val_d_l | 0x00 | Record the lower eight bits of the day of the current time |

      - RTC_VAL_D_H

RTC_VAL_D_H Overall parameter configuration offset address: 0x0004

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:0] | RO | rtc_val_d_h | 0x00 | Record the high eight bits of the day of the current time |

      - RTC_MATCH_S

RTC_MATCH_S Overall parameter configuration offset address: 0x0005

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:6] | - | reserved | - | reserve |
| [5:0] | RW | rtc_match_s | 0x00 | with the count value to see if an interrupt occurs |

      - RTC_MATCH_M

RTC_MATCH_M Overall parameter configuration offset address: 0x0006

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:6] | - | reserved | - | reserve |
| [5:0] | RW | rtc_match_m | 0x00 | with the count value to see if an interrupt occurs |

      - RTC_MATCH_H

RTC_MATCH_H Overall parameter configuration offset address: 0x0007

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:5] | - | reserved | - | reserve |
| [4:0] | RW | rtc_match_h | 0x00 | with the count value to see if an interrupt occurs |

      - RTC_MATCH_D_L

RTC_MATCH_D_L Overall parameter configuration offset address: 0x0008

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:0] | RW | rtc_match_d_l | 0x00 | with the count value to see if an interrupt occurs |

      - RTC_MATCH_D_H

RTC_MATCH_D_H Overall parameter configuration offset address: 0x0009

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:0] | RW | rtc_match_d_h | 0x00 | with the count value to see if an interrupt occurs |

      - RTC_MATCH_VEN

RTC_MATCH_VEN Overall parameter configuration offset address: 0x000A

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:1] | - | reserved | - | reserve |
| [0] | RW | rtc_match_ven | 0x00 | Match data enable signal, if it is 1 , the user set value is written to to the internal match register to prevent false triggering of interrupts. |

      - RTC_MATCH_VSTAT

RTC_MATCH_VSTAT Overall parameter configuration offset address: 0x000B

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:1] | - | reserved | - | reserve |
| [0] | RW | rtc_match_vstat | 0x00 | To prevent data uncertainty caused by power failure when configuring the match register, the user should pull this bit high after configuring the MATCH related registers, indicating that the MATCH related registers have been configured. Next time you need to configure the MATCH related registers, configure this bit to 0, and then pull it high after configuration. This bit is configured as 1 . |

      - RTC_LOAD_S

RTC_LOAD_S Overall parameter configuration offset address: 0x000C

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:6] | - | reserved | - | reserve |
| [5:0] | RW | rtc_load_s | 0x00 | Record the seconds of the preload value |

      - RTC_LOAD_M

RTC_LOAD_M Overall parameter configuration offset address: 0x000D

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:6] | - | reserved | - | reserve |
| [5:0] | RW | rtc_load_m | 0x00 | Record preload value quantile |

      - RTC_LOAD_H

RTC_LOAD_H Overall parameter configuration offset address: 0x000E

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:5] | - | reserved | - | reserve |
| [4:0] | RW | rtc_load_h | 0x00 | Record the hour digit of the preloaded value |

      - RTC_LOAD_D_L

RTC_LOAD_D_L Overall parameter configuration offset address: 0x000F

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:0] | RW | rtc_load_d_l | 0x00 | The lower eight digits of the number of days to record the preload value |

      - RTC_LOAD_D_H

RTC_LOAD_D_H Overall parameter configuration offset address: 0x0010

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:0] | RW | rtc_load_d_h | 0x00 | The high eight digits of the number of days to record the preload value |

      - RTC_CCR

RTC_CCR Overall parameter configuration offset address: 0x0011

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:3] | N/A | reserved | 0x00 | Invalid bit, default is 0 |
| [2] | RW | rtc_load | 0x00 | When this bit is 1 , count is loaded from the load register with the preset value. This bit will automatically clear to 0 after |
| [1] | RW | rtc_enable | 0x00 | Allows the user to select whether the counter should count. 0 = Counter stops counting 1= Counter starts counting |
| [0] | RW | rtc_intr_en | 0x00 | Allows the user to choose whether to mask interrupts. 0 = Interrupts are not enabled 1= Interrupt enable |

      - RTC_LOAD_VSTAT

RTC_LOAD_VSTAT Overall parameter configuration offset address: 0x0012

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:1] | - | reserved | - | reserve |
| [0] | RW | rtc_load_vstat | 0x00 | To prevent data uncertainty caused by power failure when preloading registers, the user pulls it high after configuring the LOAD related registers, indicating that the LOAD related registers have been configured. Before configuring the device, set this bit to 0 , and then set this bit to 1 after configuration . |

      - RTC_RSTAT

RTC_RSTAT Number configuration offset address: 0x0013

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:1] | - | reserved | - | reserve |
| [0] | RC | rtc_rstat | 0x00 | 0 = Interrupt not generated 1 = Interrupt occurred (ignoring priority) This bit can be read and cleared |

      - RTC_CLKDIV_NUMER_L

RTC_CLKDIV_NUMER_L Number Configuration

Offset address: 0x0014

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:0] | RW | rtc_clkdiv_numer_l | 0x00 | 8 bits of the digital frequency division , a total of 16 bits, can be set to any integer between 0 and 65535. If the frequency division number is t , then the RTC timing period is (crystal frequency/t) Hz . For example, if the RTC uses a 32.768k crystal oscillator, If you want to get a 1 Hz timing clock, the division number is 32768 . |

      - RTC_CLKDIV_NUMER_H

RTC_CLKDIV_NUMER_H Number Configuration

Offset address: 0x0015

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:0] | RW | rtc_clkdiv_numer_ h | 0x00 | 8 bits of the digital frequency division , a total of 16 bits, can be set to any integer between 0 and 65535. If the frequency division number is t , then the RTC timing period is (crystal frequency/t) Hz . For example, if the RTC uses a 32.768k crystal oscillator, If you want to get a 1 Hz timing clock, the division number is 32768 . |

      - RTC_CLKDIV_NUMER_VLD

RTC_CLKDIV_NUMER_VLD Number Configuration

Offset address: 0x0016

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:1] | - | reserved | - | reserve |
| [0] | RW | rtc_clkdiv_nummer _vld | 0x00 | Frequency division coefficient valid signal, if this bit is 0 , use the default 32768 1 , the frequency division coefficient set by the user is used. |

      - RTC_SOFT_RST_N

RTC_SOFT_RST_N Number configuration offset address: 0x0017

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:1] | - | reserved | - | reserve |
| [0] | RW | rtc_soft_rst_n | 0x01 | 0 = soft reset 1= Soft reset is invalid |

#### Watchdog

###### Overview

The watchdog is used to reset the system when it crashes.

###### Functional Description

The watchdog has two main features:

- Once an external physical reset signal arrives, the watchdog outputs a low-level reset signal for 510 seconds. System clock time, extending the physical reset time to ensure that the system is fully reset;

- To prevent the system from freezing, the software needs to send a knock signal to the watchdog within a certain period of time. If the watchdog is not stimulated within the predetermined time, it is considered that the system has frozen and will automatically reset the system. The reset level lasts for 510 seconds.

A system clock.

###### How it works

##### Enable watchdog

After the system starts, configure WD_CW. wde=1 to enable the watchdog timer.

##### Disable watchdog

The operation of disabling the watchdog is a bit complicated. This is to prevent accidental disabling of the watchdog due to software misoperation or electromagnetic interference . The disabling process is as follows:

- When wde is high, set WD_CW.wdtoe to 1 ;

- Set WD_CW.wde to 0 , and the watchdog is turned off;

- Set WD_CW.wdtoe to 0 as the initial value.

After being turned off, the initial count value of the watchdog and other control words are not changed. You can continue to use it after turning on the watchdog . The timing diagram is as follows:

开启状态

关闭状态

Watchdog Working status

wde wdtoe

CLK

Figure 21 Watchdog shutdown timing diagram

Table 31 Watchdog working status table

| wdtoe | wde | Watchdog working status |
| --- | --- | --- |
| 0 | 0 | Not allowed |
| 0 | 1 | Open |
| 1 | 0 | Close (jump in from 11 to close, jump in directly from 10 will not close ) |
| 1 | 1 | Open |

##### Normal work

In normal working mode, the system needs to periodically write to the watchdog register WD_CW. knocker . If the register is not written within the specified period, the watchdog will consider the system to be dead and will reset the system.

- Configure WD_TIMER and set the initial value of the watchdog period;

- Configure WD_CW. wde=1 to enable the watchdog timer;

- the internal counter counts from the initial value WD_TIMER to the overflow value, configure WD_CW.knocker =1 once to complete the

After knocking the door, the internal counter starts counting from WD_TIMER again.

###### Register Overview

Register Table 8 WDT Register Overview (Base Address: 0x1401_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x0 | WDG_LOCK | Watchdog Lock Register |  |
| 0x4 | WD_OT_LOAD_H | Watchdog timeout counter high |  |
| 0x8 | WD_OT_LOAD_L | Watchdog timeout counter low |  |
| 0xc | WDG_RST_LOAD | Watchdog reset time initial value |  |
| 0x10 | WDG_EN | Watchdog enable |  |
| 0x14 | WDG_KNOCK | Watchdog feeding register |  |

###### Register Description

      - WDG_LOCK

Offset address: 0x0 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | WDG_LOCK | 0x1 | 0x5ada_7200 to this register will enable write access to all registers, and writing other values to this register will disable write access to all registers. Reading this register will return the locked status: 0x0 : write allowed, unlocked 0x1 : write disabled, locked |

      - WDG_OT_LOAD_H

Offset address: 0x4 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:3] | RO | reserved | 0x0 | reserve |
| [2:0] | RW | WDG_OT_LOA D_H | 0x7 | The high initial value of the timeout counter. When the high initial value of the timeout counter overflows, it indicates Timeout, the default timeout is about 60s |

      - WDG_OT_LOAD_L

Offset address: 0x8 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | WDG_OT_LO AD_L | 0xc0000000 | The low initial value of the timeout counter, when the high counter overflows, Indicates timeout. The default timeout is about 60s . |

      - WDG_RST_LOAD

Offset address: 0x c

Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:25] | RO | reserved | 0x0 | reserve |
| [24:0] | RW | WDG_RST_LO AD | 0x03f80000 | The initial value of the watchdog reset time is reset after counting to 0x03ffffff Completed, the default reset time is about 20ms |

      - WDG_EN

Offset address: 0x10 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | RO | reserved | 0x0 | reserve |
| [0] | RW | WDG_EN | 0x0 | Watchdog enable signal 0 : Watchdog is off and no watchdog reset is generated 1 : Watchdog is working. If the watchdog is not fed within the timeout period, a watchdog reset will occur. |

      - WDG_KNOCK

Offset address: 0x14 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | WDG_KNOCK | 0x0 | Feed the dog signal, write 0x55aadd22 to feed the dog, watchdog timeout The counter restarts counting. |

## Peripheral devices

### I²C​​

###### Overview

I 2 C​​ Inter-Integrated Circuit ) interface has 2 signals: SDA (serial data or address line) and SCL ( serial clock line ) . HR_C7000 I²C​​ Module as I 2 C Bus Master , can be I 2 C Communicates from the device in accordance with I 2 C Bus protocol version 2.1 .

###### Functional Description

I²C​​ The module has the following features:

- Support standard 100kbit/s Data transmission rate and fast mode 400kbit/s ;

- Provide TX FIFO and RX FIFO ;

- Supports interruption reporting and interruption query.

I2C is transferring data , the data signal on SDA must remain stable when SCL is at a high level, and can only change phase when SCL is at a low level.

START The conditions for transmission are: When SDA is high, There is a High-to-Low jump.

STOP The conditions for transmission are: When SDA is high There is a Low-to-High This process is shown in the figure below:

MSB	0D/1	0D/21	0/1	0/1	0/1	0/1	0/1

1

2

3

4

5

6

7

8

9

P

S

SDA SCL

Start Condition

Figure 22 I²C​​ Bus data transmission process

Stop Condition

I²C​​ Each data packet on the bus is 8 bit width, plus an ACK bits , so a complete data transfer includes 8 clock cycles. Accepted 8 After the SDA Set low to indicate ACK , at this time, Transmitter Need to release SDA If Slave Receiver No ACK is sent , which means Receiver If no data is received, then the Master Can send STOP Signal terminates data transmission. If the Master Receiver If no ACK is sent , it means the transmission is over, and the Slave Transmitter can terminate the transmission.

Table 32 I²C​​ Interface signals

| signal name | direction | describe |
| --- | --- | --- |
| I 2 C_SCL | B | I²C​​ Serial clock line |
| I2C_SDA​​ | B | I²C​​ Serial data address lines |

###### Clock frequency configuration

When I2C is used as a master , the clock frequency needs to be configured. I2C supports fast mode and the maximum data rate supported is 400kb/s . The clock frequency is calculated according to formula ( 1 ). SCL_PERIOD_FS in formula ( 1 ) Indicates data rate. IC_HCNT_FS indicates fast mode, when SCL IC_LCNT_FS represents the I2C clock cycle count during the active-high period of SCL . IC_HCNT_FS is the configured value of the IC_FS_SCL_HCNT register plus 8 , and IC_LCNT_FS is the configured value of the IC_FS_SCL_LCNT register plus 1 .

Formula ( 1 )

For example, the data rate is 400kb/s When SCL_PERIOD_FS = 2.5us , I2C​​ Working clock is APB The clock is 50M , and IC_HCNT_FS can be calculated according to the above formula and IC_LCNT_FS , and then register IC_FS_SCL_HCNT and IC_FS_SCL_LCNT The configuration values are ( IC_HCNT_FS – 8 ) and ( IC_LCNT_FS – 1 Register​ The minimum value of IC_FS_SCL_HCNT is not less than 6 , and the minimum value of register IC_FS_SCL_LCNT is not less than 8 .

###### How it works

##### initialization

I²C​​ The controller initialization process is as follows:

- Configure register IC_ENABLE to enable I2C Shutdown;

- Configure register IC_INTR_MASK to mask interrupts;

- Configuration register IC_FS_SCL_HCNT and IC_FS_SCL_LCNT , set the clock frequency;

- Configuration register IC_CON ;

- Write slave address register IC_TAR ;

- Configuring the Transmit FIFO Threshold register IC_TX_TL and receive FIFO Threshold register IC_RX_TL ;

- Configure register IC_INTR_MASK to enable interrupt configuration;

- Register IC_ENABLE , enable I2C .

##### Data sending and receiving

Data sending process:

1. Register IC_DATA_CMD Write data and configure IC_START Register, start data sending;

2. In query mode, poll register IC_STATUS Monitor transmit FIFO In interrupt mode, when the transmit FIFO is detected Threshold interrupt TX_EMPTY ( Transmit FIFO When the data volume is less than or equal to the threshold, write the data to the transmit FIFO and query the register IC_STATUS at the same time. Judging FIFO Empty or full, when FIFO If it is full, stop writing data;

Notice:

- Before sending data, the transmit FIFO Cannot be empty, because once the FIFO is sent If it is empty, STOP will be triggered.

Condition, resulting in the end of data transmission.

- If you need to turn off I2C , you need to ensure that I2C  In idle state, that is, query and satisfy the register

IC_STATUS bit0 0. Data receiving process:

1. Write register IC_DATA_CMD Bit[8 ] of IC_START Register, send read data command;

2. In query mode, poll the register IC_STATUS . When the receive FIFO is detected When not empty, read register IC_DATA_CMD . In interrupt mode, the receive FIFO is detected Threshold interrupt RX_FULL ( when the receive FIFO When the data is greater than the set threshold , read the receiving register IC_DATA_CMD and query the register IC_STATUS at the same time. Read the receive FIFO to empty.

      - Disable I²C​​ Controller

In IC_ENABLE Register from 1 Assigned to 0 After that, the software can monitor IC_ENABLE_STATUS Register to determine if the hardware has completed the shutdown process.

Disable I²C​​ The controller process is:

- Define a time interval ( Ti2c_poll ) equal to I2C 10 of the maximum transmission period For example, if the highest transmission speed is 400kbps , then Ti2c_poll is 25us .

- Defines a maximum timeout parameter, MAX_T_POLL_COUNT , so that any repeated query operation greater than this value will result in an error .

- Disable initiation of new I 2 C transfers, but allows unfinished transfers to continue

- Initialize POLL_COUNT The variable is 0 .

- Set IC_ENABLE The register is 0 .

- Read IC_ENABLE_STATUE Register, Checker IC_EN Bit ( 0​ POLL_COUNT​​​ Increment by 1. If POLL_COUNT is greater than or equal to MAX_T_POLL_COUNT , an error is returned.

- If IC_EN If the bit is 1 , wait for Ti2c_poll time, and return to the previous step. Otherwise, exit successfully.

###### Register Overview

Register Table 9 I2C0 Register Overview ( Base Address 0x14060000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x0 | IC_CON | I2C Control registers |  |
| 0x4 | IC_TAR | I2C Destination Address Register |  |
| 0x10 | IC_DATA_CMD | I2C Send and receive data buffers and commands |  |
| 0x1c | IC_FS_SCL_HCNT | Fast SCL High level count |  |
| 0x20 | IC_FS_SCL_LCNT | Fast SCL Low level count |  |
| 0x2c | IC_INTR_STAT | Interrupt Status |  |
| 0x30 | IC_INTR_MASK | Interrupt mask |  |
| 0x34 | IC_RAW_INTR_STAT | Raw interrupt status |  |
| 0x38 | IC_RX_TL | Receive FIFO threshold |  |
| 0x3c | IC_TX_TL | Transmit FIFO threshold |  |
| 0x40 | IC_CLR_INTR | Clear Interrupt |  |
| 0x44 | IC_CLR_RX_UNDER | Clear interrupt RX_UNDER |  |
| 0x48 | IC_CLR_RX_OVER | Clear interrupt RX_OVER |  |
| 0x4c | IC_CLR_TX_OVER | Clear interrupt TX_OVER |  |
| 0x50 | IC_CLR_RD_REQ | Clear interrupt RD_REQ |  |
| 0x54 | IC_CLR_TX_ABRT | Clear interrupt TX_ABRT |  |
| 0x58 | IC_CLR_RX_DONE | Clear interrupt RX_DONE |  |
| 0x5c | IC_CLR_ACTIVITY | Clear interrupt activity |  |
| 0x60 | IC_CLR_STOP_DET | Clear interrupt STOP_DET |  |
| 0x64 | IC_CLR_START_DET | Clear interrupt START_DET |  |
| 0x6c | IC_ENABLE | I2C Enable register |  |
| 0x70 | IC_STATUS | I2C Status Register |  |
| 0x74 | IC_TXFLR | Transmit FIFO water level register |  |
| 0x78 | IC_RXFLR | Receive FIFO water level register |  |
| 0x80 | IC_TX_ABRT_SOURCE | I2C Transfer end status register |  |
| 0x9c | IC_ENABLE_STATUS | I2C Enable Status Register |  |
| 0xa0 | IC_START | Control data starts sending |  |

Register Table 10 I2C1 Register Overview ( base address is 0x14070000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x0 | IC_CON | I2C Control registers |  |
| 0x4 | IC_TAR | I2C Destination Address Register |  |
| 0x10 | IC_DATA_CMD | I2C Send and receive data buffers and commands |  |
| 0x1c | IC_FS_SCL_HCNT | Fast SCL High level count |  |
| 0x20 | IC_FS_SCL_LCNT | Fast SCL Low level count |  |
| 0x2c | IC_INTR_STAT | Interrupt Status |  |
| 0x30 | IC_INTR_MASK | Interrupt mask |  |
| 0x34 | IC_RAW_INTR_STAT | Raw interrupt status |  |
| 0x38 | IC_RX_TL | Receive FIFO threshold |  |
| 0x3c | IC_TX_TL | Transmit FIFO threshold |  |

| 0x40 | IC_CLR_INTR | Clear Interrupt |  |
| --- | --- | --- | --- |
| 0x44 | IC_CLR_RX_UNDER | Clear interrupt RX_UNDER |  |
| 0x48 | IC_CLR_RX_OVER | Clear interrupt RX_OVER |  |
| 0x4c | IC_CLR_TX_OVER | Clear interrupt TX_OVER |  |
| 0x50 | IC_CLR_RD_REQ | Clear interrupt RD_REQ |  |
| 0x54 | IC_CLR_TX_ABRT | Clear interrupt TX_ABRT |  |
| 0x58 | IC_CLR_RX_DONE | Clear interrupt RX_DONE |  |
| 0x5c | IC_CLR_ACTIVITY | Clear interrupt activity |  |
| 0x60 | IC_CLR_STOP_DET | Clear interrupt STOP_DET |  |
| 0x64 | IC_CLR_START_DET | Clear interrupt START_DET |  |
| 0x6c | IC_ENABLE | I2C Enable register |  |
| 0x70 | IC_STATUS | I2C Status Register |  |
| 0x74 | IC_TXFLR | Transmit FIFO water level register |  |
| 0x78 | IC_RXFLR | Receive FIFO water level register |  |
| 0x80 | IC_TX_ABRT_SOURCE | I2C Transfer end status register |  |
| 0x9c | IC_ENABLE_STATUS | I2C Enable Status Register |  |
| 0xa0 | IC_START | Control data starts sending |  |

###### Register Description

      - IC_CON

Offset address: 0x00 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [15:7] | - | reserved | - | reserve |
| [6] | RW | IC_SLAVE_DISA BLE | 0x1 | I2C functions as a slave after reset . 0 : salve enabled 1 : slave disabled |
| [5] | RW | IC_RESTART_EN | 0x1 | When I2C is used as a master , does it support RESTART ? 0 : Off 1 : Enable |
| [4] | RW | IC_10BITADDR_ MASTER | 0x1 | When acting as a master , the address mode of I2C transmission. 0 : 7-bit address mode 1 : 10-bit address mode |
| [3] | RW | IC_10BITADDR_ SLAVE | 0x1 | acting as a slave , controls the address mode of the I2C response. 0 : 7-bit address mode 1 : 10-bit address mode |
| [2:1] | RW | SPEED | 0x2 | Controls the speed mode of I2C . 1 : Standard mode ( 100 kbit/s ) 2 : Fast mode ( 400 kbit/s ) 3 : High speed mode ( 3.4 Mbit/s ) (Not supported ) |
| [0] | RW | MASTER_MODE | 0x1 | I2C is enabled as a master . 0 : Master is turned off |

|  |  |  |  | 1 : Master enabled |
| --- | --- | --- | --- | --- |

      - IC_TAR

Offset address: 0x04 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [15:12] | - | reserved | - | reserve |
| [11] | RW | SPECIAL | 0x0 | Indicates whether the software executes a general call or start byte command. ( Not supported These two functions ) |
| [10] | RW | GC_OR_START | 0x0 | only when bit 11 is set to 1. It indicates whether the software is executing a general call or a start byte command. 0 : Broadcast call 1 : Start byte |
| [9:0] | RW | IC_TAR | 0x55 | Indicates the target slave address when I2C is used as the master . |

      - IC_DATA_CMD

Offset address: 0x10 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [15:9] | - | reserved | - | reserve |
| [8] | WO | CMD | 0x0 | I2C is used as a master , read and write control bits. 1 : Read 0 : Write |
| [7:0] | RW | DAT | 0x0 | Data sent to or received from the I2C bus . |

      - IC_FS_SCL_HCNT

Offset address: 0x1C Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [15:0] | RW | IC_FS_SCL_HCN T | 0x5a | Set in fast mode, SCL clock is active high when ic_clk Number of cycles |

      - IC_FS_SCL_LCNT

Offset address: 0x20 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [15:0] | RW | IC_FS_SCL_LCN T | 0xa0 | Set in fast mode, SCL clock is active low when ic_clk Number of cycles |

      - IC_INTR_STAT

Offset address: 0x2C Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [15:12] | - | reserved | - | reserve |
| [11] | RO | R_GEN_CALL | 0x0 | IC_RAW_INTR_STAT register for detailed explanation |
| [10] | RO | R_START_DET | 0x0 |  |
| [9] | RO | R_STOP_DET | 0x0 |  |
| [8] | RO | R_ACTIVITY | 0x0 |  |
| [7] | RO | R_RX_DONE | 0x0 |  |
| [6] | RO | R_TX_ABRT | 0x0 |  |
| [5] | RO | R_RD_REQ | 0x0 |  |
| [4] | RO | R_TX_EMPTY | 0x0 |  |
| [3] | RO | R_TX_OVER | 0x0 |  |
| [2] | RO | R_RX_FULL | 0x0 |  |
| [1] | RO | R_RX_OVER | 0x0 |  |
| [0] | RO | R_RX_UNDER | 0x0 |  |

      - IC_INTR_MASK

Offset address: 0x30 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [15:12] | - | reserved | - | reserve |
| [11] | RW | M_GEN_CALL | 0x8ff | IC_RAW_INTR_STAT register for detailed explanation |
| [10] | RW | M_START_DET | 0x8ff |  |
| [9] | RW | M_STOP_DET | 0x8ff |  |
| [8] | RW | M_ACTIVITY | 0x8ff |  |
| [7] | RW | M_RX_DONE | 0x8ff |  |
| [6] | RW | M_TX_ABRT | 0x8ff |  |
| [5] | RW | M_RD_REQ | 0x8ff |  |
| [4] | RW | M_TX_EMPTY | 0x8ff |  |
| [3] | RW | M_TX_OVER | 0x8ff |  |
| [2] | RW | M_RX_FULL | 0x8ff |  |
| [1] | RW | M_RX_OVER | 0x8ff |  |
| [0] | RW | M_RX_UNDER | 0x8ff |  |

      - IC_RAW_INTR_STAT

Offset address: 0x34 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [15:12] | - | reserved | - | reserve |

| [11] | RO | GEN_CALL | 0x0 | Valid when receiving a broadcast call address. (Not supported ) |
| --- | --- | --- | --- | --- |
| [10] | RO | START_DET | 0x0 | master or slave mode, it indicates whether START or RESTART |
| [9] | RO | STOP_DET | 0x0 | master or slave mode, indicates whether STOP is detected |
| [8] | RO | ACTIVITY_NEG E | 0x0 | Indicates that I2C changes from active state to idle state. (i.e., I2C active state Falling edge ) |
| [7] | RO | RX_DONE | 0x0 | When I2C is used as a slave transmitter, if the master does not have an ACK sent byte, this position is 1 , indicating the end of transmission. |
| [6] | RO | TX_ABRT | 0x0 | I2C is used as a transmitter and the transmission is interrupted. See the cause for more information. IC_TX_ABRT_SOURCE Register |
| [5] | RO | RD_REQ | 0x0 | When I2C is used as a slave and other masters read data from I2C , this error will occur. When this interrupt is valid, I2C pulls SCL low and keeps it until the CPU responds to this interrupt and writes data to the IC_DATA_CMD register. |
| [4] | RO | TX_EMPTY | 0x0 | When the transmit FIFO data is lower than or equal to the threshold set in the IC_TX_TL register When the transmit FIFO threshold is exceeded, a transmit FIFO threshold interrupt will be generated. |
| [3] | RO | TX_OVER | 0x0 | When the transmit FIFO reaches the set depth and the processor continues to write data, Generates a transmit overflow interrupt. |
| [2] | RO | RX_FULL | 0x0 | When the receive FIFO data is greater than or equal to the threshold set by the IC_RX_TL register When the receive FIFO threshold value is reached, a receive FIFO threshold interrupt will be generated. |
| [1] | RO | RX_OVER | 0x0 | When the receive FIFO is full and continues to receive data, an overflow interrupt is generated. |
| [0] | RO | RX_UNDER | 0x0 | When the receive FIFO is empty and the processor reads data, an underflow interrupt is generated. |

      - IC_RX_TL

Offset address: 0x38 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [15:8] | - | reserved | - | reserve |
| [7:0] | RW | TX_TL | 0x6 | Set the receive FIFO threshold, ranging from 0 to 7. When the threshold is set to 7 , the receive FIFO data reaches 7 , which triggers the RX_FULL interrupt; when the threshold is set to 0-6 , RX_FULL interrupt will be triggered only when the value is greater than this threshold . |

      - IC_TX_TL

Offset address: 0x3C Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [15:8] | - | reserved | - | reserve |
| [7:0] | RW | TX_TL | 0x6 | Set the threshold of the transmit FIFO , ranging from 0 to 7. If the threshold is less than or equal to the threshold, the Send TX_EMPTY interrupt. |

      - IC_CLR_INTR

Offset address: 0x40

Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [15:1] | - | reserved | - | reserve |
| [0] | RO | CLR_INTR | 0x0 | Reading this register clears combined interrupts, all single interrupts, and IC_TX_ABRT_SOURCE Register. Only software-clearable interrupts can be cleared . |

      - IC_CLR_RX_UNDER

Offset address: 0x44 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [15:1] | - | reserved | - | reserve |
| [0] | RO | CLR_RX_UNDER | 0x0 | Clear the RX_UNDER interrupt. |

      - IC_CLR_RX_OVER

Offset address: 0x48 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [15:1] | - | reserved | - | reserve |
| [0] | RO | CLR_RX_OVER | 0x0 | Clear the RX_OVER interrupt. |

      - IC_CLR_TX_OVER

Offset address: 0x4C Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [15:1] | - | reserved | - | reserve |
| [0] | RO | CLR_TX_OVER | 0x0 | Clears the TX_OVER interrupt. |

      - IC_CLR_RD_REQ

Offset address: 0x50 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [15:1] | - | reserved | - | reserve |
| [0] | RO | CLR_RD_REQ | 0x0 | Clear RD_REQ Interrupt. |

      - IC_CLR_TX_ABRT

Offset address: 0X54 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |

| [15:1] | - | reserved | - | reserve |
| --- | --- | --- | --- | --- |
| [0] | RO | CLR_TX_ABRT | 0x0 | Reading the register can clear the TX_ABRT interrupt and IC_TX_ABRT_SOURCE register. |

      - IC_CLR_RX_DONE

Offset address: 0x58 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [15:1] | - | reserved | - | reserve |
| [0] | RO | CLR_RX_DONE | 0x0 | Clear RX_DONE Interrupt. |

      - IC_CLR_ACTIVITY

Offset address: 0x5c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [15:1] | - | reserved | - | reserve |
| [0] | RO | CLR_ACTIVITY | 0x0 | Clear ACTIVITY Interrupt. |

      - IC_CLR_STOP_DET

Offset address: 0x60 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [15:1] | - | reserved | - | reserve |
| [0] | RO | CLR_STOP_DET | 0x0 | Clear STOP_DET Interrupt. |

      - IC_CLR_START_DET

Offset address: 0x64 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [15:1] | - | reserved | - | reserve |
| [0] | RO | CLR_START_DE T | 0x0 | Clear START_DET Interrupt. |

      - IC_ENABLE

Offset address: 0x6c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [15:1] | - | reserved | - | reserve |

| [0] | RW | ENABLE | 0x0 | Controls I2C enable. 0 : I2C off 1 : I2C enabled |
| --- | --- | --- | --- | --- |

      - IC_STATUS

Offset address: 0x70 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:7] | - | reserved | - | reserve |
| [6] | RO | SLV_ACTIVITY | 0x0 | The slave state machine is active. 0 : The slave state machine is in idle state 1 : The slave state machine is not in idle state |
| [5] | RO | MST_ACTIVITY | 0x0 | The master state machine is active. 0 : The master state machine is in idle state 1 : The master state machine is not in idle state |
| [4] | RO | RFF | 0x0 | The receive FIFO is completely full. 0 : Receive FIFO is not full 1 : Receive FIFO full |
| [3] | RO | RFNE | 0x0 | The receive FIFO is not empty. 0 : Receive FIFO empty 1 : Receive FIFO is not empty |
| [2] | RO | TFE | 0x1 | The transmit FIFO is completely empty. 0 : Transmit FIFO is not empty 1 : Transmit FIFO empty |
| [1] | RO | TFNF R | 0x1 | The transmit FIFO is not full. 0 : Transmit FIFO full 1 : Transmit FIFO is not full |
| [0] | RO | ACTIVITY | 0x0 | I2C is active |

      - IC_TXFLR

Offset address: 0x74 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:4] | - | reserved | - | reserve |
| [3:0] | RO | TXFLR | 0x0 | Number of valid data in the transmit FIFO |

      - IC_RXFLR

Offset address: 0x78 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:4] | - | reserved | - | reserve |

| [3:0] | RO | RXFLR | 0x0 | Number of valid data in receive FIFO |
| --- | --- | --- | --- | --- |

      - IC_TX_ABRT_SOURCE

Offset address: 0x80 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | - | reserved | - | reserve |
| [15] | RO | ABRT_SLVRD_I NTX | 0x0 | When I2C is used as a slave-transmitter , I2C requests to send data, and the processor Write and read commands to the transmit FIFO . |
| [14] | RO | ABRT_SLV_ARB LOST | 0x0 | As a Slave-Transmitter , when transmitting data to the master , the loss of cut. |
| [13] | RO | ABRT_SLVFLUS H_TXFIFO | 0x0 | As a Slave-Transmitter , I2C receives a read command and sends the data in the FIFO. There is already some data, and a TX_ABRT interrupt will be initiated to release the original data |
| [12] | RO | ARB_LOST | 0x0 | acting as a Master-Transmitter or Slave-Transmitter , the cut |
| [11] | RO | ABRT_MASTER_ DIS | 0x0 | acting as a master , access the master when the master mode is shut down . |
| [10] | RO | ABRT_10B_RD_ NORSTRT | 0x0 | As Master-Receiver , the RESTART function is turned off, but in the 10- bit Sending a read command in address mode |
| [9] | RO | ABRT_SBYTE_N ORSTRT | 0x0 | As a master , RESTART is turned off, and the start byte is sent |
| [8] | RO | ABRT_HS_NORS TRT | 0x0 | As a master , the RESTART function is turned off, but an attempt is made to operate in high-speed mode Transfer data |
| [7] | RO | ABRT_SBYTE_A CKDET | 0x0 | As a master , I2C sends the start byte, and the start byte is acked ( positive Normally, the start byte does not need ack ) |
| [6] | RO | ABRT_HS_ACKD ET | 0x0 | acting as a master , when I2C is in high-speed mode, the high-speed master code is acked . ack is required under normal circumstances ) |
| [5] | RO | ABRT_GCALL_R EAD | 0x0 | As a master-tansmitter , I2C issues a general call when reading data |
| [4] | RO | ABRT_GCALL_N OACK | 0x0 | As a master-tansmitter , the broadcast call address issued has no slave response. |
| [3] | RO | ABRT_TXDATA_ NOACK | 0x0 | acting as a master-tansmitter , the address sent gets ack , but then The data sent out does not receive ack from the slave |
| [2] | RO | ABRT_10ADDR2 _NOACK | 0x0 | As a master , the address mode is 10-bit mode, and the second address issued is No response was received from any slave . |
| [1] | RO | ABRT_10ADDR1 _NOACK | 0x0 | I2C is used as a master , the address mode is 10-bit mode, and the first The address did not receive any slave response. |
| [0] | RO | ABRT_7B_ADDR _NOACK | 0x0 | I2C is used as a master , the address mode is 7-bit mode, and the address sent is not response was received from any slave . |

      - IC_ENABLE_STATUS

Offset address: 0x9c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:3] | - | reserved | - | reserve |
| [2] | RO | SLV_RX_DATA_ LOST | 0x0 | only when bit0 is 0 , indicating that in slave-reveive mode, after receiving >= 1 byte of data, the I2C enable is turned off and the data transmission is abandoned. When it is set to 1 , it means that the transmission has entered the data stage and after receiving at least 1 byte of data, the I2C enable is turned off. Regardless of whether ack is received , the transmission is abandoned. When it is set to 0 , it means that when I2C is turned off, The data transmission phase has not yet begun. ( I2C is not supported as a slave .) |
| [1] | RO | SLV_DISABLED_ WHILE_BUSY | 0x0 | This bit is valid only when bit0 is 0. It indicates that when I2C is used as a slave , the data transmission is terminated because the I2C enable is turned off. When I2C is receiving the address byte or Slave-Receiver mode, when I2C is receiving address and data bytes, the CPU writes 0 to the IC_ENABLE register , which will cause this position to be set to 1. (Not supported) I2C as slave )​ |
| [0] | RO | IC_EN | 0x0 | Indicates whether I2C is enabled. 0 : I2C is inactive 1 : I2C enabled |

      - IC_START

Offset address: 0xa0 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [15:1] | - | reserved | - | reserve |
| [0] | RW | START | 0x0 | Controls the I2C to start sending data. This register must be configured after the IC_ENABLE register is enabled. When there is data in the FIFO , writing 1 to this register will start sending data. 0 : Do not start 1 : Start |

### SPI

###### Overview

The SPI controller realizes serial-to-parallel and parallel-to-serial conversion of data, and can perform synchronous serial communication with external devices as a Master or Slave .

###### Functional Description

##### Features

SPI The interface has the following characteristics:

- Multiple interrupts and interrupt mask registers;

- The chip has two sets of SPI interfaces, each of which supports a maximum of 2 slaves ;

- Support configurable interrupt source priority;

- Programmable clock frequency;

- with APB bus clock;

- Data transmission / reception 16-bit width FIFO independent, FIFO depth 128 ( spi_slave FIFO depth is 8 ) ;

- Support SPI full-duplex working mode, support data frame transmission polarity and phase adjustment;

- Provide internal self-loop mode test;

- Supports 4~16bit software configurable SPI transmission mode;

- Supports configurable SCLK polarity and phase.

##### Typical Applications

SPI_MOSI SPI_MISO SPI_CLK

SPI_CS_n

slave1

SPI_MOSI SPI_MISO SPI_CLK

SPI_CS_n

slave0

SPI_CS1_n

CHIP

SPI_MOSI

SPI_MISO SPI_CLK

SPI_CS0_n

SPI The module acts as a master device, connecting two slaves The typical application of the device is shown in the figure below:

Figure 23 Master SPI Typical connection method

CHIP

APB1 BUS

apb_ssi_ slave

Figure 24 Slave SPI Typical connection method

##### Configurable clock polarity and phase

SPI Output clock SCLK_OUT The polarity and phase are configurable via registers ( CTRL0 bit [ 7 : 6 ] ) , SCPH represents the phase , when SCPH Configured as 0 , it means that the SCLK_OUT The first clock edge samples data, which is 1 Indicates that SCLK_OUT Data is sampled on the second clock edge of SCPOL . Indicates SCLK_OUT The polarity of SCPOL Configured to 0 ,

Indicates SCLK_OUT When invalid, it is low level, 1 Indicates SCLK_OUT High level when invalid. Docking SLAVE

The same applies to equipment.

Figure 25 SCPH=0 Data transmission and sampling timing

In the figure above, SCPH=0 , data sampling is on the first edge of sclk_out , and data transmission starts on the falling edge of the chip select signal ss_0_n .

Figure 26 SCPH=1 Data transmission and sampling timing

In the figure above, SCPH=1 , data sampling is at sclk_out The second edge of sclk_out The first edge of

###### How it works

##### Transmission Timing

Figure 27 SPI Data serial timing

Serial data is generated on the falling edge of the clock and sampled on the rising edge of the clock.

After the same command is executed, the chip select needs to be canceled and the chip select needs to be re-enabled for the next command. Canceling the chip select during the processing of the same command will interrupt the command.

In order to adapt to a variety of devices, the module can set the data serial transmission order by setting FRF Register implementation: 0 , from

MSB Start; 1 , from LSB start.

##### Clock and reset

Output SPI Clock SPI_CLK The clock frequency is calculated as follows:

F sclk_out = F pclk / spi_baudr

F pclk ： SPI The working reference clock, APB Bus clock.

spi_baudr Please consult register BAUDR .

##### Operation process

###### initialization

The initialization steps are as follows:

1. Set SPI_SSIENR 0x0 , disable SPI module;

2. Write register SPI_CTRLR0 to configure the frame format and transmission data width and other parameters . and EEPROM Read Transfer mode ) , skip to step 3 Then jump to step 4. If you only need to send data and do not need to receive data, jump to step 4 ;

3. Write register SPI_CTRLR1 to set the number of received data;

4. Configure the register SPI_BAUDR to configure the clock division factor;

5. In interrupt mode, set register SPI_IMR to enable the corresponding interrupt; in query mode, the corresponding interrupt should be disabled;

6. In interrupt mode, set register SPI_TXFTLR and SPI_RXFTLR .

for data transmission in query mode are as follows:

1. Set register SPI_SSIENR Register 0x1 , enable SPI module;

2. Set register SPI_SER The register is 0x1 , enabling the corresponding slave ;

3. Write the data to be sent to the SPI_DR register continuously ;

4. Poll the SPI_SR register until bit0[busy]=0 (not busy ) and [TFE]=1 ( transmit FIFO Empty )​

[RFNE]=1 ( indicates receiving FIFO Not empty ) , go to step 6 ; 5. Read data and ensure that the receiving FIFO is empty ;

6. Configuration register SPI_SSIENR The register is 0x0 , disabling SPI ;

7. Configuration register SPI_SER Register is 0x0 , prohibiting slave communication.

###### Data transmission in interrupt mode

1. Set register SPI_SSIENR Register 0x1 , enable SPI module;

2. Set register SPI_SER The register is 0x1 , enabling the corresponding slave ;

3. Write the data to be sent to the SPI_DR register continuously ;

4. Wait for interrupt RXFIS ( receive FIFO Data exceeds SPI_RXFTLR Set value), read out data, and loop until all data are read ;

5. Configuration register SPI_SSIENR The register is 0x0 , disabling SPI ;

6. Configuration register SPI_SER Register is 0x0 , prohibiting slave communication.

      - SPI Module 4 transmission mode

- Transmit & Receive model

Transmit & Receive Mode, SPI The module sends data and receives data at the same time, and the number of sent data is equal to the number of received data.

Figure 28 SPI Transmit & Receive model

- Transmit Only model

Transmit Only mode, the SPI module only transmits data; the received data is not placed in the receive FIFO after passing through the shift register .

Figure 29 SPI Transmit Only model

- Receive Only model

Receive Only Mode, SPI The module only receives data; however, in order to start data reception, it is necessary to send FIFO

If you write any data into the , the data will be sent out repeatedly.

Figure 30 SPI Receive Only model

- EEPROM Read Transfer Mode ( Master SPI only )

EEPROM Read Transfer Mode, SPI The module first sends FIFO After the data in the FIFO is sent out, the data will be received; no data will be received during the data sending process. Note that the data to be sent must be filled into the FIFO first . After that, enable the corresponding slave device in chip select.

NOTE: EEPROM Read Transfer In this mode, the command length is at least two beats, that is, sending a command requires at least writing to the FIFO

Twice; if only once, the first beat of the data read back ( frame width) will not be written to the FIFO middle.

Figure 31 SPI EEPROM Read Transfer model

###### Register Overview

Register Table 11 SPI0 Register Overview (Base Address: 0x140A_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |

| 0x00 | CTRLR0 | Transmission mode and frame length configuration |  |
| --- | --- | --- | --- |
| 0x04 | CTRLR1 | Number of received data frames |  |
| 0x08 | SSIENR | SPI module enable |  |
| 0x10 | SER | slave Chip select enable |  |
| 0x14 | BAUDR | Clock enable and frequency division selection |  |
| 0x18 | TXFTLR | Transmit FIFO threshold |  |
| 0x1C | RXFTLR | Receive FIFO threshold |  |
| 0x20 | TXFLR | Number of data in the transmit FIFO |  |
| 0x24 | RXFLR | Number of data in receive FIFO |  |
| 0x28 | SR | SPI module status |  |
| 0x2C | IMR | Interrupt mask register |  |
| 0x30 | ISR | Masked interrupt register |  |
| 0x34 | RISR | Unmasked interrupt register |  |
| 0x38 | TXOICR | Transmit FIFO overflow Interrupt clear |  |
| 0x3C | RXOICR | Receive FIFO overflow Interrupt clear |  |
| 0x40 | RXUICR | Receive FIFO underflow Interrupt clear |  |
| 0x48 | ICR | Clear ssi_txo_intr , ssi_rxu_intr , ssi_rxo_intr Interrupt |  |
| 0x58 | IDR | Module ID Register |  |
| 0x5C | SSI_COMP_VERSION | Version number |  |
| 0x60 | DR | Data transmit / receive FIFO interface |  |

Register Table 12 SPI1 Register Overview ( Base Address: 0x140B_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x00 | CTRLR0 | Transmission mode and frame length configuration |  |
| 0x04 | CTRLR1 | Number of received data frames |  |
| 0x08 | SSIENR | SPI module enable |  |
| 0x10 | SER | slave Chip select enable |  |
| 0x14 | BAUDR | Clock enable and frequency division selection |  |
| 0x18 | TXFTLR | Transmit FIFO threshold |  |
| 0x1C | RXFTLR | Receive FIFO threshold |  |
| 0x20 | TXFLR | Number of data in the transmit FIFO |  |
| 0x24 | RXFLR | Number of data in receive FIFO |  |
| 0x28 | SR | SPI module status |  |
| 0x2C | IMR | Interrupt mask register |  |
| 0x30 | ISR | Masked interrupt register |  |
| 0x34 | RISR | Unmasked interrupt register |  |
| 0x38 | TXOICR | Transmit FIFO overflow Interrupt clear |  |
| 0x3C | RXOICR | Receive FIFO overflow Interrupt clear |  |
| 0x40 | RXUICR | Receive FIFO underflow Interrupt clear |  |
| 0x48 | ICR | Clear ssi_txo_intr , ssi_rxu_intr , ssi_rxo_intr Interrupt |  |
| 0x58 | IDR | Module ID Register |  |
| 0x5C | SSI_COMP_VERSION | Version number |  |

| 0x60 | DR | Data transmit / receive FIFO interface |  |
| --- | --- | --- | --- |

Register Table 13 SPI2 Register Overview ( Base Address: 0x140E_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x00 | CTRLR0 | Transmission mode and frame length configuration |  |
| 0x04 | CTRLR1 | Number of received data frames |  |
| 0x08 | SSIENR | SPI module enable |  |
| 0x10 | SER | slave Chip select enable |  |
| 0x14 | BAUDR | Clock enable and frequency division selection |  |
| 0x18 | TXFTLR | Transmit FIFO threshold |  |
| 0x1C | RXFTLR | Receive FIFO threshold |  |
| 0x20 | TXFLR | Number of data in the transmit FIFO |  |
| 0x24 | RXFLR | Number of data in receive FIFO |  |
| 0x28 | SR | SPI module status |  |
| 0x2C | IMR | Interrupt mask register |  |
| 0x30 | ISR | Masked interrupt register |  |
| 0x34 | RISR | Unmasked interrupt register |  |
| 0x38 | TXOICR | Transmit FIFO overflow Interrupt clear |  |
| 0x3C | RXOICR | Receive FIFO overflow Interrupt clear |  |
| 0x40 | RXUICR | Receive FIFO underflow Interrupt clear |  |
| 0x48 | ICR | Clear ssi_txo_intr , ssi_rxu_intr , ssi_rxo_intr Interrupt |  |
| 0x58 | IDR | Module ID Register |  |
| 0x5C | SSI_COMP_VERSION | Version number |  |
| 0x60 | DR | Data transmit / receive FIFO interface |  |

###### Register Description

      - CTRLR0

Offset address: 0x00 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:12] | RO | reserved | 0x0 | Reserve |
| [11] | RW | SRL | 0x0 | Self-loop test mode |
| [10] | RW | eENDING ( as master ) | 0x0 | Endian control, 1 : big endian; 0 : little endian |
| [10] | RW | SLV_OE ( as slave ） | 0x0 | the slave 's txd is enabled. 0 : enable ; 1 : disable |
| [9:8] | RW | TMOD | 0x0 | Transmission Mode: 00-Transmit & Receive 01-Transmit Only - Receive Only - EEPROM Read ( master only ) SPI ) |

| [7] | RW | SCPOL | 0x0 | Clock polarity, indicating the level state when the clock is invalid. 0 : SCLK is invalid 1 : It is in low level state when CLK is invalid; 2: It is in high level state when CLK is invalid. |
| --- | --- | --- | --- | --- |
| [6] | RW | SCPH | 0x0 | Clock phase. 0 : Indicates that data is sampled on the first edge of SCLK ; 1 : on the second edge of SCLK . |
| [5] | RW | M2L | 0x0 | Select the sending order: 0 , start from MSB ; 1 , start from LSB |
| [4:0] | RW | DFS | 0x7 | Data frame length |
| [5 : 4] | RW | FRF ( slave ) | 0x0 | Select the serial protocol. 00 : Motorola SPI ( fixed to SPI ) |
| [3:0] | RW | DFS ( slave ) | 0x7 | Data frame length |

      - CTRLR1 ( for master only )

Offset address: 0x04 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RO | reserved | 0x0 | Reserve |
| [15:0] | RW | NDF | 0x0 | Number of data received |

      - SSIENR

Offset address: 0x08 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | RO | reserved | - | Reserve |
| [0] | RW | SSI_EN | 0x0 | SPI module enable, disable configuration register when 1 , clear FIFO when 0 and status register |

      - SER ( for master only )

Offset address: 0x10 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:2] | RO | reserved | 0x0 | Reserve |
| [1:0] | RW | SER | 0x0 | slave chip select |

      - BAUDR ( master only )

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RO | reserved | - | 预留 |
| [15:0] | RW | SCKDV | 0x0 | 输出时钟分频，只允许偶数分频，为0时禁用时钟 |

Offset address: 0x14 Reset mode: H/S

      - TXFTLR

Offset address: 0x18 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:8] | RO | reserved | - | Reserve |
| [7:0] | RW | TFT | 0x0 | Transmit data FIFO threshold |

      - RXFTLR

Offset address: 0x1c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:8] | RO | reserved | 0x0 | Reserve |
| [7:0] | RW | RFT | 0x0 | Receive data FIFO threshold |

      - TXFLR

Offset address: 0x20 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:8] | RO | reserved | 0x0 | Reserve |
| [7:0] | RO | TXTFL | 0x0 | Number of data in the transmit data FIFO |

      - RXFLR

Offset address: 0x24 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:8] | RO | reserved | 0x0 | Reserve |
| [7:0] | RO | RXTFL | 0x0 | Number of data in the receive data FIFO |

      - SR

Offset address: 0x28 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:5] | RO | reserved | 0x0 | Reserve |
| [4] | RO | RFF | 0x0 | Receive fifo is full. 0 : not full; 1 : full |
| [3] | RO | RFNE | 0x0 | Receive FIFO is not empty. 0 : empty; 1 : not empty |
| [2] | RO | TFE | 0x1 | Transmit FIFO is empty. 0 : Not empty; 1 : Empty |
| [1] | RO | TFNF | 0x1 | Transmit FIFO is not full. 0 : full; 1 : not full |
| [0] | RO | BUSY | 0x0 | Transferring data |

      - IMR

Offset address: 0x2c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:6] | RO | reserved | 0x0 | Reserve |
| [5] | RW | reserved | 0x1 | Reserve |
| [4] | RW | RXFIM | 0x1 | Receive data exceeds threshold interrupt mask |
| [3] | RW | RXOIM | 0x1 | Receive FIFO Overflow interrupt shield |
| [2] | RW | RXUIM | 0x1 | Receive FIFO underflow interrupt shield |
| [1] | RW | TXOIM | 0x1 | Transmit FIFO Overflow interrupt shield |
| [0] | RW | TXEIM | 0x1 | Transmit FIFO Null interrupt mask |

      - ISR

Offset address: 0x30 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:5] | RO | reserved | 0x0 | Reserve |
| [4] | RO | RXFIS | 0x0 | Masked receive data exceeds threshold interrupt |
| [3] | RO | RXOIS | 0x0 | Masked receive FIFO overflow interrupt |
| [2] | RO | RXUIS | 0x0 | Masked receive FIFO underflow interrupt |
| [1] | RO | TXOIS | 0x0 | Masked transmit FIFO overflow interrupt |
| [0] | RO | TXEIS | 0x0 | Masked transmit FIFO Empty Interrupt |

      - RISR

Offset address: 0x34 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:5] | RO | reserved | 0x0 | Reserve |
| [4] | RO | RXFIR | 0x0 | Unmasked receive data exceeds threshold interrupt |
| [3] | RO | RXOIR | 0x0 | Unmasked receive FIFO overflow interrupt |
| [2] | RO | RXUIR | 0x0 | Unmasked receive FIFO underflow interrupt |
| [1] | RO | TXOIR | 0x0 | Unmasked transmit FIFO overflow interrupt |
| [0] | RO | TXEIR | 0x0 | Unmasked transmit FIFO Empty Interrupt |

      - TXOICR

Offset address: 0x38 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | RO | reserved | 0x0 | Reserve |
| [0] | RO | TXOICR | 0x0 | Clear transmit FIFO overflow interrupt |

      - RXOICR

Offset address: 0x3c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | RO | reserved | 0x0 | Reserve |
| [0] | RO | RXOICR | 0x0 | Clear receive FIFO overflow interrupt |

      - RXUICR

Offset address: 0x40 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | RO | reserved | 0x0 | Reserve |
| [0] | RO | RXUICR | 0x0 | Clear receive FIFO underflow interrupt |

      - ICR

Offset address: 0x48 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | RO | reserved | 0x0 | Reserve |
| [0] | RO | ICR | 0x0 | Clear transmit FIFO Overflow interrupt, receive FIFO overflow interrupt, Receive FIFO underflow interrupt |

      - IDR

Offset address: 0x58 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | IDR | 0x1122334 4 | Module ID |

      - SSI_COMP_VERSION

Offset address: 0x5c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | SSI_COMP_VERS ION | 0x3331312 a | Module version number |

      - DR

Offset address: 0x60

Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | DR | 0x0 | FIFO read / write address |

### I80

###### Overview

HR_C7000 Built-in Intel I80 Interface controller for LCD And other device interfaces.

###### Functional Description

I80 The controller is connected to the system AHB on the chip Bus, connected to the external I80 outside the chip Interface devices.

###### How it works

I80 The controller automatically converts operations on its registers into I80 compliant The timing required by the bus. I80 There are multiple configuration registers inside the controller , which can realize flexible configuration of various timing parameters and level polarity of I80 .

###### Register Overview

Figure 32 8080 Interface Timing

Register Table 14 I80 Register Overview ( Base Address: 0x1200_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x0 | INDEX | Addressing register |  |

| 0x4 | DATA | Data register |  |
| --- | --- | --- | --- |
| 0x10 | WCFG | Write timing parameter configuration |  |
| 0x14 | RCFG | Read timing parameter configuration |  |
| 0x20 | SCFG | Configuration registers |  |
| 0x2c | AC_MODE | Bus Mode Register |  |

###### Register Description

      - INDEX

Offset address: 0x00 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | INDEX0 | 0x0 | Port Index Register |

      - DATA

Offset address: 0x04 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | DATA0 | 0x0 | Port data register |

      - WCFG

Offset address: 0x10 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | RO | reserved | 0x0 | reserve |
| [23:16] | RW | TCNT_WHT0 | 0x1 | Write strobe tail invalid cycle |
| [15:8] | RW | TCNT_WL0 | 0x1 | Write strobe valid period |
| [7:0] | RW | TCNT_WHF0 | 0x1 | Write strobe header invalidation cycle |

      - RCFG

Offset address: 0x14 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | reserved | 0x0 | reserve |
| [23:16] | RW | TCNT_RHT0 | 0x1 | Read strobe tail invalid cycle |
| [15:8] | RW | TCNT_RL0 | 0x1 | Read strobe valid period |
| [7:0] | RW | RCNT_RHF0 | 0x1 | Read strobe header invalidation cycle |

      - SCFG

Offset address: 0x20 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | P0_SCFG | 0x0 | Port Configuration Register |

      - AC_MODE

Offset address: 0x2c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | BUSHS | 0x0 | Bus Mode Control Register |

### UART

###### Overview

Universal Asynchronous Receiver Transmitter ( UART ) is an asynchronous serial communication interface. Its main function is to convert data from peripheral devices into serial-to-parallel data and then transmit it to the internal bus, and to convert data into parallel-to-serial data and then output it to external devices. The main function is to communicate with the UART of the external chip The HR_C7000 provides three UART units. By default, the pull - up and pull-down enable of RXD and TXD are disabled. When the UART function is not used , it is recommended to enable the pull-up and pull-down enable registers in software configuration and fix RXD to high or low level on the board-level port.

###### Functional Description

UART A frame transmission mainly includes the start signal, data, check bit and end signal, as shown in the function description . TXD​ Output from another UART RXD​ Terminal input.

Figure 33 UART Frame format

The meanings of the start signal, data, check bit, and end signal are as follows:

- Start signal bit )

A data frame starts, UART The agreement stipulates TXD A low level signal indicates the beginning of a data frame. When not transmitting data, it should remain high.

- Data signal bit )

The data bit width can be adjusted according to different application requirements and can be configured as 5bit/6bit/7bit/8bit Data bit width.

- parity​ bit )

The check digit is 1 Bit error correction signal, UART The parity bits include odd parity, even parity and fixed parity bits, and support enabling and disabling of parity bits.

- End signal ( stop bit )

The end signal is the stop bit of the data frame, supporting 1 Bit and 2 There are two configurations of bit stop bit. The end signal of the data frame is to put TXD Pull to high level.

###### How it works

Baud rate configuration

Baud rate calculation formula : Current baud rate = UART Reference clock frequency ( APB Bus Clock ) ( / 	16 x Frequency division coefficient ) .

The frequency division coefficient is configured by registers UART_DLL and UART_DLH .​ Configure the lower 8 bits and U A R T _ D L H configure the upper 8 bits.

For example, if the baud rate is configured to 9600 and the working clock pclk is 25M , the coefficient calculated by the above formula is hexadecimal A2 , that is, UART_DLL is configured to 0xa2 and UART_DLH is configured to 0x0 .

###### initialization :

1. Configure the interrupt enable register UART_IER to mask all interrupts;

2. Configure the baud rate divisor latch enable UART_LCR[7] , the number of stop bits UART_LCR[2] and the number of data bits UART_LCR[1:0] ;

3. Write registers UART_DLL and UART_DLH to configure the baud rate divisor;

4. After configuring the baud rate, clear UART_LCR[7] , that is, write UART_LCR[7]=0 ;

5. Configure the UART_FCR register . Write UART_FCR[0]=1 to enable the FIFO ; write UART_FCR[5 : 4] and UART_FCR[7 : 6] to configure the transmit FIFO and receive FIFO thresholds, respectively .

###### Data sending steps:

1. Write the transmit data into the UART_THR register to start data transmission.

2. Query mode, poll the status bit UART_LSR[5] to check whether the FIFO is full and decide whether to continue sending data to the FIFO . Interrupt mode, when an interrupt is received, read the interrupt ID Register UART_IIR , handles the read interrupt (the read interrupt is the highest priority ) and detects whether the transmit FIFO is suspended Threshold empty interrupt, if suspended, continue to send FIFO Write data and query register UART_LSR[5] to determine the send FIFO If the transmit FIFO is full, stop writing data.

The steps for sending data in interrupt mode are shown in the following flow chart.

是

读寄存器LSR判断

发送FIFO是否满

退出

读中断ID寄存器IIR,

处理挂起中断，检测发送保持寄存器阈值中断

是

写寄存器UART_THR将数据存到FIFO

否

中断入口

Figure 34 Interrupt mode data transmission flow chart

###### Data receiving steps :

In query mode, when receiving data, detect the receiving FIFO by polling UART_LSR[0] Is there data ready? If there is data available, read the data. Interrupt mode: when there is an interrupt pending, read the register UART_IIR , process the interrupt with the highest priority , and detect whether there is an interrupt available for receiving data. If so, read the receive FIFO. The interrupt mode data reception process is shown in the figure below.

中断入口

读寄存器UART_RBR

退出

读寄存器LSR判断

接收FIFO是否空

Read the interrupt ID register IIR, process the pending interrupt, and detect the reception

Data Available Interrupt

It is recommended to set the receive FIFO threshold to 1, that is, whenever

the receive FIFO , read it.

yes

no

是

Figure 35 Interrupt mode data receiving flow chart

###### Register Overview

Register Table 15 UART0 Register Overview (Base Address: 0x1403_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| RBR | 0x00 | Receive buffer register |  |
| THR | 0x00 | Transmit Holding Register |  |
| DLL | 0x00 | Frequency division latch low |  |
| DLH | 0x04 | Frequency division latch high |  |
| IER | 0x04 | Interrupt enable register |  |
| IIR | 0x08 | Interrupt Flag Register |  |
| FCR | 0x08 | FIFO Controller Registers |  |
| LCR | 0x0c | Line Control Register |  |
| MCR | 0x10 | Modem Control registers |  |
| LSR | 0x14 | Line Status Register |  |
| USR | 0x7c | FIFO Status Flags |  |
| TFL | 0x80 | The amount of data in the transmit FIFO |  |
| RFL |  | The amount of data in the receive FIFO . |  |

Register Table 16 UART1 Register Overview (Base Address: 0x1404_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| RBR | 0x00 | Receive buffer register |  |
| THR | 0x00 | Transmit Holding Register |  |
| DLL | 0x00 | Frequency division latch low |  |
| DLH | 0x04 | Frequency division latch high |  |
| IER | 0x04 | Interrupt enable register |  |
| IIR | 0x08 | Interrupt Flag Register |  |
| FCR | 0x08 | FIFO Controller Registers |  |
| LCR | 0x0c | Line Control Register |  |
| MCR | 0x10 | Modem Control registers |  |
| LSR | 0x14 | Line Status Register |  |
| USR | 0x7c | FIFO Status Flags |  |
| TFL | 0x80 | The amount of data in the transmit FIFO |  |
| RFL |  | The amount of data in the receive FIFO . |  |

Register Table 17 UART2 Register Overview (Base Address: 0x1405_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| RBR | 0x00 | Receive buffer register |  |
| THR | 0x00 | Transmit Holding Register |  |
| DLL | 0x00 | Frequency division latch low |  |
| DLH | 0x04 | Frequency division latch high |  |
| IER | 0x04 | Interrupt enable register |  |
| IIR | 0x08 | Interrupt Flag Register |  |
| FCR | 0x08 | FIFO Controller Registers |  |
| LCR | 0x0c | Line Control Register |  |
| MCR | 0x10 | Modem Control registers |  |
| LSR | 0x14 | Line Status Register |  |
| USR | 0x7c | FIFO Status Flags |  |
| TFL | 0x80 | The amount of data in the transmit FIFO |  |
| RFL |  | The amount of data in the receive FIFO . |  |

###### Register Description

      - RBR

Offset address: 0x00 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:8] | - | reserved | - | reserve |
| [7:0] | RO | receive buffer | 0x0 | Receive data from the serial input port |

      - THR

Offset address: 0x00 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:8] | - | reserved | - | reserve |
| [7:0] | WO | Transmit Holding Register | 0x0 | Data sent to the serial port |

      - DLL

Offset address: 0x00 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:8] | - | reserved | - | reserve |
| [7:0] | RW | Divisor Latch (Low) | 0x0 | Lower 8 bits of the baud rate divisor (precondition: LCR[7]=1…b1 ) |

      - DLH

Offset address: 0x04 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:8] | - | reserved | - | reserve |
| [7:0] | RW | Divisor Latch (High) | 0x0 | The high 8 bits of the baud rate divisor (precondition: LCR[7]=1 ' b1 ) |

      - IER

Offset address: 0x04 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:8] | - | reserved | - | reserve |
| [7] | RW | PTIME | 0x0 | Used to enable/disable the generation of THRE interrupt. 0 = disabled 1 = enabled |
| [6:4] | - | Reserved |  | reserve |
| [3] | RW | EDSSI | 0x0 | Enable modem status interrupt. 0 = disabled 1 = enabled |
| [2] | RW | ELSI | 0x0 | Enable receiver line status interrupt 0 = disabled 1 = enabled |

| [1] | RW | ETBEI | 0x0 | Enable transmit holding register empty interrupt 0 = disabled 1 = enabled |
| --- | --- | --- | --- | --- |
| [0] | RW | ERBFI | 0x0 | Enable receive data available interrupt and character timeout interrupt ( FIFO mode ) 0 = disabled 1 = enabled |

      - IIR

Offset address: 0x08 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:8] | - | reserved | - | reserve |
| [7:6] | RO | FIFOs Enabled | 0x0 | Used to enable FIFO 00 = disabled 11 = enabled |
| [5:4] | - | reserved |  | reserve |
| [3:0] | RO | Interrupt ID | 0x01 | Interrupt ID . Indicates the highest priority pending interrupt 0001 = no interrupt pending 0010 = THR empty 0100 = received data available 0110 = receiver line status 0111 = busy detect (When compatible with the 16550 standard, this interrupt is not available ) 1100 = character timeout |

| Interrupt ID |  |  |  | Interrupt sources and interrupt resets |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| bit3 | bit2 | bit1 | bit0 | Priority | Interrupt Type | Interrupt Source | Interrupt reset |
| 0 | 0 | 0 | 1 | - | none | none | - |
| 0 | 1 | 1 | 0 | Highest | receiver line status . | Overflow / parity / frame error ( stop bit is not 1 ). Or interval interrupt ( rx signal Always 0 ) . | Read LSR register. |
| 0 | 1 | 0 | 0 | second | received data available . | Receive FIFO data is greater than Equal to the threshold. | Receive FIFO Data Less than the threshold. |
| 1 | 1 | 0 | 0 | second | character timeout . | The receive FIFO has data. And there is no Data goes in and out of the receive FIFO . | Read register RBR . |
| 0 | 0 | 1 | 0 | third | THR empty . | Send FIFO data is less than Equal to the threshold. | Read register IIR or Transmit FIFO Big data |

|  |  |  |  |  |  |  | at the threshold. |
| --- | --- | --- | --- | --- | --- | --- | --- |

      - FCR

Offset address: 0x08 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:8] | - | reserved | - | reserve |
| [7:6] | WO | RCVR Trigger | 0x0 | Sets the threshold for generating a receive data available interrupt. When it is greater than or equal to the threshold, an interrupt is generated to indicate that data is available in the receive FIFO . 00 : 1 ; 01 : 1/4 depth of receive FIFO (i.e. set to 4 ); 10 : 1/2 depth FIFO of receive FIFO (i.e. set to 8 ) ; 11 : 2 characters short of the receive FIFO depth (set to 14 ) |
| [5:4] | WO | TX Empty Trigger | 0x0 | Sets the threshold for generating a TX empty interrupt. When the value is less than or equal to this threshold, a threshold interrupt will be generated. DMA request. 00 : FIFO null; 01 : There are 2 characters in FIFO ; 10 : 1/4 depth of FIFO (i.e. set to 4 ); 11 : FIFO 1/2 the depth (i.e. set to 8 ) . |
| [3] | WO | DMA Mode | 0x0 | DMA mode selection. 0 : Mode 0 ; 1 : Mode 1 . |
| [2] | WO | XMIT FIFO Reset | 0x0 | XMIT FIFO reset. You can reset the control logic of the transmit FIFO . FIFO considered empty |
| [1] | WO | RCVR FIFO Reset | 0x0 | RCVR FIFO reset. You can reset the control logic of the receive FIFO . FIFO considered empty |
| [0] | WO | FIFO Enable | 0x0 | Enable / disable transmit FIFO and receive FIFO . |

      - LCR

Offset address: 0x0C Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:8] | - | reserved | - | reserve |
| [7] | RW | DLAB | 0x0 | Baud rate divisor latch bit. This bit is used to enable UART_DLL and UART_DLH register configures the baud rate. After initializing the baud rate, this bit must be cleared. |
| [6] | RW | Break | 0x0 | When set to 1 , it means that there is no serial data transmission on the serial port . |
| [5] | - | Stick Parit |  | Reserved, treated as 0 |
| [4] | RW | EPS | 0x0 | Even parity selection. Used to select even parity selection. Set to 1 , even parity; |

|  |  |  |  | Set to 0 , odd parity. |
| --- | --- | --- | --- | --- |
| [3] | RW | PEN | 0x0 | Used to enable the parity bit. 0 = parity disabled 1 = parity enabled |
| [2] | RW | STOP | 0x0 | stop bits for each character . 0 = 1 stop bit 1 = 1.5 stop bits when DLS ( LCR[1:0 ]) = 0 , otherwise select 2 stop bits stop bits selected , the receiver only checks the first stop bit. |
| [1:0] | RW | DLS | 0x0 | Data length selection. Used to select the number of data bits for each character. 00 = 5 bits 01 = 6 bits 10 = 7 bits 11 = 8 bits |

      - MCR

Offset address: 0x10 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:7] | - | reserved | - | reserve |
| [6] | RW | SIRE | 0x0 | SIR mode enabled |
| [5] | RW | AFCE | 0x0 | Automatic flow control |
| [4] | RW | LoopBack | 0x0 | Loop Mode |
| [3] | RW | OUT2 | 0x0 | OUT2 |
| [2] | RW | OUT1 | 0x0 | OUT1 |
| [1] | RW | RTS | 0x0 | RTS |
| [0] | RW | DTR | 0x0 | DTR |

      - LSR

Offset address: 0x14 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:8] | - | reserved |  | reserve |
| [7] | RO | RFE | 0x0 | Receive FIFO error bit. Set to 1 when at least one of parity error, framing error, or interrupt error occurs in the FIFO . 0 : no error; 1 : There is an error |
| [6] | RO | TEMT | 0x1 | Transmitter empty flag. In FIFO mode, when the transmit shift register and FIFO When both are empty, this bit will be set. |

| [5] | RO | THRE | 0x1 | Send holding register empty flag. When UART_IER[7]=1 , it means TX FIFO is full; when UART_IER[7]=0 , it means TX FIFO is empty; |
| --- | --- | --- | --- | --- |
| [4] | RO | BI | 0x0 | Break Interrupt. Used to indicate a break sequence in the serial input data. In UART mode, when the serial input sin Keep it at 0 for longer than the time it takes to transmit a character ( start time + data bits+ parity + stop bits ) time, set; |
| [3] | RO | FE | 0x0 | Frame error flag. When the receiver does not receive a valid stop bit, it will A framing error occurred. |
| [2] | RO | PE | 0x0 | Parity error flag. 0 = no parity error 1 = parity error |
| [1] | RO | OE | 0x0 | Overrun error. In IFO mode, when the FIFO is full and the next data is received, an overrun error will occur. The FIFO data is retained and the shift register is The data in the memory is lost. |
| [0] | RO | DR | 0x0 | Data ready bit. Indicates that there is at least one character in the RBR or receive FIFO . 0 = no data ready 1 = data ready |

      - USR

Offset address: 0x7c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:5] | - | reserved | - | reserve |
| 4 | R | RFF | 0x0 | Receive FIFO Full. Used to indicate that the receive FIFO is completely full. 0 = Receive FIFO not full 1 = Receive FIFO Full |
| 3 | R | RFNE | 0x0 | Receive FIFO Not empty. Used to indicate receive FIFO There are one or more entry . 0 = Receive FIFO is empty 1 = Receive FIFO is not empty |
| 2 | R | TFE | 0x0 | Transmit FIFO Empty. Used to indicate that the transmit FIFO is completely empty. 0 = Transmit FIFO is not empty 1 = Transmit FIFO is empty |
| 1 | R | TFNF | 0x0 | Transmit FIFO Not full. Used to indicate that the send FIFO There are one or more data . 0 = Transmit FIFO is full 1 = Transmit FIFO is not full |
| 0 | R | BUSY | 0x0 | This bit is invalid. |

      - TFL

Offset address: 0x80 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:FIFO_A DDR_WIDT H+ 1] | - | reserved | - | reserve |
| [FIFO_ ADDR _WID TH:0] | RO | Transmit FIFO Level | 0x0 | Indicates the send FIFO The amount of data in . |

      - RFL

Offset address: 0x84 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:FIFO_A DDR_WIDT H+ 1] | - | reserved | - | reserve |
| [FIFO_ ADDR _WID TH:0] | R | Receive FIFO Level | 0x0 | Indicates receive FIFO The amount of data in . |

### SDIO

###### Overview

HR_C7000 Built-in SDIO controller, can be implemented with SD Card, SDIO Devices, etc.

###### Functional Description

SD/SDIO Controller for SD/SDIO The controller soft core has the following functions:

Supported interface types:

- SD Secure Card Digital Memory ) 	version 3.0 protocol

- SDIO ( Secure Digital I/ O 	version 3.0 protocol

- CE-ATA (Consumer Electronics Advanced Transport Architecture) 	version 1.1 protocol

- Multimedia Cards(MMC version4.41, eMMC version 4.5)

Support read and write modes:

- Bitstream read and write ( SIDO equipment )

- Single block data reading and writing ( mainly for small capacity SD Card )

- Continuous multi-block data reading and writing ( mainly for large-capacity SD Card )

Supported data bus modes:

- 1 bit Data Mode ( SD Mode, SDIO Mode, SPI model )

- 4bit Data bus ( SD Mode, SDIO model )

Other features:

- The controller can support 1 Zhang SD Card or SDIO equipment

- Support instruction completion signal interrupt, programmable shielding

- cclk_in The nominal clock value is 50MHz , which can be divided to get cclk_out , with a division factor of 1. or 2*n ( n=1~255 )

- cclk_in Clock 1/4 Phase accuracy can be matched to the sampling clock and the driving clock

- Supports low-power clock gating

###### How it works

##### Overall process

SD/SDIO Controller The basic process from power-on to normal data communication is as follows:

- Turn on the power of the external device and set the number of clock cycles for device detection;

- Set the clock frequency for device enumeration ( 100kHz ~ 400kHz ) and start the clock ;

- Enable global interrupts, shield unnecessary interrupts, and detect device status;

- Perform device enumeration and complete device initialization;

- Change the clock frequency ( standard mode up to 25MHz , high speed mode up to 50MHz ) ,

- FIFO related to the application layer Threshold, timeout threshold, interrupt mask and other registers;

- Enter data communication operation.

Note: SD After the card is powered on, it needs to maintain at least 74 cclk_out​ The enumeration function is allowed only after the cclk_out clock period is reached, otherwise the memory card device cannot respond normally. The cclk_out clock period is much longer than the system clock, so the software needs to leave enough time.

##### Power Management

SD/SDIO Controller Power management features:

a) Configure 	SD through registers The power switch of the device;

PWREN[0] = 1'b1;  // Enable #0 Card device power supply

##### Clock Control

The clock frequency change operation must be performed in the instruction and data communication idle state. Before changing the clock frequency, the clock must be disabled first and then the clock of the new frequency must be enabled.

- Disable the clock: Clear clk_enable register ( CLKENA@0x10 ) ; Then set start_cmd , update_clock_only , wait_data_complete ( CMD@0x2c ) are all 1 , sending the clock control instruction; wait for start_cmd to be cleared, indicating that the instruction has been sent.

CLKENA[15:0] = 16'h0; // Disable all memory card working clocks

CMD[31] = 1'b1, CMD[21] = 1'b1,CMD[13] = 1'b1; // Update clock setting parameters

- Enable clock: Set the clock division parameter CLKDIV Register (CLKDIV@0x08) selects the divided clock source

（ CLKSRC@0x0c ) , and set the clock enable CLKENA Register; then set start_cmd , update_clock_only , wait_data_complete When each bit is 1 , the clock control instruction is sent; wait for start_cmd to be cleared, indicating that the instruction has been sent.

CLKDIV[7:0] = 8'h40; // Setting #0 Frequency divider 128 Frequency division

CLKSRC[1:0] = 2'h0; // Select #0 clk_out of the memory card For #0 Divider Clock

CLKENA[0] = 1'b0; 	// Enable #0 Memory card cclk clock

CMD[31] = 1'b1, CMD[21] = 1'b1,CMD[13] = 1'b1; 	// Update clock setting parameters

- Set clock low power mode ( optional ) : by setting CLKEN@0x10 Register bit [31:16] enables the device to be in low power mode. In low power mode, the card device is in IDLE Status more than 8 After 1 clock, the controller automatically stops the clock output to reduce power consumption. When configuring low power mode, you need to use CMD The clock update command ( start_cmd , update_clock_only ) is valid. In addition, the low power mode is limited to MMC/SD For SDIO devices, the SDIO device interrupt will not be detected after the clock stops .

CLKEN[16] = 1'b1; 	// Enable #0 Card device clock low power mode

Note: Clock control instructions only work on SD/SDIO Controller Internal, not sent to external devices, so no instruction completion interrupt is generated.

##### Interrupt Control

The interrupt enable function includes: a global interrupt control register int_enable (CTRL@0x00, bit4) controls the global interrupt enable. The interrupt will be triggered only when int_enable is valid.

Three groups of registers related to interrupts: INTMASK@0x24 , MINTSTS@0x40 and RINTSTS@0x044 , where INTMASK[15:0] bits control mask 16 The INTMAX [ 31:16 ] bits control the masking of the corresponding 16 interrupts with specific functions . SDIO interrupt, RINTSTS[31:0] shows the interrupt trigger status ( same as INTMAST Status is irrelevant ) , and MINTSTS[31:0] indicates the unmasked trigger interrupt, and the three satisfy the following relationship:

MINTSTS = MINTSTS & RINTSTS;

INTMASK When the register is cleared, the corresponding interrupt is masked. After the interrupt is triggered, it will only be Update interrupt status in register , in MINTSTS No updates. Response to RINTSTS/MINSTS After the interrupt, write 1 to this register Clear the interrupt status.

##### Status Detection

STATUS@0x48 Registers are used for FIFO when reading and writing device data Before the controller communicates with the device, it should query this register to determine the controller status.

CMD@0x2c Registers are used for the master to send command control, device initialization, and response format control.

##### Device Enumeration

Before a device enters the data transmission process, it must first undergo device enumeration to identify and initialize the device. The enumeration clock frequency is 100-400KHz .

SD The device enumeration process flow chart is as follows, please refer to SD Specifications Part 1 Physical Layer Specification Version 3.00 :

CMD3

CMD2

Power- on

CMD5

Arg=0

Resp

No resp

SD Mem 	SDIO device , read card voltage

V2.0 SD ( voltage mismatch )

ACMD41

HCS =0

CMD5

or V1.x SD , or non -SD

No response

CMD8

N

Vaild?

Y

ACMD41

HCS =0/1

Compatible volt?

Unusable card

CMD5

Setting the voltage

Resp0[27] 	COMBO card

Unusable card

Timeout

Card Ready?

Busy

Busy

Card Ready?

Timeout

Unusable card

CMD3

CCS in R

CCS=1

V1.x SDSC

CCS=0

V2.0 SDSC

V2.0 SDHC

S18R=1

S18R S18A?

&S18A=1

CMD11

S18R=0 or S18A=0

SD Mode 	SDIO Mode

Figure 36 SD Card enumeration and initialization process

- Send CMD0 Initialize the card device. Send CMD0 When sending, you need to set the send_initialization ( CMD@0x2c , bit15 ) value to 1 , and the controller automatically sends an initialization sequence of 80 clocks;

- Identify the card type.

First send CMD5 , if the correct response is received, it is SDIO Device, jump to step d) for SDIO Device enumeration ; if the response is over or the response is wrong, CMD8 is sent , and its Argument content is as follows:

If the controller receives CMD8 If the correct answer is R7 , it means that SD2.0 is supported. Large capacity device of the above protocol; then send Argument The content of ACMD41 is as follows Instruction ( ACMD41 Need to send CMD55 first , then send CMD41 , the device will automatically recognize CMD as A C M D4 1 ) ;

If the controller receives ACMD41 The correct answer is R3 , then SD Memory card; otherwise MMC , CEATA Or invalid card ( the project does not support MMC , CEATA The device does not require it, so I will not repeat the enumeration and read and write operations of these two cards ) ; Detection response R3 busy​ bit Status: If it is zero, it means the card device is in the initialization process and needs to send ACMD41 instructions repeatedly, waiting for the busy bit in the response to be set; if the response times out, the card is invalid;

If the controller does not receive CMD8 If the correct response R3 is received , it means that the device does not support SD2.0 protocol; then send CMD0 to reset the card device; then resend the ACMD41 command with the following Argument ;

At this time, if the controller receives ACMD41 If the card responds to R3 , it is an SD card. Memory, otherwise MMC Or CEATA ; Note: You must send CMD8 first and then ACMD41 ;

- Repeatedly send ACMD41 Until the response R3 is detected busy​ The bit is set to indicate that the device power-on initialization is completed;

- Enumerate the card device according to the specified command sequence. CMD2 obtains the card device's CID information in the response , and CMD3 provides the card device with a new offset address. To publish a new offset address, resend CMD3 .

SD Card: send CMD0 , CMD8 , ACMD41 , CMD2 , CMD3 ;

SDIO : Send CMD5 , if the function number is correct then send CMD3 ; if it is memory present If it is set, it means it is a COMBO card with a storage area, and then the enumeration is completed according to the SD memory card;

MMC : Send CMD0 , CMD1 , CMD2 , CMD3 ;

- Identifying MMC/CE-ATA Card

Send CMD8 Get device EXT_CSD If the value of register S_CMD_SET[4] is 1 , the device supports ATA mode . Then by setting EXT_CSD Register CMD_SET[4] activates ATA Mode instruction set, SD/SDIO Controller changes the instruction mode through CMD6 ;

If the device responds that it does not support ATA Mode: Send CMD39 If a response is received and contains CE- ATA

The logo is CE-ATA v1.0 device; otherwise, MMC Card.

##### Command Operation

SD/SDIO Controller Instructions include clock update instructions and SD Protocol standard instructions. Clock update instructions are not sent to the external instruction bus. Only one instruction requiring data transmission can exist on the instruction bus at a time. By setting the instruction register

( CMD@8 h2c ) sends a command.

Before sending certain specific commands, you need to first send an argument The register ( CMDARG@0x28 ) is filled with the corresponding function value. The protocol specifies that for instructions without arguments , it is recommended to clear the corresponding argument content segment to zero.

After receiving a response from the card device, the controller automatically sets Command done ( RINTSTS@0x44) to 1 , triggering an interrupt . Response errors may trigger Response timeout error , Response CRC error , or Response error interrupts. Short responses are stored in RESP0 @ 0x30 ; long responses are stored in registers RESP0 @0x30 , RESP1 @0x34 , RESP2@0x38 , and RESP3@0x3c , respectively.

The registers that need to be set when sending the command ( CMD@0x2c ) are shown in the following table:

Table 33 Send instruction register setting

| IP Main Features | Values | Comments |
| --- | --- | --- |
| Default Properties |  |  |
| Bit[31], start_cmd | 1 | Set to 1 Indicates that the start command is sent; when CIU After taking the instruction, start_cmd Set to 0 ; when start_cmd A value of 1 When the software still sends instructions, the hardware Locked Write Error ( RINTSTS @0x40 ) will be set to 1 , triggering an error interrupt. |
| Bit[29], use_hold_reg | 0 | CMD and DATA Without HOLD Reg send |
| Bit[28], volt_switch | 0 | No voltage switching |
| Bit[27:22] | 0 | With SD and SDIO Unrelated |
| Bit[21], update_clock_registers_only​ | 0/1 | 0 : SD Instructions, CIU Send the instruction to the card device; 1 : Clock control instruction, CIU Do not send instructions to the card device, only update the clock related registers. |
| Bit[20:16], card_number | x | Card device offset address |
| Bit[15], send_initalization | 0/1 | 0 : Do not initialize the sequence 1 : Send 80 Initialize the card device |
| Bit[14], stop_abort_cmd | 0 | 0 : No effect on the current command 1 : Stop or abandon the current instruction |
| Bit[12], send_auto_stop | 0 | 0 : After data transmission is completed, there is no automatic stop instruction |

|  |  | 1 : After data transmission is completed, there is an automatic stop instruction |
| --- | --- | --- |
| Bit[11], transfer_mode | 0/1 | 0 : Block transfer ( SDHC ) 1 : Data stream transmission ( SDIO ) |
| Bit[10], read/write | 0/1 | 0 : Read from the card device; 1 : Write to the card device |
| Bit[9], data_expected | 0/1 | 0 : No data transmission 1 : Data transmission instruction. |
| Bit[7], response_length | 0/1 | 0 : short response; 1 : long response. |
| Bit[6], response_expect | 1 | 0 : CMD0/CMD4/CMD15 No response is required for other commands; 1 : Response required. |
| Bit[5:0], cmd_index | x | Instruction number |
| User attributes |  |  |
| Bit[13], wait_prvdata_complete | 1 | Wait for the data transmission operation to be completed before sending the next instruction. Set to 1 (unless a status query command or a transmission abort command is sent ) |
| Bit[8], check_response_crc | 1 | Response CRC check |

##### Data transmission

SD Memory data transfer instructions and SDIO Device I/O The instruction sets used for operations are different, SD The memory can use multiple data read and write instructions, such as CMD17 , CMD18 , CMD24 , CMD25 etc., while SDIO Device data read and write operations have only CMD53 Instructions. In addition, SD Storage and SDIO The state transition of devices during data transmission is also different . The data transmission process status of SDIO mode is shown in the figure below. The data transmission process status of SDIO mode is shown in the figure below. The extended interface technology of the protocol, so SD The association strives to simplify SDIO from multiple aspects such as instruction set and state machine protocol to enable SDIO Can become fully compatible with SD Electrical standards, easy to use, transparent IO to upper-layer software Interface. For detailed description, refer to SD Specifications Part 1 Physical Layer Specification Version 3.00 and SD Specifications Part E1 SDIO Specification Version 3.00 .

Figure 37 SD Memory data transfer state transition diagram

Figure 38 SDIO Device data transmission state transition diagram The data transmission process will be in RINTSTS@0x44 The corresponding interrupt is triggered in the register: the data transmission process will be triggered in RINTSTS@0x44 The corresponding interrupt is triggered in the register:

End Bit Error/Write no CRC(EBE) : End bit error when reading the card, or CRC not received after writing the card confirm;

Auto Command Done : After the multi-block data transmission is completed, CMD12 is automatically sent Instruction completion interrupt;

Start Bit Error (SBE) : 4 bits When the bus width card is read, if DAT0 The start bit 0 is sent , and other data lines do not send 0 , triggering the interrupt;

Hardware Locked write Error(HLE) : in hardware_locked Status, write register to trigger the interrupt;

FIFO Under/Over Run Error(FRUN) ： Host In FIFO When full / empty, writing / reading data triggers this interrupt;

Data Starvation by Host Timeout(HTO) ： host FIFO​ This interrupt is triggered when the FIFO is not read or written in time when it is full or empty ;

Data Read Timeout (DRTO) : Data read timeout, the card device does not send data to the Host within the specified time. The end sends data;

Data CRC Error(DCRC) : CRC during data reception Verification error, at End Flag bit error;

Receive FIFO Data Request(RXDR) ： FIFO The data in the FIFO exceeds the threshold, and the software can Read data from the FIFO; Transmit FIFO Data Request (TXDR) : The data in the FIFO is lower than the threshold, and the software can write data to the FIFO ; Data Transfer Over (DTO) : Data transfer completion interrupt;

Notice:

- If the data transmission instruction response times out, it means that the instruction fails to take effect and no more data will be transmitted ;

- DCRC occurs during data transmission , SBE or EBE When errors occur, DTO may or may not be triggered , so CMD12 must be triggered when any data error occurs. or CMD53 Forced termination of data transmission to ensure DTO is triggered (DesignWare DWC Mobile Storage Host Databook, Version 2.60a , p56) .

Before sending the data transmission instruction, you need to send CMD13 Instruction read SD Current state of memory (argument [31:16] of the segment is the offset address, [15:0] is the filling byte ) , and the response is READY_FOR_DATA The bit indicates whether the memory can currently receive new data. Then send CMD7 ( argument Segment [31:16] is the offset address, [15:0] is the filling byte) selects the card device and puts it into the transmission state.

###### Read data in single or multiple blocks

  - To register CTRL@0x00 fifo_reset Write "1" to reset the FIFO Pointer, query and wait for this bit to be automatically cleared.

  - To register BYTCNT@0x20 Write the number of bytes of data to be transmitted;

  - To register BLKSIZ@0x1c Write block size ( SDHC The block size is fixed at 512 byte ) ;

  - To register CMDARG@0x28 The starting address of the written data;

  - Set register CMD . For SD/MMC Memory, use instructions CMD17/CMD18 respectively Perform single block / multi-block read operations; for SDIO Device, only use command CMD53 Perform single block / multi-block read operation. Once register CMD is written, SD/SDIO Controller Start executing the command. When the command is sent and the correct response is received , the Command Done interrupt is triggered;

  - Check register STATUS@0x48 The fifo_empty status bit is cleared to "0" and data is read from register FIFO@0x200 so that SD/SDIO Controller Receive the following data; at the same time check the data error interrupt, that is, register RINTSTS bit[7] and RINTSTS bit[15] . At this point, the program can send a stop command to terminate the data transmission.

  - Read FIFO Data, until STATUS fifo_empty The status position is "1" . After the data is read, write "1" to RINTSTS bit [3] to clear the transfer completion interrupt;

  - If multiple blocks are read, and register CMD is set send_auto_stop is set to "1" , SD/SDIO Controller automatically sends CMD12 after data reading is completed Stop command: If necessary, you can actively send the data transmission stop command CMD12 to stop data transmission.

Table 34 SD Memory read operation instruction setting

| IP Main Features | Values | Comments |
| --- | --- | --- |
| Default Properties |  |  |
| Bit[31], start_cmd | 1 | Set to 1 Indicates that the start command is sent; when CIU After taking the instruction, start_cmd Set to 0 ; when start_cmd A value of 1 When the software still sends instructions, the hardware Locked Write Error ( RINTSTS @0x40 ) will be set to 1 , triggering an error interrupt. |
| Bit[29], use_hold_reg | 0/1 | CMD and DATA Whether it has passed HOLD Reg send |
| Bit[28], volt_switch | 0 | No voltage switching |
| Bit[27:22] | 0 | With SD and SDIO Unrelated |
| Bit[21], update_clock_registers_only​ | 0 | 0 : SD Instructions, CIU Send the instruction to the card device; 1 : Clock control instruction, CIU Do not send instructions to the card device, only update the clock related registers. |
| Bit[20:16], card_number | x | Card device offset address |
| Bit[15], send_initalization | 0 | 0 : Do not initialize the sequence 1 : Send 80 Initialize the card device |
| Bit[14], stop_abort_cmd | 0 | 0 : No effect on the current command 1 : Stop or abandon the current command |
| Bit[12], send_auto_stop | 0/1 | 0 : After data transmission is completed, there is no automatic stop instruction 1 : After data transmission is completed, there is an automatic stop instruction |
| Bit[11], transfer_mode | 0 | 0 : Data block transmission; 1 : Data stream transmission |
| Bit[10], read/write | 0 | 0 : Read from the card device; 1 : Write to the card device |
| Bit[9], data_expected | 1 | 0 : non-data transfer operation; 1 : data transfer operation |
| Bit[7], response_length | 0 | 0 : short response; 1 : long response. |
| Bit[6], response_expect | 1 | 0 : No response required ( such as CMD0/CMD4/CMD15 wait ) 1 : Response required |
| Bit[5:0], cmd_index | x | Instruction number |

| User attributes |  |  |
| --- | --- | --- |
| Bit[13], wait_prvdata_complete | 1 | Wait for the data transmission operation to be completed before sending the next instruction. Set to 1 (unless a status query command or a transmission abort command is sent ) |
| Bit[8], check_response_crc | 1 | Response CRC check |

###### Single-block and multi-block writing

  - Register CTRL fifo_reset Write "1" to reset FIFO Pointer, query waits until the bit is automatically cleared;

  - BYTCNT register Write the size of the data to be transferred;

  - BLKSIZ register Write block size ( SDHC The block size is fixed at 512 byte ) ;

  - Register CMDARG The starting address of the written data;

  - Read STATUS Register information, after judging the FIFO status, write the data into the FIFO , that is, write register 0x200 . Usually, the FIFO should be filled at the beginning ;

  - Set register CMD . For SD/MMC Card, use command CMD24/CMD25 respectively Perform single / multi-block write operations. For SDIO cards, use the CMD53 instruction to perform single / multi-block write operations.

  - Check register STATUS@0x48 fifo_empty status position " 1 " , or RINTSTS [TXDR] interrupt position "1" , write 0x200 Register, to FIFO Fill data; at the same time, check the data error interrupt, that is, check the register RINTSTS [ DCRC] bit, RINTSTS [EBE] bit, RINTSTS [HLE] bit. If necessary, the program can send a stop instruction to terminate the data transmission. DTO Set to "1" , data transmission is completed, write back to "1" , clear the interrupt;

    - If multiple blocks are written and the register CMD send_auto_stop is set to "1" , the controller automatically sends a stop command to end a data transmission. If necessary, the data transmission stop command CMD12 can be actively sent. Stop data transmission;

    - Query and wait for the register CMD [ data_busy] to change from "1" to "0" .

Table 35 SD Memory write operation instruction setting

| IP Main Features | Values | Comments |
| --- | --- | --- |
| Default Properties |  |  |
| Bit[31], start_cmd | 1 | Set to 1 Indicates that the start command is sent; when CIU After taking the instruction , set start_cmd to 0 ; when the start_cmd value is 1 , the software still sends the instruction . Locked Write Error ( RINTSTS @0x40 ) will be set to 1 , triggering an error Interrupt. |
| Bit[29], use_hold_reg | 0/1 | CMD and DATA Whether it has passed HOLD Reg send |
| Bit[28], volt_switch | 0 | No voltage switching |
| Bit[27:22] | 0 | With SD and SDIO Unrelated |
| Bit[21], update_clock_registers_only | 0 | 0 : SD Instructions, CIU Send the instruction to the card device; 1 : Clock control instruction, CIU Do not send instructions to the card device , only update the clock related registers. |
| Bit[20:16], card_number | x | Card device offset address |
| Bit[15], send_initalization | 0 | 0 : Do not initialize the sequence 1 : Send 80 Initialize the card device |
| Bit[14], stop_abort_cmd | 0 | 0 : No effect on the current command 1 : Stop or abandon the current instruction |
| Bit[12], send_auto_stop | 0/1 | 0 : After data transmission is completed, there is no automatic stop instruction 1 : After data transmission is completed, there is an automatic stop instruction |
| Bit[11], transfer_mode | 0 | 0 : Data block transmission; 1 : Data stream transmission |

| Bit[10], read/write | 1 | 0 : Read from the card device; 1 : Write to the card device |
| --- | --- | --- |
| Bit[9], data_expected | 1 | 0 : No data transmission; 1 : Data transmission instruction |
| Bit[7], response_length | 0 | 0 : short response; 1 : long response. |
| Bit[6], response_expect | 1 | 0 : CMD0/CMD4/CMD15 No response is required for other commands; 1 : Response required. |
| Bit[5:0], cmd_index | x | Instruction number |
| User attributes |  |  |
| Bit[13], wait_prvdata_complete | 1 | Wait for the data transfer operation to be completed before sending the next instruction. Normally set to 1 (unless a status query command or a transmission abort command is sent ) |
| Bit[8], check_response_crc | 1 | Response CRC check |

      - Auto-stop operate

In the multi-block read and write instruction operation, a stop instruction is required to complete the data transmission. The stop instruction can be sent by software by actively sending CMD12 , or by setting Auto-stop in CMD (0x2c). Function enables SD/SDIO Controller Automatically send CMD12 so that the card can return the corresponding status. After completion, RINTSTS (0x44) will be triggered auto_command_done Interrupt to determine whether the stop instruction is completed, and the response content is saved in register RESP1 ( 0x34) . The application scenarios of this function are as follows

Table 36 Auto-Stop Supported card types and transfer modes

| Memory card type | Transmission Mode | Number of bytes (0x20) | Can send_auto_stop be used? | illustrate |
| --- | --- | --- | --- | --- |
| MMC | Stream read/write | 0 | no | Open-end Streaming |
| MMC | Stream read/write | >0 | yes | Auto- stop after byte transfer |
| MMC | Single-block read/write | >0 | no | Byte count = 0 is illegal |
| MMC | Multiple-block read/write | >0 | yes | Auto- stop after the transfer is completed |
| SDMEM | Single-block read/write | >0 | no | Byte count = 0 is illegal |
| SDMEM | Multiple-block read/write | 0 | no | Open-end Multi-block operations |
| SDMEM | Multiple-block read/write | >0 | yes | Auto- stop after the transfer is completed |
| SDIO | Single-block read/write | >0 | no | Byte count = 0 is illegal |
| SDIO | Multiple-block read/write | 0, or >0 | no | SDIO does not support this function |

##### Stop data transfer

- Command CMD12 For MMC/SD Memory card, SDIO The data transfer of the storage function stops. Note that you need to set CMD (0x2c) bit14 = 1'b1, bit13 = 1'b0

- Command CMD52 Setting up SDIO Card register CCCR 0x06 Bit 0-1 (AS0-AS1) in the address is used for SDIO

I /O The transmission of the function stops.

Note that you need to set CMD (0x2c) bit14 = 1′b1, bit13 = 1 b0

##### Erase Operation

- Pre-erase operation

Before the multi-block write operation, use ACMD23 When the instruction sets these blocks to be written to be pre-erasable mode, the writing speed will be faster than the same operation without setting . Tell the memory card the number of blocks to be written / pre-erase. If the host sends a write termination command (CMD12) in the middle of the process , the contents of the remaining data blocks that have been set to pre-erase mode will be in

Unknown state, the content may have been erased or not. If the number of data blocks sent by the host is more than the data block value given in ACMD23 , the memory card will erase the data blocks one by one when receiving new data blocks. After completing the multi-block write operation, this value (pre-erase-count ) will be reset to the default value of 1. The protocol recommends that you use the default value in CMD25 . Send before writing command The ACMD23 command is used to increase the memory card writing speed. Note that the ACMD23 command must be issued before the write command, otherwise the pre-erase-count will be cleared by other commands.

- Normal erase operation

Use the ERASE_WR_BLK_START ( CMD32 ) and ERASE_WR_BLK_END (CMD33) instructions

To define the start and end addresses of the erased data block, send ERASE (CM38) to start the erase operation. These three instructions must be executed in sequence , otherwise the memory card will respond with ERASE_SEQ_ERROR Error, and reset the entire sequence. If other instructions are inserted (SEND_STATUS The memory card sets ERASE_RESET , resets the erase instruction queue, and executes the last instruction ( the inserted instruction ) . If the erase address range contains write-protected data segments, these data segments must be skipped and only the non-write-protected areas must be erased , and WP_ERASE_SKIP is set at the same time . The minimum erase range is determined by the CSD . Sector section in the register specifies.

In addition, the erase operation is also time-consuming. The master can use an interleaved operation similar to the write operation to improve efficiency. The DATA_STAT_AFTER_ERASE (bit 55) in the SCR register defines whether the state after the erase is 1 or 0 .

###### Register Overview

Register Table 18 SDIO Register Overview ( Base Address: 0x1500_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x00 | CTRL | Control registers |  |
| 0x04 | PWREN | Power-on enable register |  |
| 0x08 | CLKDIV | Clock divider register |  |
| 0x0c | CLKSRC | Clock source register |  |
| 0x10 | CLKENA | Clock enable register |  |
| 0x14 | TMOUT | Timeout register |  |
| 0x18 | CTYPE | Card Type Register |  |
| 0x1c | BLKSIZ | Block size register |  |
| 0x20 | BYTCNT | Byte Count Register |  |
| 0x24 | INTMASK | Interrupt Mask Register |  |
| 0x28 | CMDARG | Instruction content segment register |  |
| 0x2c | CMD | Command register |  |
| 0x30 | RESP0 | Response reply register 0 |  |
| 0x34 | RESP1 | Response reply register 1 |  |
| 0x38 | RESP2 | Response reply register 2 |  |
| 0x3c | RESP3 | Response reply register 3 |  |
| 0x40 | MINTSTS | Interrupt Mask Register |  |
| 0x44 | RINTSTS | Interrupt Status Register |  |
| 0x48 | STATUS | Status register, mainly used to check the controller status |  |
| 0x4c | FIFOTH | FIFO Threshold Register |  |
| 0x50 | CDETECT | Card detection register |  |
| 0x54 | WRTPRT | Write protection register |  |
| 0x58 | GPIO | Not used |  |
| 0x5c | TCBCNT | CIU Byte transmitted by the module Number register |  |

| 0x60 | TBBCNT | Host with BIU_FIFO byte of interaction data Number register |  |
| --- | --- | --- | --- |
| 0x64 | DEBNCE | Card detection bounce register ( unit is host clock number ) |  |
| 0x68 | USRID | User ID Register |  |
| 0x6C | VERID | Soft core version ID register |  |
| 0x70 | HCON | Hardware Configuration Registers |  |
| 0x74 | UHS_REG | Not used |  |
| 0x78 | RST_n | Hard reset register |  |
| 0x80~0x98 | DMA_CTRL | DMA Related, not used |  |
| 0x100 | CardThrCtl | Card read threshold enable register |  |
| 0x104 | Back_end_power | Back_end Power register |  |
| 0x108 | UHS_REG_ECT | Not used |  |
| 0x10c | EMMC_DDR_REG | Not used |  |
| 0x110 | ENABLE_SHIFT | Conversion Phase Control Register |  |
| >=0x200 | DATA | FIFO read and write space |  |

###### Register Description

      - CTRL ( Control Register )

Offset address: 0x00 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:26] | - | reserve | 0x0 | - |
| [25] | - | use_internal_dmac | 0x0 | Not used |
| [twenty four] | RW | enable_OD_pullup | 0x0 | Chip external open_drain Pull-up: 0- disabled, 1- enabled. |
| [23:20] | RW | card_voltage_b | 0x0 | Card regulator_B Voltage setting, output to card_volt_a . Optional, The Ports The port can be used as an output port. |
| [19:16] | RW | card_voltage_b | 0x0 | Card regulator_A Voltage setting, output to card_volt_a . Optional, The Ports The port can be used as an output port. |
| [15:12] | - | reserve | 0x0 | - |
| [11] | RW | ceata_device_interrup t_status | 0x0 | - Turn off CE_ATA Interrupts in the device, - Enable CE_ATA Interrupts in the device. |
| [10] | RW | send_auto_stop_ccsd | 0x0 | Send CCSD to CE_ATA device, sends the internally generated STOP . send_ccsd must also be set with send_auto_stop_ccsd |
| [9] | RW | send_ccsd | 0x0 | Send command completion signal disable to CE_ATA set up Preparation |
| [8] | RW | abort_read_data | 0x0 | 0- no change, 1- reset data state machine, the data state machine returns to IDLE BIT​ The bit is automatically cleared. |
| [7] | RW | send_irq_response | 0x0 | 0- no change, 1- send automatic IRQ reply |
| [6] | RW | read_wait | 0x0 | 0- clear read wait, 1- read wait valid |
| [5] | RW | dma_enable | 0x0 | DMA Transfer mode enable: value 1 Enable, value 0 Disable Enable |
| [4] | RW | int_enable | 0x0 | Global interrupt enable signal: value 1 Enable, value 0 Disable Enable |

| [3] | - | reserve | 0x0 | - |
| --- | --- | --- | --- | --- |
| [2] | RW | dma_reset | 0x0 | Reset internal DMA Interface control logic, in two AHB clocks Then it is automatically cleared to zero. |
| [1] | RW | fifo_reset | 0x0 | Reset FIFO , automatically cleared to zero after the reset operation is completed. |
| [0] | RW | controller_reset | 0x0 | 1​ When the controller is reset, the two AHB and two cclk_in clock bit in the cycle The bit is automatically cleared. |

Note: H/S Represents hardware reset and software reset

      - PWREN ( Power-On Enable Register )

Offset address: 0x4 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | - | reserve | 0x0 | - |
| [0] | RW | power_enable | 0x0 | The project uses one controller to operate one SD Card configuration, power-on, clock division, clock enable, etc. all use 1 bit The control bits are controlled independently. The following only describes the configuration of a single controller. The same operation is used for the other controller and will not be repeated here. power_enable[0 ] = 1′b1 , the device is powered on power_enable[0 ] = 1′b0 , the device is powered off. Other bits are reserved and cleared. |

      - CLKDIV ( Clock Divider Register )

Offset address: 0x8 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:8] | - | clk_divider1/2/3 | 0x0 | Not used |
| [7:0] | RW | clk_divider0 | 0x0 | #0 Divider ( cclk_in Divide the frequency to get cclk_out , the division rate = 2 where 0 means no frequency division , 255 means 510 times frequency division, and so on. [7:0] is configured in the project , and the other bits [31:8] = 24′h0 |

      - CLKSRC ( Clock Source Register )

Offset address: 0xc Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | clk_source | 0x0 | Select each SD The driving clock of the memory has the following relationship 00b : Select Clock divider_0 , 01 : Select Clock divider_1 , 10b : Select Clock divider_2 , 11 : Select Clock divider_3 , each controller in the project only contains clk_divder0 , which is fixed to 0 |

      - CLKENA ( Clock Enable Register )

Offset address: 0x10 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RW | cclk_low_power | 0x0 | 16 SD memory or 1 Zhang MMC Memory low power control 0 : Non-low power mode; 1 : Low power mode , the project only configures bit[16] , and all other bits are fixed to 0 |
| [15:0] | RW | cclk_enable | 0x0 | 16 SD memory or 1 Zhang MMC Memory clock enable control , 0 : Clock is invalid; 1 : Clock is enabled The project only configures bit[0] , and all other bits are fixed to 0 |

      - TMOUT(Timeout register )

Offset address: 0x14 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:8] | RW | data_timeout | 0xffffff | The memory card data reading timeout value is set, and the memory card clock starts counting after it ends. cclk_out of the selected card The period value is the unit time |
| [7:0] | RW | response_timeout | 0x40 | Reply timeout setting, with cclk_out The period value is the unit time |

      - CTYPE ( Card Type Register )

Offset address: 0x18 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RW | card_width1 | 0x0 | Indicates whether each memory card is 8-bit type 0 : non- 8-bit Type; 1 : 8-bit If there is no 8-bit application in the type project , write all 0s in this segment. |
| [15:0] | RW | card_width0 | 0x0 | Indicates whether each memory card is 1-bit or 4-bit type 0 : 1-bit Type ; 1 : 4-bit type The project only configures bit[0] and all other bits are written to 0 |

      - BLKSIZ ( block size register )

Offset address: 0x1c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | - | reserved | - | reserve |
| [15:0] | RW | block_size | 0x200 | Block size (bytes ) |

      - BYTCNT (Byte Count Register )

Offset address: 0x20

Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | byte_count | 0x200 | The number of bytes of data to be transferred For unknown data size transmission, 0 must be written |

      - INTMASK (Interrupt Mask Register )

Offset address: 0x24 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RW | sdio_int_mask | 0x0 | Shield 16 SDIO memory card Interrupt, 0 : mask; 1 : enable The project only configures bit[16] , and all other bits are written to 0 |
| [15:0] | RW | int_mask | 0x0 | Shield unwanted interrupts, 0 : Shield; 1 : Enable bit15 : End-bit error(read)/Write no CRC(EBE) bit14: Auto Command Done(ACD) bit13: Start Bit Error(SBE)/Busy Complete Iterrupt(BCI) bit12: Hardware Locked write Error(HLE) bit11: FIFO underrun/overrun Error(FRUN) bit10: Data starvation-by-host timeout(HTO)/Volt_switch_int bit9: Data Read Timeout(DRTO) bit8: Response Timeout(RTO) bit7: Data CRC Error(DCRC) bit6: Responese CRC Error (RCRC) bit5: Receive FIFO Data Request(RXDR) bit4: Transmit FIFO Data Request(TXDR) bit3: Data Transfer Over(DTO) bit2: Command done(CD) bit1: Response Error(RE) bit0: Card Detect(CD) |

      - CMDARG ( command_argument register )

Offset address: 0x28 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | cmd_arg | 0x0 | 32bit transmitted to the card Instruction content |

      - CMD (Command Register )

Offset address: 0x2c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RW | start_cmd | 0x0 | Send instructions, CIU After receiving the instruction, this bit is automatically cleared |

| [30] | - | reserved | 0x0 | reserve |
| --- | --- | --- | --- | --- |
| [29] | RW | use_hold_reg | 0x1 | 0 : CMD and DATA Not through HOLD Registers sent to memory card 1 : CMD and DATA through HOLD Register sent to memory card |
| [28] | RW | volt_switch | 0x0 | 0 : No voltage switching 1 : Voltage switching enabled, send CMD11 Must be set when |
| [27] | RW | boot_mode | 0x0 | Boot Mode selection: 0 : Must boot operate 1 : Optional boot operate |
| [26] | RW | disable_boot | 0x0 | Boot invalid |
| [25] | RW | expect_boot_ack | 0x0 | ( CIU ) hopes to obtain boot Response |
| [twenty four] | RW | enable_boot | 0x0 | Enable boot |
| [twenty three] | RW | ccs_expected | 0x0 | 0 : CE_ATA The interrupt in the device is invalid, or the command does not need to get the device CCS (Command Complete Signal ) . 1 : CE_ATA Device internal interrupt enable, RW_BLK Need from CE_ATA The device's CCS (Command Completion Signal ) . If there is no CE-ATA application, this bit is written as 0. |
| [twenty two] | RW | read_ceata_device | 0x0 | 0 : The host does not send data to CE_ATA. Device reads data 1 : Master end to CE-ATA If the device reads data without CE-ATA application, this bit is written as 0. |
| [twenty one] | RW | update_clock_register rs_only | 0x0 | 0 : Normal instruction sequence 1 : Do not send instructions, only update the clock register |
| [20:16] | RW | card_number | 0x0 | Current valid card number |
| [15] | RW | send_initialization | 0x0 | 0 : Do not send the initialization sequence before sending the command 1 : Send initialization sequence before sending command 80 must be executed before sending any command to the card. clocks​ Initialize the card. |
| [14] | RW | stop_abort_cmd | 0x0 | 0 : Do not stop or abandon the current data transfer 1 : Stop or abandon the current data transmission |
| [13] | RW | wait_prvdata_comple te | 0x0 | 0 : Send new command regardless of whether the previous data transmission is completed 1 : Wait for the last data transmission to complete before sending a new command |
| [12] | RW | send_auto_stop | 0x0 | 0 : After data transmission is completed, no (automatic) stop command is issued 1 : After data transmission is completed, a (automatic) stop command is issued |
| [11] | RW | transfer_mode | 0x0 | 0 : Data block transmission instruction; 1 : Data stream transmission instruction |
| [10] | RW | read/write | 0x0 | 0 : read; 1 : write |
| [9] | RW | data_expected | 0x0 | 0 : No data reading or writing; 1 : Data reading or writing |
| [8] | RW | check_response_crc | 0x0 | 0 : No CRC response Check; 1 : Perform reply CRC check |
| [7] | RW | response_length | 0x0 | 0 : short reply; 1 : long reply |
| [6] | RW | response_expect | 0x0 | 0 : No reply; 1 : Reply |
| [5:0] | RW | cmd_index | 0x0 | Instruction number |

      - RESP0 ( Response Reply Register 0 )

Offset address: 0x30

Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | response 0 | 0x0 | Reply register 0 , short response or long response bit [31:0] |

      - RESP1 ( Response Reply Register 1 )

Offset address: 0x34 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | response 1 | 0x0 | Reply register 1 , long response bits [63:32] auto_Stop response is stored in this register |

      - RESP2 ( Response Reply Register 2 )

Offset address: 0x38 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | response 2 | 0x0 | Reply register 2 , long response bit [95:64] |

      - RESP3 ( Response Reply Register 3 )

Offset address: 0x3c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | response 3 | 0x0 | Reply register 3 , long response bit [127:96] |

      - MINTSTS (Mask Interrupt Register )

Offset address: 0x40 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RO | sdio_interrupt | 0x0 | Unshielded memory card SDIO Interrupt status, each bit For a card: Bit[31] corresponds to card[15], bit[16] corresponds to card[0] . 0 : No SDIO interrupt is sent from the memory card 1 : With SDIO Interrupt sent from memory card The project only uses bit[16] |
| [15:0] | RO | int_status | 0x0 | Interrupt status signal: bit15: End-bit error(read)/Write no CRC(EBE) bit14: Auto Command Done(ACD) bit13: Start Bit Error(SBE)/Busy Complete Iterrupt(BCI) bit12: Hardware Locked write Error(HLE) bit11: FIFO underrun/overrun Error(FRUN) bit10: Data starvation-by-host timeout(HTO)/Volt_switch_int bit9: Data Read Timeout (DRTO) |

|  |  |  |  | bit8: Response Timeout(RTO) bit7: Data CRC Error (DCRC) bit6: Responese CRC Error (RCRC) bit5: Receive FIFO Data Request(RXDR) bit4: Transmit FIFO Data Request(TXDR) bit3: Data Transfer Over(DTO) bit2: Command done(CD) bit1: Response Error(RE) bit0: Card Detect(CD) |
| --- | --- | --- | --- | --- |

Note: MISTATUS = RINTSTS and INTMASK

      - RINTSTS (Raw Interrupt Status Register )

Offset address: 0x44 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RO/W C | sdio_interrupt | 0x0 | 16 SDIO memory card Interrupt status, regardless of the mask . Corresponding to a card: bit[31] corresponds to card[15], bit[16] corresponds to card[0] . 0 : No SDIO interrupt is issued from the memory card 1 : With SDIO Interrupt sent from memory card The project only uses bit[16] |
| [15:0] | RO/R W | int_status | 0x0 | Many interrupt enable signals have nothing to do with shielding. For the following bits Bit write 1 Can be cleared. bit15: End-bit error(read)/Write no CRC(EBE) bit14: Auto Command Done(ACD) bit13: Start Bit Error(SBE)/Busy Complete Iterrupt(BCI) bit12: Hardware Locked write Error(HLE) bit11: FIFO underrun/overrun Error(FRUN) bit10: Data starvation-by-host timeout(HTO)/Volt_switch_int bit9: Data Read Timeout(DRTO) bit8: Response Timeout(RTO) bit7: Data CRC Error(DCRC) bit6: Responese CRC Error (RCRC) bit5: Receive FIFO Data Request(RXDR) bit4: Transmit FIFO Data Request(TXDR) bit3: Data Transfer Over(DTO) bit2: Command done (CD) bit1: Respones Error(RE) bit0: Card Detect(CD) |

      - STATUS (Status Register)

Offset address: 0x48

Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RO | dma_req | 0x0 | DMA Request signal status bit |
| [30] | RO | dma_ack | 0x0 | DMA Response signal status bit |
| [29:17] | RO | fifo_count | 0x0 | The amount of data written into the FIFO |
| [16:11] | RO | response_index | 0x0 | Previous reply index number |
| [10] | RO | data_state_mc_busy | 0x1 | Data sending or receiving state machine is busy |
| [9] | RO | data_busy | 0x0 | The opposite value of card_data [0] 0 : Memory card data is not busy; 1 : Memory card data is busy |
| [8] | RO | data_3_status | 0x0 | Select the storage card card_datap[3] signal to detect whether there is a card. 0 : Memory card present; 1 : No memory card present |
| [7:4] | RO | command_fsm_states | 0x0 | Indicates the current state of the command state machine: - IDLE - Send init sequence - Tx cmd start bit - Tx cmd bit - Tx cmd index + arg - Tx cmd crc7 - Tx cmd end bit - Rx resp start bit - Rx resp IRQ response - Rx resp tx bit - Rx resp cmd idx - Rx resp data - Rx resp crc7 - Rx resp end bit - Cmd path wait NCC - Wait , CMD_to_response turnaround |
| [3] | RO | fifo_full | 0x0 | FIFO full flag |
| [2] | RO | fifo_empty | 0x1 | FIFO empty flag |
| [1] | RO | fifo_tx_watermark | 0x1 | FIFO reaches the send watermark value |
| [0] | RO | fifo_rx_watermark | 0x0 | FIFO reaches the receive waterline value |

      - FIFOTH​​ Threshold Register )

Offset address: 0x4c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | - | reserved | 0x0 | reserve |
| [30:28] | RW | DW_DMA_Multiple_Transaction_Size | 0x0 | Burst The burst (block) size during operation must be the same as DW_DMA The SRC/DEST_MSIZE size in the controller is consistent 000:1 transfer ; 001:4 ; 010:8 ; 011:16 ; 100:32 ; 101:64 ; 110:128 ; 111:256 1 transfer size = H_DATA_WIDTH |
| [26:16] | RW | RX_WMark | 0x7f | Accept threshold setting, when FIFO When the amount of data in the system is greater than the threshold, Receive data ( FIFO_DEPTH in the project is 128 ) |
| [15:12] | RW | reserved | 0x0 | reserve |
| [11:0] | RW | TX_WMark | 0x0 | Send threshold setting, when FIFO When the amount of data in the system is less than the threshold, Sending Data |

      - CDETECT (Card Detection Register )

Offset address: 0x50 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:30] | RO | reserved | 0x0 | reserve |
| [29:0] | RO | card_detect_n | card_detect _n_inputs | Input port card_detect_n The value on the card ( 1 bit for each card ), 0 : card detected; 1 : card not detected The project only uses bit[0] |

      - WRTPRT (Write Protection Register )

Offset address: 0x54 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:30] | RO | reserved | 0x0 | reserve |
| [29:0] | RO | write_protect | card_write_prt_inputs | The value on the input port card_write_prt ( 1 bit per card ), 0 : card not write protected; 1 : card write protected The project only uses bit[0] |

      - TCBCNT ( CIU Byte transmitted by the module Number register )

Offset address: 0x5c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | trans_card_byte_count | 0x0 | By module CIU The number of data bytes sent to the Card |

      - TBBCNT ( Host with BIU_FIFO Interaction data byte Number register )

Offset address: 0x60 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | trans_fifo_tyte_count | 0x0 | Host with BIU-FIFO The number of bytes of data transferred between |

      - DEBNCE (Card detection debounce register ( unit is host clock Number​​

Offset address: 0x64 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | RW | reserved | 0x0 | reserve |
| [23:0] | RW | debounce_count | 0xffffff | The value of the debounce filter logic is in the unit of the master clock cycle. The typical value is |

|  |  |  |  | 5-25ms​ |
| --- | --- | --- | --- | --- |

      - USRID ( User ID register )

Offset address: 0x68 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | usrid | 0x45700311 | User ID register, set by the user. |

      - VERID (soft core version number register )

Offset address: 0x6c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | Verid | 0x5342250a | Synopsys Soft core version number register |

      - HCON (Soft Core Configuration Register )

Offset address: 0x70 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | hcon | 0x00c44cc1 | User-generated IP Before, configure this register to associate with the hardware configuration Software Development |

      - RST_n (Card Reset Register )

Offset address: 0x78 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | - | reserved | - | reserve |
| [15:0] | RW | card_reset | 0x1 | MMC Card reset control ( generated on the top-level port rst_n Signal ) 0 : Reset; 1 : Activate The project only uses bit[0] |

      - CardThrCtl (Read Threshold Enable Register )

Offset address: 0x100 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:28] | - | reserved | 0x0 | reserve |
| [N:16] | RW | CardRD Threshold | 0x0 | Storage card reading threshold size, N by FIFO Size determines. |

|  |  |  |  | FIFO_DEPTH in the project is 128 , so N=27 |
| --- | --- | --- | --- | --- |
| [15:2] | - | reserved | 0x0 | reserve |
| [1] | RW | BsyClrIntEn | 0x0 | Enable busy clear interrupt 0 ： busy Clear interrupt invalid; 1 : busy Clear interrupt enable |
| [0] | RW | CardRdThrEn | 0x0 | Memory card read threshold enable 0 : Read threshold invalid; 1 : Read threshold enabled |

      - Back_end_power ( Backend Power register )

Offset address: 0x104 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | - | reserved | 0x0 | reserve |
| [15:0] | RW | back_end_power | 0x0 | back_end power supply, 0 : power off; 1 : power on, one pin per card |

      - ENABLE_SHIFT (Phase Adjustment Control Register )

Offset address: 0x110 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | Enable_shift | 0x0 | The default phase shift value in the control design is controlled by 1 for every 2 bits . memory cards 00 : Default phase shift; 01 : Enable phase shift on the next clock rising edge 10 : Enable phase shift at the next clock falling edge; 11 : Reserved |

      - DATA ( FIFO Read and write entry address )

Offset address: >=0x200 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | FIFO | 0x0 | Data read and write FIFO entry address, Random reading and writing of FIFO may cause logic disorder |

### USB

###### Overview

HR_C7000 Non -OTG Device Configuration mode, implement USB1.1 Device Function.

###### Functional Description

Non-OTG Device The main configuration parameters are as follows:

- Working mode: Non-OTG Device

- Structure: Internal DMA

- SG-DMA : None

- HS PHY Interface: None

- USB1.1 FS Transceiver interface: dedicated FS interface

- Total EP Quantity: 1 CTRL IN/OUT(EP0) + 2 IN(EP1/3) + 2 OUT(EP2/4) , total 5 EP​

- TxFIFO Mode: Dedicated TxFIFO

- Total FIFO Depth: 128 (FIFO The bit width is 35 bits)

###### How it works

Dev Mode, soft core MAC Receiving Host Various packages sent and the integrity of the packages are detected. The specific process is as follows:

- When receiving OUT or SETUP When signaling packets, soft core MAC Wait and detect the PID of the following data packet After the data packet is received , the soft core MAC Check the integrity of the data packet and send it to the Host Make corresponding response. If the soft core receives OUT Signaling but RxFIFO When not available, M A C Reply NAK ;

- When receiving IN Signaling packet and TxFIFO When available, M AC Automatically from TxFIFO Reads the data and sends the data packet, then waits for a possible response from the Host . If IN signaling is received but there is no data in the TxFIFO , the MAC responds with NAK .

When the soft core sends and receives data packets, various interrupts and configurations have a strict timing relationship. If the configuration content or configuration time point is incorrect, the soft core will not work properly. This section extracts the general interrupt trigger timing and configuration timing to facilitate the query of upper-level software developers . The legend is as follows:

Table 37 Legend symbol definition

| serial number | Use Case Name | describe |
| --- | --- | --- |
| 1 |  | Host Bag |
| 2 |  | Dev Bag |
| 3 | Yellow text | Trigger GINTSTS.RxFLvl Interrupt, a packet is written to RxFIFO |
| 4 | Blue text | Setting IN/OUT Port transmission properties If there is data to be sent, it can be sent to TxFIFO Push data |
| 5 | Black text | Transmission completion interrupt, indicating that the transmission is completed |
| 6 | Time sequence | From left to right, from top to bottom |

- CTRL with data output transmission

SETUP Stage

DATA Stage 	Status Stage

SETUP	DATA0

ACK

OUT	DATA1	NAK	OUT	DATA1	ACK

IN	DATA	ACK

setup_pkt_rcv POP Data

setup_xfer_done POP pkt out0_setup_done

out_pkt_rcv

POP Data out_xfer_done POP pkt

in0_xfer_compl

Set EP OUT0 	out0_xfer_compl

Figure 39 CTRL OUT Transmission Timing

- CTRL with data input Transmission Timing

SETUP Stage

DATA Stage 	Status Stage

SETUP

DATA0

ACK

IN

DATA1	ACK

OUT	DATA1	ACK

setup_pkt_rcv

POP Data Set EP IN0

setup_xfer_done

POP pkt

out0_setup_done

in0_xfer_compl Set EP OUT0

out_pkt_rcv

POP pkt out_xfer_done

Push Data 	Set EP IN0 Push Data

POP pkt out0_xfer_compl

Figure 40 CTRL IN Transmission Timing

- CTRL without data Transmission Timing

SETUP Stage 	Status Stage

SETUP

DATA0

ACK

IN

DATA	ACK

setup_pkt_rcv POP Data Set EP IN0

setup_xfer_done POP pkt out0_setup_done

Set EP OUT0

in0_xfer_compl

Figure 41 No data CTRL Transmission Timing

- Single OUT Affairs

Upon receiving OUT Signaling, OUT has not been set correctly Timing of port parameters; if it has been configured before receiving

OUT Port parameter, then OUT The port directly responds with ACK ;

OUT	DATA0	NAK

OUT	DATA0	ACK

out_OUTTknEPdis Set EP OUT

out_pkt_rcv

POP Data out_xfer_done

POP pkt

Figure 42 Single OUT Transaction Timing

- Multiple OUT Affairs

Set OUT After the port receives multiple packets, the application software In the interrupt service routine, read the RxFIFO Continuous reading

When all data transmission is completed, the soft core triggers the completion interrupt. If the Host does not receive an ACK during this process , the Host The data packet will be resent, and the soft core will automatically respond with ACK and no RxFLvl Interrupt triggering, no software intervention is required;

OUT	DATA0	ACK

OUT	DATA1	ACK

OUT	DATA1	ACK

Set EP OUT

out_pkt_rcv

POP Data out_xfer_done

POP pkt

out_pkt_rcv

POP Data out_xfer_done

POP pkt

out_pkt_rcv

POP Data out_xfer_done

POP pkt out_xfer_compl

Figure 43 Multiple OUT Transaction Timing

- Single IN Transaction Timing

IN The port receives the IN TxFIFO for signaling Not ready timing; if receiving IN Before signaling, IN When the port settings and TxFIFO are ready, the IN port directly responds to the data packet;

IN

DATA	ACK

NAK

IN

in_INTknFEmp Set EP IN

Push Data

Figure 44 Single IN Transaction Timing

in_xfer_compl

- Multiple IN Transaction Timing

Set IN After the port sends multiple packets continuously, the application software In the interrupt service routine, the TxFIFO Continuously push data in, and when all data transmission is completed ( that is, all received Host The soft core triggers a completion interrupt if the Dev does not receive an ACK during this process . The soft core automatically resends the data packet without software intervention.

DATA1	ACK

IN

DATA1	ACK

IN

DATA0	ACK

IN

Set EP IN

in_TxFEmp Push Data

in_TxFEmp Push Data

in_xfer_compl

Figure 45 Multiple IN Transaction Timing

###### Register Overview

Register Table 19 SDIO Register Overview ( Base Address: 0x1500_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x008 | GAHBCFG | AHB Configuration , global AHB Configuration registers |  |
| 0x00c | GUSBCFG | USB Configuration , Global USB Configuration registers |  |
| 0x010 | GRSTCTL | Reset , global reset control register |  |
| 0x014 | GINTSTS | Interrupt , global interrupt status register |  |
| 0x018 | GINTMSK | Interrupt Mask , global interrupt mask control register |  |
| 0x01c | GRXSTSR | Receive Status Debug Read , global receive status reads the send register |  |
| 0x020 | GRXSTSP | Status Read and Pop , global receive status readout register |  |
| 0x024 | GRXFSIZ | Receive FIFO Size , RxFIFO Allocate control registers |  |
| 0x028 | GNPTXFSIZ | Non-Periodic Transmit FIFO Size , Non- Periodic TxFIFO Allocate control registers |  |
| 0x038 | GGPIO | General Purpose IO , general IO register |  |
| 0x03c | GUID | User ID , user ID register |  |
| 0x040 | GSNPSID | Synopsys ID , version number read-only register (0x4f54_270a) |  |
| 0x044 | GHWCFG1 | User HW Config1 , soft core configuration status register 1 |  |
| 0x048 | GHWCFG2 | User HW Config2 , soft core configuration status register 2 |  |
| 0x04c | GHWCFG3 | User HW Config3 , soft core configuration status register 3 |  |
| 0x050 | GHWCFG4 | User HW Config4 , soft core configuration status register 4 |  |
| 0x054 | GUSRSTS | User States , user-defined observation signals |  |
| 0x104 | DIEPTXF1 | IN Endpoint1 TxFIFO Depth , TxFIFO of port 1 Depth Configuration Register |  |
| 0x108 | DIEPTXF3 | IN Endpoint3 TxFIFO Depth , TxFIFO of port 3 Depth Configuration Register |  |
| 0x800 | DCFG | Device Configuration , device overall configuration register |  |
| 0x804 | DCTL | Device Control , device overall control register |  |
| 0x808 | DSTS | Device Status , device overall status register |  |
| 0x810 | DIEPMSK | Device IN Endpoint Common Interrupt Mask , equipment IN port overall interrupt mask |  |
| 0x814 | DOEPMSK | Device OUT Endpoint Common Interrupt Mask , set Standby OUT Port Global Interrupt Mask Register |  |
| 0x818 | DAINT | Device All Endpoints Interrupt , all ports of the device Off Status Register |  |
| 0x81c | DAINTMSK | Device All Endpoints Interrupt Mask , all devices Port Interrupt Mask Control Register |  |
| 0x830 | DTHRCTL | Device Threshold Control , device threshold control register |  |
| 0x834 | DIEPEMPMSK | Device IN Endpoint FIFO Empty Interrupt Mask , Equipment IN Port FIFO Empty Interrupt Mask Register |  |
| 0x900 | DIEPCTL0 | Device Control IN Endpoint0 Control , device IN EP0 Control registers |  |
| 0x908 | DIEPINT0 | Device IN Endpoint0 Interrupt , device IN EP0 Interrupt Status Register |  |
| 0x910 | DIEPTSIZ0 | Device IN Endpoint0 Transfer Size , device IN EP0 |  |

|  |  | Transfer size setting register |  |
| --- | --- | --- | --- |
| 0x914 | DIEPDMA0 | Device IN Endpoint0 DMA Addr , device IN EP0 DMA address |  |
| 0x918 | DTXFSTS0 | Device IN Endpoint0 Transmit FIFO Status , device IN EP0 TxFIFO status |  |
| 0xb00 | DOEPCTRL0 | Device Control OUT Endpoint0 Control , device OUT EP0 Control registers |  |
| 0xb08 | DOEPINT0 | Device OUT Endpoint0 Interrupt , device OUT EP0 Interrupt Status Register |  |
| 0xb10 | DOEPTSIZ0 | Device OUT Endpoint0 Transfer Size , device OUT EP0 Transfer size setting register |  |
| 0xb14 | DOEPDMA0 | Device OUT Endpoint0 DMA Addr , device OUT EP0 DMA address |  |
| 0x920 | DIEPCTL1 | Device Control IN Endpoint1 Control , device IN EP1 Control registers |  |
| 0x928 | DIEPINT1 | Device IN Endpoint1 Interrupt , device IN EP1 Interrupt Status Register |  |
| 0x930 | DIEPTSIZ1 | Device IN Endpoint1 Transfer Size , device IN EP1 Transfer size setting register |  |
| 0x934 | DIEPDMA1 | Device IN Endpoint1 DMA Addr , device IN EP1 DMA address |  |
| 0x938 | DTXFSTS1 | Device IN Endpoint1 Transmit FIFO Status , device IN EP1 TxFIFO status |  |
| 0xb40 | DOEPCTRL2 | Device Control OUT Endpoint2 Control , device OUT EP2 Control registers |  |
| 0xb48 | DOEPINT2 | Device OUT Endpoint2 Interrupt , device OUT EP2 Interrupt Status Register |  |
| 0xb50 | DOEPTSIZ2 | Device OUT Endpoint2 Transfer Size , device OUT EP2 Transfer size setting register |  |
| 0xb54 | DOEPDMA2 | Device OUT Endpoint2 DMA Addr , device OUT EP2 DMA address |  |
| 0x960 | DIEPCTL3 | Device Control IN Endpoint3 Control , device IN EP3 Control registers |  |
| 0x968 | DIEPINT3 | Device IN Endpoint3 Interrupt , device IN EP3 Interrupt Status Register |  |
| 0x970 | DIEPTSIZ3 | Device IN Endpoint3 Transfer Size , device IN EP3 Transfer size setting register |  |
| 0x974 | DIEPDMA3 | Device IN Endpoint3 DMA Addr , device IN EP3 DMA address |  |
| 0x978 | DTXFSTS3 | Device IN Endpoint3 Transmit FIFO Status , device IN EP3 TxFIFO status |  |
| 0xb80 | DOEPCTRL4 | Device Control OUT Endpoint4 Control , device OUT EP4 Control registers |  |

| 0xb88 | DOEPINT4 | Device OUT Endpoint4 Interrupt , device OUT EP4 Interrupt Status Register |  |
| --- | --- | --- | --- |
| 0xb90 | DOEPTSIZ4 | Device OUT Endpoint4 Transfer Size , device OUT EP4 Transfer size setting register |  |
| 0xb94 | DOEPDMA4 | Device OUT Endpoint4 DMA Addr , device OUT EP4 DMA address |  |
| 0xe00 | PCGCCTL | Power and Clock Gating Control , power and clock gating Control registers |  |
| 0x1000 | DEVEP0FF | EP0 FIFO PUSH/POP Entry address |  |
| 0x2000 | DEVEP1FF | EP1 FIFO PUSH/POP Entry address |  |
| 0x3000 | DEVEP2FF | EP2 FIFO PUSH/POP Entry address |  |
| 0x4000 | DEVEP3FF | EP3 FIFO PUSH/POP Entry address |  |
| 0x5000 | DEVEP4FF | EP4 FIFO PUSH/POP Entry address |  |

###### Register Description

      - GAHBCFG

Offset address: 0x008 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:9] | - | reserved | 0x0 | reserve |
| [8] | - | Non- usage | 0x0 | Not used |
| [7] | RW | NPTxFEmpLvl | 0x0 | Non-Periodic TxFIFO Empty Level , dedicated FIFO In dev mode , 0 : DIENPINTn.TxFEmp indicates that the IN EP TxFIFO is half full. 1 : DIENPINTn.TxFEmp , indicates IN EP TxFIFO is full |
| [6] | - | reserved | 0x0 | reserve |
| [5] | RW | DMAEn | 0x0 | DMA Enable 0 : Slave mode; 1 : DMA mode |
| [4:1] | RW | HBstLen | 0x0 | Burst Length/Type , configure dma_burst[3:0] in DMA mode When internal DMA is configured 4'h0: Single 4'h1: INCR 4'h3: INCR4 4'h5: INCR8 4'h7: INCR16 Other : Reserved |
| [0] | RW | GlbIntrMsk | 0x0 | Global Interrupt Mask 0 : Mask all interrupts 1 : Unmask all interrupts |

      - GUSBCFG

Offset address: 0x00c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RW | corrtxpkt | 0x0 | Corrupt Tx Packet Only used in debug mode, do not set it to 1 |
| [30:29] | - | Non- usage | 0x0 | Not used |
| [28:26] | - | reserved | 0x0 | reserve |
| [25:15] | RW | Non- usage | 0x0 | Non- usage |
| [14] | - | reserved | 0x0 | reserve |
| [13:10] | RW | USBTrdTim | 0x5 | USB Turnaround Time Calculation formula: (4*AHB_CLK + 1*PHY_CLK)/PHY_CLK |
| [9:7] | RW | Non- usage | 0x0 | Non- usage |
| [6] | RO | PHYSel | 0x1 | USB2.0 HSPHY or USB1.1 FS Serial Transceiver 1 : USB1.1 FS Serial Transceiver 0 : USB2.0 high-speed PHY |
| [5] | RW | FSIntf | 0x0 | FS Serial Interace Select 0 : 6-pin unidirectional FS interface 1 : 3-pin bidirectional FS interface |
| [4:3] | RW | Non- usage | 0x0 | Non- usage |
| [2:0] | RW | TOutCal | 0x0 | HS/FS Timeout Calibration (USB defines the FS mode timeout value as 16-18 bits time , which is the delay added to accommodate different PHY delays ) Calculation formula: 0.25bit time * The value |

      - GRSTCTL

Offset address: 0x010 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RO | AHBIdle | 0x1 | AHB Master Idle , indicating AHB The Master state machine is in the idle state |
| [30] | RO | DMAReq | 0x0 | DMA Request Signal , indicating DMA Req request, used for debugging |
| [29:11] | - | reserved | 0x0 | reserve |
| [10:6] | RW | FxFNum | 0x0 | TxFIFO Number 'h0 : Reset host mode Non-periodic TxFIFO Reset dev mode Non-periodic TxFIFO ( shared ) Reset dev mode TxFIFO0 ( dedicated ) 'h1 : Reset host mode Periodic TxFIFO Reset dev mode Periodic TxFIFO1 ( shared ) Reset dev mode TxFIFO1 ( dedicated ) 'h2 : Reset dev mode Periodic TxFIFO2 ( shared ) Reset dev mode TxFIFO2 ( dedicated ) ... |

|  |  |  |  | 'hf : reset dev mode Periodic TxFIFO15 ( shared ) Reset dev mode TxFIFO15 ( dedicated ) 'h10 : Reset all TxFIFOs |
| --- | --- | --- | --- | --- |
| [5] | WS/SC | TxFFLSH | 0x0 | TxFIFO reset enable Before setting this bit, you must confirm that there is no read or write operation in TxFIFO , and the application must wait until this bit is automatically cleared (8 phy_clk or hclk ) before performing new read or write operations. |
| [4] | WS/SC | RxFFLSH | 0x0 | RxFIFO reset enable Before setting this bit, you must confirm that there is no read or write operation in the RxFIFO , and the application must wait until this bit is automatically cleared (8 phy_clk or hclk ) before performing new read or write operations. |
| [3:2] | - | Non- usage | 0x0 | Not used |
| [1] | WS/SC | HSftRst | 0x0 | hclk soft reset , hclk clock domain soft reset |
| [0] | WS/SC | CSftRst | 0x0 | hclk and phy_clk clock domain soft reset |

      - GINTSTS

Offset address: 0x014 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RO/WC | WkUpInt | 0x0 | Resume/Remote Wakeup Dectected interrupt Dev mode: detect USB Set when Resume |
| [30:29] | - | Non- usage | 0x5 | Not used |
| [28] | RO/WC | ConIDStsChng | 0x0 | Connector ID Status Change interrupt |
| [27] | - | reserved | 0x0 | reserve |
| [26:24] | - | Non- usage | 0x4 | Not used |
| [twenty three] | - | reserved | 0x0 | reserve |
| [twenty two] | RO/WC | FetSusp | 0x0 | Data Fetch Suspended , valid only in DMA mode |
| [twenty one] | RO/WC | incomplSOOUT | 0x0 | Incomplete Isochronous OUT Transfer (dev- only) dev mode, there is at least one ISO OUT EP has not completed transmission in the current frame , and is set at the same time as the EOPF interrupt |
| [20] | RO/WC | incompleSOIN | 0x0 | Incomplete Isochronous IN Transfer (dev- only) dev mode, there is at least one ISO IN EP has not completed transmission in the current frame , and is set at the same time as the EOPF interrupt |
| [19] | RO | OEPInt | 0x0 | OUT Endpoints interrupt (dev- only) dev mode, if there is OUT Set when the EP interrupt is not processed. The application must read DAINT and DOEPINTn to determine the specific interrupt event to be processed. |
| [18] | RO | IEPInt | 0x0 | IN Endpoints interrupt (dev- only) dev mode, if there is IN Set when the EP interrupt is not processed. The application must read DAINT and DIEPINTn to determine the specific interrupt event to be processed. |
| [17] | RO/WC | EPMis | 0x0 | Endpoint Mismatch interrupt (dev- only) Only valid when shared FIFO is in operation. After token , FIFO When the data in does not match, it triggers |
| [16] | - | reserved | 0x0 | reserve |

| [15] | RO/WC | EOPF | 0x0 | End of Periodic Frame interrupt (dev- only) Reach the Periodic value set by DCFG.PerFrInt Frame Triggered when the Interval period is reached |
| --- | --- | --- | --- | --- |
| [14] | RO/WC | ISOOutDrip | 0x0 | ISO OUT Packet Dropped interrupt (dev- only) When the RxFIFO is insufficient and the soft core cannot receive ISO Triggered when OUT packet |
| [13] | RO/WC | EnumDone | 0x0 | Enumeration Done interrupt (dev- only) Triggered after speed enumeration is completed. The application must read DSTS to obtain the enumeration result. |
| [12] | RO/WC | USBRst | 0x0 | USB Reset interrupt (dev- only) Soft core monitoring to USB Triggered after the reset signal |
| [11] | RO/WC | USBSusp | 0x0 | USB Suspend interrupt (dev- only) Triggered after the soft core detects the suspend state |
| [10] | RO/WC | ErlySusp | 0x0 | Early Suspend interrupt (dev- only) The soft core is triggered when it detects that the USB bus is in idle state for more than 3ms . |
| [9:8] | - | Non- usage | 0x0 | Not used |
| [7] | RO | GOUTNakEff | 0x0 | Global OUT NAK Effective interrupt (dev-only) Triggered when DCTL.SGOUTNak is set, DCTL.CGOUTNak is set Time Clear |
| [6] | RO | GINNakEff | 0x0 | Global IN Non-periodic NAK Effective interrupt (dev- only) DCTL.SGNPInNak is set, cleared when DCTL.CGNPInNak is set |
| [5] | RO | Non- usage | 0x1 | Non-Periodic TxFIFO Empty interrupt Only valid in shared TxFIFO mode |
| [4] | RO | RxFv | 0x0 | RxFIFO Non- Empty Triggered when there is at least one packet data waiting to be read in the RxFIFO |
| [3] | RO/WC | Sof | 0x0 | Start of Frame Interrupt dev mode, the soft core is triggered when it receives SOF , and the application then reads DSTS Get the frame number |
| [2] | RO | OTGInt | 0x0 | OTG interrupt Triggered when an OTG protocol event occurs |
| [1] | RO/WC | ModeMis | 0x0 | Mode Mismatch interrupt Triggered when the register in dev mode is used as host mode, and the AHB bus responds OKAY , but the soft core ignores the register access operation |
| [0] | RO | CurMod | 0x0 | Current Mode of Operation 0 : dev , 1 : host |

      - GINTMSK

Offset address: 0x018 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RW | WkUpIntMsk | 0x0 | Resume/Remote Wakeup Dectected interrupt mask 0 : Shield , 1: Enable (same below ) |

| [30:29] | RW | Non- usage | 0x0 | Not used |
| --- | --- | --- | --- | --- |
| [28] | RW | ConIDStsChngMsk | 0x0 | Connector ID Status Change interrupt mask |
| [27] | - | reserved | 0x0 | reserve |
| [26:24] | RW | Non- usage | 0x0 | Not used |
| [twenty three] | - | reserved | 0x0 | reserve |
| [twenty two] | RW | FetSuspMsk | 0x0 | Data Fetch Suspended interrupt mask, only valid in DMA mode |
| [twenty one] | RW | incomplSOOUTMs k | 0x0 | Incomplete Isochronous OUT Transfer interrupt mask (dev-only) |
| [20] | RW | incompleSOINMsk | 0x0 | Incomplete Isochronous IN Transfer interrupt mask (dev- only) |
| [19] | RW | OEPIntMsk | 0x0 | OUT Endpoints interrupt mask (dev- only) |
| [18] | RW | IEPIntMsk | 0x0 | IN Endpoints interrupt mask (dev- only) |
| [17] | RW | EPMisMsk | 0x0 | Endpoint Mismatch interrupt (dev- only) |
| [16] | - | reserved | 0x0 | reserve |
| [15] | RW | EOPFMsk | 0x0 | End of Periodic Frame interrupt mask (dev- only) |
| [14] | RW | ISOOutDripMsk | 0x0 | ISO OUT Packet Dropped interrupt mask (dev- only) |
| [13] | RW | EnumDoneMsk | 0x0 | Enumeration Done interrupt mask (dev- only) |
| [12] | RW | USBRstMsk | 0x0 | USB Reset interrupt mask (dev- only) |
| [11] | RW | USBSuspMsk | 0x0 | USB Suspend interrupt mask (dev- only) |
| [10] | RW | ErlySuspMsk | 0x0 | Early Suspend interrupt mask (dev- only) |
| [9:8] | RW | Non- usage | 0x0 | Not used |
| [7] | RW | GOUTNakEffMsk | 0x0 | Global OUT NAK Effective interrupt mask (dev- only) |
| [6] | RW | GINNakEffMsk | 0x0 | Global IN Non-periodic NAK Effective interrupt mask (dev-only) |
| [5] | RW | NPTxFEmpMsk | 0x0 | Non-Periodic TxFIFO Empty interrupt mask |
| [4] | RW | RxFLvlMsk | 0x0 | RxFIFO Non-Empty Shield |
| [3] | RW | SofMsk | 0x0 | Start of Frame interrupt mask |
| [2] | RW | OTGIntMsk | 0x0 | OTG interrupt mask |
| [1] | RW | ModeMisMsk | 0x0 | Mode Mismatch interrupt mask |
| [0] | - | reserved | 0x0 | reserve |

      - GRXSTSR/ GRXSTSP

Offset address: 0x01c/0x020 Reset mode: H/S

This register entity is stored in the RxFIFO In, read GRXSTSR and GRXSTSP The return value is exactly the same when reading the register, the difference is that GRXSTSP By the way, the 32 Bit header information from RxFIFO Medium POP Come out, and GRXSTSR Just read .

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:25] | - | reserved | 0x0 | reserve |
| [24:21] | RO | FrmNum | 0x0 | Frame Number , ISO The lower four bits of the OUT frame number |
| [20:17] | RO | PktSts | 0x0 | Packet Status , indicating the status of the received packet 'h1 : Global OUT NAK ( trigger interrupt ) |

|  |  |  |  | 'h2 : OUT packet received 'h3 : OUT transmission ends ( trigger interrupt ) 'h4 : SETUP transmission ends ( trigger interrupt ) 'h6 : SETUP packet received Others: Reserved |
| --- | --- | --- | --- | --- |
| [16:15] | RO | DPID | 0x0 | Data PID , indicating the receiving packet status 'h0 : DATA0 'h1 : DATA2 'h2 : DATA1 'h3 : MDATA |
| [14:4] | RO | BCnt | 0x0 | Packet Status , indicating the length of the received packet data |
| [3:0] | RO | EPNum | 0x0 | Endpoint Number , the destination port number of the current data |

Note: GRXSTSP For POP only The header information seems to be not POPable Data information in the package, data information requires POP Corresponding RxFIFO address

      - GRXFSIZ

Offset address: 0x024 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | - | reserved | 0x0 | reserve |
| [15:0] | RW | RxFDep | 0x80 | RxFIFO depth ( width 35 bits ), min=16 This segment is adjustable according to the actual depth of SPRAM |

      - GNPTXFSIZ

Offset address: 0x028 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RW | INEPTxF0Dep | 0x80 | IN EP TxFIFO depth ( width 35 bits ), min=16 , depth adjustable |
| [15:0] | RW | INEPTxF0StAddr | 0x80 | IN EP TxFIFO start address, adjustable |

      - GGPIO

Offset address: 0x038 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | - | Reserved | 0x0 | reserve |
| [30] | RW | PADRcvEn | 0x1 | PAD differential receive enable 0 : Disable; 1 : Enable |
| [29:28] | RW | PADDPPullUp | 0x0 | PAD DP line DPPU control 'h0/1 : soft core dp_rpu1_en signal control 'h2 : Forced pull-up enabled 'h3 : Forced pull-up is invalid |

| [27:26] | RW | PADDPPullUpLO | 0x0 | PAD DP line DPPULO control 'h0: soft core dp_rpu2_en signal control 'h1: soft core dp_rpu1_en signal control 'h2 : forced pull-up enable 'h3 : Forced pull-up is invalid |
| --- | --- | --- | --- | --- |
| [25:24] | RW | PADDMPullUp | 0x0 | PAD DM line DMPU control 'h0/1 : soft core dm_rpu_en signal control 'h2 : Forced pull-up enabled 'h3 : Forced pull-up is invalid |
| [23:22] | RW | PADDMPullUpL O | 0x0 | PAD DM line DMPULO control 'h0/1 : soft core dm_rpu_en signal control 'h2 : Forced pull-up enabled 'h3 : Forced pull-up is invalid |
| [twenty one] | RW | PADPullDown | 0x0 | PAD pull-down resistor control 0 : Pull-down disabled; 1 : Pull-down enabled |
| [20] | - | Reserved | 0x0 | reserve |
| [19] | RW | PADSpeed | 0x1 | PAD rate selection 0 : LS mode; 1: FS mode |
| [18:17] | RW | PADSuspend | 0x0 | PAD Suspend Control 'h0/1 : soft core control Suspend signal 'h2 : Forced suspend enabled 'h3 : Forced suspension is invalid |
| [16] | RW | PADPowerDown | 0x0 | PAD offline 0 : Power on; 1 : Power off |
| [15:0] | - | gp_in | 0x0 | Not used |

      - GUID

Offset address: 0x03c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | User ID | 0x4570_0 322 | User-defined ID number |

      - GSYNPSID

Offset address: 0x040 Reset method: N/A

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | Synopsys ID | 0x4f54_270a | USB soft core version number |

      - GHWCFG1

Offset address: 0x044 Reset method:

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:10] | RO | Non- usage | 0x0 | EP direction, two bits indicate the EP direction 'h0 : IN/OUT , ' h1 : IN, 'h2 : OUT , ' h3 : Reserved |
| [9:8] | RO | EP4Dir | 0x2 | EP4 direction: OUT |
| [7:6] | RO | EP3Dir | 0x1 | EP3 Direction: IN |
| [5:4] | RO | EP2Dir | 0x2 | EP2 direction: OUT |
| [3:2] | RO | EP1Dir | 0x1 | EP1 Direction: IN |
| [1:0] | RO | EP0Dir | 0x0 | EP0 direction: IN/OUT |

      - GHWCFG2

Offset address: 0x048 Reset method:

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | - | reserved | 0x0 | reserve |
| [30:22] | RO | Non- usage | 0x8a | Not used |
| [21:20] | - | reserved | 0x0 | reserve |
| [19] | RO | DynFifoSizing | 0x1 | Support dynamic FIFO allocation |
| [18:14] | RO | Non- usage | 0x3 | Not used |
| [13:10] | RO | NumDevEps | 0x4 | Supports 4 Dev ports ( except EP0 )​​ |
| [9:8] | RO | FSPhyType | 0x1 | FS PHY interface type: dedicated FS interface |
| [7:6] | RO | HSPhyType | 0x0 | HS PHY interface type: HS not supported PHY |
| [5] | RO | SingPnt | 0x0 | Point-to-Point ( not used ) |
| [4:3] | RO | OtgArch | 0x2 | Soft core architecture: internal DMA |
| [2:0] | RO | OtgMode | 0x4 | Working mode: Non-OTG Dev |

      - GHWCFG3

Offset address: 0x04c Reset method:

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RO | DfifoDepth | 0x7a | DFIFO effective depth: 122 |
| [15:12] | - | reserved | 0x0 | reserve |
| [11] | RO | RstType | 0x0 | Soft core reset mode: asynchronous reset |
| [10] | RO | OptFeature | 0x1 | Remove softcore optional features: No optional features |
| [9] | RO | VndctlSupt | 0x0 | Support Vendor Control Interface: Not supported |
| [8] | RO | I2CINtSel | 0x0 | Support I2C interface: Not supported |
| [7] | RO | OtgEn | 0x0 | Support OTG : No |

| [6:4] | RO | PktSizeWidth | 0x0 | Packet counter bit width: 4 bits |
| --- | --- | --- | --- | --- |
| [3:0] | RO | XferSizeWidth | 0x0 | Transfer byte counter bit width: 11 bits |

      - GHWCFG4

Offset address: 0x050 Reset method:

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RO | SGDMA | 0x0 | Support SG-DMA dynamic configuration: Not supported |
| [30] | RO | SGDMAConfig | 0x0 | Is there SG-DMA configuration: No |
| [29:26] | RO | INEPs | 0x2 | Supports 3 IN ports ( including EP0 ) |
| [25] | RO | DedFifoMode | 0x1 | Dedicated TxFIFO : Yes |
| [24:20] | RO | Non- usage | 0x3 | Not used |
| [19:16] | RO | NumCtlEps | 0x0 | Supports 0 control EP number ( except EP0 )​ |
| [15:14] | RO | PhyDataWidth | 0x0 | PHY interface width: 8 bits |
| [13:6] | - | reserved | 0x0 | reserve |
| [5] | RO | AhbFreq | 0x1 | AHB clock greater than 60MHz ? Yes |
| [4] | RO | EnablePwrOpt | 0x0 | Support power consumption optimization: Not supported |
| [3:0] | RO | Non- usage | 0x0 | Not used |

      - GUSRSTS

Offset address: 0x054 Reset method:

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | RO | RxRevSpc | 0x10 | RxFIFO is reserved for CTRL transmission, double word unit |
| [23:16] | RO | RxAvailSpc | 0x70 | RxFIFO non- CTRL transmission available space, double word unit |
| [15] | - | Reserved | 0x0 | reserve |
| [14] | RO | DFBusy | 0x0 | FIFO busy 1 : Busy, 0 : Idle |
| [13:9] | RO | RxPktNum | 0x0 | Number of packets in RxFIFO |
| [8:5] | RO | RxTknPID | 0x0 | Receive packet PID 4′h:1 OUT 4 h2: ACK 4′h3: DATA0 4'h6: SOF 4′h9: IN 4ha: NAK 4 hb: DATA1 4′hc: PRE 4′hd: SETUP 4'he: STALL Ohters: Reserved |

| [4:0] | RO | TxRxFSMState | 0x0 | MAC layer state machine current state 5′h0: IDLE 5′h1-5: Not used (Host mode ) 5′h6: Receive Error 5′h7: Receive Data 5′h8: Send Handshake 5′h9: Send EOP 5ha: Send Data 5 hb: Wait Handshake 5 hc: Check CRC16 5 hd: Wait Data 5′he: Test Mode 5′hf: Wait Bus Idle 5′h10: Wait Turnaround 5 h11: Data To Status Others: Reserved |
| --- | --- | --- | --- | --- |

      - DIEPTXF1/3

Offset address: 0x104/0x108 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RW | INEPnTxFDep | 0x80 | IN EP TxFIFOn depth ( effective bit width 32) , configurable |
| [15:0] | RW | INEPnTxFStAddr | 0x100 | IN EP FIFOn starting address, DIEPTXF3 : 0x180 |

Note: For detailed configuration methods, see Section

      - DCFG

Offset address: 0x800 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:26] | - | reserved | 0x0 | reserve |
| [25:18] | - | Non- usage | 0x8 | Not used |
| [17:13] | - | reserved | 0x0 | reserve |
| [12:11] | RW | PerFrInt | 0x0 | Periodic Frame Interval , from SOF to soft core setting End of Periodic Frame interruption duration 'h0: 80% of the frame interval 'h1: 85% of the frame interval 'h2: 90% of the frame interval 'h3: 95% of the frame interval |
| [10:4] | RW | DevAddr | 0x0 | Dev address, the application software must set it after receiving the SetAddr instruction This paragraph |
| [3] | - | reserved | 0x0 | reserve |

| [2] | RW | NZStsOUTHshk | 0x0 | Status with non-zero length OUT handshake core 's response mode in the Status Stage when receiving a non-zero length OUT transaction in a control transfer 1 : Respond to the STALL packet and report the data to the application 0 : Report data to the application and respond to the handshake packet according to DCTL.NAK/STALL |
| --- | --- | --- | --- | --- |
| [1:0] | RW | DevSpd | 0x0 | Dev speed 'h0: HS(USB2.0 PHY CLK=30/60MHz) 'h1: FS(USB2.0 PHY CLK=30/60MHz) 'h2: LS(USB1.1 PHY CLK=6MHz) 'h3: FS(USB1.1 PHY CLK = 48MHz) |

      - DCTL

Offset address: 0x804 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | - | reserved | 0x0 | reserve |
| [15:13] | - | Non- usage | 0x0 | Not used |
| [12] | - | reserved | 0x0 | reserve |
| [11] | RW | PWROnPrgDone | 0x0 | Power-On Programming Done Mark the soft core from power Register configuration is completed after waking up from down mode |
| [10] | WO | CGOUTNak | 0x0 | Clear Global OUT NAK Clear Global OUT NAK |
| [9] | WO | SGOUTNak | 0x0 | Set Global OUT NAK When set, all OUT ports in the soft core should NAK packets Before enabling this bit, be sure to clear GINTSTS.GOUTNakEff |
| [8] | WO | CGNPInNak | 0x0 | Clear Global Non-periodic IN NAK Clear Global Non-periodic IN NAK bit |
| [7] | WO | SGNPInNak | 0x0 | Set Global Non-periodic IN NAK When set, all IN ports in the soft core should NAK packets Before enabling this bit, be sure to clear GINTSTS.GINNakEff |
| [6:4] | RW | TstCtl | 0x0 | Test Controls 'h0 : Test mode is invalid 'h1 : Test J mode 'h2 : Test K mode 'h3 : Test SE0_NAK mode 'h4 : Test Packet mode 'h5 : Test Force_Enable mode Others: Reserved |
| [3] | RO | GOUTNakSts | 0x0 | Global OUT NAK status 0 : A handshake packet has occurred according to the FIFO status and NAK/STAL settings. 1 : No data is written to RxFIOF . Except for SETUP transactions, all packets |

|  |  |  |  | Reply NAK and discard all ISO OUT . |
| --- | --- | --- | --- | --- |
| [2] | RO | GNPINNakSts | 0x0 | Global Non-periodic IN NAK state 0 : A handshake packet has occurred according to the FIFO status 1 : All non-periodic IN port responds with NAK |
| [1] | RW | SftDiscon | 0x0 | Soft Disconnect After setting, the soft core is in a disconnected state and no data interaction occurs. |
| [0] | RW | RmtWkUpSig | 0x0 | Remote Wakeup Signaling After setting, the soft core initiates a remote signal to wake up the USB The host application must set this bit to exit the soft core suspend state. According to the USB 2.0 specification, the application must clear this bit 1-15ms after setting it . |

      - DSTS

Offset address: 0x808 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:22] | - | reserved | 0x0 | reserve |
| [21:8] | RO | SOFFrmNum | 0x0 | Frame Number of Received SOF , receive SOF frame number |
| [7:4] | - | reserved | 0x0 | reserve |
| [3] | RO | ErrticErr | 0x0 | Uncertain error UTMI PHY status error reports this error, and the software enters the suspended state, and GINTSTS.ErlySusp is set. At this time, the application software can only be restored through soft disconnect |
| [2:1] | RO | EnumSpd | 0x1 | Enumerated Speed , indicating the operating frequency after the soft core is powered on and enumerated. 'h0: HS (USB2.0 PHY CLK=30/60MHz) 'h1: FS(USB2.0 PHY CLK=30/60MHz) 'h2: LS(USB1.1 PHY CLK=6MHz) 'h3: FS(USB1.1 PHY CLK = 48MHz) |
| [0] | RO | SuspSts | 0x0 | Suspend Status Dev mode, the soft core detects a suspend condition and sets the bit. The soft core then enters the suspend state. The soft core exits the suspend state under any of the following conditions: - There is a level change on the phy_lin_state_i signal line; - DCTL.RmtWkUpSig is set |

      - DIEPMSK

Offset address: 0x810 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:10] | - | reserved | 0x0 | reserve |
| [9] | RW | BNAInIntrMsk | 0x0 | Buffer Not Available interrupt mask, 0 : mask , 1: enable (same below ) |
| [8] | RW | TxfifoUndrnMsk | 0x0 | FIFO Underrun Shield |

| [7] | - | reserved | 0x0 | reserve |
| --- | --- | --- | --- | --- |
| [6] | RW | INEPNakEffMSK | 0x0 | IN Endpoint NAK Effective Mask |
| [5] | RW | INTknEPMisMsk | 0x0 | IN Token received with EP Mismatch Mask |
| [4] | RW | INTknTXFEEmp Msk | 0x0 | IN Token Received When TxFIFO Empty Mask |
| [3] | RW | TimeOUTMsk | 0x0 | Timeout Condition Mask |
| [2] | RW | AHBErrMsk | 0x0 | AHB Error Mask , valid for DMA |
| [1] | RW | EPDisbldMsk | 0x0 | Endpoint Disabled Interrupt Mask |
| [0] | RW | XferComplMsk | 0x0 | Transfer Completed Interrupt Mask |

      - DOEPMSK

Offset address: 0x814 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:10] | - | reserved | 0x0 | reserve |
| [9] | RW | BNAInIntrMsk | 0x0 | BNA interrupt mask, 0 : mask , 1: enable (same below ) |
| [8] | RW | OutPktErrMsk | 0x0 | OUT Packet Error Mask |
| [7] | - | reserved | 0x0 | reserve |
| [6] | RW | Back2BackSETup | 0x0 | Back-to-Back Packets Received Mask |
| [5] | - | reserved | 0x0 | reserve |
| [4] | RW | OUTTknEPdisMsk | 0x0 | OUT Token Received when Endpoint Disabled mask |
| [3] | RW | SetUPMsk | 0x0 | SETUP Phase Done Mask , only used for control ports |
| [2] | RW | AHBErrMsk | 0x0 | AHB Error Mask |
| [1] | RW | EPDisbldMsk | 0x0 | Endpoint Disabled Interrupt Mask |
| [0] | RW | XferComplMsk | 0x0 | Transfer Completed Interrupt Mask |

      - DAINT

Offset address: 0x818 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RO | OutEPInt | 0x0 | Each bit indicates an OUT port bit16 : OUT EP0 bit31 : OUT EP15 |
| [15:0] | RO | InEpInt | 0x0 | Each bit indicates an IN port bit0 : IN EP0 bit15 : IN EP15 |

      - DAINTMSK

Offset address: 0x81c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RO | OutEPIntMSK | 0x0 | Each bit indicates an OUT port, 0 : shield , 1: enable (same below) bit16 : OUT EP0 bit31 : OUT EP15 |
| [15:0] | RO | InEpIntMSK | 0x0 | Each bit indicates an IN port bit0 : IN EP0 bit15 : IN EP15 |

      - DTHRCTL

Offset address: 0x830 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:28] | - | reserved | 0x0 | reserve |
| [27] | RW | ArbPrkEn | 0x1 | Arbiter Parking Enable Controls arbitration of the internal DMA pause IN port |
| [26] | - | reserved | 0x0 | reserve |
| [25:17] | RW | RxThrLen | 0x8 | Receive Threshold Length Receive threshold, minimum value is 8 DWord (32bit) |
| [16] | RW | RxThrEn | 0x0 | Receive Threshold Enable Receive threshold enable |
| [15:11] | - | reserved | 0x0 | reserve |
| [10:2] | RW | TxThrLen | 0x8 | TransmitThreshold Length Send threshold, minimum value is 8 DWord (32bit) |
| [1] | RW | ISOThrEn | 0x0 | ISO IN Endpoints Threshold Enable ISO IN port receive threshold enable |
| [0] | RW | NonISOThrEn | 0x0 | Non-ISO IN Endpoints Threshold Enable Non- ISO IN port receive threshold enable |

      - DIEPMPMSK

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | - | reserved | 0x0 | 保留 |
| [15:0] | RO | InEpTxfEmpMsk | 0x0 | 每个位标示一个IN端口，用于控制DIEPINTn.TxfEmp 0: 屏蔽, 1:使能 bit0：IN EP0 |

Offset address: 0x834 Reset mode: H/S

|  |  |  |  | bit15 : IN EP15 |
| --- | --- | --- | --- | --- |

      - DIEPCTL0

Offset address: 0x900 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | WS/SC | EPEna | 0x0 | Endpoint Enable In non- SG-DMA mode, when set, indicates data is ready. The soft core automatically clears this bit before asserting any of the following interrupts: endpoint disable transfer completed |
| [30] | WS/SC | EPDis | 0x0 | Endpoint diable , can only be used after setting EPEna When set, it forces the data transmission to be terminated ( even if the data has not yet ended ) . The soft core automatically clears this bit before setting the Endpoint disable interrupt. |
| [29:28] | - | reserved | 0x0 | reserve |
| [27] | WO | SNAK | 0x0 | Set NAK When set, the control port responds with NAK |
| [26] | WO | CNAK | 0x0 | Clear NAK , clear the SNAK bit |
| [25:22] | RW | TxFNum | 0x0 | Specify a dedicated TxFIFO number Indicates allocation to IN EP0 TxFIFO number, must be zero |
| [twenty one] | WS/SC | Stall | 0x0 | Stall handshake The application can only set this bit after receiving the SETUP signaling. This bit has higher priority than NAK , Global Non-Periodic IN NAK and Global OUT NAK bits. |
| [20] | - | reserved | 0x0 | reserve |
| [19:18] | RO | EPType | 0x0 | Port type, EP0 is only used for control |
| [17] | RO | NAKSts | 0x0 | NAK status 0 : The soft core sends a non-NAK handshake packet based on the FIFO status 1 : The soft core sends a NAK handshake packet When set, the soft core forces the termination of data transfer |
| [16] | - | reserved | 0x0 | reserve |
| [15] | RO | USBActEP | 0x1 | USB Active port, EP0 is always active |
| [14:11] | RW | NextEp | 0x0 | Next port ( only for shared FIFO configuration, non-usage) |
| [10:2] | - | reserved | 0x0 | reserve |
| [1:0] | RW | MPS | 0x0 | Maximum Packet Size 'h0: 64 bytes 'h1 : 32 bytes 'h2 : 16 bytes 'h3: 8 bytes |

      - DIEPINT0/1/3

Offset address: 0x908/0x928/0x968 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:10] | - | reserved | 0x0 | reserve |
| [9] | RO/WC | Non- usage | 0x0 | Not used |
| [8] | RO/WC | TxfifoUndrn | 0x0 | Set DATA1 PID , IN Only valid in configurations with dedicated TxFIFO and threshold |
| [7] | RO | TxFEmp | 0x0 | TxFIFO Empty when the TxFIFO is half empty or completely empty. GAHBCFG.NPTXFempLvl determines whether the interrupt is half-empty or full-empty. |
| [6] | RO/WC | INEPNakEff | 0x0 | IN Endpoint NAK Effective the soft core detects that NAK is set, the interrupt is triggered. DIEPCTLn.CNAK clears the interrupt |
| [5] | - | INTknEPMis | 0x0 | Receive IN that does not match EP Token (non-periodic valid ) The port receives IN Token , non-periodic Triggered when the data at the TxFIFO output port is not the EP |
| [4] | RO/WC | INTknTXFEmp | 0x0 | Receive IN Token TxFIFO empty ( non-periodic valid ) receiving the IN command, the TxFIFO of the corresponding EP is empty. |
| [3] | RO/WC | Timeout | 0x0 | Timeout Condition , the applicable conditions are as follows: Dedicated TxFIFO configuration : CTRL The soft core is triggered after detecting that the last IN instruction of USB has timed out. |
| [2] | RO/WC | AHBErr | 0x0 | AHB Error In internal DMA mode, trigger when AHB read or write error occurs |
| [1] | RO/WC | EPDisbld | 0x0 | Endpoint Disable Triggered when the port is closed |
| [0] | RO/WC | XferCompl | 0x0 | Transfer Complete Triggered when the specified number of transfers have completed |

      - DIEPTSIZ0

Offset address: 0x910 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:21] | - | reserved | 0x0 | reserve |
| [20:19] | RW | PktCnt | 0x0 | Packet Count EP0 The number of USB packets in this transaction . After the soft core sends a packet to the TxFIFO , the segment is automatically reduced by 1. |
| [18:7] | - | reserved | 0x0 | reserve |
| [6:0] | RW | XferSize | 0x0 | Transfer Size The total byte length of this transaction of EP0 , that is, the sum of the lengths of PktCnt packets. After the soft core pushes the sent data into TxFIFO , the value of this segment is automatically reduced by 1 . |

|  |  |  |  | it reaches 0 , the transfer completion interrupt is triggered. |
| --- | --- | --- | --- | --- |

      - DIEPDMA0/1/3

Offset address: 0x914/0x934/0x974 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | DMAAddr | 0x0 | EP0 IN DMA address |

      - DTXFSTS0/1/3

Offset address: 0x918/0x938/0x978 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | - | reserved | 0x0 | reserve |
| [15:0] | RO | InEpTxFSpcAvail | 0x0 | The corresponding port TxFIFO space is available 'h0 ： EP0 TxFIFO full 'h1 : remaining 1 word … 'hn : remainder n words(0 ≤ n ≤ 32768) 'h8000 : 32768 words remaining Others: Reserved |

      - DOEPCTL0

Offset address: 0xb00 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | WS/SC | EPEna | 0x0 | Endpoint Enable In non- SG-DMA mode, when set, it indicates that the FIFO allocation is ready to receive Host data. The soft core automatically clears this bit before setting any of the following interrupts: SETUP Phase Done Transfer Completed Endpoint Disable ( illegal ) |
| [30] | RO | EPDis | 0x0 | Endpoint diable , OUT EP0 is always in a valid state and cannot be disabled. effect |
| [29:28] | - | reserved | 0x0 | reserve |
| [27] | WO | SNAK | 0x0 | Set NAK When set, the control port responds with NAK |

| [26] | WO | CNAK | 0x0 | Clear NAK , clear the SNAK bit |
| --- | --- | --- | --- | --- |
| [25:22] | - | reserved | 0x0 | reserve |
| [twenty one] | WS/SC | Stall | 0x0 | Stall handshake The application can only set this bit after receiving the SETUP signaling. This bit has a higher priority than NAK and Global OUT NAK bits. |
| [20] | RW | Snp | 0x0 | Snoop Mode In Snoop mode, the soft core does not check whether the OUT packet is correct, but directly reports it to the application |
| [19:18] | RO | EPType | 0x0 | Port type, EP0 is only used for control |
| [17] | RO | NAKSts | 0x0 | NAK status 0 : The soft core sends a non-NAK handshake packet based on the FIFO status 1 : The soft core sends a NAK handshake packet When set, the soft core forces the termination of data transfer |
| [16] | - | reserved | 0x0 | reserve |
| [15] | RO | USBActEP | 0x1 | USB Active port, EP0 is always active |
| [14:2] | - | reserved | 0x0 | reserve |
| [1:0] | RO | MPS | 0x0 | Maximum Packet Size , same as DIEPCTL0.MPS'h0 : 64 bytes 'h1 : 32 bytes 'h2 : 16 bytes 'h3: 8 bytes |

      - DOEPINT0/2/4

Offset address: 0xb08/0xb48/0xb88 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:10] | - | reserved | 0x0 | reserve |
| [9] | RO/WC | Non- usage | 0x0 | Not used |
| [8] | RO/WC | OutPktErr | 0x0 | OUT Packet Error Only valid in configurations with dedicated TxFIFO and threshold |
| [7] | - | Non- usage | 0x0 | Not used |
| [6] | RO/WC | Back2BackSETup | 0x0 | Back2BackSETup (CTRL is valid ) the soft core receives three or more consecutive SETUP packets |
| [5] | - | Non- usage | 0x0 | Not used |
| [4] | RO/WC | OUTTknEPdis | 0x0 | OUT Token Received when EP Disa (CTRL valid ) OUT is received , EP is not enabled yet |
| [3] | RO/WC | SetUp | 0x0 | SETUP Phase DONE (CTRL is valid ) SETUP​ Triggered when the Stage is completed and there are no subsequent SETUP packets. After this interrupt, the application can start parsing the SETUP packet. |
| [2] | RO/WC | AHBErr | 0x0 | AHB Error In internal DMA mode, trigger when AHB read or write error occurs |

| [1] | RO/WC | EPDisbld | 0x0 | Endpoint Disable Triggered when the port is closed |
| --- | --- | --- | --- | --- |
| [0] | RO/WC | XferCompl | 0x0 | Transfer Complete The specified number of transfers has completed |

      - DOEPTSIZ0

Offset address: 0xb10 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | - | reserved | 0x0 | reserve |
| [30:29] | RW | SUPCnt | 0x0 | SETUP Packet Count SETUP packets that the port can accept . After the soft core reads a packet from the RxFIFO , this segment is automatically reduced by 1. 'h1: 1 , 'h2: 2, 'h3: 3 |
| [28:20] | - | reserved | 0x0 | reserve |
| [19] | RW | PktCnt | 0x0 | Packet Count EP0 The number of USB packets in this transaction . After the soft core reads a packet from the RxFIFO , this segment is automatically reduced to 0. |
| [18:7] | - | reserved | 0x0 | reserve |
| [6:0] | RW | XferSize | 0x0 | Transfer Size EP0 total byte length of this transaction. After the soft core receives a byte and reads it from the RxFIFO , the value of this segment automatically decreases by 1. When it decreases to 0 , the transmission completion interrupt is triggered. |

      - DOEPDMA0/2/4

Offset address: 0xb14/0xb54/0xb94 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | DMAAddr | 0x0 | EP0 OUT DMA Address |

      - DIEPCTL1/3

Offset address: 0x920/0x960 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | WS/SC | EPEna | 0x0 | Endpoint Enable Set to indicate that the data is ready to be sent, and the soft core automatically clears this bit before setting any of the following interrupts Endpoint disable |

|  |  |  |  | Transfer completed |
| --- | --- | --- | --- | --- |
| [30] | WS/SC | EPDis | 0x0 | Endpoint Disable When set, it forces the termination of data transmission ( even if the data has not yet ended ) , and the soft core automatically clears this bit before setting the Endpoint disable interrupt. be used after setting EPEna |
| [29] | WO | SetD1PID | 0x0 | Set DATA1 PID Set port Data PID value is set to DATA1 |
| [28] | WO | SetD0PID | 0x0 | Set DATA0 PID Set port Data PID value is set to DATA0 |
| [27] | WO | SNAK | 0x0 | Set NAK When set, the control port responds with NAK |
| [26] | WO | CNAK | 0x0 | Clear NAK , IN/OUT Clear the SNAK bit |
| [25:22] | RW | TxFNum | 0x0 | Specify a dedicated TxFIFO number Configure mapping to EP1 IN and EP3 IN TxFIFO number DIEPCTL1.TxFNum = 0x1 DIEPCTL3.TxFNum = 0x2 |
| [twenty one] | RW | Stall | 0x0 | Stall handshake ( valid in non-ISO ) After setting, the port only responds to STALL . This bit has higher priority than NAK , Global Non-Periodic IN NAK and Global OUT NAK bit, the application should clear this bit. |
| [20] | - | Non- usage | 0x0 | Not used |
| [19:18] | RO | EPType | 0x0 | Port transmission type 'h0 : Control ( not supported ) 'h1: ISO 'h2: Bulk 'h3: Intr |
| [17] | RO | NAKSts | 0x0 | NAK status 0 : The soft core sends a non-NAK handshake packet based on the FIFO status 1 : The soft core sends a NAK handshake packet When set, data is transferred as follows: Non-ISO IN port: The soft core is forced to stop sending any data; ISO IN port: The soft core sends a zero-length data packet; |
| [16] | RO | DPID | 0x0 | Endpoint Data PID (Intr/Bulk valid ) PID information of the packet that the port is about to receive / send . After the port is enabled, the application must set the SetD0PID or SetD1PID information for a receive / send packet . 0 : DATA0 , 1 : DATA1 |
| [15] | RW/SC | USBActEP | 0x0 | USB Active Endpoint Indicates whether the port is active under the current configuration and interface descriptor. The application must determine whether to configure this bit based on the SetConfiguration and SetInterface instructions. The soft core detects the USB This bit is automatically cleared after reset |
| [14:11] | - | Non- usage | 0x0 | Not used |

| [10:0] | RW | MPS | 0x0 | Maximum Packet Size The software must write the maximum packet length in bytes to this section. |
| --- | --- | --- | --- | --- |

      - DIEPTSIZ1/3

Offset address: 0x930/0x970 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | - | reserved | 0x0 | reserve |
| [30:29] | RW | MC | 0x0 | Multi Count For the Periodic IN port, this field specifies the number of ISO packets per frame . The soft core uses this value to calculate the ISO data PID . 'h1: 1 , 'h2: 2, 'h3: 3 For non-periodic IN ports (RO) , this field is used only in internal DMA mode. This field specifies the number of packets that the soft core must prepare for the current EP before switching to the next EP . |
| [28:19] | RW | PktCnt | 0x0 | Packet Count Specify the number of transaction packets. When TxFIFO sends a packet, the segment is automatically decremented. |
| [18:0] | RW | XferSize | 0x0 | Transfer Size The total byte length of this transaction on the port. Every time a byte is written to the TxFIFO , the segment is automatically reduced by 1 and an interrupt is triggered when it reaches zero. |

      - DOEPCTL2/4

Offset address: 0xb40/b80 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | WS/SC | EPEna | 0x0 | Endpoint Enable When set, indicates data is ready. The soft core automatically clears this bit before asserting any of the following interrupts: Endpoint disable Transfer completed |
| [30] | WS/SC | EPDis | 0x0 | Endpoint disable When set, it forces the data transfer to terminate ( even if the data has not yet ended ) . The soft core automatically clears this bit before setting the Endpoint Disable interrupt. be used after setting EPEna |
| [29] | WO | SetD1PID | 0x0 | Set DATA1 PID (Intr/Bulk valid ) Set port Data PID value is set to DATA1 |
| [28] | WO | SetD0PID | 0x0 | Set DATA0 PID (INT/Bulk valid ) Set port Data PID value is set to DATA0 |
| [27] | WO | SNAK | 0x0 | Set NAK When set, the control port responds with NAK |

| [26] | WO | CNAK | 0x0 | Clear NAK Clear the SNAK bit |
| --- | --- | --- | --- | --- |
| [25:22] | - | Non- usage | 0x0 | Not used |
| [twenty one] | RW | Stall | 0x0 | Stall handshake ( valid for Non-ISO/CTRL ) After setting, the port only responds to STALL . This bit has higher priority than NAK , Global Non-Periodic IN NAK and Global OUT NAK bit, the application should clear this bit. |
| [20] | RW | Snp | 0x0 | Snoop Mode In Snoop mode, the soft core does not check whether the OUT packet is correct, but directly reports it to the application |
| [19:18] | RO | EPType | 0x0 | Port transfer type 'h0: Control 'h1: ISO 'h2: Bulk 'h3: Int |
| [17] | RO | NAKSts | 0x0 | NAK status 0 : The soft core sends a non-NAK handshake packet based on the FIFO status 1 : The soft core sends a NAK handshake packet When set, data is transferred as follows: OUT port: The soft core is forced to stop receiving any data; |
| [16] | RO | DPID | 0x0 | Endpoint Data PID (Intr/Bulk valid ) PID information of the packet that the port is about to receive / send . After the port is enabled, the application must set the SetD0PID or SetD1PID information for a receive / send packet . 0 : DATA0 , 1 : DATA1 |
| [15] | RW/SC | USBActEP | 0x0 | USB Active Endpoint Indicates whether the port is active under the current configuration and interface descriptor. The application must determine whether to configure this bit based on the SetConfiguration and SetInterface instructions. The soft core detects the USB This bit is automatically cleared after reset |
| [14:11] | - | Non- usage | 0x0 | Not used |
| [10:0] | RW | MPS | 0x0 | Maximum Packet Size The software must write the maximum packet length in bytes to this section. |

      - DOEPTSIZ2/4

Offset address: 0xb50/0xb90 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | - | reserved | 0x0 | reserve |
| [30:29] | RO | RxDPID | 0x0 | Received Data PID (ISO valid ) PID number of the last packet 'h0: DATA0 |

|  |  |  |  | 'h1: DATA2 'h2: DATA1 'h3: MDATA |
| --- | --- | --- | --- | --- |
| [28:19] | RW | PktCnt | 0x0 | Packet Count Specify the number of transaction packets. When RxFIFO receives a packet, the segment is automatically reduced by 1. |
| [18:0] | RW | XferSize | 0x0 | Transfer Size The total byte length of this transaction on the port. Each time a byte is read from the RxFIFO , the segment is automatically reduced by 1 and an interrupt is triggered when it reaches zero. |

      - PCGCCTL

Offset address: 0xe00 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:4] | - | reserved | 0x0 | reserve |
| [3] | RW | RstPdwnModule | 0x0 | Reset Power-Down Modules is only valid in partial power-down mode . The application is set after the power is turned off and the power is turned on and the PHY This bit is cleared after the clock starts. |
| [2] | - | Non- usage | 0x0 | Not used |
| [1] | RW | GateHclk | 0x0 | Gate Hclk When USB is suspended, the application can set this bit to turn off hclk |
| [0] | RW | StopPclk | 0x0 | Stop Pclk When Dev is in suspend or disconnect state, the application software can set this bit to turn off phy_clk |

### GPIO

###### Overview

HR_C7000 Supports 3 groups of GPIO ( GPIO OA ~ GPIO C ) , each group can have up to 32 GPIOs . Each GPIO The ports can be individually set to input/output mode, and support interrupt input in input mode. Some GPIO and function pins are multiplexed.

###### Functional Description

GPIO The module supports the following functions:

- Supports every GPIO The pin can be set as input or output and can be used as an external interrupt source input;

- When set to output mode, each GPIO The pins can be set or cleared independently;

- In interrupt mode, it supports rising edge, falling edge, double edge, high level, low level and other 5 Trigger interrupt mode;

- Supports de-jitter function, and the de-jitter sampling time is configurable.

###### How it works

GPIO The pin is in input state after reset.

###### General Purpose Input and Output

Each pin can be configured as input or output. The specific steps are:

1. Refer to IO Multiplexing register description, configure the corresponding bits of the pins, and enable the GPIOs that need to be used Pin function;

2. Configure register GPIO_SWPORTX(AD)_DDR and select GPIO whether as input or output;

3. When configured as an input pin, read the register GPIO_EXT_PORTX(AD) to view the input signal value; when configured as an output pin, write the register GPIO_SWPORTX(AD)_DR Controlling GPIO Pin output level.

Note: When GPIO As an output, it should not be GPIO Otherwise, an interrupt will be generated when the output signal meets the interrupt condition .

###### Interrupt operation

Only port A With interrupt function, ports B , C , D They do not have interrupt functions and can only be used as normal input and output pins. GPIO Interrupts can be enabled via GPIO_INTTYPE_LEVEL and GPIO_INT_POLARITY When multiple interrupts occur simultaneously, they will be aggregated into one interrupt report .

The configuration process is as follows:

1. Configure the register GPIO_INTTYPE_LEVEL and select edge trigger or level trigger;

2. Configure the register GPIO_INT_POLARITY to select falling edge / rising edge trigger and high level / low level trigger;

3. Configure register GPIO_PORTA_EOI 0xff , clear interrupt;

4. Configure register GPIO_INTEN Enable interrupts.

NOTE: GPIO The pin signal remains stable to avoid false interrupts.

Edge-triggered interrupt: Software must clear the interrupt to enable the next interrupt.

Level-triggered interrupt: The external interrupt source should maintain the level until the processor recognizes the interrupt.

###### Register Overview

Register Table 20 GPIOA Register Overview (Base Address: 0x1402_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x00 | SWPORTA_DR | GPIO Data register |  |
| 0x04 | SWPORTA_DDR | GPIO Direction register |  |
| 0x30 | INTEN | Interrupt enable register |  |
| 0x34 | INTMASK | Interrupt mask register |  |
| 0x38 | INTTYPE_LEVEL | Interrupt Typer |  |
| 0x3c | INT_POLARITY | Interrupt Polarity Register |  |
| 0x40 | INTSTATUS | Interrupt Status Register |  |
| 0x44 | RAW_INTSTATUS | Raw Interrupt Status Register |  |
| 0x48 | DEBOUNCE | Debounce Enable |  |
| 0x4c | PORTA_EOI | Interrupt Clear Register |  |
| 0x50 | EXT_PORTA | Port Status |  |
| 0x78 | GPIO_INTR_BOTH_EDGE | Dual edge triggered interrupt |  |

Register Table 21 GPIOB Register Overview (Base Address: 0x1410_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x00 | SWPORTA_DR | GPIO Data register |  |
| 0x04 | SWPORTA_DDR | GPIO Direction register |  |

| 0x30 | INTEN | Interrupt enable register |  |
| --- | --- | --- | --- |
| 0x34 | INTMASK | Interrupt mask register |  |
| 0x38 | INTTYPE_LEVEL | Interrupt Typer |  |
| 0x3c | INT_POLARITY | Interrupt Polarity Register |  |
| 0x40 | INTSTATUS | Interrupt Status Register |  |
| 0x44 | RAW_INTSTATUS | Raw Interrupt Status Register |  |
| 0x48 | DEBOUNCE | Debounce Enable |  |
| 0x4c | PORTA_EOI | Interrupt Clear Register |  |
| 0x50 | EXT_PORTA | Port Status |  |
| 0x78 | GPIO_INTR_BOTH_EDGE | Dual edge triggered interrupt |  |

Register Table 22 GPIOC Register Overview (Base Address: 0x1411_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x00 | SWPORTA_DR | GPIO Data register |  |
| 0x04 | SWPORTA_DDR | GPIO Direction register |  |
| 0x30 | INTEN | Interrupt enable register |  |
| 0x34 | INTMASK | Interrupt mask register |  |
| 0x38 | INTTYPE_LEVEL | Interrupt Typer |  |
| 0x3c | INT_POLARITY | Interrupt Polarity Register |  |
| 0x40 | INTSTATUS | Interrupt Status Register |  |
| 0x44 | RAW_INTSTATUS | Raw Interrupt Status Register |  |
| 0x48 | DEBOUNCE | Debounce Enable |  |
| 0x4c | PORTA_EOI | Interrupt Clear Register |  |
| 0x50 | EXT_PORTA | Port Status |  |
| 0x78 | GPIO_INTR_BOTH_EDGE | Dual edge triggered interrupt |  |

###### Register Description

      - SWPORTA_DR

Offset address: 0x00 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | PortA Data Register | 0x0 | Port A output data |

      - SWPORTA_DDR

Offset address: 0x04 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | PortA Data Direction Register | 0x0 | Control port A data direction |

      - INTEN

Offset address: 0x30 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | Interrupt enable | 0x0 | Interrupt enable |

      - INTMASK

Offset address: 0x34 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | Interrupt mask | 0x0 | Controls whether to mask interrupts |

      - INTTYPE_LEVEL

Offset address: 0x38 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | Interrupt level | 0x0 | Controls the interrupt type of port A |

      - INT_POLARITY

Offset address: 0x3c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | Interrupt polarity | 0x0 | Controls the polarity of the interrupt on port A |

      - INTSTATUS

Offset address: 0x40 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | Interrupt status | 0x0 | Port A interrupt status |

      - RAW_INTSTATUS

Offset address: 0x44 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | Raw interrupt status | 0x0 | port A before masking |

      - DEBOUNCE

Offset address: 0x48 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | Debounce | 0x0 | Debounce Enable Register |

      - PORTA_EOI

Offset address: 0x4c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | PortA_EOL | 0x0 | PORTA interrupt clear register. 0 : Do not clear interrupt; 1 : Clearing Break |

      - EXT_PORTA

Offset address: 0x50 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | External PortA | 0x0 | When port A is configured as input, read this register to read the input data. |

      - INTR_BOTH_EDGE

Offset address: 0x78 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | Gpio_intr_both_ed ge | 0x0 | 1 : Dual-edge triggered interrupt 0 : Non-double-edge triggered interrupt. Note: To configure dual-edge interrupts, first configure gpio_inttype_level to edge trigger, and then configure register 0x78 . When dual-edge interrupts are not used, write 0 to register 0x78 . |

### PWM

###### Overview

PWM The module generates a periodic pulse waveform, where the period frequency and duty cycle can be configured through registers. HR_C7000 provides three independent PWM modules.

###### Functional Description

PWM The module supports the following functions:

- The cycle frequency can be set, and the maximum supported is 54M ;

- The duty cycle can be set.

PWM The output timing diagram is shown in the figure, where period and high and low level phase Configurable length:

phase0 	phase1

period

Figure 46 PWM Output timing diagram

PWM The pulse generation mode is divided into ONE_SHOT Mode and Continuous Mode. ONE_SHOT The number of pulse cycles generated by the mode can be controlled by register PWM_RPT CONTINUOUS​ The mode can update configuration information during the process .

Start 	Stop

Start

Stop

pwm_fp

pwm_per

start

inactout = 0

fpout = 1

pwm_rpt = 2

interrupt

Figure 47 ONE_SHOT Normal mode diagram

para_a para_b

para_c

pwm_fp_a

pwm_fp_b

pwm_fp_b

pwm_fp_c

pwm_per_a

pwm_per_b

pwm_per_b

pwm_per_c

pwm_fp pwm_per

start

inactout = 0

fpout = 1

interrupt

soft_rst

Figure 48 CONTINUOUS Mode diagram

###### How it works

      - Continuous model

Configure PWM_CFG Bit[ 5 :4] is 2′b10 , indicating continuous Then configure the first phase of a cycle

The number of continuous clock beats is configured by PWM_FP The number of clock beats in a cycle can be configured by PWM_PER

to achieve.

开始

配置0x0pwm_ctrl寄存器配置控制信息

(除cfg_update和start位)

配置0xc pwm_fp

配置0x8 pwm_per

配置0x0pwm_ctrl寄存器配置

cfg_update

配置0x0pwm_ctrl寄存器配置start

读0xff0中断状态寄存器

结束

example

0x328

0xa

0x5

0x32c

0x32d

Figure 49 CONTINUOUS Mode software operation diagram

      - One Shot model

Configure PWM_CFG Bit[5:4] is 2′b01 , indicating one shot Then configure the number of clock beats for the first phase of a cycle by configuring PWM_FP The number of clock beats in a cycle can be set by configuring PWM_PER . The number of repeated cycles can be set by configuring PWM_RPT .

ONE_SHOT The mode is configured as follows:

1. Configuration register PWM_CTRL Related information, except bit0[start] and bit2[cfg_update] ; 2. Configuration registers PWM_RPT , PWM_PER and PWM_FP ;

3. Configuration register PWM_CTRL Bit 2 of , write 1 ;

4. Configuration register PWM_CTRL Bit 0 of , write 1 ;

5. Check PWM_STATUS as needed and PWM_INTR_STA state.

###### Register Overview

Register Table 23 PWM Register Overview ( Base Address: 0x140C_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x0000 | PWM0_CFG | PWM0 Configuration registers |  |
| 0x0004 | PWM0_RPT | PWM0 ONE_SHOT Number of transmission repetition cycles |  |
| 0x0008 | PWM0_PER | PWM0 The periodic waveform lasts for the number of beats of the system clock |  |
| 0x000C | PWM0_FP | PWM0 The number of beats that the first phase of a cycle lasts |  |

| 0x0010 | PWM0_STATUS | PWM0 Status Register |  |
| --- | --- | --- | --- |
| 0x0020 | PWM1_CFG | PWM1 Configuration registers |  |
| 0x0024 | PWM1_RPT | PWM1 ONE_SHOT Number of transmission repetition cycles |  |
| 0x0028 | PWM1_PER | PWM1 The periodic waveform lasts for the number of beats of the system clock |  |
| 0x002C | PWM1_FP | PWM1 The number of beats that the first phase of a cycle lasts |  |
| 0x0030 | PWM1_STATUS | PWM1 Status Register |  |
| 0x0040 | PWM2_CFG | PWM2 Configuration registers |  |
| 0x0044 | PWM2_RPT | PWM2 ONE_SHOT Number of transmission repetition cycles |  |
| 0x0048 | PWM2_PER | PWM2 The periodic waveform lasts for the number of beats of the system clock |  |
| 0x004C | PWM2_FP | PWM2 The number of beats that the first phase of a cycle lasts |  |
| 0x0050 | PWM2_STATUS | PWM2 Status Register |  |
| 0xff0 | P WM_INTR_STA | All PWM Interrupt Status |  |

###### Register Description

      - PWM0_CFG

Offset address: 0x0 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:10] | - | reserved | - | reserve |
| [9] | RW | inactout | 0x0 | pwm The output level when not enabled. |
| [8] | RW | fpout | 0x0 | pwm Output level of the first phase in one cycle |
| [7:6] | RW | evtrig | 0x0 | In ONE_SHOT Event trigger mode in mode 1: rising edge trigger 2: Falling edge trigger 0,3: Normal mode, that is, not PWM according to external signals Signal output |
| [5:4] | RW | mode | 0x0 | PWM Mode, 1:ONE_SHOT model 2: CONTINUOUS model 0,3: Reserved, no PWM Waveform output |
| [3] | RW | inten | 0x0 | Interrupt enable |
| [2] | RW | cfg_update | 0x0 | IN CONTINUOUS mode, it will be updated in the middle PWM_PER and PWM_FP Two registers. When the register value is changed, cfg_update Start from 0 to 1 Configuration ( indicates two send Register values are updated simultaneously ) |
| [1] | RW | soft_rst | 0x0 | Software reset enable, used to turn off PWM at any time Waveform input out. |
| [0] | RW | start | 0x0 | pwm Start encoding, rising edge is valid ( ie from 0 Change to 1) . |

      - PWM0_RPT

Offset address: 0x4 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | - | reserved | - | reserve |
| [15:0] | RW | inactout | 0x0 | ONE_SHOT​ The number of cycles that the transmission is repeated. |

      - PWM0_PER

Offset address: 0x8 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | pwm0_rpt | 0x0 | One cycle of the waveform lasts for the number of beats of the system clock. |

      - PWM0_FP

Offset address: 0xC Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | pwm0_fp | 0x0 | The number of beats that the first phase of a cycle waveform lasts. If pwm_fp pwm_per is greater than pwm_per , all valid signals are fpout . If pwm_fp is 0 , all valid signals are !fpout. |

      - PWM0_STATUS​

Offset address: 0x10 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:10] | - | reserved | - | reserve |
| [9] | RW | pwm_out | 0x0 | pwm Current value of the waveform |
| [8] | RW | pwm_busy | 0x0 | State 0 of the pwm state machine : idle state 1 : Working status |
| [7:0] | RW | pwm_cycle | 0x0 | In CONTINUOUS mode, the same group the pwm_per & pwm_fp parameters have output |

      - PWM1_CFG

Offset address: 0x20 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:10] | - | reserved | - | reserve |
| [9] | RW | inactout | 0x0 | pwm The output level when not enabled. |
| [8] | RW | fpout | 0x0 | pwm Output level of the first phase in one cycle |
| [7:6] | RW | evtrig | 0x0 | In ONE_SHOT Event trigger mode in mode 1: rising edge trigger 2: Falling edge trigger |

|  |  |  |  | 0,3: Normal mode, that is, not PWM according to external signals Signal input out |
| --- | --- | --- | --- | --- |
| [5:4] | RW | mode | 0x0 | PWM Mode, 1:ONE_SHOT model 2: CONTINUOUS model 0,3: Reserved, no PWM Waveform output |
| [3] | RW | inten | 0x0 | Interrupt enable |
| [2] | RW | cfg_update | 0x0 | IN CONTINUOUS mode, it will be updated in the middle PWM_PER and PWM_FP Two registers. When the register value is changed, cfg_update Start from 0 to 1 Configuration ( indicates two send Register values are updated simultaneously ) |
| [1] | RW | soft_rst | 0x0 | Software reset enable, used to turn off PWM at any time Waveform input out. |
| [0] | RW | start | 0x0 | pwm Start encoding, rising edge is valid ( ie from 0 Change to 1) . |

      - PWM1_RPT

Offset address: 0x24 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | - | reserved | - | reserve |
| [15:0] | RW | inactout | 0x0 | ONE_SHOT​ The number of cycles that the transmission is repeated. |

      - PWM1_PER

Offset address: 0x28 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | pwm0_rpt | 0x0 | One cycle of the waveform lasts for the number of beats of the system clock. |

      - PWM1_FP

Offset address: 0x2C Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | pwm0_fp | 0x0 | The number of beats that the first phase of a cycle waveform lasts. If pwm_fp pwm_per is greater than pwm_per , all valid signals are fpout . If pwm_fp is 0 , all valid signals are !fpout. |

      - PWM1_STATUS​​

Offset address: 0x30

Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:10] | - | reserved | - | reserve |
| [9] | RW | pwm_out | 0x0 | pwm Current value of the waveform |
| [8] | RW | pwm_busy | 0x0 | State 0 of the pwm state machine : idle state 1 : Working status |
| [7:0] | RW | pwm_cycle | 0x0 | In CONTINUOUS mode, the same group the pwm_per & pwm_fp parameters have output |

      - PWM2_CFG

Offset address: 0x40 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:10] | - | reserved | - | reserve |
| [9] | RW | inactout | 0x0 | pwm The output level when not enabled. |
| [8] | RW | fpout | 0x0 | pwm Output level of the first phase in one cycle |
| [7:6] | RW | evtrig | 0x0 | In ONE_SHOT Event trigger mode in mode 1: rising edge trigger 2: Falling edge trigger 0,3: Normal mode, that is, not PWM according to external signals Signal output |
| [5:4] | RW | mode | 0x0 | PWM Mode, 1:ONE_SHOT model 2: CONTINUOUS model 0,3: Reserved, no PWM Waveform output |
| [3] | RW | inten | 0x0 | Interrupt enable |
| [2] | RW | cfg_update | 0x0 | IN CONTINUOUS mode, it will be updated in the middle PWM_PER and PWM_FP Two registers. When the register value is changed, cfg_update Start from 0 to 1 Configuration ( indicates two send Register values are updated simultaneously ) |
| [1] | RW | soft_rst | 0x0 | Software reset enable, used to turn off PWM at any time Waveform input out. |
| [0] | RW | start | 0x0 | pwm Start encoding, rising edge is valid ( ie from 0 Change to 1) . |

      - PWM2_RPT

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | - | reserved | - | 保留 |
| [15:0] | RW | inactout | 0x0 | 一次 ONE_SHOT 传输重复的周期数。 |

Offset address: 0x44 Reset mode: H/S

      - PWM2_PER

Offset address: 0x48 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | pwm2_rpt | 0x0 | One cycle of the waveform lasts for the number of beats of the system clock. |

      - PWM2_FP

Offset address: 0x4C Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | pwm2_fp | 0x0 | The number of beats that the first phase of a cycle waveform lasts. If pwm_fp pwm_per is greater than pwm_per , all valid signals are fpout . If pwm_fp is 0 , all valid signals are !fpout. |

      - PWM1_STATUS​​

Offset address: 0x30 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:10] | - | reserved | - | reserve |
| [9] | RW | pwm_out | 0x0 | pwm Current value of the waveform |
| [8] | RW | pwm_busy | 0x0 | State 0 of the pwm state machine : idle state 1 : Working status |
| [7:0] | RW | pwm_cycle | 0x0 | In CONTINUOUS mode, the same group the pwm_per & pwm_fp parameters have output |

      - PWM_INTR_STA

Offset address: 0xff0 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:3] | - | reserved | - | reserve |
| [2] | RO | pwm2_intr | 0x0 | PWM2 output interrupt status |
| [1] | RO | pwm1_intr | 0x0 | PWM1 output interrupt status |
| [0] | RO | pwm0_intr | 0x0 | pwm0 output interrupt status |

#### ADC Controller

###### Overview

HR_C7000 Built- in ADC . The controller is used to control the ADC according to the upper software configuration Sampling is done normally and the sampling data is uploaded.

###### Functional Description

- ADC The controller has the following functions:

- Data conversion rate 200KSPS

- Sampling accuracy 10 bit

- 8 ADCs​ aisle

- Support single channel scanning function

- Support continuous scanning function

- Supports low power mode during continuous scanning, ADC Automatically enter or exit low power state

- Use maskable interrupt to report to ADC Sampling data

- Supports configuration of sampling initial value and change threshold, automatically filters jitter of sampling signal

- Support ADC Abnormal status interrupt and design ADC Forced soft reset function

- Supports adjustable ADC Power-on / power-off protection time interval

- Support ADC The sampled data is directly output serially to the chip

###### How it works

Valid area , report interruption

ADC_INTR_DELTA

ADC_INTR_DELTA

none  effect  district

Invalid 		area

DATA

ADC_INTR_THRESHOLE

DATA

Valid area, report interruption

Figure 50 ADC_INTR_THRESHOLE with ADC_INTR_DELTA Relationship diagram

In the figure above, from ADC The sampled data DATA is compared with ADC_INTR_THRESHOLE Do the difference, if the absolute value of the difference is greater than the ADC_INTR_DELTA data, an interrupt will be reported.

##### Single scan processing flow

In single scan mode, configure the scan mode ( ADC_CTRL [ 2 ] = 1 ′ b 0 ) , whether to support low power consumption ( ADC_CTRL [ 0 ] ) , and the valid channel number ADC_CH_VLD Other related control information, ADC_CTRL Start ( ADC_START ) , start a scan, wait for the ADC conversion result, ADC_CTRL obtains the ADC conversion value, and reports an interrupt. A single scan does not filter out glitch .

The specific operation process is as follows:

- Configure the controller ADC_CTRL (0x0) = 0x8 , soft reset ADC modules and controllers;

- Configure bit 1 of the controller ADC_CTRL_STOP ( 0x20 ) register to soft reset the controller.

- Read ADC_CTRL_STATE (0x18) and ADC_INTR_STA(0x0c) , confirm the current status of the controller.

ADC_CTRL_STATE Register ADC_CTRL_BUSY bits and SAMP_FSM_STATE If the bit is not zero, you need to repeat step 3 ) and continue resetting the controller until the controller enters the IDLE state;

- Configure ADC_CTRL(0x0) = 0x0 to enable single scan mode;

- Configure ADC_INTR_THRESHOLE (0x1c) to set the sampling data trigger interrupt level;

- Configure ADC_DATA_DELTA (0x4) to set the error range between the sampling signal and the trigger level value;

- Configure ADC_PD_SEOC_TIME(0x28) to set PD Power-on and power-off protection time periods;

- Configure ADC_P2S_EN (0x2c) to turn ADC on or off Data serial output;

- Configure ADC_INTR(0x08)[16] = 1′b1 and ADC_INTR[8:0] to enable the corresponding interrupt;

- Configure ADC_CH_VLD (0x24) to enable the sampling channel. If multiple channels are configured, the controller will only output the data of the lowest channel number. For example, if ADC_CH_VLD = 0x6 , only channel B data will be output.

- Set ADC_START(0x14)=0x1 to start ADC Single-shot single-channel sampling;

- waiting for ADC_INTR_STA (0x0c) corresponding channel number interrupt to be set, start from 0x30 or 0x34 or 0x38 Read the corresponding sampling data from the register and clear the interrupt register (0x0c) .

Notice:

  - Steps 1)-2) and 5)-9) only need to be configured during the first operation and do not need to be repeated thereafter; each time a new scan is started, the other steps need to be repeated.

  - ADC_INTR_STA (0x0c) and configure ADC_STOP = 0x2 before starting sampling Reset controller,

  - Must not be in ADC_START Then to ADC_CTRL_BUSY Write to any register except soft reset ( that is, ADC_CTRL(0x00) bit4 and ADC_CTRL_STOP(0x20) ) before clearing it to 0 .

##### Single scan software operation process:

否

是

12）设置ADC_START，启动扫描

11）

置ADC_CH_VLD，使能采样通道

3）软复位控制器

4）确认控制器当前状态是否空闲

5）、6）、7）、8）、9）配置相关控制信息

10）配置ADC_INTR，使能中断

一次扫描结束

13）等待中断，读取数据，并清除ADC_INTR_SAT

2）软复位ADC

1）配置全局控制器寄存器

Next startup

Figure 51 ADC_CTRL Single scan software operation process

In the figure above, the black arrow indicates the first scan software operation after reset. The red arrow indicates the restart software operation.

##### Continuous scanning process

In continuous scan mode, configure the scan mode ( ADC_CTRL [ 2 ]= 1 ′ b 1 ) , whether to filter glitch ( ADC_CTRL [ 1 ] ) , whether to support low power mode ( ADC_CTRL [ 0] ) , continuous scan time interval ADC_SCAN_TIME , and valid channel number . ADC_CH_VLD , ADC_CTRL Start ( ADC_S T A R T ) , start scanning, in a time interval Tscan The scan of a valid channel is completed within 1 second, and the scan of the next valid channel is started when the next scan time comes.

Channel A ScanChannel 	A Scan

Channel A ScanChannel 	A Scan

Start the next channel scan according to the valid channel number configured by ADC_CH_SEL

Tscan

Tscan 	Tscan

Tscan

T0 	T1 	T2 	T3 	T4

4 times

Figure 52 ADC_CTRL Continuous Scan Channel Polling Diagram

The specific operation process is as follows:

- Configure the controller ADC_CTRL(0x0) = 0x8 to reset the ADC modules and controllers;

- Configure bit 1 of the controller ADC_CTRL_STOP ( 0x20 ) register to soft reset the controller

- Read ADC_CTRL_STATE (0x18) and ADC_INTR_STA(0x0c) , confirm the current state of the controller, if ADC_CTRL_STATE Register ADC_CTRL_BUSY bits and SAMP_FSM_STATE If the bit is not zero, you need to repeat step 3 ) and continue resetting the controller until the controller enters the IDLE state;

- Configure ADC_CTRL (0x0) to enable continuous scan mode, where ADC_CTRL = 0x4 Indicates enabling continuous scan mode; ADC_CTRL = 0x5 Indicates enabling low-power continuous scan mode; ADC_CTRL = 0x6 Indicates that the glitch filtering continuous scanning mode is enabled; ADC_CTRL = 0x7 indicates that the low-power glitch filtering continuous scanning mode is enabled;

- Configure ADC_INTR_THRESHOLE (0x1c) to set the sampling data trigger interrupt level;

- Configure ADC_DATA_DELTA (0x4) to set the error range between the sampling signal and the trigger level value;

- Configure ADC_PD_SEOC_TIME(0x28) to set PD Power-on and power-off protection time periods ; ( Step 8 ) must be configured before Step 9 )

- Configure ADC_SCAN_TIME (0x18) to set the scan time. If the scan time value is less than ADC_PD_SEOC_TIME (0x28) EOC_PD_TIME+12 , the actual written value is EOC_PD_TIME+12 , and ADC_CTRL_STATE

(0x18) Register SCAN_TIME_ERR Set ;

- Configure ADC_P2S_EN (0x2c) to turn ADC on or off Data serial output;

- Configure ADC_INTR(0x08)[16] = 1′b1 and ADC_INTR[8:0] to enable the corresponding interrupt;

- Configure ADC_CH_VLD (0x24) to enable the sampling channel. The controller samples each channel in order from low to high. For example, if ADC_CH_VLD = 0x6 is configured , the controller outputs the B and C channel data in sequence.

- Set ADC_START(0x14)=0x1 to start ADC Continuous sampling;

- waiting for ADC_INTR_STA (0x0c) corresponding channel number interrupt to be set, start from 0x30 or 0x34 or 0x38 Read the corresponding sampling data from the register and clear the interrupt register (0x0c) .

Notice:

  - If the relevant configuration parameters remain unchanged, steps 1)-2) and 5)-10) only need to be configured during the first operation and do not need to be repeated thereafter . Each time a new scan is started, the other steps need to be repeated.

  - ADC_INTR_STA (0x0c) and configure ADC_STOP = 0x3 before starting sampling Reset controller;

  - ADC at the end of continuous sampling CTRL_STOP=0x1 , terminate the continuous sampling mode;

  - Must not be in ADC_START Then to ADC_CTRL_BUSY Write to any register except soft reset ( that is, ADC_CTRL(0x00) bit4 and ADC_CTRL_STOP(0x20) ) before clearing it to 0 .

##### Continuous scanning software operation process

否

是

4）确认控制器当前状态是否空闲

13）设置ADC_START，启动扫描

12）

置ADC_CH_VLD，使能采样通道

11）配置ADC_INTR，使能中断

5）、6）、7）、8）、9）、10）配置相关控制信息

3）软复位控制器

一次扫描结束

14）等待中断，读取数据，并清除ADC_INTR_SAT

2）软复位ADC

1）配置全局控制器寄存器

Next startup

Figure 53 ADC_CTRL Continuous scanning software operation flow chart

In the figure above, ADC_CTRL In the continuous scanning software operation flow chart, when starting up next time, configure the relevant control steps.

If the control information such as whether to filter glitch or support low power consumption remains unchanged, you only need to reconfigure ADC_CH_VLD and ADC_START

If other configurations need to be changed, other registers can also be reconfigured.

The black arrow indicates the first scan software operation after reset. The red arrow indicates the restart software operation.

##### Burr filtering process

In continuous scanning mode, the glitch filtering process is as follows: each channel is filtered with 4 The data is a unit, and each sampling is 4 The data is averaged and output. After filtering out the glitch, the valid scan value is reported as an interrupt and the scan completion flag is set. Setting the ADC_CTRL (0x1) GLITCH bit enables this function.

##### Low power flow

In continuous scan mode, when low power mode is enabled, each ADC After the conversion is completed, it enters low power mode. When the next scan time comes, the ADC Wake up from low power mode and start ADC again Setting the ADC_CTRL(0x1) POWRR_DOWN bit enables this function.

Twork(12个clk)

Tsleep

1

2

3

4

5

6

7

8

9

10

clk

PD

TimeGen

Tscan

Figure 54 ADC_CTRL Low power mode timing diagram

ADC_CTRL The scanning time interval is Tscan , ADC The conversion time is Twork , Twork Later, ADC Enter low power mode, the low power consumption time is Tsleep , wait for the next Tscan time to arrive, and enter Twork again .

###### Register Overview

Register Table 24 ADC Register Overview (Base Address: 0x140D_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x0 | ADC_CTRL | ADC Control registers |  |
| 0x4 | ADC_INTR_DELTA | ADC Interruption error range |  |
| 0x8 | ADC_INTR | ADC Interrupt Control Register |  |
| 0xc | ADC_INTR_STA | ADC Interrupt Status Register |  |
| 0x10 | ADC_SCAN_TIME | ADC Scan interval setting |  |
| 0x14 | ADC_START | ADC start up |  |
| 0x18 | ADC_CTRL_STATE | ADC Control and Status Registers |  |
| 0x1c | ADC_INTR_THRESHOLE | ADC Interrupt initial value |  |
| 0x20 | ADC_CTRL_STOP | ADC Configuration registers |  |
| 0x24 | ADC_CH_VLD | ADC Channel Configuration Register |  |
| 0x28 | ADC_PD_SEOC_TIME | ADC Power-off time register |  |

| 0x2c | ADC_P2S_EN | ADC Serial data output register |  |
| --- | --- | --- | --- |
| 0x30 | ADC_DATA_AB | ADC Channel AB Scan Value |  |
| 0x34 | ADC_DATA_CD | ADC Channel CD Scan Value |  |
| 0x38 | ADC_DATA_EF | ADC Channel EF Scan Value |  |
| 0x3c | ADC_DATA_GH | ADC Channel GH Scan Value |  |
| 0x40 | ADC_CTRL_VERID | ADC Version Register |  |
| 0x44 | ADC_DIR_DAT_OP | ADC EOC Then directly output ADC data |  |

###### Register Description

      - ADC_CTRL

Offset address: 0x00 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:4] | - | reserved |  | reserve |
| [3] | RW | PD_FORCE_GIGH | 0x0 | Write 1 to soft reset the ADC and controller. For normal operation, write 0 to clear it . |
| [2] | RW | MODE_SEL | 0x0 | Scan mode selection. 0 : Single scan; 1 : Continuous scan |
| [1] | RW | GLITCH | 0x0 | Whether to filter burrs. 0 : Do not filter burrs; 1 : Filter burrs. ( Single scan does not filter burrs. Need to filter burrs ) |
| [0] | RW | POWER_DOWN | 0x0 | Whether to support low power mode. 0 : not supported; 1 : supported |

      - ADC_INTR_DELTA

Offset address: 0x04 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:10] | - | reserved |  | reserve |
| [9:0] | RW | ADC_DATA_DELT A | 0x0 | Set the interrupt error range. The controller can trigger data interrupt and output data only when the difference between the sampled data and the ADC_INTR_THRESHOLE (0x24) register configuration value is greater than ( not equal to ) . Figure 50 . |

      - ADC_INTR

Offset address: 0x08 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:17] | - | reserved | - | reserve |
| [16] | RW | INTR_EN | 0x0 | Global interrupt enable. 1 : enable ; 0 : disable |
| [15:9] | - | reserved | - | reserve |

| [8] | RW | INTR_ADC_ERR | 0x0 | ADC abnormal interrupt enable |
| --- | --- | --- | --- | --- |
| [7] | RW | INTR_EN_H | 0x0 | CHH scan valid value interrupt enable. 1 : enable ; 0 : disable |
| [6] | RW | INTR_EN_G | 0x0 | CHG scan valid value interrupt enable. 1 : enable ; 0 : disable |
| [5] | RW | INTR_EN_F | 0x0 | CHF scan valid value interrupt enable. 1 : enable ; 0 : disable |
| [4] | RW | INTR_EN_E | 0x0 | CHE scan valid value interrupt enable. 1 : enable ; 0 : disable |
| [3] | RW | INTR_EN_D | 0x0 | CHD scan effective value interrupt enable. 1 : enable ; 0 : disable |
| [2] | RW | INTR_EN_C | 0x0 | CHC scan valid value interrupt enable. 1 : enable ; 0 : disable |
| [1] | RW | INTR_EN_B | 0x0 | CHB scan valid value interrupt enable. 1 : enable ; 0 : disable |
| [0] | RW | INTR_EN_A | 0x0 | CHA scan valid value interrupt enable. 1 : enable ; 0 : disable |

      - ADC_INTR_STA

Offset address: 0x0c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:25] | - | reserved |  | reserve |
| [twenty four] | RO | ADC_ERR_RAW | 0x0 | Unmasked ADC exception interrupt, software reads 0 the EOC signal does not appear 11 cycles after the SOC signal . |
| [twenty three] | RO | CHANNEL_H_INTR_RAW | 0x0 | Unmasked channel H interrupt flag, software reads 0. 0 : No interrupt, 1 : Interrupt |
| [twenty two] | RO | CHANNEL_G_INTR_RAW | 0x0 | Unmasked channel G interrupt flag, software reads 0. 0 : No interrupt, 1 : Interrupt |
| [twenty one] | RO | CHANNEL_F_INTR_RAW | 0x0 | Unmasked channel F interrupt flag, software reads 0. 0 : No interrupt, 1 : Interrupt |
| [20] | RO | CHANNEL_E_INTR_RAW | 0x0 | Unmasked channel E interrupt flag, software reads 0. 0 : No interrupt, 1 : Interrupt |
| [19] | RO | CHANNEL_D_INTR_RAW | 0x0 | Unmasked channel D interrupt flag, software reads 0. 0 : No interrupt, 1 : Interrupt |
| [18] | RO | CHANNEL_C_INTR_RAW | 0x0 | Unmasked channel C interrupt flag, software reads 0. 0 : No interrupt, 1 : Interrupt |
| [17] | RO | CHANNEL_B_INTR_RAW | 0x0 | Unmasked channel B interrupt flag, software reads 0. 0 : No interrupt, 1 : Interrupt |
| [16] | RO | CHANNEL_A_INTR_RAW | 0x0 | Unmasked channel A interrupt flag, software reads 0. 0 : No interrupt, 1 : Interrupt |
| [15:9] | - | reserved |  | reserve |
| [8] | RO | ADC_ERR_INTR | 0x0 | ADC abnormal interrupt, software reads clear 0 is triggered when the EOC signal does not appear in the 11th cycle after the SOC signal . |
| [7] | RO | CHANNEL_H_INTR | 0x0 | Channel H interrupt flag, software reads 0 . 0 : No interrupt, 1 : Interrupt |
| [6] | RO | CHANNEL_G_INTR | 0x0 | Channel G interrupt flag, software reads 0 . 0 : No interrupt, 1 : Interrupt |
| [5] | RO | CHANNEL_F_INTR | 0x0 | Channel F interrupt flag, software reads 0 . |

|  |  |  |  | 0 : No interrupt, 1 : Interrupt |
| --- | --- | --- | --- | --- |
| [4] | RO | CHANNEL_E_INTR | 0x0 | Channel E interrupt flag, software reads 0 . 0 : No interrupt, 1 : Interrupt |
| [3] | RO | CHANNEL_D_INTR | 0x0 | Channel D interrupt flag, software reads 0 . 0 : No interrupt, 1 : Interrupt |
| [2] | RO | CHANNEL_C_INTRN | 0x0 | Channel C interrupt flag, software reads 0 . 0 : No interrupt, 1 : Interrupt |
| [1] | RO | CHANNEL_B_INTR | 0x0 | Channel B interrupt flag, software reads 0 . 0 : No interrupt, 1 : Interrupt |
| [0] | RO | CHANNEL_A_INTR | 0x0 | Channel A interrupt flag, software reads 0. 0 : No interrupt, 1 : Interrupt |

      - ADC_SCAN_TIME

Offset address: 0x10 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | SCAN_TIME | 0x2c | Scan time interval setting, in units of clk number. The minimum configuration value is ( a d c _ e o c _ d _ t i m e + 1 2 , decimal number ) . If it is less than the minimum value, it is calculated according to the minimum value , and A DC_C T R L _ STAT E [ 3 ] is set, and the 0 x 2 0 register SCAN_TIME_ERR is set . Note : Before configuring this register, the 0 x 3 4 register must be configured first . |

      - ADC_START

Offset address: 0x14 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | - | reserved |  | reserve |
| [0] | WO | ADC_START | 0x0 | Write 1 to start the adc_ctrl module. In single scan mode, the next beat of the EOC signal will clear this bit to 0 ; in continuous scan mode, the adc_ctrl_stop register A write enable signal will clear this bit to 0. |

      - ADC_CTRL_STATE

Offset address: 0x18 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:9] | - | reserved |  | reserve |
| [8] | R/W | EOC_WIDTH_ERR | 0x0 | Set when the EOC signal width is greater than one clk . Cleared to 0 for any value . |
| [7] | R/W | SOC_EOC_TIME_OUT | 0x0 | Set when the interval between SOC and EOC is not equal to 11 clks . Write any value to clear 0 . |

| [6] | R/W | SCAN_TIME_ERROR | 0x0 | The ADC_SCAN_TIME register configuration value is less than the minimum value. The meaning value is cleared to 0 . |
| --- | --- | --- | --- | --- |
| [5:1] | RO | SAMP_FSM_STATE | 0x0 | ADC_CTRL sampling data state machine state 9 : transition state, G_SAMPLE_S 8 : Sampling channel H data, G_SAMPLE_H . 7 : Sampling channel G data, G_SAMPLE_G . 6 : Sampling channel F data, G_SAMPLE_F . 5 : Sampling channel E data, G_SAMPLE_E . 4 : Sampling channel D data, G_SAMPLE_D . 3 : Sampling channel C status, G_SAMPLE_C . 2 : Sampling channel B status, G_SAMPLE_B . 1 : Sampling channel A status, G_SAMPLE_A . 0 : Idle state, G_IDLE |
| [0] | RO | ADC_CTRL_BUSY | 0x0 | ADC_CTRL module status indication. 0 : idle 1 : busy |

      - ADC_INTR_THRESHOLE

Offset address: 0x1c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | - | reserved |  | reserve |
| [9 : 0] | RW | ADC_INTR_THRES HOLD | 0x0 | ADC Interrupt initial value, when the difference between the conversion result and this initial value is greater than adc_intr_delta When the error threshold is exceeded, the report interrupt will be triggered . Figure 50 |

      - ADC_CTRL_STOP

Offset address: 0x20 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:2] | - | reserved |  | reserve |
| [1] | WO | ADC_CTRL_RESET | 0x0 | Write 1 to soft reset the controller, self-clearing |
| [0] | WO | ADC_CTRL_STOP | 0x0 | Only works in continuous scan mode. In continuous scan mode, it stops automatically scanning after scanning the last valid channel and clears itself to 0. (In continuous scan mode, writing 1 to this register can stop the automatic scanning function of adc_ctrl . It is necessary to rewrite the ADC_START and ADC_CH_VLD registers . to restart automatic scanning ) |

      - ADC_CH_VLD

Offset address: 0x24 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:8] | - | reserved |  | reserve |
| [7] | RW | ADC_CHH_VLD | 0x0 | In continuous scan mode, whether channel H is valid. 0 : invalid; 1 : valid. |
| [6] | RW | ADC_CHG_VLD | 0x0 | In continuous scan mode, is channel G valid? 0 : invalid; 1 : valid. |
| [5] | RW | ADC_CHF_VLD | 0x0 | In continuous scan mode, is channel F valid? 0 : invalid; 1 : valid. |
| [4] | RW | ADC_CHE_VLD | 0x0 | In continuous scan mode, is channel E valid? 0 : invalid; 1 : valid. |
| [3] | RW | ADC_CHD_VLD | 0x0 | In continuous scan mode, whether channel D is valid. 0 : invalid; 1 : valid. |
| [2] | RW | ADC_CHC_VLD | 0x0 | In continuous scan mode, is channel C valid? 0 : invalid; 1 : valid. |
| [1] | RW | ADC_CHB_VLD | 0x0 | In continuous scan mode, whether channel B is valid. 0 : invalid; 1 : valid. |
| [0] | RW | ADC_CHA_VLD | 0x0 | In continuous scan mode, is channel A valid? 0 : invalid; 1 : valid. |

Note: Single scan mode, ADC_CH_VLD Only the lowest-order channel is valid; when scanning multiple times, scan from the lowest-order channel to the highest-order channel in sequence.

      - ADC_PD_SEOC_TIME

Offset address: 0x28 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | - | reserved |  | reserve |
| [15:8] | RW | EOC_PD_TIME | 0xa | EOC and the rising edge of PD . the configured value is smaller than the default value, the default value is used. |
| [7:0] | RW | PD_SOC_TIME | 0x20 | SOC and the falling edge of PD . the configured value is smaller than the default value, the default value is used. |

      - ADC_P2S_EN

Offset address: 0x2c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | - | reserved |  | reserve |
| [0] | RW | ADC_P2S_EN | 0x0 | Enables serial output of sampled data, for debugging only. 1 : enable ; 0 : disable |

      - ADC_DATA_AB

Offset address: 0x30 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [ 31:26 ] | - | reserved |  | reserve |
| [25:16] | RO | CHANNEL_B_DATA | 0x0 | Channel B scan value |
| [15:10] | - | reserved |  | reserve |

| [9 : 0] | RO | CHANNEL_A_DATA | 0x0 | Channel A scan value |
| --- | --- | --- | --- | --- |

      - ADC_DATA_CD

Offset address: 0x34 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:26] | - | reserved |  | reserve |
| [25:16] | RO | CHANNEL_D_DATA | 0x0 | Channel D scan value |
| [15:10] | - | reserved |  | reserve |
| [9:0] | RO | CHANNEL_C_DATA | 0x0 | Channel C scan value |

      - ADC_DATA_EF

Offset address: 0x38 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [ 31:26 ] | - | reserved |  | reserve |
| [25:16] | RO | CHANNEL_F_DATA | 0x0 | Channel F scan value |
| [15:10] | - | reserved |  | reserve |
| [9 : 0] | RO | CHANNEL_E_DATA | 0x0 | Channel E scan value |

      - ADC_DATA_GH

Offset address: 0x3c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [ 31:26 ] | - | reserved |  | reserve |
| [25:16] | RO | CHANNEL_H_DATA | 0x0 | Channel H scan value |
| [15:10] | - | reserved |  | reserve |
| [9 : 0] | RO | CHANNEL_G_DATA | 0x0 | Channel G scan value |

      - ADC_CTRL_VERID

Offset address: 0x40 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | ADC_CTRL_VERID | 0x20151216 | Controller version number (for debugging only ) |

      - ADC_DIR_DAT_OP

Offset address: 0x44 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:10] | - | reserved |  | reserve |

| [9:0] | RO | ADC_DIR_DAT_OP | 0x0 | ADC data directly after EOC (for debug only ) |
| --- | --- | --- | --- | --- |

#### DAC Controller

###### Overview

HR_C7000 Built-in 3 Channel DAC , used to output low-speed analog control signals.

###### Functional Description

DAC controller is used with the internal DAC Interface, implement CPU DAC​ control.

###### How it works

HR_C7000 The DAC controller directly interacts with the internal DAC through the CPU configuration register, Send data , enable and low power control signals.

###### Register Overview

Register Table 25 DAC Register Overview ( Base Address: 0x140D_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x0 | DAC_PD_CTRL | DAC Enable |  |
| 0x4 | DAC_PD_MODE_EN | DAC Low power enable |  |
| 0x8 | DAC_DATA_A | DAC Channel A data |  |
| 0xc | DAC_DATA_B | DAC Channel B data |  |
| 0x10 | DAC_DATA_C | DAC Channel C data |  |
| 0x14 | DAC_CTRL_VERID | DAC Controller version |  |

###### Register Description

      - DAC_PD_CTRL

Offset address: 0x00 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:3] | - | reserved |  | reserve |
| [2] | RW | DAC_PD_C | 0x1 | DAC channel C power on and off control. 1 : Power off; 0 : Power on |
| [1] | RW | DAC_PD_B | 0x1 | DAC channel B power on and off control. 1 : Power off; 0 : Power on |
| [0] | RW | DAC_PD_A | 0x1 | DAC channel A power on and off control. 1 : Power off; 0 : Power on |

      - DAC_PD_MODE_EN

Offset address: 0x04 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:3] | - | reserved |  | reserve |
| [2] | RW | DAC_PD_MODE_C EN | 0x1 | DAC channel C low power mode enable. 1 : Enable; 0 : Shutdown |

| [1] | RW | DAC_PD_MODE_B EN | 0x1 | DAC channel B low power mode enable. 1 : Enable; 0 : Shutdown |
| --- | --- | --- | --- | --- |
| [0] | RW | DAC_PD_MODE_A EN | 0x1 | DAC channel A low power mode enable. 1 : Enable; 0 : Disable Break |

      - DAC_DATA_A

Offset address: 0x08 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:12 ] | - | reserved |  | reserve |
| [11:0] | RW | DAC_DATA_A | 0x0 | DAC Channel A data |

      - DAC_DATA_B

Offset address: 0x0c Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:12 ] | - | reserved |  | reserve |
| [11:0] | RW | DAC_DATA_B | 0x0 | DAC channel B data |

      - DAC_DATA_C

Offset address: 0x10 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:12 ] | - | reserved |  | reserve |
| [11:0] | RW | DAC_DATA_C | 0x0 | DAC Channel C data |

      - DAC_CTRL_VERID

Offset address: 0x14 Reset mode: H/S

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RW | DAC_CTRL_VERID | 0x20160 229 | Controller version number (for debugging only ) |

## Codec application

#### Overview

HR_C7000 The chip has built-in high-performance audio Codec IP , supports differential and single-ended Mic Input, ADC Support AGC Automatic gain control. Output supports two-way lineout single-ended output and PWM output mode.

#### Functional Description

Figure 55 Codec Module functional block diagram

Codec It has the following features:

- Main clock frequency 24MHz ;

- Support Pop noise elimination;

- 24bit-16bit Linear PCM Mode output;

- Support 8K , 16K , 24K , 48K sampling frequency;

- Fast ADC Typical wake-up time: 100ms ;

- Support automatic gain control ( AGC ) ;

- Support programmable wind noise filtering ( WNF ) ;

- Two-way differential / single-ended Mic input support;

- Two single-ended Lineout outputs are supported, and Lineout is PWM modulated output.

- ADC working master mode

- The DAC can be operated in master mode or slave mode.

#### Reference Circuit

###### MIC enter

Codec ADC​ Input supports differential Mic Input mode and single-ended Mic The following figure shows the schematic diagram of two different connection methods.

Figure 56 Codec Mic Differential and single-ended schematics

The external pickup circuit needs to pass through a set of filter capacitors Cbyline With Codec Internal isolation, the recommended capacitor size is 1uF . For the above differential or single-ended Microphone In order to obtain better performance and reduce high-frequency interference, it is necessary to effectively utilize the Micbias application circuit.

Micbias The application circuit is shown in the figure below.

Figure 57 Codec Micbias Differential and single-ended schematics

Micbias The output voltage can be configured with different typical output values according to different configuration parameters. and

Rmic .

Table 38 Codec Micbias Parameter Description

###### LINEOUT Output

HR_C7000 built-in Codec DAC of the module The output is through PWM Digital output signal modulation to achieve DAC

function.

Two Lineouts of the chip Output ( Lineout1 and Lineout2 For the same PWM The modulation outputs the same waveform. Through respective enable control, two lineouts can be output simultaneously, or any one of the lineouts can be controlled to output single-ended.

Figure 58 Codec PWM Output schematic

The above PWM The reference components for the output external rectification circuit are recommended as follows.

Table 39 Codec PWM Output parameter description

#### How it works

###### Working mode description

Table 40 Codec Working mode definition

| Mode | SB | SB_SLEEP | SB_DAC | SB_DAC2 | SB_AICR_ADC | SB_AICR_DAC | SB_MIC1 | SB_MICBIAS1 | SB_ADC | ADC_LEFT_ONLY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Register signal values after a reset mode | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-down mode | 1 | X | 1 | 1 | 1 | 1 | X | X | 1 | X |
| Playback mode with analog | 0 | 1 | 0 | 1 | 1 | 0 | X | X | 1 | X |

| power-down, DAC to BO* ouputs |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sleep mode | 0 | 1 | 1 | 1 | 1 | 1 | X | X | 1 | X |
| Analog Record mode with mono inputs, MIC1 input | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 |

      - Power-off mode

Turn off CODEC Operating voltage.

      - Power-down mode

After the chip is powered on, CODEC Reset and enter power-down mode. In this mode, CODEC The relevant analog and digital power supplies are all in the on state, but the global standby ( SB ) signal is valid and the codec The power consumption is at its lowest state.

      - Playback mode​

PWM on the voice playback path The modulator is turned on. The analog power can be turned off by " SB=1 ".

In the current mode, codec is disabled. Reset operation.

      - Sleep mode

PWM on the voice playback path The modulator is turned on. The analog power can be turned off by " SB=1 ".

In the current mode, codec is disabled. Reset operation.

      - Analog record mode

CODEC ADC​ In working mode.

###### Working mode switching

      - Power-down Mode switches to voice playback mode

- Configuration registers SB_AICR_DAC 	(0x1600_09C8[4] =0) , Open DAC Audio interface enabled ;

- Configuration registers SB_DAC 	(0x1600_09CD[4] =0) , Open DAC

- Configuration registers DAC_SOFT_MUTE 	(0x1600_09CD[7] =0) , closure DAC Soft Mute

Among them, Tmute The maximum delay does not exceed 24ms .

      - Power-down Mode switches to sleep mode

Set SB=0 , CODEC Will be powered down The state switches to sleep mode.

      - Sleep Mode to CODEC ADC Opening

- Configuration registers SB 	(0x1600_09D2[0] =0) , CODEC Depend on power-down Mode switches to sleep mode ;

- Configuration registers SB_SLEEP 	(0x1600_09D2[1] =0) , Switch from sleep mode to working mode ;

- Configure register SB_MIC1 ( 0x1600_09CB[4]=0) to enable MIC1 port

- Configure register SB_AICR_ADC ( 0x1600_09C9[4]=0) to enable ADC Audio Interface

- Configuration register SB_ADC ( 0x1600_09CF [ 4 ] = 0 ) , turn on ADC​​

- Configuration register ADC_SOFT_MUTE ( 0x1600_09CF [ 7 ] = 0 ) , turn off ADC​​ Soft mute function.

Among them Tsbyu With Tshd-mic Both are 50~100ms

      - Sleep mode to audio playback mode

- Configuration registers SB 	(0x1600_09D2[0] =0) , CODEC Depend on power-down Mode switches to sleep mode ;

- Configuration registers SB_AICR_DAC 	(0x1600_09C8[4] =0) , Open DAC Audio interface enabled ;

- Configuration registers SB_DAC 	(0x1600_09CD[4] =0) , Open DAC

- Configuration registers DAC_SOFT_MUTE 	(0x1600_09CD[7] =0) , closure DAC Soft Mute

- Query register 0x1100_0088[31] , CPU Get STANDBY_LO status to turn the speaker on / off.

Among them Tstbon In 12ms Within.

      - Switch to power-down mode model

- Configuration registers SB 	(0x1600_09D2[0] =1) , CODEC Enter power-down model .

- Configuration registers SB_AICR_DAC 	(0x1600_09C8[4] =1) , closure DAC Audio interface enabled .

- Configuration registers SB_DAC 	(0x1600_09CD[4] =1) , closure DAC .

- Configure register SB_AICR_ADC ( 0x1600_09C9[4]=1) to turn off ADC Audio interface.

- Configuration register SB_ADC ( 0x1600_09CF [ 4 ] = 1 ) , turn off ADC .​​

###### Codec Workflow

##### Audio interface mode settings

HR_C7000 Integrated CODEC The audio interface is a specific configuration, and the CODEC After reset, the default values of its internal registers are different from the expected values of the application, so they need to be reconfigured. Master and slave modes, signal bit width, etc.

|   | Configuration register 0x1600_09C8[7:6] = ' 1 1 ', select DAC The input signal width is 24 bits ; Configuration registers 0x1600_09C8[5] = ' 0 ' , select DAC Works in master mode ; |  |
| --- | --- | --- |
|  | Configuration register 0x1600_09C8[1:0] = | ' 00 ', select DAC The audio interface is in parallel mode; |
|  | Configuration register 0x1600_09C9[7:6] = | ' 1 1 ', select ADC The output signal width is 24 bits ; |
|  | Configuration register 0x1600_09C9[1:0] = | ' 00 ', select ADC The audio interface is in parallel mode; |
| 6.4.3.2 | Audio Send ( Analog record ） |  |

MIC

HR_C7000

DAC

16bit

PCM

TxI

TxQ

Moderm

Data Buffer

AHB Master wrap	8/32

IRQ

RAM

32KB

System

ahb

40组

32bit音频数据

AHB

8bit

RAMdata

32bit

Regdata

RAM 5.8KB

Codec

CPU_SYS

声码器

CK803S

Figure 59 Voice sending data flow diagram

As shown in the figure above, it is MIC The data flow diagram of the voice signal from the end to the sending end. First, MIC The audio from the end is transmitted through CODEC ADC Convert to 16bit Parallel data and corresponding 8KHz Enable signal. Then, there is a distinction between digital and analog:

In word mode, audio data flows through AHB Output to the vocoder, and then return to the MODEM after audio encoding is completed DMR in the data cache Encode and send. In simulation mode, codec ADC The output audio data is directly on the MODEM The corresponding voice audio modulation is completed internally and finally sent to the transmitter. For the specific process, please refer to the detailed description in the data interface manual.

In summary, whether in digital or analog mode, for MIC The audio sending process of the end, CODEC The configuration is the same, open codec ADC .

- Configuration registers 0x1100_0080[0] = 0Select 	audio source as CODEC ADC ;

- Configuration registers SB_SLEEP 	(0x1600_09D2[1] =0) , Switch from sleep mode to working mode ;

- Configure register SB_MIC1 ( 0x1600_09CB[4]=0) to enable MIC1 port;

- Configure register SB_AICR_ADC ( 0x1600_09C9[4]=0) to enable ADC Audio interface;

- Configuration register SB_ADC ( 0x1600_09CF [ 4 ] = 0 ) , turn on ADC ;​​

- Configuration register ADC_SOFT_MUTE ( 0x1600_09CF [ 7 ] = 0 ) , turn off ADC​​ Soft mute function.

      - Audio Receive ( Play back ）

SPK

HR_C7000

CPU_SYS

ADC

RxI	RxQ

声码器	16bit

PCM

CK803S

Moderm

AHB Master wrap

40组 32bit

RAM 5.8KB

Data Buffer 8/32

IRQ

RAM

32KB

System

ahb

音频

数据

AHB

8bit

RAMdata

32bit

Regdata

Codec

Figure 60 Voice reception data flow diagram

As shown in the figure above, the demodulated audio signal is received by CODEC. Line_out/HPout For analog mode, the RF receiving signal is in MODEM Internally completed FM Demodulation, the output audio data is directly given to CODEC DAC For digital mode, DMR The demodulated data is sent to the vocoder for decoding, and the decoded audio data is returned to the MODEM end and finally to the CODEC DAC end.

In summary, whether it is digital or analog audio playback process, CODEC The configuration is the same.

codec DAC .

        - Configuration registers SB_SLEEP 	(0x1600_09D2[1] =0) , Switch from sleep mode to working mode ;

        - Configuration registers SB_AICR_DAC 	(0x1600_09C8[4] =0) , Open DAC Audio interface enabled ;

        - Configuration registers SB_DAC 	(0x1600_09CD[4] =0) , Open DAC ;

        - Configuration registers DAC_SOFT_MUTE 	(0x1600_09CD[7] =0) , closure DAC Soft mute .

###### Soft mute function mute​

CODEC Built-in ADC/DAC Soft mute function , respectively through register 0x1600_09CF [ 7 ( ]​ 0x1600_09CD[7] ( dac_soft_mute ) is set to ' 1 ' to be valid.

adc_ s o f t _ m u t e ) ,

CODEC The output signal will be in Mute sequence ( unmute The signal amplitude is gradually increased (decreased) over a time period called a sequence . Typically, this time period lasts less than 24ms .

###### AGC

Figure 61 CODEC Automatic Gain Control ( AGC ) System Block Diagram

The picture above is CODEC ADC Automatic Gain Control ( AGC ) system block diagram. ADC The generated signal first passes through the signal envelope detection module ( signal envelope de t ec t or ) , calculate the signal mean, and then AGC Complete 4 in the control module The controller determines the threshold and generates a control instruction which is returned to the ADC unit.

The definitions of the four groups of thresholds are as follows:

Critical threshold t h r e s ho l d ) : Determine ADC Is the current output value saturated ? When the output value is greater than the threshold , AGC Will quickly reduce ADC Output gain. CRIT_THR = FS – 3dB , that is, the boundary threshold is set by CODEC The full scale of the ADC determines the value and does not require configuration by the user.

High Target High Target​​​​​ Low ) : These two sets of thresholds are used to define the expected signal amplitude .

The intervals are Target_high = ( Target + 2 dB ) and Target_low = ( Target – 4 dB ) . When the signal is within this interval , AGC No adjustment is made. AGC The duration of a signal amplitude increase or decrease is set by registers DCY and ATK respectively.

Noise Threshold Gate ) : This threshold is used to determine the current ADC The output of the ADC is either noise or a valid audio signal. Output value is lower than NG_THR When it is judged as a noise signal, the AGC No adjustment is made. NG_THR Configured by registers. It is defined as NG_THR = MIN_NG_THR + a ( in dB ) . Taking a 1KHz single tone signal as an example, MIN_NG_THR represents -68dB .

Application example ( TARGET = full scale - 6dB ):

- Configuration register (0x1600_09F0 =0x18) ;

- Configure register (0x1600_09EF = 0x40) ; latch the data cached in reg_0x1600_09F0 into the register with indirect address 00 (ADC_AGC_0) ;

- Configuration Register (0x1600_09EF =0x80) , turn on ADC AGC​ Function; Note: AGC must be enabled separately after parameter configuration is completed!

###### Soft clipping DRC

When the DAC When the input data exceeds the set threshold, audio compression is performed on the excess part to reduce the audio amplitude.

###### Wind Noise Filter

CODEC There is a built-in wind noise filter, which can be turned on or off by register FCR_ADC ( 0x1600_0017 ), and supports 3 groups of noise reduction mode settings.

Table 41 Codec Wind noise filter parameters

| Filter characteristics |  | 24MHz |  |
| --- | --- | --- | --- |
| Wind Noise Filter corner frequency | Mode 1 | - 3dB | 59 Hz |
|  | Mode 2 | - 3dB | 117 Hz |
|  | Mode 3 | - 3dB | 235 Hz |

#### register

###### Register Overview

Register Table 26 Codec Register Overview (Base Address: 0x1600_09C0 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x0 | SR1 | Status Register |  |
| 0x1 | SR2 | Status Register |  |
| 0x7 | MR | ADC Mute control |  |
| 0x8 | AICR_DAC | DAC Audio interface control |  |
| 0x9 | AICR_ADC | ADC Audio interface control |  |
| 0xB | CR_MIC1 | MIC1 Control registers |  |
| 0xC | CR_MIC2 | MIC2 Control registers |  |
| 0xD | CR_DAC | DAC Control registers |  |
| 0xF | CR_ADC | ADC Control registers |  |
| 0x10 | CR_MIX | Digital Aliasing Control Register |  |
| 0x11 | DR_MIX | Digital Aliasing Control Indirect Register |  |
| 0x12 | CR_VIC | Codec sleep​ Mode Control Register |  |
| 0x13 | CR_CK | MCLK Control registers |  |
| 0x14 | FCR_DAC | DAC Sampling Clock Configuration Register |  |
| 0x17 | FCR_ADC | ADC Sampling Clock Configuration Register |  |
| 0x18 | CR_TIMER_h | Counter high 8 bits |  |
| 0x19 | CR_TIMER_l | Counter lower 8 bits |  |
| 0x1A | ICR | Interrupt Control Register |  |
| 0x1B | IMR | Interrupt Mask Control registers |  |
| 0x1C | IFR | Interrupt flag definition |  |
| 0x1D | IMR2 | Timer flag MASK Control registers |  |
| 0x1E | IFR2 | Timer flag definition |  |
| 0x1F | GCR_DACL | DAC Gain Control |  |
| 0x23 | GCR_MIC1 | Mic1 Gain Control |  |
| 0x25 | GCR_ADCL | ADC Gain Control |  |
| 0x27 | GCR_MIXDACL | Aliasing DAC Gain Control |  |
| 0x29 | GCR_MIXADCL | Aliasing ADC Gain Control |  |
| 0x2B | CR_DAC_AGC | DAC Gain Control Register |  |
| 0x2C | DR_DAC_AGC | DAC AGC Read and write data of indirect address |  |
| 0x2F | CR_ADC_AGC | ADC Gain Control Register |  |
| 0x30 | DR_ADC_AGC | ADC AGC Read and write data of indirect address |  |

###### Register Description

      - SR1

Offset address: 0x00 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |

| [7] | RO | pon_ack | 0x0 | codec power on status response 0 : codec is not in working mode; 1 : codec is in working mode |
| --- | --- | --- | --- | --- |
| [6] | RO | irq_ack | 0x0 | Interrupt pending status register 0 : No pending interrupts are included; 1 : There is currently an unresponsive interrupt. |
| [5] | RO | reserved | 0x0 |  |
| [4] | RO | dac_locked | 0x0 | DAC ADAS lock status, 1 is valid ( this function is in the current application Not involved ) |
| [3:0] | RO | reserved | 0x0 |  |

      - SR2

Offset address: 0x01 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:5] | RO | reserved | 0x0 |  |
| [4] | RO | dac_unknown_fs | 0x0 | DAC_ADAS response flag after locking: 0 : The FS of the input signal is within the frequency range supported by the codec ; 1 : The FS of the input signal exceeds the range supported by the codec ( the The performance index cannot be guaranteed). This function is not involved in the current application . |
| [3:0] | RO | reserved | 0x0 |  |

      - MR

Offset address: 0x07 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:5] | RO | reserved | 0x0 |  |
| [4:3] | RO | adc_mute | 0x0 | 00 : ADC is not muted; 01 : ADC is being muted; 10 : ADC is about to end the mute state; 11 : ADC is in mute state |
| [2] | RO | reserved | 0x0 |  |
| [1:0] | RO | dac_mute | 0x0 | 00 : DAC is not muted; 01 : DAC is being muted; 10 : DAC is about to end the mute state; 11 : DAC is in mute mode |

      - AICR_DAC

Offset address: 0x08 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:6] | RW | dac_adwl | 0x3 | In serial transmission mode, the DAC audio data bit width is set as follows: 00 : 16bit ; 01 : 18bit ; 10 : 20bit ; 11 : 24bit . After reset, the initial setting is 24 bits , and the internal 16-bit signal of the modem has been Expansion processing. |
| [5] | RW | dac_slave | 0x0 | DAC audio interface master-slave mode configuration, 0 : Master ; 1 : Slave |
| [4] | RW | sb_aicr_dac | 0x1 | DAC audio interface enable, 0 : active ; 1 : power- down |
| [3:2] | RW | reserved | 0x0 |  |
| [1:0] | RW | dac_audioif | 0x3 | DAC audio interface mode selection, 00 : Parallel ; 01 : Left-justified ; 10 : DSP ; 11 : I2S After reset, the initial configuration is I2S mode, which needs to be fixed Parallel mode |

      - AICR_ADC

Offset address: 0x09 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:6] | RW | adc_adwl | 0x3 | In serial transmission mode, the ADC audio data bit width is set. 00 : 16bit ; 01 : 18bit ; 10 : 20bit ; 11 : 24bit After reset, the initial setting is 24 bits , and the 16-bit signal inside the modem has been expanded. |
| [5] | RW | adc_slave | 0x0 | ADC audio interface master-slave mode configuration, 0 : Master ; 1 : Slave |
| [4] | RW | sb_aicr_adc | 0x1 | ADC audio interface enable, 0 : active ; 1 : power- down |
| [3:2] | RW | reserved | 0x0 |  |
| [1:0] | RW | adc_audioif | 0x3 | ADC audio interface mode selection, 00 : Parallel ; 01 : Left-justified ; 10 : DSP ; 11 : I2S After reset, the initial configuration is I2S mode, which needs to be fixed Parallel mode |

      - CR_MIC1

Offset address: 0x0B Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |

| [7] | RW | reserved | 0x0 |  |
| --- | --- | --- | --- | --- |
| [6] | RW | micdiff1 | 0x0 | MIC1 input mode selection, 0 : single-ended input; 1 : differential input |
| [5] | RW | sb_micbias1 | 0x1 | MIC1 BIAS enable, 0 : active ; 1 : power- down |
| [4] | RW | sb_mic1 | 0x1 | MIC1 Enable, 0 : active ; 1 : power- down |
| [3] | RW | micbias1_v | 0x0 | MIC bias Voltage selection, 0 : 2.08V ; 1 : 1.66V |
| [2:0] | RW | reserved | 0x0 |  |

      - CR_MIC2

Offset address: 0x0C Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7] | RW | reserved | 0x0 |  |
| [6] | RW | micdiff2 | 0x0 | MIC1 input mode selection, 0 : AIPL1/AINL1 ; 1 : AIPL2/AINL2 |
| [5:0] | RW | reserved | 0x30 |  |

      - CR_DAC

Offset address: 0x0D Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7] | RW | dac_soft_mute | 0x1 | DAC soft mute mode, 0 : soft mute off; 1 : soft mute on |
| [6:5] | RW | reserved | 0x1 |  |
| [4] | RW | sb_dac | 0x1 | Mercury DAC operation enable, 0 : active ; 1 : power-down |
| [3:1] | RW | reserved | 0x0 |  |
| [0] | RW | dac_zero_n | 0x0 | DO_LO_PWM*N pin output disabled, 0 : active ; 1 : clamp Made in VSSD |

      - CR_ADC

Offset address: 0x0F Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7] | RW | adc_soft_mute | 0x1 | ADC soft mute mode, 0 : soft mute off; 1 : soft mute on |
| [6] | RW | adc_dmic_sel | 0x0 | Digital filter input selection, 0 : analog mic ; 1 : digital mic (none) |
| [5] | RW | adc_left_only | 0x1 | ADC right channel shield, 0 : enable right channel; 1 : disable right channel |
| [4] | RW | sb_adc | 0x1 | ADC enable, 0 : on; 1 : off |
| [3:0] | RW | reserved | 0x0 |  |

      - CR_MIX

Offset address: 0x10

Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7] | RW | mix_en | 0x0 | Digital aliasing enabled, 0 : Digital aliasing is off; 1 : Digital aliasing is on |
| [6] | RW | mix_load | 0x0 | Digital aliasing indirect address space read and write control 0 : read; 1 : write |
| [5:0] | RW | mix_add | 0x0 | Digital aliased indirect address |

|  |  | [7:6] | RW | aidacl_sel | 0x0 | DAC1 modulator Input audio left channel |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Source selection, |
|  |  |  |  |  |  | 00 : I_DAC1_INL = DAC1_INL; |
|  |  |  |  |  |  | 01 : I_DAC1_INL = DAC1_INR; |
|  |  |  |  |  |  | 10 : I_DAC1_INL = |
|  |  |  |  |  |  | (DAC1_INL+DAC1_INR)/2; |
|  |  |  |  |  |  | 11 : I_DAC1_INL = 0 |
|  | Digital | [5:4] | RW | aidacr_sel | 0x0 | DAC1 modulator Input audio right channel |
| MIX_0 ( 0x00 ) | mixer control |  |  |  |  | Source selection, 00 : I_DAC1_INR = DAC1_INR; |
|  | register 0 |  |  |  |  | 01 : I_DAC1_INR = DAC1_INL; |
|  |  |  |  |  |  | 10 : I_DAC1_INR = |
|  |  |  |  |  |  | (DAC1_INL+DAC1_INR)/2; |
|  |  |  |  |  |  | 11 : I_DAC1_INR = 0 |
|  |  | [3:1] | RW | reserved | 0x0 |  |
|  |  | [0] | RW | dac_mix | 0x0 | DAC1 Input audio aliasing enabled, |
|  |  |  |  |  |  | 0 : DAC only ; 1 : DAC+ADC |
|  |  | [7:6] | RW | mixdacl_sel | 0x0 | DAC Left channel of input audio to the mixer |
|  |  |  |  |  |  | Audio source selection, |
|  |  |  |  |  |  | 00 : I_DILMIX = DIL; |
|  |  |  |  |  |  | 01 : I_DILMIX = DIR; |
|  | Digital |  |  |  |  | 10 : I_DILMIX = (DIL+DIR)/2; |
| MIX_1 ( 0x01 ) | mixer control |  |  |  |  | 11 : I_DILMIX = 0 |
|  |  | [5:4] | RW | mixdacr_sel | 0x0 | DAC Right channel of input audio to the mixer |
|  | register 1 |  |  |  |  | Audio source selection, |
|  |  |  |  |  |  | 00 : I_DIRMIX = DIR; |
|  |  |  |  |  |  | 01 : I_DIRMIX = DIL; |
|  |  |  |  |  |  | 10 : I_DIRMIX = (DIL+DIR)/2; |
|  |  |  |  |  |  | 11 : I_DIRMIX = 0 |

|  |  | [3:0] | RW | reserved | 0x0 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | [7:6] | RW | aiadcl_sel | 0x0 | ADC Give the left channel audio source selection to the port, |
|  |  |  |  |  |  | 00 : DOL = I_DOL; |
|  |  |  |  |  |  | 01 : DOL = I_DOR; |
|  |  |  |  |  |  | 10 : DOL = (I_DOL+I_DOR)/2; |
| MIX_2 ( 0x02 ) | Digital mixer control register 2 |  |  |  |  | 11 : DOL = 0 |
|  |  | [5:4] | RW | aiadcr_sel | 0x0 | ADC Give the right channel audio source selection to the port, 00 : DOR = I_DOR; 01 : DOR = I_DOL; 10 : DOR = (I_DOL+I_DOR)/2; |
|  |  |  |  |  |  | 11 : DOR = 0 |
|  |  | [3:1] | RW | reserved | 0x0 |  |
|  |  | [0] | RW | mix_rec | 0x0 | ADC Aliasing of output data is enabled, |
|  |  |  |  |  |  | 0 : ADC only ; 1 : ADC+DAC |
|  |  | [7:6] | RW | mixadcl_sel | 0x0 | ADC Left channel of input audio to the mixer |
|  |  |  |  |  |  | Audio source selection, |
|  |  |  |  |  |  | 00 : I_ADC_INL = ADC_INL; |
|  |  |  |  |  |  | 01 : I_ADC_INL = ADC_INR; |
|  |  |  |  |  |  | 10 : I_ADC_INL = |
| MIX_3 ( 0x03 ) | Digital mixer control register 3 |  |  |  |  | (ADC_INL+ADC_INR)/2; 11 : I_ADC_INL = 0 |
|  |  | [5:4] | RW | mixadcr_sel | 0x0 | ADC The right channel audio source selection of the input audio to the mixer, 00 : I_ADC_INR = ADC_INR; |
|  |  |  |  |  |  | 01 : I_ADC_INR = ADC_INL; |
|  |  |  |  |  |  | 10 : I_ADC_INR = |
|  |  |  |  |  |  | (ADC_INL+ADC_INR)/2; |
|  |  |  |  |  |  | 11 : I_ADC_INR = 0 |
|  |  | [3:0] | RW | reserved | 0x0 |  |

      - DR_MIX

Offset address: 0x11 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:0] | RW | mix_data | 0x0 | Reading and writing data with digital aliased indirect addresses |

      - CR_VIC

Offset address: 0x12 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:2] | RW | reserved | 0x03 |  |
| [1] | RW | sb_sleep | 0x1 | sleep mode, 0 : active ; 1 : sleep |
| [0] | RW | sb | 0x1 | All codecs Simulate partial shutdown mode, 0 : active ; 1 : analog power- down |

      - CR_CK

Offset address: 0x13 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7] | RW | reserved | 0x0 |  |
| [6] | RW | mclk_div | 0x1 | 0 : MCLK/1 ( reserved for Dolphin application ); 1 : MCLK/2 |
| [5] | RW | reserved | 0x0 |  |
| [4] | RW | shutdown_clock | 0x0 | Turn off MCLK Clock, 0 : active ; 1 : MCLK Close ( Constant = 0 ), power-down Can be turned off in mode |
| [3:0] | RW | crystal | 0x0 | Indicates MCLK Frequency ( when MCLK_DIV=1 ), 0000 : MCLK=24MHz ; 0010 : MCLK=26MHz |

      - FCR_DAC

Offset address: 0x14 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:4] | RW | reserved | 0x0 |  |
| [3:0] | RW | dac_freq | 0x0 | DAC Sampling clock selection, 0000: 8KHz ; others omitted |

      - SFCCR_DAC

Offset address: 0x15 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7] | RW | dac_freq_valid | 0x0 | 0 : Inactive ; 1 : master FS mode Calibration function is on ( this The configuration is fixed in the application and does not need to be changed ) |
| [6:0] | RW | dac_freq_adj_h | 0x0 |  |

      - SFFCR_DAC

Offset address: 0x16 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:0] | RW | dac_freq_adj_l | 0x0 | FS Coordination configuration word with differences between FS0 dac_freq_adj=128* ( MCLK/2 ) *(1/FS-1/FS0) ( This function is not involved in the current application ) |

      - FCR_ADC

Offset address: 0x17 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7] | RW | reserved | 0x0 |  |
| [6] | RW | adc_hpf | 0x0 | ADC High-pass filter enable, 0 : off; 1 : on |
| [5:4] | RW | adc_wnf | 0x0 | ADC Wind noise filter enable, 00 : off; 01 : ADC Wind noise filter Mode1 ; 10 : ADC Wind noise filter Mode2 ; 11 : ADC Wind noise filter Mode3 ; |
| [3:0] | RW | adc_freq | 0x0 | ADC Sampling clock selection, 0000: 8KHz ; others omitted |

      - CR_TIMER_h

Offset address: 0x18 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:0] | RW | count_h | 0x0 | The timer counts down with a slow clock ( usually 512MCLK ). When the count reaches 0 , an interrupt is generated. The minimum timing cycle is ( 3+count ) *slow_clock ; The maximum timing cycle is ( 4+count ) *slow_clock+10*MCLK |

      - CR_TIMER_l

Offset address: 0x19 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:0] | RW | count_l | 0x0 | Used with count_h |

      - ICR

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:6] | RW | int_form | 0x0 | 中断信号输出方式, 00:IRQ 为持续高电平；01:IRQ 为持续低电平； 10:IRQ 为正向脉冲，脉冲宽度为 8 个 MC_CLK （Parallel）或者 8 个 MCLK（I2C）； |

Offset address: 0x1a Reset mode: H

|  |  |  |  | 11 : IRQ It is a negative pulse with a pulse width of 8 MC_CLK​ ( Parallel ) or 8 MCLK ( I2C ) ; |
| --- | --- | --- | --- | --- |
| [5:0] | RW | reserved | 0x0 |  |

      - IMR

Offset address: 0x1b Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7] | RW | adas_lock_mask | 0x1 | Block ADAS Lock sign |
| [6:3] | RW | reserved | 0xf |  |
| [2] | RW | adc_mute_mask | 0x1 | Shield ADC Mute sign |
| [1] | RW | reserved | 0x1 |  |
| [0] | RW | dac_mute_mask | 0x1 | Shielded DAC Mute sign |

      - IFR

Offset address: 0x1c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7] | RW | adas_lock_event | 0x0 | ADAS Lock event flag, when DAC ADAS Locked and lost This flag is generated when switching between locks ( this function is not involved in the current application , DAC always works in Master mode ) |
| [6:3] | RW | reserved | 0x0 |  |
| [2] | RW | adc_mute_event | 0x0 | ADC Soft mute mode event flag, when ADC Soft mute mode on This flag is generated when the function is enabled or disabled ( this function is not involved in the current application ) |
| [1] | RW | reserved | 0x0 |  |
| [0] | RW | dac_mute_event | 0x0 | DAC Soft mute mode event flag, when DAC This flag is generated when the soft mute mode is turned on or off ( this function is not available in the current application) . Involved ) |

      - IMR2

Offset address: 0x1d Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:5] | RW | reserved | 0x7 |  |
| [4] | RW | TIMER_END_MASK | 0x1 | Timer interrupt mask |
| [3:0] | RW | reserved | 0xf |  |

      - IFR2

Offset address: 0x1e

Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:5] | RW | reserved | 0x0 |  |
| [4] | RW | TIMER_END | 0x0 | Timer count end interrupt |
| [3:0] | RW | reserved | 0x0 |  |

      - GCR_DACL

Offset address: 0x1f Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7] | RW | lrgod | 0x0 | DAC1 left and right channels sharing the left channel is enabled. 0 : Independent configuration; 1 : Shared left channel configuration |
| [6] | RW | reserved | 0x0 |  |
| [5:0] | RW | godl | 0x0 | DAC1 Left channel gain configuration, 011111 : -31dB ;​ 011110 : -30dB ;​ Accumulate 1dB 000001 : -1dB 000000: 0dB ; 111111: 1dB ; 111110 : 2dB ; Accumulated 1dB 100001 : 31dB ; 100000 : 32dB ; |

      - GCR_DACR

Offset address: 0x20 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:6] | RW | reserved | 0x0 |  |
| [5:0] | RW | godr | 0x0 | DAC1 Right channel gain configuration, 011111 : -31dB ;​ 011110 : -30dB ;​ Accumulate 1dB 000001 : -1dB 000000: 0dB ; 111111: 1dB ; 111110 : 2dB ; Accumulated 1dB 100001 : 31dB ; 100000 : 32dB ; |

      - GCR_MIC1

Offset address: 0x23 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:3] | RW | reserved | 0x0 |  |
| [2:0] | RW | gim1 | 0x0 | MIC1 Gain step size ( boost stage ) 000: 0dB ; 001: 4dB ; 010: 8dB ; 011: 12dB ; 100 : 16dB ; other configurations are 20dB ; |

      - GCR_ADCL

Offset address: 0x25 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7] | RW | lrgid | 0x0 | ADC The left channel is shared by the left and right channels. 0 : Independent configuration; 1 : Shared left channel configuration |
| [6] | RW | reserved | 0x0 |  |
| [5:0] | RW | gidl | 0x0 | ADC Left channel gain configuration, 000000 : 0dB ; 000001 : 1dB ; Accumulated 1dB 101010 : 42dB ; Others: 43dB ; When gidl When the value changes, the ADC Data output at 64 In addition, the activation of the high-pass filter will also introduce a delay in the path. |

      - GCR_ADCR

Offset address: 0x26 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:6] | RW | reserved | 0x0 |  |
| [5:0] | RW | gidr | 0x0 | ADC Right channel gain configuration, 000000 : 0dB ; 000001 : 1dB ; Accumulated 1dB 101010 : 42dB ; Others : 43dB ; |

      - GCR_MIXDACL

Offset address: 0x27

Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7] | RW | lrgomix | 0x0 | Aliasing DAC The left and right channel gains share the configuration of the left channel. 0 : Independent configuration; 1 : Shared left channel configuration |
| [6] | RW | reserved | 0x0 |  |
| [5:0] | RW | gomixl | 0x0 | Aliasing DAC Left channel gain configuration, 000000 : 0dB ; 000001 : -1dB ;​ Decrease by 1dB 011111 : -31dB ; |

      - GCR_MIXDACR

Offset address: 0x28 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:6] | RW | reserved | 0x0 |  |
| [5:0] | RW | gomixr | 0x0 | Aliasing DAC Right channel gain configuration, 000000 : 0dB ; 000001 : -1dB ;​ Decrease by 1dB 011111 : -31dB ; |

      - GCR_MIXADCL

Offset address: 0x29 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7] | RW | lrgimix | 0x0 | Aliasing ADC The left and right channel gains share the configuration of the left channel. 0 : Independent configuration; 1 : Shared left channel configuration |
| [6] | RW | reserved | 0x0 |  |
| [5:0] | RW | gimixl | 0x0 | Aliasing ADC Left channel gain configuration, 000000 : 0dB ; 000001 : -1dB ;​ Decrease by 1dB 011111 : -31dB ; |

      - GCR_MIXADCR

Offset address: 0x2a Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |

| [7:6] | RW | reserved | 0x0 |  |
| --- | --- | --- | --- | --- |
| [5:0] | RW | gimixr | 0x0 | Aliasing ADC Right channel gain configuration, 000000 : 0dB ; 000001 : -1dB ;​ Decrease by 1dB 011111 : -31dB ; |

      - CR_DAC_AGC

Offset address: 0x2b Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7] | RW | dac_agc_en | 0x0 | DAC AGC Enable, 0 : Automatic gain off; 1 : Automatic gain on |
| [6] | RW | dac_agc_load | 0x0 | DAC AGC Indirect address space read and write control 0 : read; 1 : write |
| [5:0] | RW | dac_agc_add | 0x0 | DAC AGC Indirect Address |

      - DR_DAC_AGC

Offset address: 0x2c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:0] | RW | dac_agc_data | 0x0 | DAC AGC Read and write data of indirect address |

| DAC_AGC_0 (0x00 ) | DAC1 DRC Control register 0 |  | [7] | RW | lr_drc | 0x0 | DAC1 DRC The left channel is shared by the left and right channels. 0 : Independent configuration; 1 : Shared left channel configuration |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | [6:5] | RW | reserved | 0x0 |  |
|  |  |  | [4:0] | RW | lthres | 0x0 | DAC1 Left channel DRC compression threshold ,​ 00000 : 0dB ; 00001 : -1dB ;​ Decrease by 1dB 11111 : -31dB ; |
| DAC_AGC_1 (0x01 ) | DAC1 |  | [7:3] | RW | reserved | 0x0 |  |

|  | DRC Control register 1 |  | [2:0] | RW | lcomp | 0x0 | DAC1 Left channel DRC Compression rate : 0 : 1​​ 001 : 2 ; step X 2 101 : 32 ; |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DAC_AGC_2 (0x02 ) | DAC1 DRC Control register 2 |  | [7:5] | RW | reserved | 0x0 |  |
|  |  |  | [4:0] | RW | rthres | 0x0 | DAC1 Right channel DRC compression threshold ,​ 00000 : 0dB ; 00001 : - 1dB ; Decrease by 1dB 11111 : -31dB ; |
| DAC_AGC_3 (0x03 ) | DAC1 DRC Control register 3 |  | [7:3] | RW | reserved | 0x0 |  |
|  |  |  | [2:0] | RW | rcomp | 0x0 | DAC1 Right channel DRC Compression rate : 0 : 1​​ 001 : 2 ; step X 2 101 : 32 ; |

      - CR_ADC_AGC

Offset address: 0x2f Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7] | RW | adc_agc_en | 0x0 | ADC AGC Enable, 0 : Automatic gain off; 1 : Automatic gain on |
| [6] | RW | adc_agc_load | 0x0 | ADC AGC Indirect address space read and write control 0 : read; 1 : write |
| [5:0] | RW | adc_agc_add | 0x0 | ADC AGC Indirect Address |

      - DR_ADC_AGC

Offset address: 0x30 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [7:0] | RW | adc_agc_data | 0x0 | ADC AGC Read and write data of indirect address |

|  |  |  | [7] | RW | reserved | 0x0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | [6] | RW | agc_stereo | 0x0 | ADC AGC Configuration, left and right |
|  |  |  |  |  |  |  | Channels share the left channel |
|  |  |  |  |  |  |  | Set to enable, |
|  | ADC |  |  |  |  |  | 0 : Independent configuration; 1 : |
|  | AGC |  |  |  |  |  | Shared left channel configuration |
| ADC_AGC_0 (0x00) | Control |  | [5:2] | RW | target | 0xd | ADC Output amplitude target area |
|  | Register |  |  |  |  |  | between |
|  | 0 |  |  |  |  |  | 0000 : Max_target ; |
|  |  |  |  |  |  |  | 0001 : |
|  |  |  |  |  |  |  | Max_target -1.5dB ; |
|  |  |  |  |  |  |  | Decrease by 1.5dB |
|  |  |  |  |  |  |  | 1111 : |
|  |  |  |  |  |  |  | Max_target -22.5dB |
|  |  |  | [1:0] | RW | reserved | 0x0 |  |
|  |  |  | [7] | RW | ng_en | 0x0 | noise gate Enable, 1 is valid |
|  |  |  | [6:4] | RW | ng_thr | 0x0 | noise gate threshold |
|  |  |  |  |  |  |  | 000 : MIN_NG_THR; |
|  | ADC |  |  |  |  |  | 001 : |
|  | AGC |  |  |  |  |  | MIN_NG_THR+6dB ; |
| ADC_AGC_1(0x01) | Control |  |  |  |  |  | Increase by 6dB |
|  | Register |  |  |  |  |  | 111 : |
|  | 1 |  |  |  |  |  | MIN_NG_THR+42dB |
|  |  |  | [3:0] | RW | hold | 0x7 | AGC Before the function is activated |
|  |  |  |  |  |  |  | Signal holding time |
|  |  |  |  |  |  |  | 0000 : 0ms |
|  |  |  |  |  |  |  | 0001: 2ms |
|  |  |  |  |  |  |  | step * 2 |
|  |  |  |  |  |  |  | 1000: 256ms |
|  |  |  |  |  |  |  | ……. |
|  |  |  |  |  |  |  | 1111 : 32.768s |

| ADC_AGC_2 (0x02) | ADC AGC Control Register 2 |  | [7:4] | RW | atk | 0x4 | Gain ramp Fall time 0000 to 0001 : Reserved 0010 : 96ms increments 32ms 1111 : 512ms |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | [3:0] | RW | dcy | 0x4 | Gain ramp Rise time 0000 to 0001 : Reserved 0010 : 96ms increments 32ms 1111 : 512ms |
|  |  |  | [7:5] | RW | reserved | 0x0 |  |
|  |  |  | [4:0] | RW | agc_max | 0x1f | ADC Maximum channel gain |
|  |  |  |  |  |  |  | 00000 : 0 |
|  | ADC |  |  |  |  |  | Increase by 1.5dB |
|  | AGC |  |  |  |  |  | 01111 : 22.5 |
| ADC_AGC_3 (0x03) | Control |  |  |  |  |  | 10000 : 23 |
|  | Register |  |  |  |  |  | 10001 : 23 |
|  | 3 |  |  |  |  |  | 10010 ： 23 |
|  |  |  |  |  |  |  | 10011 : 24.5 |
|  |  |  |  |  |  |  | Increment 1.5 |
|  |  |  |  |  |  |  | 11111 : 42.5 |
|  |  |  | [7:5] | RW | reserved | 0x0 |  |
| ADC_AGC_4 (0x04) | ADC AGC Control Register 4 |  | [4:0] | RW | agc_min | 0x00 | ADC Minimum channel gain 00000 : 0 1.5dB increment 01111 : 22.5 10000 : 23 10001 : 23 |
|  |  |  |  |  |  |  | 10010 ： 23 |
|  |  |  |  |  |  |  | 10011 : 24.5 |
|  |  |  |  |  |  |  | Increment 1.5 |
|  |  |  |  |  |  |  | 11111 : 42.5 |

## Audio path

#### DMR model

###### Overview

HR_C7000 Built-in vocoder module, supports the encoding and decoding of audio data. In digital mode, it can realize the codec It can encode and send audio data, and decode and play it. It also supports digital call recording.

###### How it works

##### Digital audio transmission

As shown in the figure below, it is the audio data link diagram sent in digital mode.

MIC

HR_C7000

DAC

16bit TxI	TxQ PCM

Moderm

Data Buffer

AHB Master wrap	8/32

IRQ

RAM

32KB

System

ahb

40组

32bit音频数据

AHB

8bit

RAMdata

32bit

Regdata

RAM 5.8KB

Codec

CPU_SYS

声码器

CK803S

Audio MIC/LINE IN The signal passes through the built-in codec ADC Realize analog-to-digital conversion and output 16bit bit-wide PCM Audio code stream, the data stream is transmitted through two adjacent groups of 16 bits Data is combined in pairs to form 32 bits Data ( first generated 16 bits The data is stored in the low bit of 32 bits , and the data generated later is stored in the high bit ) . 32-bit group The combination of data is cached to the built-in RAM 0x1800_0000~1800_009F area, and output interrupt PCM_wr_data_interp ( irq28 , see 3.6.3 for details interrupt vector description ).

80 groups of PCM

data

(10ms audio )

0x0106

0x1F23

0x5E5F

0x0705

0x0204

0xF526

0x08B5

0xF919

„ 0x0527

0x7932

0x5024

0x3261

160

| 地址 | 数据 |
| --- | --- |
| 0x1800_0000 | 0x1F23_0106 |
| 0x1800_0004 | 0x0705_5E5F |
| 0x1800_0008 | 0xF526_0204 |
| 0x1800_000C | 0xF919_08B5 |

byte RAM space

„ 	„

| 0x1800_0098 | 0x7932_0527 |
| --- | --- |
| 0x1800_009C | 0x3261_5024 |

CPU Detected 28th When an internal interrupt source is detected, the above RAM The data in the output is sent to the voice code function area for encoding, and the interrupt is cleared ( to the Modem Register 0x1100_03B0 bit4​ Write " 1 ") , the encoding result is output to TX RAM The range of 0x1600_0960~1600_097A .

##### Digital audio reception

As shown in the figure below, this is the process of receiving audio playback in digital mode.

SPK

HR_C7000

CPU_SYS

ADC

RxI

RxQ

声码器

16bit

PCM

Moderm

RAM 5.8KB

CK803S

IRQ

RAM

32KB

System

ahb

40组

32bit音频数据

AHB

8bit

RAMdata

32bit

Regdata

Codec

Data Buffer

8/32

AHB Master wrap

27 bytes obtained by digital reception Data passes through AHB Read ( 8bit in the figure RAMdata Channel, Address 0x1600_0960~1600_097A , this process can refer to AHB Read RX RAM process ) and output to the CPU Built-in voice coding function area.

After decoding, the length is 10ms Audio data ( 160 bytes ) , the data is temporarily stored in the address range 0x1800_00A0~1800_0140 Field. Built-in CODEC By AHB The bus obtains audio data from this area and periodically generates an interrupt (irq27 ) indicating that data reading is complete at 10ms intervals . Detected 27 When the internal interrupt source is triggered, the new data decoded by the built-in vocoder is written to 0x1800_00A0~1800_0140 again , and the interrupt is cleared ( write " 1 " to bit 5 of the Modem register 0x1100_03B0 ) .

##### Digital call recording function

        - Digital Send Recording

During digital transmission, the CPU can choose to save the audio data before encoding ( 0x1800_0000 ~ 1800_009F ) or the audio data after encoding , that is , the data written by the CPU to 0x1600_0960 ~ 1600_097A .

        - Digital receiving recording

During digital reception, the CPU You can choose to save the audio data before decoding, that is, CPU From 0x1600_0960~1600_097A

Read the obtained data and save the decoded audio data ( 0x1800_00A0 ~ 1800_0140 ) .

#### FM model

###### Overview

- Support sending and receiving analog audio data;

- Supports the relay transmission of audio data in relay mode;

- Support analog call recording and saving

###### How it works

##### Analog audio transmission

There are two data sources for analog audio transmission: CODEC ADC Output PCM Code stream and save in TX RAM The simulation in sends audio data.

        - Audio source is CODEC ADC

Audio PCM Stream to FM The sending path, its data flow does not require CPU To move, you only need to configure HR_C7000 Work in

FM Mode, in PTT Send within the effective time, while ensuring CODEC ADC The enable is already enabled.

Modem register: 0x1100_0080[0] = 0 	selects the audio source as CODEC ADC 0x1100_0100[7] = 1	 choose DH4570 Work in FM model

0x1100_0560[0] 	PTT Send enable, high level valid

CODEC register : 0x1600_09C9[5] = 0 	CODEC ADC Work in MASTER model

0x1600_09C9[4] = 0 	CODEC ADC Function enabled

0x1600_09C9[1:0] = 00 	CODEC ADC Output parallel PCM Bitrate

        - Audio source is TX RAM

Modem Register: 0x1100_0080[0] = 1Select 	the audio source as TX RAM The data in .

This method is usually used in relay mode, where the base station forwards the audio data source as TX  RAM  of

0x1600_0030~0x1600_042F The interval is divided into 2 Slice Data Space ( 0x1600_0030~0x1600_022F and

0x1600_0230 ~ 0x1600_042F ) .​​​​​​​​​​ or SPI Ping-pong access is performed, with 512 byte , can save 256 16 -bit audio data ( 32ms audio playback length ) .

##### Analog audio reception

FM Receive the demodulated audio data output to the built-in CODEC DAC port, whose data flow does not require CPU To migrate, you only need to configure the DH4570 to work in FM mode and ensure that the CODEC DAC is enabled and turned on.

Modem Register: 0x1100_0080[0] = 0 	Select to pass audio through CODEC DAC Play

0x1100_0100[7] = 1 	selects DH4570 Working in simulation mode

CODEC Register: 0x1600_09C8[5] = 0 	CODEC DAC Work in MASTER model

0x1600_09C8[4] = 0 	CODEC DAC Function enabled

0x1600_09C8[1:0] = 00 	CODEC DAC Input parallel PCM Bitrate

##### Analog call recording function

        - Analog sending recording

Configuration Modem Register: 0x1100_0080[0] = 0Select 	audio source as CODEC ADC

0x1100_0100[7] = 1 	selects DH4570 Working in FM model

At this time, the analog recording function is enabled by default . ADC The output audio data is given to FM When the module is sent, it is also saved to the 0x1600_0030~0x1600_042F range in TX RAM .

CPU Access the area and move and save the data.

        - Analog receiving recording

Configuration Modem Register: 0x1100_0100[7] = 1Select	​ DH4570 Work in FM model

At this time, the analog receiving recording function is enabled by default . The audio signal generated by the receiver demodulation is output to the CODEC DAC At the same time, it is also saved to the 0x1600_04E0~0x1600_08DF range in RX RAM .

CPU Access the area and move and save the data.

##### Analog sidetone playback

Configuration Modem Register: 0x1100_0080[3] = 1. 	Enable the sidetone function .

DTMF and SelCall in analog mode In scenarios such as , the signaling signal sent will be played locally for voice playback ( in this case, the corresponding CODEC DAC needs to be enabled ) .

#### Prompt tone channel

###### Overview

AHB The bus supports writing of prompt sound data. There are two methods to choose from: writing based on query mode and writing based on interrupt mode.

###### Functional Description

##### Query-based prompt tone writing

AHB Write to 0x1600_0980~1600_09BF Interval, addresses are accumulated cyclically without interval.

By querying the baseband register 0x1100_0080 Bit31 , judge RAM The interval is full or empty ( " 1 " is full , " 0 " is empty ) . The disadvantage of this method is that it requires the CPU to constantly query.

##### Writing prompt sound based on interruption mode

Write the prompt tone data to 0x1 8 00 _ 00 A 0~ 1 800 _ 014 0 , a total of 80 16 bits Audio signal ( 10 ms ) . CODEC The DAC will be at 8KHz The data is automatically taken away at the same rate and an internal interrupt is generated . Detected 27 If the internal interrupt source irq27 is enabled , new data will be written to 0x1800_00A0~1800_0140 again .

This method is more convenient to use than the query method, and the CPU No need to wait.

## RF interface

#### DAC send

###### Overview

HR_C7000 It has two built-in high-performance DACs with single-ended output, supports two-point modulation and single-point modulation RF interfaces, and the amplitude and offset of the two signals are adjustable separately.

The user can configure the registers to select the corresponding transmit interface, the bias of the two output signals, and the amplitude of the two output signals. In addition, to control the power consumption of the chip, the user can set the registers to turn off the DAC when it is not working.

###### Functional Description

Table 42 HR_C7000 Baseband transmit control register configuration

| Offset address | name | describe | other |
| --- | --- | --- | --- |
| 0x70 | DAC_CONTROL | DAC Control registers |  |
| 0x104 | RF_MODE | Send RF Mode Settings |  |
| 0x108 | SIG_CENTER | Send IQ Two-way signal bias setting or two-point modulation Signal bias setting |  |
| 0x10c | TX_IF_FREQ | Send IF frequency word configuration |  |
| 0x110 | RF_CONTROL | RF Send advance and send interrupt advance settings |  |
| 0x114 | RF_MOD_BIAS_CTRL | [7:0] ： Mod1 Send signal amplitude configuration |  |

|  |  | [15:8] ： Mod2 Send signal amplitude configuration; Two-point modulation bias |  |
| --- | --- | --- | --- |
|  |  | [25:16] ： Mod1 Control level configuration, used for External 1846s Chip crystal oscillator stability correction |  |

###### How it works

##### Two-point modulation

MOD2

TV

AMP

RF_PA

MOD1

PLL

LPF

APC

APC

TX_VCO

HR_C7000

16.8MHz VC-TCXO

LPF

By setting the register RF_MODE[6:5] to 0x3 , HR_C7000 Working in two-point modulation mode, you can configure SIG_CENTER The registers adjust the offset values of the two-point modulation signals respectively. As shown in the figure below, the two-point modulation signals are adjusted by two op amps to obtain MOD1 and MOD2. The two signals control the crystal oscillator and VCO respectively to achieve two-point modulation. The two APCs are analog signals output by the DAC_MCU of the HR_C7000 , which can be used to adjust the voltage amplitude.

Figure 62 Two-point modulation interface signal

2240mV when sending random signals .

The modulation frequency deviation mapping relationship can be adjusted by configuring the register RF_MODE[7] . When RF_MODE[7]=0 , the relationship between the corresponding symbol and modulation frequency deviation is:

| symbol | Modulation frequency deviation |
| --- | --- |
| +3 | 1944 Hz |
| +1 | 648 Hz |
| - 1 | -648 Hz |
| - 3 | -1944 Hz |

RF_MODE[7]=1 is configured , the relationship between the corresponding symbol and modulation frequency deviation is:

| symbol | Modulation frequency deviation |
| --- | --- |
| +3 | -1944 Hz |
| +1 | -648 Hz |
| - 1 | 648 Hz |
| - 3 | 1944 Hz |

Output MOD1 can be adjusted by setting register SIG_CENTER[31:24] The circuit bias has an adjustment range of approximately ±422mV and a minimum adjustment step of 3.3mV .

Output MOD2 can be adjusted by setting register SIG_CENTER[23:16] The circuit bias has an adjustment range of approximately ±422mV and a minimum adjustment step of 3.3mV .

By setting register RF_MODE[9] , you can choose to send a gently rising starting point. If you configure RF_MODE[9]=1′b0 , the gently rising starting point is 1.65V . If you configure RF_MODE[9]=1′b1 , the gently rising starting point is 0V .

By setting register RF_MODE[10]=1′b1 , 40Hz can be sent Square wave, used for two-point modulation debugging.

By setting the register RF_CONTROL [ 13:8] , you can configure the RF control interrupt RF_TX_INTER Relative 30ms

The time slot boundary advance is adjustable from 0μs to 6300μs , with a minimum adjustment step of 100μs .

The output MOD1 can be adjusted by setting the register RF_MOD_BIAS_CTR[7:0]. Road amplitude, adjustable range is about

8.75mV~2240mV , the minimum adjustment step is 8.75mV .

The output MOD2 can be adjusted by setting the register RF_MOD_BIAS_CTR[15:8]. Road amplitude, adjustable range is about

8.75mV~2240mV , the minimum adjustment step is 8.75mV .

When sending in two-point debugging mode, if the receiving RF channel requires HR_C7000 The output DC voltage is used to control the voltage of the crystal oscillator, so you need to configure DAC_CONTROL Register enables the DAC In the normally open state . At this time , you can set MOD1 by configuring the register RF_MOD_BIAS_CTR [ 25:16 ] . The output voltage value in the receiving state is adjustable from 0 to 3.3V .

##### Single-point modulation

By setting the register RF_MODE[6:5] to 0x3 , HR_C7000 Working in two-point modulation mode, if only HR_C7000 is used in two-point mode Baseband DAC Output 1 Road Mod1 The signal is modulated by sending signal, which is single-point modulation. The bias value of single-point modulation signal can be adjusted by configuring SIG_CENTER[31:24] register. As shown in the figure below, the single- point modulation signal passes through RF Dedicated chip RDA1846S After passing PA Amplified and low-pass transmitted. APC By HR_C7000 The built- in DAC_MCU outputs an analog signal that can be used to adjust the voltage amplitude.

HR_C7000

26MHz VC-TCXO

MOD1

RDA1846S

PA

I2C

APC

低通

Figure 63 Single-point modulation interface signal

#### ADC take over

###### Overview

HR_C7000 Built-in two high-performance ADCs , supporting AF It supports RF interfaces such as IF and RF interfaces, and supports adjustable amplitude and offset of two channels .

Control two ADCs through register configuration The voltage when the signal is at full scale input.

the MCU can be configured to select Automatically convert the corresponding ADC according to the receiving time slot

the MCU is set to sleep mode, or Control the corresponding ADC working status.

###### Functional Description

Table 43 HR_C7000 Baseband receive control register configuration

| Offset address | name | describe | other |
| --- | --- | --- | --- |
| 0x74 | ADC_CONTROL | ADC Control registers |  |

| 0x104 | RF_MODE | Receive RF Mode Settings |  |
| --- | --- | --- | --- |
| 0x108 | SIG_CENTER | Receive IQ Two-way bias setting |  |
| 0x1b0 | RX_IF_FREQ | Receive IF frequency word configuration |  |
| 0x120 | THRESHOLD_VALUE | Configure the detection threshold of the arrival detection module and the timing synchronization module. Block detection threshold |  |
| 0x128 | PHASE_OUT | AF Phase amplitude (including gain) in receive mode Interval. Represented by maximum and minimum values respectively. |  |

###### How it works

      - AF take over

By setting RF_MODE[24]=1′b1 , HR_C7000 can Working in AF The receiving block diagram using AF mode is shown below.

HR_C7000

26MHz VC-TCXO

AF_out

RDA1846S

I2C

低通

LNA

Figure 64 AF Receive reference interface circuit

AF Receive mode, enter HR_C7000 The signal is a frequency modulated signal and needs to be modulated within the chip Gain and offset of the signal.

AF​ The signal gain is adjusted by register RF_MODE[23:16] . 0~255 Respectively represent 0~15.9375 Multiplier gain, that is, the unit step is 1/16 Times. By reading the register PHASE_OUT in real time Get the AF under the current gain configuration Phase amplitude range (for standard signal strength of -47dBm , modulation frequency deviation 2.749KHz) Plus or minus 3 Signal, AF Phase amplitude range is -1300~1300 To further improve digital reception performance, AF gain fine-tuning can be performed simultaneously with the receive bit error rate test to obtain an empirical configuration value for RF_MODE[23:16] that matches the current peripheral hardware design.

AF Signal bias then CPU DAC on the bus Provide an offset point and configure AF Receive differential ADC N​ The AF signal terminal is connected to the P terminal of the ADC .

In addition to the above-mentioned amplitude and bias adjustments, it is recommended to turn on register AF_MODE [ 25 ] and set “ 1 ” to be valid, which can effectively avoid the interference of the high-frequency signal envelope on the AF phase signal under strong RF signals.

##### IF reception

By setting R F _ MODE [ 14 : 1 3 ] = 2 ′ b0 0 , HR_C7000 can be enabled . Working in IF mode, the bias of the received IF signal can be adjusted by configuring the SIG_CENTER register, and the RX_IF_FREQ register can be configured to adjust the bias of the received IF signal. For HR_C7000 Set the received intermediate frequency. Set the appendix register THRESHOLD_VALUE to set the detection threshold of the timing synchronization module and the detection threshold of the arrival detection module.

The receiving block diagram using IF mode is shown below.

LNA

低通

21.7MHz VC-TCXO

BK4815

HR_C7000

I2C

IF

Figure 65 IF receiving reference interface circuit

The received signal is filtered and amplified before being sent to BK4815 The intermediate frequency IF obtained by demodulation of the dedicated chip The signal is directly sent to the AD of HR_C7000 Pin ADC_IVINP , while ADC The negative terminal of ADC_IVINN With VCM It is also connected to the bias voltage of the intermediate frequency signal . It should be noted that the low intermediate frequency filter is mainly used to filter out adjacent channel interference signals. When the channel spacing is 25KHz When the channel spacing is 12.5KHz, the filter bandwidth can be selected as ± 7.5KHz . When the channel spacing is 12.5KHz , the filter bandwidth can be selected as ± 3.75KHz or ± 4.5KHz .

#### Digital reception

###### Overview

HR_C7000 Supports digital interface receiving mode.

###### Functional Description

| RF IC |  |  |  |
| --- | --- | --- | --- |
|  | AD_FS | AD_SCLK | AD_S |

Digital interface uses SPI One-way receiving interface, the interface diagram is shown in the figure below.

DO

HR_C7000

Figure 66 HR_C7000 Digital interface diagram

###### How it works

Receive as shown above, where AD_FS For ADC Sampling clock, frequency is 38.4KHz ; AD_SCLK For ADC Serial data clock, frequency is 2.4576MHz ; AD_SDO is ADC serial output data.

Figure 67 Digital SPI Interface Timing Diagram

The detailed interface timing is shown in the figure above.

AD_FS Used to control ADC IQ​ Two channel input, AD_SDO Support up to 24 bits Precision input, but due to HR_C7000 The receiving channel supports up to 16 bits Receiving accuracy, can support the selection of AD_SDO 24 bits of serial data Any 16-bit data in the ADC is used as ADC sampling data input.

AD_SDO Optional status data S13-S0 in Including RSSI Value, AGC Status values and other information via SPI Input to HR_C7000 , and this information can be obtained through the internal related registers.

## DMR Physical layer applications

#### Overview

HR_C7000 Chip baseband DMR The digital part adopts a flexible layered design model, flexibly opening different levels for users to use according to different user needs.

The layered design adopts a three-layer structure, as shown in the figure.

控制和数据业务

语音业务

呼叫控制层

数据链路层

物理层

Figure 68 DMR Digital baseband physical layer structure

The physical layer (one layer) mode mainly solves the channel filtering and signal modulation and demodulation process of baseband or low intermediate frequency signals.

Using the physical layer mode requires you to handle channel encoding and decoding, as well as all communication protocol stack processing. This offers the greatest development flexibility and reduces development workload. This mode is primarily used for testing transceiver performance indicators. However, some basic physical layer parameters also apply to other operating modes.

#### register

###### Register Overview

Register Table 27 Physical Layer Register Overview ( Base Address: 0x1100_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x100 | WORK_MODE | Baseband working mode setting, this article sets it to digital layer mode |  |
| 0x104 | RF_MODE | Transceiver RF mode settings |  |
| 0x108 | SIG_CENTER | Send and receive IQ two-way bias setting |  |
| 0x10c | TX_IF_FREQ | Send IF frequency word configuration |  |
| 0x110 | RF_CONTROL | RF transmit and receive interrupt advance setting |  |
| 0x114 | RF_MOD_BIAS_CTRL | Sending signal amplitude adjustment |  |
| 0x118 | DEV_LIMITER | Send frequency deviation threshold |  |
| 0x11c | RSSI_BIAS_VALUE | Signal Energy Detection Calculation Value and ADC DC bias detection |  |
| 0x120 | THRESHOLD_VALUE | Signal synchronization detection threshold setting |  |
| 0x128 | PHASE_OUT | Phase input amplitude detection in AF receive mode |  |
| 0x12c | SEND_DATA_SYNC_H | Send data synchronization frame field |  |
| 0x130 | SEND_DATA_SYNC_L | Send data synchronization frame field |  |
| 0x134 | SEND_RC_SYNC_H | Send RC Sync frame field |  |
| 0x138 | SEND_RC_SYNC_L | Send RC Sync frame field |  |
| 0x13c | RECV_MS_SYNC_H | Receiving MS Data synchronization frame field |  |
| 0x140 | RECV_MS_SYNC_L | Receiving MS Data synchronization frame field |  |
| 0x144 | RECV_BS_SYNC_H | Receive BS data synchronization frame field |  |
| 0x148 | RECV_BS_SYNC_L | Receive BS data synchronization frame field |  |
| 0x14c | RECV_TDMA1_SYNC_H | Receive TDMA1 Data synchronization frame field |  |
| 0x150 | RECV_TDMA1_SYNC_L | Receive TDMA1 Data synchronization frame field |  |
| 0x154 | RECV_TDMA2_SYNC_H | Receive TDMA2 Data synchronization frame field |  |
| 0x158 | RECV_TDMA2_SYNC_L | Receive TDMA2 Data synchronization frame field |  |
| 0x15c | RECV_RC_SYNC_H | Receive RC Data synchronization frame field |  |
| 0x160 | RECV_RC_SYNC_L | Receive RC Data synchronization frame field |  |
| 0x164 | RECV_INFO_COUNT | Receive and calculate FSKError |  |
| 0x16c | RECVFIR_COEF1 | Receive FIR coefficients |  |
| 0x170 | RECVFIR_COEF2 | Receive FIR coefficients |  |
| 0x174 | RECVFIR_COEF3 | Receive FIR coefficients |  |
| 0x178 | RECVFIR_COEF4 | Receive FIR coefficients |  |
| 0x17c | RECVFIR_COEF5 | Receive FIR coefficients |  |
| 0x180 | RECVFIR_COEF6 | Receive FIR coefficients |  |
| 0x184 | RECVFIR_COEF7 | Receive FIR coefficients |  |
| 0x188 | RECVFIR_COEF8 | Receive FIR coefficients |  |
| 0x18c | RECVFIR_COEF9 | Receive FIR coefficients |  |
| 0x190 | RECVFIR_COEF10 | Receive FIR coefficients |  |

| 0x194 | RECVFIR_COEF11 | Receive FIR coefficients |  |
| --- | --- | --- | --- |
| 0x198 | RECVFIR_COEF12 | Receive FIR coefficients |  |
| 0x19c | RECVFIR_COEF13 | Receive FIR coefficients |  |
| 0x1a0 | RECVFIR_COEF14 | Receive FIR coefficients |  |
| 0x1a4 | RECVFIR_COEF15 | Receive FIR coefficients |  |
| 0x1a8 | RECVFIR_COEF16 | Receive FIR coefficients |  |
| 0x1ac | RECVFIR_COEF17 | Receive FIR coefficients |  |
| 0x1b0 | RX_IF_FREQ | Receive IF frequency word configuration |  |
| 0x1b4 | RF_RECV_DELAY | Receiving channel delay offset setting |  |
| 0x1b8 | MODEM_AK_CLK_CALB | AK2401 Clock Adjustment Control Register |  |
| 0x1bc | AK2401_ADC_STATE | AK2401ADC Receive Status Register |  |
| 0x1c0 | PHY_RECEIVE_STATE | Physical layer receiving status information |  |
| 0x400 | LAYER2_CONTROL | Physical layer transceiver mode control, multiplexing with Layer 2 |  |
| 0x408 | LAYER2_TXRX_CTRL | Physical layer transceiver control, multiplexing with Layer 2 |  |

###### Register Description

      - WORK_MODE

Offset address: 0x100 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:9] | - | reserved |  |  |
| [8] | RW | digital_analog_en | 0x0 | The digital and analog simultaneous detection switch is required to be turned on in the default configuration digital mode. After receiving a digital signal or an analog signal, it switches to the digital mode or analog mode accordingly. 1 : Indicates that the simultaneous digital and analog reception control is enabled; 0 : Indicates that only digital or analog reception is enabled. According to Bit7 setting. |
| [7] | RW | modulator_mode | 0x0 | Working mode settings: 0 : Indicates DMR digital mode; 1 : Indicates FM analog mode. |
| [6] | RW | is_tier1_mode | 0x1 | 0 : indicates Tier I mode; 1 : Indicates Tier II mode. |
| [5] | RW | is_continuemode | 0x1 | 0 : Indicates Continue mode; 1 : Indicates TimeSlot mode. |
| [4:3] | RW | layermode | 0x1 | Hierarchy mode settings: 0 : indicates physical layer mode; 1 : indicates Layer 2 mode; 2 : Indicates three-layer mode. |
| [2] | RW | is_repeater | 0x0 | 0 : indicates non-relay mode; |

|  |  |  |  | 1 : Indicates relay mode. |
| --- | --- | --- | --- | --- |
| [1] | RW | is_aligned | 0x1 | 0 : indicates offset mode (offset in non-relay mode indicates single-frequency mode); 1 : Indicates alignment mode. |
| [0] | RW | work_slot_sel | 0x1 | The Layer 3 non-relay mode must be set to 1 ; 0 : indicates time slot 1 ; 1 : Indicates time slot 2 . |

      - RF_MODE

Offset address: 0x104 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RW | reserved | 0x0 | In the physical layer receiving test mode, it is required to be configured as 1 . Normal mode set to 0 |
| [30] | RW | reserved | 0x0 |  |
| [29] | RW | reserved | 0x0 | In the physical layer receiving test mode, it is required to be configured as 1 . Normal mode set to 0 |
| [28:26] | - | reserved |  |  |
| [25] | RW | phase_avg_en | 0x0 | AF mode average calculation switch, high effective |
| [twenty four] | RW | af_mode | 0x0 | AF mode settings : 0 : AF signal reception disabled; 1 : AF signal reception enabled. |
| [23:16] | RW | af_recv_ctrl | 0x0 | AF receiving mode ADC input signal is directly phase signal |
| [15] | RW | rf_recviq_mode | 0x0 | PHY demodulation reception and actual RF reception ( confirmed by the actual RF reception solution): 0 : means unchanged; 1 : Indicates the opposite. |
| [14:13] | RW | rf_recv_mode | 0x0 | RF receiving mode: 0x0 : indicates intermediate frequency receiving mode; 0x1 : indicates intermediate frequency IQ receiving mode; 0x2 : Indicates baseband IQ reception mode. |
| [12] | RW | adjustsyncmax | 0x1 | Control and adjust the synchronization head detection threshold enable: 0 : indicates a fixed synchronization detection threshold setting value; 1 : Indicates automatic dynamic synchronization detection threshold. Note: Required to be set to 0 when cc_opt of the LOCAL_CC register is set to 1 . |
| [11] | RW | use_dual_phy | 0x0 | Turn on the two receiver channel control switches in time slots : 0 : Disable dual receiving channels; 1 : Enable dual receiving channels. (Applicable to relay chip receiving mode and DCDM mode) |
| [10] | RW | two_point_ctr | 0x0 | Two-point modulation test enable: |

|  |  |  |  | 0 : means off; 1 : Indicates output of 40Hz sine wave. |
| --- | --- | --- | --- | --- |
| [9] | RW | up_ctr | 0x0 | smooth rise and fall of signal strength at timeslot boundaries : 0 : means off; 1 : Enabled. |
| [8] | RW | iq_just_debug | 0x0 | IQ two-way balance debugging test enable: 0 : means unchanged; 1 : Indicates adding an offset value. |
| [7] | RW | rf_transiq_mode | 0x1 | PHY modulation transmission and actual RF transmission (confirmed by the actual RF transmission scheme): 0 : means unchanged; 1 : Indicates the opposite. |
| [6:5] | RW | rf_trans_mode | 0x3 | RF transmission mode: 0x0 : indicates intermediate frequency transmission mode; 0x1 : indicates intermediate frequency IQ transmission mode; 0x2 : indicates baseband IQ transmission mode; 0x3 : Indicates two-point modulation transmission mode. |
| [4] | RW | ak2401_mode | 0x0 | AK2401 mode setting: 0 : AK2401 mode off; 1 : AK2401 mode is on. |
| [3:0] | RW | ak_adc_shift | 0x0 | AK2401 mode input ADC signal shift control Control register |

      - SIG_CENTER

Offset address: 0x108 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | RW | trans_i_sig_center | 0x0 | Send I channel bias value setting 0~255 |
| [23:16] | RW | trans_q_sig_center | 0x0 | Send Q channel bias value setting 0~255 |
| [15:8] | RW | recv_i_sig_center | 0x0 | Receive I channel bias value setting 0~255 |
| [7:0] | RW | recv_q_sig_center | 0x0 | Receive Q channel bias value setting 0~255 |

      - TX_IF_FREQ

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:0] | RW | if_freq_tx | 0x0bb800 | 发送中频频率字配置 |

Offset address: 0x10c Reset mode: H

      - RF_CONTROL

Offset address: 0x110 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:21] | - | reserved |  |  |
| [20:16] | RW | tx_pre_on | 0x4 | Send the configuration value of the advance, step 100us |
| [15:14] | - | reserved |  |  |
| [13:8] | RW | rf_pre_on_tx | 0xa | RF transmit interrupt advance setting, step 100us |
| [7:6] | - | reserved |  |  |
| [5:0] | RW | rf_pre_on_rx | 0xa | RF receive interrupt advance setting, step 100us |

      - RF_MOD_BIAS_CTRL

Offset address: 0x114 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:26] | - | reserved |  |  |
| [25:16] | RW | two_point_bias | 0x0 |  |
| [15:8] | RW | sig_reduce | 0x0 | Adjust the IQ channel amplitude in IQ mode. Adjust the Mod2 amplitude in two-point mode . 0 remains unchanged, 1~255 changes from small to large |
| [7:0] | RW | phase_reduce | 0x0 | Mod1 amplitude in two-point mode . 0 remains unchanged, 1~255 changes from small to large |

      - DEV_LIMITER

Offset address: 0x118 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | - | reserved |  |  |
| [15:0] | RW | dev_limiter | 0xff00 | Limit the sending frequency deviation threshold |

      - RSSI_BIAS_VALUE

Offset address: 0x11c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RO | rssi_value | 0x0 | Signal energy detection calculation value |
| [15:8] | RO | adc_bias_i | 0x0 | ADC I input DC offset detection |
| [7:0] | RO | adc_bias_q | 0x0 | ADC Q - channel input DC bias detection |

      - THRESHOLD_VALUE

Offset address: 0x120

Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:29] | - | reserved |  |  |
| [28:24] | RW | sync_real_limit | 0x9 | Signal synchronization detection ratio threshold, when the synchronization detection value meets the synchronization head detection low threshold, but does not meet the synchronization head detection high threshold, the ratio needs to be judged. Decision |
| [23:16] | RW | maxvalue_low_limit | 0x64 | Signal synchronization head detection low threshold |
| [15:8] | RW | maxvalue | 0x78 | Signal synchronization head detection high threshold |
| [7:0] | RW | sig_cor_value | 0x67 | Signal reaches detection threshold |

      - PHASE_OUT

Offset address: 0x128 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:30] | - | reserved |  |  |
| [29:16] | RO | phase_max_out | 0x0 | When detecting the phase input amplitude in AF receiving mode, 14-bit phase signal maximum value |
| [15:14] | - | reserved |  |  |
| [13:0] | RO | phase_min_out | 0x0 | When detecting the phase input amplitude in AF receiving mode, 14-bit phase signal minimum value |

      - SEND_DATA_SYNC_H

Offset address: 0x12c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:16] | RW | send_data_sync1 | 0xd5 | Send Data Sync Frame Field [47:40] |
| [15:8] | RW | send_data_sync2 | 0xd7 | Send Data Sync Frame Field [39:32] |
| [7:0] | RW | send_data_sync3 | 0xf7 | Send data synchronization frame field [31:24] |

      - SEND_DATA_SYNC_L

Offset address: 0x130 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:16] | RW | send_data_sync4 | 0x7f | Send data synchronization frame field [23:16] |
| [15:8] | RW | send_data_sync5 | 0xd7 | Send data synchronization frame field [15:8] |
| [7:0] | RW | send_data_sync6 | 0x57 | Send data synchronization frame field [7:0] |

      - SEND_RC_SYNC_H

Offset address: 0x134 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:16] | RW | send_rc_sync1 | 0x77 | Send RC Sync Frame Field [47:40] |
| [15:8] | RW | send_rc_sync2 | 0xd5 | Send RC Sync Frame Field [39:32] |
| [7:0] | RW | send_rc_sync3 | 0x5f | Send RC Sync Frame Field [31:24] |

      - SEND_RC_SYNC_L

Offset address: 0x138 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:16] | RW | send_data_sync4 | 0x7f | Send data synchronization frame field [23:16] |
| [15:8] | RW | send_data_sync5 | 0xd7 | Send data synchronization frame field [15:8] |
| [7:0] | RW | send_data_sync6 | 0x57 | Send data synchronization frame field [7:0] |

      - RECV_MS_SYNC_H

Offset address: 0x13c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RW | recv_ms_sync_en | 0x1 | Receive MS data synchronization frame enable switch, high effective (Except three-layer mode ) |
| [30:24] | - | reserved |  |  |
| [23:16] | RW | recv_ms_sync1 | 0xd5 | Receive MS data synchronization frame field [47:40] |
| [15:8] | RW | recv_ms_sync2 | 0xd7 | Receive MS data synchronization frame field [39:32] |
| [7:0] | RW | recv_ms_sync3 | 0xf7 | Receive MS data synchronization frame field [31:24] |

      - RECV_MS_SYNC_L

Offset address: 0x140 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:16] | RW | recv_ms_sync4 | 0x7f | Receive MS data synchronization frame field [23:16] |
| [15:8] | RW | recv_ms_sync5 | 0xd7 | Receive MS data synchronization frame field [15:8] |
| [7:0] | RW | recv_ms_sync6 | 0x57 | Receive MS data synchronization frame field [7:0] |

      - RECV_BS_SYNC_H

Offset address: 0x144

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RW | recv_bs_sync_en | 0x1 | Receive BS data synchronization frame enable switch, high effective ( except (Except the three-layer model ) |
| [30:24] | - | reserved |  |  |
| [23:16] | RW | recv_bs_sync1 | 0xdf | Receive BS data synchronization frame field [47:40] |
| [15:8] | RW | recv_bs_sync2 | 0xf5 | Receive BS data synchronization frame field [39:32] |
| [7:0] | RW | recv_bs_sync3 | 0x7d | Receive BS data synchronization frame field [31:24] |

      - RECV_BS_SYNC_L

Offset address: 0x148 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:16] | RW | recv_bs_sync4 | 0x75 | Receive BS data synchronization frame field [23:16] |
| [15:8] | RW | recv_bs_sync5 | 0xdf | Receive BS data synchronization frame field [15:8] |
| [7:0] | RW | recv_bs_sync6 | 0x5d | Receive BS data synchronization frame field [7:0] |

      - RECV_TDMA1_SYNC_H

Offset address: 0x14c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RW | recv_tdma1_sync_en | 0x1 | Receive TDMA1 data synchronization frame enable switch, high Effective (except three-layer mode ) |
| [30:24] | - | reserved |  |  |
| [23:16] | RW | recv_tdma1_sync1 | 0xf7 | Receive TDMA1 data synchronization frame field [47:40] |
| [15:8] | RW | recv_tdma1_sync2 | 0xfd | Receive TDMA1 data synchronization frame field [39:32] |
| [7:0] | RW | recv_tdma1_sync3 | 0xd5 | Receive TDMA1 data synchronization frame field [31:24] |

      - RECV_TDMA1_SYNC_L

Offset address: 0x150 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:16] | RW | recv_tdma1_sync4 | 0xdd | Receive TDMA1 data synchronization frame field [23:16] |
| [15:8] | RW | recv_tdma1_sync5 | 0xfd | Receive TDMA1 data synchronization frame field [15:8] |
| [7:0] | RW | recv_tdma1_sync6 | 0x55 | Receive TDMA1 data synchronization frame field [7:0] |

      - RECV_TDMA2_SYNC_H

Offset address: 0x154

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RW | recv_tdma2_sync_en | 0x1 | Receive TDMA2 data synchronization frame enable switch, high Effective (except three-layer mode ) |
| [30:24] | - | reserved |  |  |
| [23:16] | RW | recv_tdma2_sync1 | 0xd7 | Receive TDMA2 data synchronization frame field [47:40] |
| [15:8] | RW | recv_tdma2_sync2 | 0x55 | Receive TDMA2 data synchronization frame field [39:32] |
| [7:0] | RW | recv_tdma2_sync3 | 0x7f | Receive TDMA2 data synchronization frame field [31:24] |

      - RECV_TDMA2_SYNC_L

Offset address: 0x158 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:16] | RW | recv_tdma2_sync4 | 0x5f | Receive TDMA2 data synchronization frame field [23:16] |
| [15:8] | RW | recv_tdma2_sync5 | 0xf7 | Receive TDMA2 data synchronization frame field [15:8] |
| [7:0] | RW | recv_tdma2_sync6 | 0xf5 | Receive TDMA2 data synchronization frame field [7:0] |

      - RECV_RC_SYNC_H

Offset address: 0x15c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:16] | RW | recv_rc_sync1 | 0x77 | Receive RC Sync Frame Field [47:40] |
| [15:8] | RW | recv_rc_sync2 | 0xd5 | Receive RC Sync Frame Field [39:32] |
| [7:0] | RW | recv_rc_sync3 | 0x5f | Receive RC synchronization frame field [31:24] |

      - RECV_RC_SYNC_L

Offset address: 0x160 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:16] | RW | recv_rc_sync4 | 0x7d | Receive RC Sync Frame Field [23:16] |
| [15:8] | RW | recv_rc_sync5 | 0xfd | Receive RC synchronization frame field [15:8] |
| [7:0] | RW | recv_rc_sync6 | 0x77 | Receive RC synchronization frame field [7:0] |

      - RECV_INFO_COUNT

Offset address: 0x164 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |

| [31:24] | RO | sync_error_num | 0x0 | Synchronization frame type of each frame Synchronization field error of the frame statistics |
| --- | --- | --- | --- | --- |
| [23:9] | - | reserved |  |  |
| [8] | RW | fsk_error_en | 0x0 | FSKError output enable switch: 1 : indicates output of FSKError statistics; 0 : indicates output of EVM statistics. |
| [7:0] | RO | fsk_error | 0x0 | Statistics of FSKError values or EVM statistics |

      - RECVFIR_COEF1

Offset address: 0x16c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:28] | - | reserved |  |  |
| [27:16] | RW | recvfir_coef0 | 0x003 | Receive FIR coefficient 1 |
| [15:12] | - | reserved |  |  |
| [11:0] | RW | recvfir_coef1 | 0x003 | Receive FIR coefficient 2 |

      - RECVFIR_COEF2

Offset address: 0x170 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:28] | - | reserved |  |  |
| [27:16] | RW | recvfir_coef2 | 0x001 | Receive FIR coefficient 3 |
| [15:12] | - | reserved |  |  |
| [11:0] | RW | recvfir_coef3 | 0xffe | Receive FIR coefficient 4 |

      - RECVFIR_COEF3

Offset address: 0x174 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:28] | - | reserved |  |  |
| [27:16] | RW | recvfir_coef4 | 0xffb | Receive FIR coefficient 5 |
| [15:12] | - | reserved |  |  |
| [11:0] | RW | recvfir_coef5 | 0xffb | Receive FIR coefficient 6 |

      - RECVFIR_COEF4

Offset address: 0x178 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:28] | - | reserved |  |  |

| [27:16] | RW | recvfir_coef6 | 0xfff | Receive FIR coefficient 7 |
| --- | --- | --- | --- | --- |
| [15:12] | - | reserved |  |  |
| [11:0] | RW | recvfir_coef7 | 0x006 | Receive FIR coefficient 8 |

      - RECVFIR_COEF5

Offset address: 0x17c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:28] | - | reserved |  |  |
| [27:16] | RW | recvfir_coef8 | 0x00c | Receive FIR coefficient 9 |
| [15:12] | - | reserved |  |  |
| [11:0] | RW | recvfir_coef9 | 0x009 | Receive FIR coefficient 10 |

      - RECVFIR_COEF6

Offset address: 0x180 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:28] | - | reserved |  |  |
| [27:16] | RW | recvfir_coef10 | 0xffe | Receive FIR coefficient 11 |
| [15:12] | - | reserved |  |  |
| [11:0] | RW | recvfir_coef11 | 0xfef | Receive FIR coefficient 12 |

      - RECVFIR_COEF7

Offset address: 0x184 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:28] | - | reserved |  |  |
| [27:16] | RW | recvfir_coef12 | 0xfe9 | Receive FIR coefficient 13 |
| [15:12] | - | reserved |  |  |
| [11:0] | RW | recvfir_coef13 | 0xff4 | Receive FIR coefficient 14 |

      - RECVFIR_COEF8

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:28] | - | reserved |  |  |
| [27:16] | RW | recvfir_coef14 | 0x00c | 接收FIR系数15 |
| [15:12] | - | reserved |  |  |
| [11:0] | RW | recvfir_coef15 | 0x024 | 接收FIR系数16 |

Offset address: 0x188 Reset mode: H

      - RECVFIR_COEF9

Offset address: 0x18c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:28] | - | reserved |  |  |
| [27:16] | RW | recvfir_coef16 | 0x026 | Receive FIR coefficient 17 |
| [15:12] | - | reserved |  |  |
| [11:0] | RW | recvfir_coef17 | 0x00a | Receive FIR coefficient 18 |

      - RECVFIR_COEF10

Offset address: 0x190 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:28] | - | reserved |  |  |
| [27:16] | RW | recvfir_coef18 | 0xfdc | Receive FIR coefficient 19 |
| [15:12] | - | reserved |  |  |
| [11:0] | RW | recvfir_coef19 | 0xfbd | Receive FIR coefficient 20 |

      - RECVFIR_COEF11

Offset address: 0x194 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:28] | - | reserved |  |  |
| [27:16] | RW | recvfir_coef20 | 0xfc9 | Receive FIR coefficient 21 |
| [15:12] | - | reserved |  |  |
| [11:0] | RW | recvfir_coef21 | 0x006 | Receive FIR coefficient 22 |

      - RECVFIR_COEF12

Offset address: 0x198 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:28] | - | reserved |  |  |
| [27:16] | RW | recvfir_coef22 | 0x052 | Receive FIR coefficient 23 |
| [15:12] | - | reserved |  |  |
| [11:0] | RW | recvfir_coef23 | 0x076 | Receive FIR coefficient 24 |

      - RECVFIR_COEF13

Offset address: 0x19c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |

| [31:28] | - | reserved |  |  |
| --- | --- | --- | --- | --- |
| [27:16] | RW | recvfir_coef24 | 0x047 | Receive FIR coefficient 25 |
| [15:12] | - | reserved |  |  |
| [11:0] | RW | recvfir_coef25 | 0xfca | Receive FIR coefficient 26 |

      - RECVFIR_COEF14

Offset address: 0x1a0 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:28] | - | reserved |  |  |
| [27:16] | RW | recvfir_coef26 | 0xf47 | Receive FIR coefficient 27 |
| [15:12] | - | reserved |  |  |
| [11:0] | RW | recvfir_coef27 | 0xf23 | Receive FIR coefficient 28 |

      - RECVFIR_COEF15

Offset address: 0x1a4 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:28] | - | reserved |  |  |
| [27:16] | RW | recvfir_coef28 | 0xfaf | Receive FIR coefficient 29 |
| [15:12] | - | reserved |  |  |
| [11:0] | RW | recvfir_coef29 | 0x0ed | Receive FIR coefficient 30 |

      - RECVFIR_COEF16

Offset address: 0x1a8 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:28] | - | reserved |  |  |
| [27:16] | RW | recvfir_coef30 | 0x281 | Receive FIR coefficient 31 |
| [15:12] | - | reserved |  |  |
| [11:0] | RW | recvfir_coef31 | 0x3d2 | Receive FIR coefficient 32 |

      - RECVFIR_COEF17

Offset address: 0x1ac Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:28] | - | reserved |  |  |
| [27:16] | RW | recvfir_coef32 | 0x455 | Receive FIR coefficient 33 |
| [15:0] | - | reserved |  |  |

      - RX_IF_FREQ

Offset address: 0x1b0 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:0] | RW | if_freq_rx | 0x0bb800 | Receive IF frequency word configuration |

      - RF_RECV_DELAY

Offset address: 0x1b4 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:19] | - | reserved |  |  |
| [18:0] | RW | rf_recv_delay | 0x0 | The receiving channel delay is set in steps of 100ns. In AF receiving mode, it is required to be set to 2000 . The default value for other schemes is 0 . |

      - MODEM_AK_CLK_CALB

Offset address: 0x1b4 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | - | reserved |  |  |
| [15] | RW | clk_inner_calb_en | 0x0 | AK2401 receiving mode (asynchronous crystal oscillator mode), baseband clock adaptive adjustment function switch 0 : Off, 1 : On, the default is off |
| [14] | RW | clk_ext_calb_en | 0x0 | AK2401 receiving mode (asynchronous crystal oscillator mode), baseband clock fixed configuration adjustment function switch 0 : Off, 1 : On, the default is off |
| [13:9] | - | reserved |  |  |
| [8:0] | RW | clk_ext_calb_coef | 0x0 | The baseband clock has a fixed configuration adjustment coefficient. The position of en_sample is adjusted every 2048 baseband clock cycles of clk_ext_calb_coef . Satisfy the equation: ( 1 ± (1/2048*clk_ext_calb_coef))*en_sample = ak_ad_fs ; Where en_sample is the 38.4KHz sampling of the baseband ak_ad_fs is the ADC output enable when the AK is externally connected to the crystal oscillator clock . |

      - AK2401_ADC_STATE

Offset address: 0x1bc Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:14] | - | reserved |  |  |
| [13:8] | RO | ak_rssi | 0x0 | AK ADC RSSI value |
| [7:0] | RO | ak_status | 0x0 | AK ADC receiving status value |

      - PHY_RECEIVE_STATE

Offset address: 0x1c0 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:0] | RO | phy_receive_state | 0x0 | Physical layer receiving status information |

      - LAYER2_CONTROL

Offset address: 0x400 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:8] | - | reserved |  |  |
| [7] | RW | txen | 0x0 | Physical layer transmit synchronization enable switch, 0 is off, 1 is on Open |
| [6] | RW | rxen | 0x0 | Physical layer receive synchronization enable switch, 0 is off, 1 is on Open |
| [5:2] | - | reserved |  |  |
| [1] | RW | tx_master_mode | 0x0 | Physical layer transmit time slot establishment mode, 0 passive mode, 1 Active Mode |
| [0] | RW | rx_master_mode | 0x0 | Physical layer receive slot establishment mode, 0 passive mode, 1 Active mode, the default requirement is set to 0 . |

      - LAYER2_TXRX_CTRL

Offset address: 0x408 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [twenty three] | RW | syncfail_layer2_rx | 0x0 | Physical layer loses receive timeslot synchronization control switch: 0 : remain unchanged; 1 : Release the synchronization time slot and re-establish it. |
| [22:8] | - | reserved |  |  |
| [7] | RW | txnextsloten | 0x0 | Next time slot transmission enable: 0 : The upcoming time slot interrupt will not be sent at the beginning; 1 : The upcoming time slot interrupt starts transmission. |
| [6] | RW | rxnextsloten | 0x0 | Next time slot reception enable: 0 : The upcoming time slot interrupt will not be received at the beginning; 1 : The upcoming time slot interrupt starts reception. |

| [5:1] | - | reserved |  |  |
| --- | --- | --- | --- | --- |
| [0] | RW | autotest | 0x0 | Physical layer test enable, high effective |

#### Interrupt vector

DMR In digital physical layer mode, the interrupt types are defined as shown in the following table.

Table 44 List of physical layer interrupt types

| Serial number | name | type | describe |
| --- | --- | --- | --- |
| 1 | RF_TX_INTERP | Level trigger, high effective | RF switching interrupt, RF Sending start interrupt, through INTERP_CLEAR Register corresponding bit Bit write 1 Clear |
| 2 | RF_RX_INTERP | Level trigger, high effective | RF switching interrupt, RF Receive start interrupt, through INTERP_CLEAR Register corresponding bit Bit write 1 Clear |
| 3 | SYS_INTERP | Level trigger, high effective | Baseband system interrupt, sub-interrupt see the table below, through SYS_INTERP_CLEAR Write 1 to the corresponding bit of the register Clear |
| 4 | TIME_SLOT_TX_INTERP | Level trigger, high effective | Baseband digital 30ms Send time slot interrupt, through INTERP_CLEAR Register corresponding bit Bit write 1 Clear |
| 5 | TIME_SLOT_RX_INTERP | Level trigger, high effective | Baseband digital 30ms Receive time slot interrupt, through INTERP_CLEAR Register corresponding bit Bit write 1 Clear |

Table 45 Physical layer system interrupt sub-interrupt type list

| Serial number | name | type | describe |
| --- | --- | --- | --- |
| 1 | PHY_RDY_INTERP | Level trigger, high effective | Physical layer processing interrupt: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Via SYS_INTERP_CLEAR Register corresponding bit Bit Write 1 Clear. |
| 2 | PHY_RX_INTERP | Level trigger, high effective | Physical layer reception deframing interruption: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Via SYS_INTERP_CLEAR Register corresponding bit Bit Write 1 Clear. |
| 3 | PHY_RECV_ABNORMAL | Level trigger, high effective | The physical layer receives an abnormal time slot boundary and prompts an interruption: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Through SYS_INTERP_CLEA |

#### Memory Mapping

Baseband Ram Module is HR_C7000 The chip baseband digital function's main user data interaction storage space is divided into two RAMs, send and receive, according to different functional designs . The data bit width is 8 bits and the depth is 1200 .

###### The physical layer sends Ram

The physical layer sends Ram The base address is 0x16000000 , the depth is 1200 , and the following table defines the content allocation of each address segment.

Table 46 The physical layer sends Ram distribute

| Offset Address | name | describe |
| --- | --- | --- |
| 0x00~0x23 | SEND_DATA0 | Physical layer sends 1 Frame data, same as SEND_DATA1 alternately |
| 0x24~0x47 | SEND_DATA1 | Physical layer sends 1 Frame data, same as SEND_DATA0 alternately |

###### Physical layer receiving Ram

Physical layer receiving Ram The base address is 0x160004b0 and the depth is 1200. The following table defines the content allocation of each address segment.

Table 47 Physical Receive Ram distribute

| Offset Address | name | describe |
| --- | --- | --- |
| 0x30~0x53 | RECV_INFO | In time slot mode, 0x30~0x50 264 bits Deframing information; In continuous mode, 0x30~0x32 CACH​ 24 bits Information, 0x33 to 0x53 264 bits Deframing information |

#### Physical layer transmission

###### Overview

Physical layer transmission is primarily used for transmission performance testing and in conjunction with reception performance testing. The physical layer transmission test is relatively simple, requiring only register configuration to initiate transmission and the data content to be transmitted. The data is sent directly without any encoding.

###### Functional Description

's briefly introduce the content of the sent data. Standard corresponding to 2 One bit of data is a symbol, and its symbol mapping relationship is shown in the following table.

| Bit | Mapped symbols |
| --- | --- |
| 00 | +1 |
| 01 | +3 |
| 10 | - 1 |
| 11 | - 3 |

Therefore, the physical layer supports six basic signal tests: +1 , -1 , +3 , -3 , +1-1 , and +3-3. For example, if you need to send +3-3 , the configuration data value is 0x5f . Each frame of data is 36 bytes , which are written into the two addresses of the send data storage.

Of course, the physical layer transmission must support standard DMR The structure frame is shown in the figure below.

| C A C H | Voice(108) | SYNC(48) | Voice(108) | C A C H |
| --- | --- | --- | --- | --- |

Speech frame A

Speech frame

Data Frame

| CC | PI | LCSS |  |
| --- | --- | --- | --- |

| CACH | Voice(108) | EMB(8) | Embeded signalling​ (32) | EMB(8) | Voice(108) | CACH |
| --- | --- | --- | --- | --- | --- | --- |

| CACH | Info(98) | SlotType(1 0) | SYNC(48) | SlotType(1 0) | Info(98) | CACH |
| --- | --- | --- | --- | --- | --- | --- |

FEC Parity

FEC Parity

DataType

CC

Figure 69 DMR Standard sending frame structure

###### How it works

Table 48 Physical layer transmission configuration table

| Serial number | Register /Ram address | Configuration Values | describe |
| --- | --- | --- | --- |
| 1 | WORK_MODE | 0x22 or 0x02 | 0x22 Indicates the physical layer time slot mode; 0x02 Indicates physical layer continuous mode |
| 2 | RF_MODE | 0x60 | Two-point transmission intermediate frequency receiving mode |
| 3 | LAYER2_CONTROL | 0x82 | Physical layer active transmission mode |
| 4 | LAYER2_TXRX_CTRL | 0x81 | Physical layer active transmission enable |
| 5 | 0x16000000~0x16000023 |  | Send data storage address 1 |
| 6 | 0x16000024~0x16000047 |  | Send data storage address 2 |

Usually we fill the SYNC field area with standard synchronization words and fill the data part with random numbers to generate a frame of 36 bytes of data, which are written into the two addresses of the send data storage.

The reference error test random numbers are as follows:

{0x80,0x0c,0x4F,0xC3,0x26,0xFE,0xBB,0xC1,0xD6,0x26,0x23,0x14,0x6F,0xFD,0x9E,0xD5,0x D7,0xF7,0x7F,0xD7,0x57,0x61,0xE8,0x5C,0x92,0x83,0x94,0xEA,0xFB,0x6B,0x3E,0x59,0xC4,0x0D, 0x60,0x01} .

#### Physical layer reception

###### Overview

Physical layer reception is mainly used for receiving bit error rate testing.

###### Functional Description

- Support data control frame reception

- Support voice frame reception

- Supports both time slot reception and continuous reception

The following figure defines the information content of the physical layer receiving demodulation output, corresponding to the information from the receiving Ram Can be obtained in.

| CACH (twenty four) | Slot Type (10) | Slot Type (10) | SYNC (48) | Info (98) | Info (98) |
| --- | --- | --- | --- | --- | --- |

SyncType = 2

| CACH (twenty four) | SYNC or embedded signalling (48) | Info (108) | Info (108) |
| --- | --- | --- | --- |

SyncType = 0/1

###### How it works

Figure 70 Physical layer parsing data order

Table 49 Physical layer receive configuration table

| Serial number | Register /Ram address | Configuration Values | describe |
| --- | --- | --- | --- |
| 1 | WORK_MODE | 0x22 or 0x02 | 0x22 Indicates the physical layer time slot mode; 0x02 Indicates physical layer continuous mode |
| 2 | RF_MODE | 0xa0000060 | Two-point transmission intermediate frequency receiving mode |
| 3 | LAYER2_CONTROL | 0x40 | Physical layer passive receiving mode |
| 4 | LAYER2_TXRX_CTRL | 0x41 | Physical layer passive reception enable |

The physical layer receiving state process is shown in the figure below.

IDLE

检测到系统中断

①

SYS_INTERP_LI ST==0x800

①Read register SYS_INTERP_LIST to get interrupt type, write register

SYS_INTERP_CLEAR clears the interrupt

②Read register RX_TYPE_INFO to obtain the received frame type, and read the received Ram to obtain the deframed data

| Physical layer deframing interruption |  |
| --- | --- |
|  | ② |

读取帧类型、

解帧数据

Figure 71 Physical layer receiving state flow chart

## DMR Link layer applications

#### Overview

HR_C7000 Chip baseband DMR The digital part adopts a flexible layered design model, flexibly opening different levels for users to use according to different user needs.

Link layer (Layer 2) mode primarily opens up all physical layer content and performs channel encoding and decoding, interleaving and deinterleaving, and verification, as shown in the gray dashed box in the figure below. Users only need to handle the communication protocol stack's processing flow, offering greater development flexibility and a moderate workload.

控制和数据业务

语音业务

呼叫控制层

数据链路层

物理层

Figure 72 DMR Digital baseband link layer structure

#### register

###### Register Overview

Register Table 28 Link Layer Register Overview ( Base Address: 0x1100_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x100 | WORK_MODE | Baseband working mode setting, this article sets it to digital layer 2 mode |  |
| 0x350 | PI_CRC_MASK | PI Frame CRC16 Mask and initial value settings |  |
| 0x354 | RS_H_MASK | RS24 of voice frame header Code mask settings |  |
| 0x358 | RS_H_INIT | RS24 of voice frame header Initial value setting of code |  |
| 0x35c | RS_T_MASK | Mask setting of RS24 code at the end of the frame |  |
| 0x360 | RS_T_INIT | RS24 code at the end of the frame |  |
| 0x364 | CSBK_CRC_MASK | CSBK Frame CRC16 mask and initial value settings |  |
| 0x368 | MBCH_CRC_MASK | MBC CRC16 mask and initial value setting of frame header |  |
| 0x36c | DATAH_CRC_MASK | CRC16 of the data frame header Mask and initial value settings |  |
| 0x380 | CRC8_INIT | ShortLC CRC8 of the frame Initial value setting |  |
| 0x384 | LOCAL_CC | This machine sends and receives CC set up |  |
| 0x388 | VOICE_EMB_CTRL | Speech frame EMB Regional PI Setting and speech frame F EMB​ Fill Settings |  |
| 0x38c | SCRAMBLE_REG | Simple voice encryption mode key setting |  |
| 0x390 | RX_TYPE_INFO | Receive frame information description |  |
| 0x39c | RX_INFO | Receive RC Frame Information |  |
| 0x400 | LAYER2_CONTROL | Layer 2 mode control |  |
| 0x404 | LAYER2_SLOTON | Layer 2 timeslot switch control |  |
| 0x408 | LAYER2_TXRX_CTRL | Layer 2 transmit and receive enable control |  |
| 0x40c | LAYER2_SLOT_CNT | Setting the initial value of Layer 2 timeslot count |  |
| 0x410 | LAYER2_MAX_CNT | Setting the maximum count correction value of the Layer 2 timeslot |  |
| 0x414 | LAYER2_SLOT_UPDATE | Layer 2 timeslot update settings |  |
| 0x418 | LAYER2_SEND_TYPE | Layer 2 sending frame type test |  |
| 0x41c | LAYER2_STATUS | Layer 2 timeslot status indication |  |
| 0x424 | LAYER2_SLOT_PRE | Pre-configure the interruption location for Layer 2 processing |  |
| 0x428 | LAYER2_BS_MODE | Repeater mode settings |  |
| 0x430 | BS_SECOND_REG | Repeater Simulcast Mode GPS Seconds time setting register |  |
| 0x434 | BS_STATUS | BS repeater simulcast mode |  |
| 0x438 | BS_CACH | The repeater sends CACH set up |  |

###### Register Description

      - WORK_MODE

Offset address: 0x100 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:9] | - | reserved |  |  |
| [8] | RW | digital_analog_en | 0x0 | The digital and analog simultaneous detection switches are required to be turned on in the default configuration digital mode. After the signal or analog signal is switched to digital |

|  |  |  |  | Digital mode or analog mode. 1 : Indicates that the simultaneous digital and analog reception control is enabled; 0 : Indicates that only digital or analog reception is enabled, depending on the setting of Bit7 . |
| --- | --- | --- | --- | --- |
| [7] | RW | modulator_mode | 0x0 | Working mode settings: 0 : Indicates DMR digital mode; 1 : Indicates FM analog mode. |
| [6] | RW | is_tier1_mode | 0x1 | 0 : indicates Tier I mode; 1 : Indicates Tier II mode. |
| [5] | RW | is_continuemode | 0x1 | 0 : Indicates Continue mode; 1 : Indicates TimeSlot mode. |
| [4:3] | RW | layermode | 0x1 | Hierarchy mode settings: 0 : indicates physical layer mode; 1 : indicates Layer 2 mode; 2 : Indicates three-layer mode. |
| [2] | RW | is_repeater | 0x0 | 0 : indicates non-relay mode; 1 : Indicates relay mode. |
| [1] | RW | is_aligned | 0x1 | 0 : indicates offset mode (offset in non-relay mode indicates single-frequency mode); 1 : Indicates alignment mode. |
| [0] | RW | work_slot_sel | 0x1 | The Layer 3 non-relay mode must be set to 1 ; 0 : indicates time slot 1 ; 1 : Indicates time slot 2 . |

      - PI_CRC_MASK

Offset address: 0x350 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RW | pi_crc16_init_reg | 0x0 | Initial value of PI frame CRC16 |
| [15:0] | RW | pi_crc16_mask | 0x6969 | PI frame CRC16 mask |

      - RS_H_MASK

Offset address: 0x354 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:0] | RW | rs24_head_mask | 0x969696 | Voice frame header RS24 mask |

      - RS_H_INIT

Offset address: 0x358 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:0] | RW | rs24_head_init_reg | 0x0 | Initial value of the voice frame header RS24 |

      - RS_T_MASK

Offset address: 0x35c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:0] | RW | rs24_term_mask | 0x969696 | Voice or data frame tail RS24 mask |

      - RS_T_INIT

Offset address: 0x360 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:0] | RW | rs24_term_init_reg | 0x0 | Initial value of RS24 at the end of a voice or data frame |

      - CSBK_CRC_MASK

Offset address: 0x364 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RW | csbk_crc16_init_reg | 0x0 | Initial value of CRC16 of CSBK frame |
| [15:0] | RW | csbk_crc16_mask | 0xa5a5 | CSBK frame CRC16 mask |

      - MBCH_CRC_MASK

Offset address: 0x368 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RW | mbch_crc16_init_reg | 0x0 | Initial value of MBC frame header CRC16 |
| [15:0] | RW | mbch _crc16_mask | 0xa5a5 | MBC frame header CRC16 mask |

      - DATAH_CRC_MASK

Offset address: 0x36c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RW | datah_crc16_init_reg | 0x0 | Initial value of the data frame header CRC16 |
| [15:0] | RW | datah _crc16_mask | 0xa5a5 | Data frame header CRC16 mask |

      - CRC8_INIT

Offset address: 0x380 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:8] | - | reserved |  |  |
| [7:0] | RW | crc8_init_reg | 0x0 | Initial value of CRC8 in shortlc |

      - LOCAL_CC

Offset address: 0x384 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:17] | - | reserved |  |  |
| [16] | RW | encryption_type | 0x0 | Voice encryption type settings: 0 : indicates normal encryption; 1 : Indicates enhanced encryption. |
| [15:10] | - | reserved |  |  |
| [9:8] | RW | cc_opt | 0x0 | Receive CC matching method: 0 : Deframing is performed only when the received CC is correct and equal to the local CC; 1 : Match any correctly received CC and perform de-framing; 2 , 3 : Do not deframe any received frames. |
| [7:4] | - | reserved |  |  |
| [3:0] | RW | cc | 0x1 | Local CC Settings |

      - VOICE_EMB_CTRL

Offset address: 0x388 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:2] | - | reserved |  |  |
| [1] | RW | burstf_emb_ctrl | 0x0 | Voice F frame embedded information selection: write 0 to indicate direct from CPU Get; Writing 1 indicates internal RC encoding. |
| [0] | RW | pi_bit | 0x0 | Speech frame BCDE EMB area PI information configuration |

      - SCRAMBLE_REG

Offset address: 0x38c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RW | dscramble_reg | 0x0 | Simple voice encryption method, receiving end decryption register Configuration |

| [15:0] | RW | scramble_reg | 0x0 | Simple voice encryption method, encryption register at the sending end Configuration |
| --- | --- | --- | --- | --- |

      - RX_TYPE_INFO

Offset address: 0x390 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | - | reserved |  |  |
| [30] | RO | crc8_error_flag | 0x0 | CRC8 check result of shortlc information : 0 means correct; 1 indicates error. |
| [29] | RO | golay_check_flag | 0x0 | Golay decoding results of the SLOT_TYPE area of the data frame : 0 means correct; 1 indicates error. |
| [28] | RO | qr_check_flag | 0x0 | QR decoding result of the EMB area of the speech frame is: 0 means correct; 1 indicates error. |
| [27] | RO | cs_check_flag | 0x0 | Parity check result of access information ( Checksum ) after EMB : 0 means correct; 1 indicates error. |
| [26] | RO | crc16_error_flag | 0x0 | CRC16 check method: 0 means correct; 1 indicates error. |
| [25] | RO | rs24_error_flag | 0x0 | Data frame check result of RS24 check method: 0 means correct; 1 indicates error. |
| [twenty four] | RO | data_block_error | 0x0 | CRC9 data frame and CRC32 feedback data packet body check results: 0 means correct; 1 indicates error. |
| [twenty three] | RO | cach_at | 0x0 | Receive the acquired AT |
| [twenty two] | RO | cach_tc | 0x0 | Receive the acquired TC |
| [21:20] | RO | cach_lcss | 0x0 | Receive the obtained CACH LCSS [1:0] |
| [19] | RO | current_rx_slot | 0x0 | Current receiving time slot number |
| [18] | RO | save_soft_finish | 0x0 | Soft information storage completion flag |
| [17] | RO | pi | 0x0 | Received PI |
| [16:15] | RO | lcss | 0x0 | Received voice LCSS |
| [14:12] | RO | current_voice_burst | 0x0 | The currently received speech frames are numbered 1 to 6 , representing speech Audio frame ABCDEF |
| [11:8] | RO | data_type | 0x0 | Received data frame type |

| [7:4] | RO | cc | 0x0 | Received color code |
| --- | --- | --- | --- | --- |
| [3:2] | RO | recv_sync_type | 0x0 | Type of synchronization frame sequence received 00 : MS 01 : BS 10 : TDMA1 11 : TDMA2 |
| [1:0] | RO | sync_type | 0x0 | 00 means no synchronization header, 01 means voice, 10 means Data, 11 means RC |

      - RX_INFO

Offset address: 0x394 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:25] | - | reserved |  |  |
| [24:16] | RO | tdma_slot_num | 0x0 | Multiframe obtained from shortlc information in trunking mode Number |
| [15:11] | - | reserved |  |  |
| [10:0] | RO | rc_info | 0x0 | Received RC 11 bits of information |

      - LAYER2_CONTROL

Offset address: 0x400 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:8] | - | reserved |  |  |
| [7] | RW | txen | 0x0 | Layer 2 transmission synchronization enable switch, 0 is off, 1 is on start |
| [6] | RW | rxen | 0x0 | Layer 2 receive synchronization enable switch, 0 is off, 1 is on start |
| [5] | RW | layer2slot | 0x0 | Configure the current Layer 2 timeslot status, which is used for smooth switching from Layer 2 mode to Layer 3 timeslot: 0 : The current time slot is a non-working time slot; 1 : The current time slot is a working time slot. |
| [4] | RW | txtmp_master_mode | 0x0 | Conventional Layer 2 sending temporary reserved timeslot establishment mode, 0 Passive mode, 1 active mode |
| [3] | RW | rx_master_auto_updat e | 0x0 | In the active mode of Layer 2 reception, the receiving time slot boundary is updated Flag, 0 means no update, 1 means automatic update with reception. |
| [2] | RW | tx_slave_auto_mode | 0x0 | Conventional Layer 2 transmission, timeslot establishment method in passive mode: 1 , automatically establish the transmission and reception synchronization; 0 , manually establish the reception start transmission time slot. |
| [1] | RW | tx_master_mode | 0x0 | Conventional Layer 2 send slot establishment mode, 0 passive mode, 1 Active mode |
| [0] | RW | rx_master_mode | 0x0 | Layer 2 receive timeslot establishment mode, 0 passive mode, 1 active mode |

|  |  |  |  | In dynamic mode, the default setting is 0 . |
| --- | --- | --- | --- | --- |

      - LAYER2_ SLOTON

Offset address: 0x404 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RO | o_dll_tx_slot_on | 0x0 | Conventional Layer 2 mode sends a timeslot start flag, high valid |
| [30] | RO | o_dll_tx_slot_on_tmp | 0x0 | Conventional Layer 2 mode temporary reserved timeslot opening flag, high efficient |
| [29] | RO | o_dll_rx_slot_on | 0x0 | Conventional Layer 2 mode receive timeslot start flag, high valid |
| [28] | RO | o_bs_tx_slot_on | 0x0 | Repeater chip sending mode time slot start flag, high effective |
| [27] | RO | o_bs_rx_slot_on | 0x0 | Repeater chip receiving mode time slot start flag, high effective |
| [26:14] | - | reserved |  |  |
| [13] | WC | dll_rx_close_txtmp | 0x0 | Temporarily reserve the timeslot end switch in conventional Layer 2 mode. Writing 1 is valid and the chip is automatically cleared ( Bit1 dll_tx_slot_on_tmp is also required to be valid). |
| [12] | WC | dll_rx_open_txtmp | 0x0 | In conventional Layer 2 mode, connect the Husou time slot to establish a temporary reserved time slot start switch. Write 1 to be valid. The chip will automatically clear the time slot after it establishes the time slot (at the same time, Bit1 dll_tx_slot_on_tmp is valid and txtmp_master_mode is passive mode) . |
| [11] | RW | dll_tx_slot_frm_tmp | 0x0 | In conventional Layer 2 mode, a temporary reserved timeslot is established to start Off, controlled by temporary time slot |
| [10] | WC | dll_tx_close_txtmp | 0x0 | Temporarily reserve the timeslot end switch in conventional Layer 2 mode. Writing 1 is valid and the chip is automatically cleared ( Bit1 dll_tx_slot_on_tmp is also required to be valid). |
| [9] | WC | dll_txtmp_close_tx | 0x0 | In conventional Layer 2 mode, temporarily reserve the send timeslot to establish the send timeslot end switch. Writing 1 is valid and the chip automatically clears it. ( Bit0 dll_tx_slot_on is valid and txtmp_master_mode is passive mode ) |
| [8] | WC | dll_rx_close_tx | 0x0 | In conventional Layer 2 mode, the passive mode establishes the end switch of the transmission time slot . Writing 1 is effective and the chip automatically clears it. (At the same time , tx_slave_auto_mode must be set to 0 and Bit0 dll_tx_slot_on is valid ) |
| [7] | WC | dll_tx_open_txtmp | 0x0 | In conventional Layer 2 mode, the sending time slot establishes a temporary reserved time slot start switch. Writing 1 is valid and the chip automatically clears it after establishing the time slot (it also requires that Bit1 dll_tx_slot_on_tmp is valid and txtmp_master_mode is passive mode) . |
| [6] | WC | dll_txtmp_open_tx | 0x0 | In the conventional layer 2 active mode, temporarily reserve the transmission time slot to establish the transmission time slot start switch. Writing 1 is valid and the chip automatically clears it. ( Bit0 dll_tx_slot_on is valid at the same time as Bit11 dll_tx_slot_frm_tmp is valid ) |

| [5] | WC | dll_rx_open_tx | 0x0 | In conventional Layer 2 mode, the passive mode establishes the transmit time slot start switch. Writing 1 is valid and the chip automatically clears it after establishing the time slot . (At the same time, tx_slave_auto_mode must be set to 0.) And in Bit0 dll_tx_slot_on is valid ) |
| --- | --- | --- | --- | --- |
| [4] | RW | bs_rx_slot_on | 0x0 | The relay chip actively receives the mode to establish the receiving time slot. Yes, high level is valid |
| [3] | RW | bs_tx_slot_on | 0x0 | The relay chip sends the mode to establish the sending time slot enable, high Level valid |
| [2] | RW | dll_rx_slot_on | 0x0 | In the conventional Layer 2 active receiving mode, a receiving time slot is established. Yes, high level is valid |
| [1] | RW | dll_tx_slot_on_tmp | 0x0 | In conventional Layer 2 mode, the temporary reserved time slot is established to send Yes, high level is valid |
| [0] | RW | dll_tx_slot_on | 0x0 | In conventional Layer 2 mode, the send timeslot is established and enabled. Flat and effective |

      - LAYER2_TXRX_CTRL

Offset address: 0x408 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | WC | tx_complete_cpu | 0x0 | The sending end configuration in the layer 2 continuous sending mode is used to switch Change RF interrupt control, write 1 to be effective |
| [30:24] | - | reserved |  |  |
| [twenty three] | RW | syncfail_layer2_rx | 0x0 | Layer 2 loses the receive timeslot synchronization control switch: 0 : remain unchanged; 1 : Release the synchronization time slot and re-establish it. |
| [twenty two] | RW | syncfail_layer2_tx | 0x0 | Layer 2 loss of transmission synchronization control switch: 0 : remain unchanged; 1 : Release the synchronization time slot and re-establish it. |
| [21:16] | - | reserved |  |  |
| [15] | RW | begin_v_layer2 | 0x0 | In Layer 2 mode, the voice access success flag is set. Enter 1 After that, the voice starts to access the voice normally, but the bit Set to zero. |
| [14:8] | - | reserved |  |  |
| [7] | RW | txnextsloten | 0x0 | Next time slot transmission enable: 0 : The upcoming time slot interrupt will not be sent at the beginning; 1 : The upcoming time slot interrupt starts transmission. |
| [6] | RW | rxnextsloten | 0x0 | Next time slot reception enable: 0 : The upcoming time slot interrupt will not be received at the beginning; 1 : The upcoming time slot interrupt starts reception. |
| [5:1] | - | reserved |  |  |
| [0] | RW | autotest | 0x0 | Physical layer test enable, high effective |

      - LAYER2_SLOT_CNT

Offset address: 0x40c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RW | rx_slot_flag_init | 0x0 | Initial value of the conventional layer 2 or relay receiving time slot number ( main (in dynamic mode ) |
| [30] | RW | tx_slot_flag_init | 0x0 | Initial value of the conventional layer 2 or relay transmission time slot number ( main (in dynamic mode ) |
| [29] | WC | txtmpslot_cnt_set_en | 0x0 | Initial value configuration of regular Layer 2 or relay transmission timeslot count Enable (in active mode ) |
| [28:20] | RW | txtmpslot_cnt_set | 0x0 | Initial value configuration of regular Layer 2 or relay transmission timeslot count (In active mode ) |
| [19] | WC | rxslot_cnt_set_en | 0x0 | Conventional Layer 2 or relay receive timeslot count initial value configuration Enable (in active mode ) |
| [18:10] | RW | rxslot_cnt_set | 0x0 | Conventional Layer 2 or relay receive timeslot count initial value configuration (In active mode ) |
| [9] | WC | txslot_cnt_set_en | 0x0 | Initial value configuration of regular Layer 2 or relay transmission timeslot count Enable (in active mode ) |
| [8:0] | RW | txslot_cnt_set | 0x0 | Initial value configuration of regular Layer 2 or relay transmission timeslot count (In active mode ) |

      - LAYER2_MAX_CNT

Offset address: 0x410 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | - | reserved |  |  |
| [15] | RW | fine_tune_sign_rx | 0x0 | Conventional Layer 2 or relay receive time slot correction symbol 1 : Shorten the time slot 0 : Extend the time slot |
| [14:12] | - | reserved |  |  |
| [11:8] | RW | fine_tune_cnt_rx | 0x0 | Regular Layer 2 or relay receiving time slot correction value, each increment Subtract 1 to adjust 100us |
| [7] | RW | fine_tune_sign_tx | 0x0 | Conventional Layer 2 or relay transmission time slot correction symbol 1 : Shorten the time slot 0 : Extend the time slot |
| [6:4] | - | reserved |  |  |
| [3:0] | RW | fine_tune_cnt_tx | 0x0 | Regular Layer 2 or relay transmission time slot correction value, each increment Subtract 1 to adjust 100us |

      - LAYER2_SLOT_UPDATE

Offset address: 0x414

Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:14] | - | reserved |  |  |
| [13] | WC | rx_sync_txtmp_flag_en | 0x0 | Conventional Layer 2 receive timeslot is synchronized to the transmit temporary reservation Time slot, time slot number update flag, high effective |
| [12] | - | reserved |  |  |
| [11] | WC | txtmp_sync_tx_flag_en | 0x0 | Conventional Layer 2 transmission temporarily reserves time slots for synchronization to transmission Time slot, time slot number update flag, high effective |
| [10] | WC | tx_sync_txtmp_flag_en | 0x0 | Conventional Layer 2 send timeslot is synchronized to send temporary reservation Time slot, time slot number update flag, high effective |
| [9:6] | - | reserved |  |  |
| [5] | WC | rx_sync_txtmp_en | 0x0 | Conventional Layer 2 receive timeslot is synchronized to the transmit temporary reservation Time slot control |
| [4] | WC | bs_tx_sync_rx_en | 0x0 | Relay transmit time slot is synchronized to relay receive time slot control |
| [3] | WC | txtmp_sync_tx_en | 0x0 | Conventional Layer 2 transmission temporarily reserves time slots for synchronization to transmission Time slot control |
| [2] | WC | tx_sync_txtmp_en | 0x0 | Conventional Layer 2 send timeslot is synchronized to send temporary reservation Time slot control |
| [1] | WC | rx_sync_tx_en | 0x0 | Conventional Layer 2 receive timeslot synchronization to transmit timeslot control |
| [0] | WC | tx_sync_rx_en | 0x0 | Conventional Layer 2 transmit timeslot synchronization to receive timeslot control |

      - LAYER2_SEND_TYPE

Offset address: 0x418 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:30] | RW | encodetype1011 | 0x0 | Codec type selection for custom data type frames ( DataType is 1011 ): 00 BPTC96​ Codec; 01 Represents convolution 3/4 Codec; - Indicates no codec; - BPTC72​ Codec; |
| [29:28] | RW | encodetype1100 | 0x0 | Codec type selection for custom data type frames ( DataType is 1100 ): 00 BPTC96​ Codec; 01 Represents convolution 3/4 Codec; - Indicates no codec; - BPTC72​ Codec; |
| [27:26] | RW | encodetype1101 | 0x0 | Codec type selection for custom data type frames ( DataType is 1101 ): 00 BPTC96​ Codec; 01 Represents convolution 3/4 Codec; - Indicates no codec; - BPTC72​ Codec; |

| [25:24] | RW | encodetype1110 | 0x0 | Codec type selection for custom data type frames ( DataType is 1110 ): 00 BPTC96​ Codec; 01 Represents convolution 3/4 Codec; - Indicates no codec; - BPTC72​ Codec; |
| --- | --- | --- | --- | --- |
| [23:22] | RW | encodetype1111 | 0x0 | Codec type selection for custom data type frames ( DataType is 1111 ): 00 BPTC96​ Codec; 01 Represents convolution 3/4 Codec; - Indicates no codec; - BPTC72​ Codec; |
| [21:13] | - | reserved |  |  |
| [12] | RW | data_embrc_en | 0x0 | SYNC area of the data frame is embedded with RC information to control Energy, high efficiency |
| [11] | RW | data_embrc_ctrl | 0x0 | Data control frame EMB area inserts RC information selection control 0 : Select to directly from CPU Get; 1 : Indicates internal RC coding processing |
| [10] | WC | bs_tx_data_rdy | 0x0 | BS downlink forwarding frame type data preparation completion control: 0 : Data forwarding is not completed, the chip automatically frames and sends IDLE frames; 1 : Downlink forwarding data is ready, at the time slot boundary The chip automatically clears the flag. |
| [9] | WC | bs_tx_data_source | 0x0 | BS downlink forwarding data frame data selection: 0 : the default conventional chip completes the encoding and framing; 1 : Send 196 -bit signal directly without encoding The chip automatically clears the flag at the time slot boundary. |
| [8] | RW | crc_calc_sel | 0x0 | CRC check mode selection control: 0 : The chip automatically completes the check calculation; 1 : CRC calculation is completed by the CPU . |
| [7:4] | RW | localdatatype | 0x0 | data_type value of each data frame sent in each time slot , Voice A~F frames and RC frame type values |
| [3] | RW | localvod | 0x0 | Send frame type selection: 0 : data frame; 1 : Voice or RC frame. |
| [2] | RW | seconddataheader | 0x0 | When data_type is the data frame header, the corresponding data second header control selection is: 0 : regular data frame header; 1 : Second data frame header. |
| [1:0] | RW | locallcss | 0x0 | LCSS of each transmit time slot |

      - LAYER2_STATUS

Offset address: 0x41c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RO | tx_slot_choose | 0x0 | Layer 2 transmit timeslot number |
| [30] | RO | rx_slot_choose | 0x0 | Layer 2 receive timeslot number |
| [29] | RO | tx_slot_choose_tmp | 0x0 | Layer 2 temporary transmission time slot number |
| [28:27] | - | reserved |  |  |
| [26:18] | RO | tx_bit_cnt_tmp | 0x0 | Read the temporary transmit timeslot count value on the second layer |
| [17:9] | RO | rx_bit_cnt | 0x0 | Read the Layer 2 receive timeslot count value |
| [8:0] | RO | tx_bit_cnt | 0x0 | Read the second layer transmit timeslot count value |

      - LAYER2_SLOT_PRE

Offset address: 0x424 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:16] | RW | rdy_slot_pre | 0x55 | Default value ( cannot be modified at will ) |
| [15:8] | RW | rdy_interp_pre_rx | 0x5 | Receive Layer 2 processing interrupt location pre-configured |
| [7:0] | RW | rdy_interp_pre_tx | 0x5 | Send Layer 2 processing interrupt location advance configuration |

      - LAYER2_BS_MODE

Offset address: 0x428 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:12] | - | reserved |  |  |
| [11:8] | RW | bs_idle_type | 0x9 | Dual-frequency relay BS downlink transmission data type configuration |
| [7:6] | - | reserved |  |  |
| [5:4] | RW | gps_update_rate | 0x0 | Timeslot update adjustment period setting in co-frequency and co-broadcast mode: 00 : every 6 seconds ; 01 : every 3 seconds ; 10 : indicates every 1s . |
| [3] | RW | gps_pps_en | 0x0 | Same-frequency and same-broadcast mode |
| [2] | RW | bs_tx_mode | 0x0 | 1 means the chip works in BS sending mode |
| [1] | RW | bs_rx_mode | 0x0 | 1 means the chip works in BS receiving mode |
| [0] | RW | bs_single_freq | 0x0 | BS mode selection: 0 : Dual-frequency repeater mode; 1 : Single-frequency repeater mode. |

      - BS_SECOND_REG

Offset address: 0x430

Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:6] | - | reserved |  |  |
| [5:0] | RW | bs_second_reg | 0x0 | Seconds time value, range 0~59 |

      - BS_STATUS

Offset address: 0x434 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:30] | RO | gps_lcss_cnt | 0x0 | GPS -created LCSS cycle counter |
| [29:28] | RO | lcss_counter | 0x0 | LCSS cycle counter of CACH |
| [27:19] | RO | gps_bit_cnt | 0x0 | 30ms time slot created by GPS in co-frequency simulcast mode counter |
| [18:8] | RO | bs_slot_num | 0x0 | In the same frequency and broadcast mode, the frame number is counted in 60ms , and 60s is a cycle. Period cycle. |
| [7:6] | - | reserved |  |  |
| [5:0] | RO | bs_second_lock | 0x0 | The time lock value in seconds used for time slot startup |

      - BS_CACH

Offset address: 0x438 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:2] | - | reserved |  |  |
| [1] | RW | bs_cach_at | 0x0 | BS sends mode configuration AT information |
| [0] | RW | bs_cach_tc | 0x0 | BS sends mode configuration TC information |

#### Interrupt vector

DMR In digital link layer mode, the interrupt types are defined as shown in the following table.

Table 50 List of link layer interrupt types

| Serial number | name | type | describe |
| --- | --- | --- | --- |
| 1 | RF_TX_INTERP | Level trigger, high effective | RF switching interrupt, RF Sending start interrupt, through INTERP_CLEAR Register corresponding bit Bit write 1 Clear |
| 2 | RF_RX_INTERP | Level trigger, high effective | RF switching interrupt, RF Receive start interrupt, through INTERP_CLEAR Register corresponding bit Bit write 1 Clear |
| 3 | SYS_INTERP | Level trigger, high effective | Baseband system interrupt, sub-interrupt see the table below, through SYS_INTERP_CLEAR Write 1 to the corresponding bit of the register Clear |
| 4 | TIME_SLOT_TX_INTERP | Level trigger, high effective | Baseband digital 30ms Send time slot interrupt, through INTERP_CLEAR Register corresponding bit Bit write 1 Clear |
| 5 | TIME_SLOT_RX_INTERP | Level trigger, high effective | Baseband digital 30ms Receive time slot interrupt, through INTERP_CLEAR Register corresponding bit Bit write 1 Clear |

| 6 | PCM_WR_INTERP | Level trigger, high effective | Codec Audio data stream write interrupt, through INTERP_CLEAR Register corresponding bit Bit write 1 Clear |
| --- | --- | --- | --- |
| 7 | PCM_RD_INTERP | Level trigger, high effective | Codec Audio data stream reading interrupt, through INTERP_CLEAR Register corresponding bit Bit write 1 Clear |

Table 51 Link layer system interrupt sub-interrupt type list

| Serial number | name | type | describe |
| --- | --- | --- | --- |
| 1 | DLL_RDY_INTERP_TX | Level trigger, high effective | Layer 2 sending processing interruption: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Via SYS_INTERP_CLEAR Register corresponding bit Bit Write 1 Clear. |
| 2 | DLL_RDY_INTERP_RX | Level trigger, high effective | Layer 2 receive processing interruption: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Via SYS_INTERP_CLEAR Register corresponding bit Bit Write 1 Clear. |
| 3 | DLL_LATELC_RX_INTERP | Level trigger, high effective | Layer 2 voice access interruption: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Through SYS_INTERP_CLEA |
| 4 | DLL_FRAME_RX_INTERP | Level trigger, high effective | Interruption of receiving various types of frame information on Layer 2: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Through SYS_INTERP_CLEA |
| 5 | DLL_SHORTLC_RX_INTERP | Level trigger, high effective | Second floor shortlc Information reception interruption: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Through SYS_INTERP_CLEA |
| 6 | DLL_TX_SLOT_ABNORMAL | Level trigger, high effective | Abnormal interruption of the Layer 2 sending timeslot: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Through SYS_INTERP_CLEA |
| 7 | DLL_RX_SLOT_ABNORMAL | Level trigger, high effective | Abnormal interruption of the Layer 2 receiving timeslot: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Through SYS_INTERP_CLEA |
| 8 | PHY_RECV_ABNORMAL | Level trigger, high effective | The physical layer receives an abnormal time slot boundary and prompts an interruption: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Through SYS_INTERP_CLEA |

#### Memory Mapping

Baseband Ram Module is HR_C7000 The chip baseband digital function's main user data interaction storage space is divided into two RAMs, send and receive, according to different functional designs . The data length is 8 and the depth is 1200 .

###### Link layer sends Ram

Link layer sends Ram The base address is 0x16000000 , the depth is 1200 , and the following table defines the content allocation of each address segment.

Table 52 Link layer sends Ram distribute

| Offset Address | name | describe |
| --- | --- | --- |
| 0x00~0x0b | PI Header | 0x00~0x09 : 80 bits in total To control information; 0x0a~0x0b : 16 bits in total To verify the information, MCU Optional. |
| 0x00~0x0b | LC Header | 0x00~0x08 : 72 bits in total For control letter; 0x09~0x0b : 24 bits in total To verify the information, MCU Optional. |
| 0x00~0x0b | Terminator | 0x00~0x08 : 72 bits in total For control letter; 0x09~0x0b : 24 bits in total To verify the information, MCU Optional. |
| 0x00~0x0b | CSBK | 0x00~0x09 : 80 bits in total To control information; 0x0a~0x0b : 16 bits in total To verify the information, MCU Optional. |
| 0x00~0x0b | MBC Header | 0x00~0x09 : 80 bits in total To control information; 0x0a~0x0b : 16 bits in total To verify the information, MCU Optional. |
| 0x00~0x0b | MBC Block | 0x00~0x0b : 96 bits in total To control information; |
| 0x00~0x0b | DataHeader | 0x00~0x09 : 80 bits in total To control information; 0x0a~0x0b : 16 bits in total To verify the information, MCU Optional. |
| 0x00~0x0b | DataRate1_2 | 0x00~0x0b : 96 bits in total For data information; |
| 0x00~0x11 | DataRate3_4 | 0x00~0x0b : 144 bits in total For data information; |
| 0x00~0x17 | DataRate1 | 0x00~0x0b : 192 bits in total For data information; |
| 0x00~0x0a | C_RC ( PDT ) | 0x00 , 0x01 Bit7-bit5 : a total of 11 bits RC information; 0x02~0x08 : 56 in total bit Control information; 0x09~0x0a : 16 in total bit Verification information; |
| 0x00~0x09 | EMB LC | 0x00~0x08 : 72 bits in total For control letter; 0x09 : bit7-bit3 5 bits in total To verify the information, MCU Optional. This information is prepared when voice frame A is about to be sent . |
| 0x29~0x2C | EMBF | Voice F Frame filling information or 0x29 Stores the superframe sequence number ( Key ID ) , 0x2A​ High 3 bits Stores the encrypted serial number (ALOG ID) |
| 0x30~0x4a | VoiceBurstA | 216 bits in total For voice frame information, only when the voice data is sent internally Ram Valid in mode |
| 0x30~0x4a | VoiceBurstB | 216 bits in total For voice frame information, only when the voice data is sent internally Ram Valid in mode |
| 0x30~0x4a | VoiceBurstC | 216 bits in total For voice frame information, only when the voice data is sent internally Ram Valid in mode |
| 0x30~0x4a | VoiceBurstD | 216 bits in total For voice frame information, only when the voice data is sent internally Ram Valid in mode |

| 0x30~0x4a | VoiceBurstE | 216 bits in total For voice frame information, only when the voice data is sent internally Ram Valid in mode |
| --- | --- | --- |
| 0x30~0x4a | VoiceBurstF | 216 bits in total For voice frame information, only when the voice data is sent internally Ram Valid in mode |
| 0x4b~0x50 | EMB RC | Data control frame embedded 48 bits RC​ Information or 0x4b , 0x4c of high 11bit RC​ Encoder input |
| 0x30~0x48 | 196 bits info | 196 bits Control Information |
| 0x495~0x4af | SendVoiceKey | A total of 216 bits , 27 bytes |

###### Link layer receives Ram

Link layer receives Ram The base address is 0x160004b0 and the depth is 1200. The following table defines the content allocation of each address segment.

Table 53 Link layer receives Ram distribute

| Offset Address | name | describe |
| --- | --- | --- |
| 0x00~0x0b | PI Header | 0x00~0x09 : 80 bits in total To control information; 0x0a~0x0b : 16 bits in total To verify the information. |
| 0x00~0x0b | LC Header | 0x00~0x08 : 72 bits in total For control letter; 0x09~0x0b : 24 bits in total To verify the information. |
| 0x00~0x0b | Terminator | 0x00~0x08 : 72 bits in total For control letter; 0x09~0x0b : 24 bits in total To verify the information. |
| 0x00~0x0b | CSBK | 0x00~0x09 : 80 bits in total To control information; 0x0a~0x0b : 16 bits in total To verify the information. |
| 0x00~0x0b | MBC Header | 0x00~0x09 : 80 bits in total To control information; 0x0a~0x0b : 16 bits in total To verify the information. |
| 0x00~0x0b | MBC Block | 0x00~0x0b : 96 bits in total To control information; |
| 0x00~0x0b | DataHeader | 0x00~0x09 : 80 bits in total To control information; 0x0a~0x0b : 16 bits in total To verify the information. |
| 0x00~0x0b | DataRate1_2 | 0x00~0x0b : 96 bits in total For data information; |
| 0x00~0x11 | DataRate3_4 | 0x00~0x11 : 144 bits in total For data information; |
| 0x00~0x17 | DataRate1 | 0x00~0x17 : 192 bits in total For data information; |
| 0x00~0x0b | Idle | 0x00~0x0b : 96 bits in total To control information; |
| 0x00~0x08 | C_RC ( PDT ) | 0x00~0x06 : 56 in total bit Control information; 0x07~0x08 : 16 in total bit Verification information; RC Decoding result 11 bits Information is stored in registers. |
| 0x1F~0x24 | EMB_48_INFO_0 | Time Slot 0 Received 48-bit EMB Regional Information |
| 0x25~0x2a | EMB_48_INFO_1 | Time Slot 1 Received 48-bit EMB Regional Information |
| 0x2b~0x2f | ShortLC | Receiver CACH Field 36 bits ShortLC Data, where 0x2F The lower 4 bits Useless data |

| 0x36~0x50 or 0x39~0x53 | 216bit_voice_info _0 | Current time slot 0 Parsing results, 216 bits of voice frame Information slot receiving mode test, 0x36~0x50 ; In continuous receive mode, 0x39~0x53 ; |
| --- | --- | --- |
| 0x66~0x80 or 0x69~0x83 | 216bit_voice_info _1 | Current time slot 1 Parsing results, 216 bits of voice frame Information time slot receiving mode test, 0x66~0x80 ; In continuous receive mode, 0x69~0x83 ; |
| 0x495~0x4af | RecvVoiceKey | A total of 216 bits , 27 bytes |

#### Link layer timeslot management

###### Overview

In the link layer mode, timeslot management mainly involves the process of establishing and synchronously updating timeslots. According to their functions, they are mainly divided into sending timeslots and receiving timeslots. At the same time, sending timeslots also include a mechanism for sending temporarily reserved timeslots.

###### Sending time slot

The establishment modes of the transmission time slot include active mode and passive mode, wherein the passive mode includes three modes: automatically establishing the transmission time slot by receiving, manually establishing the transmission time slot by receiving, and establishing the transmission time slot by sending a temporary time slot.

The synchronization process of the sending time slot includes two processes: sending synchronization to other time slots and the sending time slot being synchronized.

##### Send time slot active establishment mode

The steps to implement the active establishment mode of sending time slots are as follows:

1. Configure the sending work enable txen and the sending time slot establishment mode, configure the register LAYER2_CONTROL , and the reference value is 0xd2 ; ( By default, rxen and temporary time slot establishment active mode are enabled)

2. Configure the initial value and initial number of the time slot count, and configure the register LAYER2_SLOT_CNT ;

3. Configure the transmit time slot start enable switch and configure the register LAYER2_SLOTON Bit 0 is 1 ;

4. After the time slot is enabled, the time slot counter automatically counts from 0 to 287. Cycle count, count up to 287 30ms later Send time slot interrupt, the time slot count value can be read through the register LAYER2_STATUS ;

5. Generate an independent transmit time slot interrupt through register INTERP_CLEAR Clear 30ms Transmit slot interrupt.

1  2  3  4  5  6  7  8  9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 43 44 45 4

sys_clk

i_syn_txen

slot_cnt_set

i_txslot_cnt_wr_en

o_dll_tx_bit_cnt

287

287

287

dll_tx_slot_on i_en_bit o_dll_trans_slot_c

o_dll_tx_slot_choose

6

Figure 73 Sending time slot actively establishes timing diagram

TimeGen

##### Send time slot receive setup mode

The steps to implement the mode of establishing the sending time slot by the receiving time slot are as follows:

1. Configure the transmit enable txen and transmit time slot establishment mode, configure the register LAYER2_CONTROL , reference value 0xd0 Indicates that the receive time slot needs to be manually established after it is generated. 0xd4 Indicates that the receive time slot is automatically established after it is generated ; ( By default, rxen and temporary time slot establishment active mode are enabled )

2. Configure the transmit time slot start enable switch and configure the register LAYER2_SLOTON Bit 0 is 1 ;

3. If it is set to automatic establishment mode, the sending time slot will be automatically established when the receiving time slot interrupt occurs;

4. If set to manual establishment mode, you need to detect the receiving time slot 30ms first After the interrupt, the configuration register

LAYER2_SLOTON 0x21​ After establishing and sending 30ms time slot;

5. After the time slot is enabled, the time slot counter automatically counts from 0 to 287. Cycle count, count up to 287 30ms later Send time slot interrupt, the time slot count value can be read through the register LAYER2_STATUS ;

6. Generate an independent transmit time slot interrupt through register INTERP_CLEAR Clear 30ms The transmit slot is interrupted.

1  2  3  4  5  6  7  8  9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 43 44 45 46

sys_clk i_syn_txen dll_rx_slot_on rx_slot_choose_l2 recv_slot_c

o_dll_tx_bit_cnt

0

287

287

287

dll_tx_slot_on i_en_bit o_dll_trans_slot_c

o_dll_tx_slot_choose

TimeGen

Figure 74 Sending time slot receiving time slot establishment timing diagram

##### Send time slot temporary establishment mode

The steps to implement the sending time slot by temporary time slot establishment mode are as follows:

1. Configure the sending work enable txen and the sending time slot establishment mode, configure the register LAYER2_CONTROL , and the reference value is 0xd 0 ; ( By default, rxen and temporary time slot establishment active mode are enabled )

2. Configure the transmit time slot start enable switch and configure the register LAYER2_SLOTON Bit 0 is 1 ;

3. It is necessary to detect the temporary time slot of 30ms first After it is turned on, configure the register LAYER2_SLOTON to 0x843 and then establish a 30ms time slot for sending (assuming the temporary time slot has been established ) ;

4. After the time slot is enabled, the time slot counter automatically counts from 0 to 287. Cycle count, count up to 287 30ms later Send time slot interrupt, the time slot count value can be read through the register LAYER2_STATUS ;

5. Generate an independent transmit time slot interrupt through register INTERP_CLEAR Clear 30ms The transmit slot is interrupted.

1  2  3  4  5  6  7  8  9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 43 44 45 4

sys_clk

i_syn_txen

dll_tx_slot_on_tmp

tx_slot_choose_tmp

trans_slot_c_tmp

o_dll_tx_bit_cnt

0

287

287

287

dll_tx_slot_on

i_en_bit

o_dll_trans_slot_c

o_dll_tx_slot_choose

6

Figure 75 Transmitting time slot temporary time slot establishment timing diagram

TimeGen

###### Send temporary time slot

The establishment modes of the sending temporary time slot include active mode and passive mode, wherein the passive mode includes two modes: establishing the sending temporary time slot by receiving time slot and establishing the sending temporary time slot by sending time slot.

The synchronization process of sending a temporary time slot includes two processes: sending the temporary synchronization to other time slots and sending the synchronized temporary time slot.

##### Temporary time slot active establishment mode

The steps to implement the temporary time slot active establishment mode are as follows:

1. Configure the temporary time slot establishment mode, configure the register LAYER2_CONTROL , and set the reference value to 0xd2 ;

2. Configure the initial value and initial number of the time slot count, and configure the register LAYER2_SLOT_CNT ;

3. Configure the temporary time slot start enable switch and configure the register LAYER2_SLOTON Bit 0 is 2 ;

4. After the temporary time slot is enabled, the temporary time slot counter automatically increases from 0 to 287. The cycle count and temporary time slot count value can be read through the register LAYER2_STATUS ;

##### Temporary timeslot reception establishment mode

The steps to implement the temporary timeslot reception establishment mode are as follows:

1. Configure the temporary time slot establishment mode, configure the register LAYER2_CONTROL , reference value 0xc2 ;

2. Configure the temporary time slot start enable switch and configure the register LAYER2_SLOTON Bit 1 is 2 ;

3. Configure the receive to establish a temporary time slot start switch, configure the register LAYER2_SLOTON Bit12​ is 1 ;

4. After the temporary time slot is started, the temporary time slot counter automatically increases from 0 to 287. The cycle count and temporary time slot count value can be read through the register LAYER2_STATUS ;

##### Temporary timeslot transmission establishment mode

The steps to implement the temporary timeslot transmission establishment mode are as follows:

1. Configure the temporary time slot establishment mode, configure the register LAYER2_CONTROL , reference value 0xc2 ;

2. Configure the temporary time slot start enable switch and configure the register LAYER2_SLOTON Bit 1 is 2 ;

3. Configure the send to establish a temporary time slot start switch, configure the register LAYER2_SLOTON Bit7​ is 1 ;

4. After the temporary time slot is started, the temporary time slot counter automatically increases from 0 to 287. The cycle count and temporary time slot count value can be read through the register LAYER2_STATUS ;

###### Receive time slot

is a process of automatically establishing after the physical layer receives the synchronization timeslot .

##### Receive time slot active establishment mode

The steps to implement the receive timeslot active establishment mode are as follows:

1. Configure the receiving work enable rxen and receive time slot establishment mode, configure the register LAYER2_CONTROL , reference value 0xd 3 ; ( by default, txen and temporary time slot establishment active mode are enabled )

2. Configure the initial value and initial number of the time slot count, and configure the register LAYER2_SLOT_CNT ;

3. Configure the receive time slot start enable switch and configure the register LAYER2_SLOTON Bit2​ is 1 ;

4. After the time slot is enabled, the time slot counter automatically counts from 0 to 287. Cycle count, count up to 287 30ms later Receive time slot interrupt, the time slot count value can be read through register LAYER2_STATUS ;

5. Generate an independent receive time slot interrupt through register INTERP_CLEAR Clear 30ms Receive slot interrupt.

##### Receive slot passive establishment mode

The steps to implement the passive establishment mode of the receive timeslot are as follows:

1. Configure the receiving work enable rxen and receive time slot establishment mode, configure the register LAYER2_CONTROL , reference value 0xd 2 ; ( by default, txen and temporary time slot establishment active mode are enabled )

2. The physical layer automatically starts the receive timeslot counter after receiving the timeslot boundary;

3. After the time slot is started, the time slot counter automatically counts from 0 to 287. Cycle count, count up to 287 30ms later Receive time slot interrupt, the time slot count value can be read through register LAYER2_STATUS ;

4. Generate an independent receive time slot interrupt through register INTERP_CLEAR Clear 30ms Receive time slot interrupt;

5. The time slot boundary automatically follows the time slot boundary most recently detected by the physical layer.

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 276 |  |  |  |  |  |  |  | 287 |  |  |  |  |  |  |  |  |  |  |  |  |  | 287 |  |  |  |  |  |  |  |  |  |  |  |  |  | 287 |  |  |  |  |  |  |  |  |  |  |  |  |  | 287 |  |  |  |  |  |  |  |  |  |

1	2	3	4	5	6	7	8	9  10  11  12  13  14  15  16  17  18  19  20  21  22  23  24  25  26  27  28  29  30

sys_clk i_syn_rxen i_recv_time_slot_l

i_recv_time_slot_c

60

recv_time_slot_en o_dll_rx_slot_on o_dll_rx_bit_cnt o_dll_recv_slot_c

o_dll_rx_slot_choose

Figure 76 Passive establishment timing diagram of receiving time slot

TimeGen

31

###### Receive timeslot synchronization strategy

The time slot synchronization strategy is mainly aimed at the synchronization between the sending time slot, temporary time slot and receiving time slot. It mainly includes the following strategies:

1. The sending time slot is synchronously updated to the receiving time slot;

2. The receiving time slot is synchronously updated to the sending time slot;

3. The sending time slot is synchronously updated to the sending temporary time slot;

4. Send temporary time slot and update it to the sending time slot synchronously;

5. The receiving time slot is synchronously updated to the sending temporary time slot;

##### Transmit and receive time slots are synchronized

The synchronization strategies between the sending time slot and the receiving time slot mainly include synchronizing the sending time slot to the receiving time slot and synchronizing the receiving time slot to the sending time slot.

The transmit time slot is synchronized to the receive time slot by configuring the register LAYER2_SLOT_UPDATE 0x1​ The corresponding reference timing diagram is shown below. The specific implementation is to send 30ms The time slot boundary generates a synchronization pulse to the receiving time slot. The receiving time slot confirms whether the current synchronization is updated to the sending time slot boundary based on the synchronization requirements and the received time slot count value. The count value range is [276,287] or [0,11] , the receiving time slot is automatically updated to the sent time slot count value 0 , otherwise the receiving time slot will give a time slot boundary abnormality prompt interrupt.

发送时隙30ms边界同步到接收时隙边界

发送同步到收时序（接收时隙早于发送时隙1.2ms之内）

Sending time slot

Receive time slot

发送时隙30ms边界同步到接收时隙边界

发送同步到收时序（接收时隙晚于发送时隙1.2ms之内）

Figure 77 Timing diagram of transmitting time slot synchronization to receiving time slot

The receive time slot is synchronized to the transmit time slot by configuring the register LAYER2_SLOT_UPDATE 0x2​ The corresponding reference timing diagram is shown in the figure below. The specific implementation is to receive 30ms The time slot boundary generates a synchronization pulse to the sending time slot. The sending time slot confirms whether the current synchronization is updated to the receiving time slot boundary based on the synchronization requirements and the sent time slot count value. The count value range is [276,287] or [0,11] , the sending time slot is automatically updated to the received time slot count value 0 , otherwise the sending time slot will give a time slot boundary abnormality prompt interrupt.

接收时隙30ms边界同步到发送时隙边界

接收同步到发送时隙时序（接收时隙早于发送时隙1.2ms之内）

Sending time slot

Receive time slot

接收时隙30ms边界同步到发送时隙边界

接收同步到发送时隙时序（接收时隙晚于发送时隙1.2ms之内）

Figure 78 Timing diagram of receiving time slot synchronization to transmitting time slot

##### Transmit time slot and transmit temporary time slot synchronization

The synchronization strategies between the sending time slot and the receiving time slot mainly include synchronizing the sending time slot to the sending temporary time slot and synchronizing the sending temporary time slot to the sending time slot.

The sending time slot is synchronously updated to the sending temporary time slot, and the time slot number is automatically updated:

By configuring the register LAYER2_SLOT_UPDATE 0x404​ The corresponding reference timing diagram is shown below. Specifically, the synchronization pulse is generated by sending a 30ms time slot boundary to the temporary time slot. The temporary time slot count is directly updated to the transmitted time slot count value of 0. The condition for the temporary time slot's 30ms boundary update pulse is that the count value is greater than 257 when the update pulse is sent . The temporary time slot number is automatically updated according to the settings.

The sending time slot is synchronously updated to the sending temporary time slot, and the time slot number is not updated;

by configuring the LAYER2_SLOT_UPDATE register to 0x4 . The corresponding reference timing diagram is shown below. Specifically, a 30ms slot boundary is sent to generate a synchronization pulse to the temporary slot. The temporary slot count is directly updated to the transmitted slot count value of 0. The condition for the temporary slot's 30ms boundary update pulse is that the count value is greater than 257 when the update pulse is sent . The temporary slot number is not updated according to the setting.

Send time slot number

30ms boundary of the sending time slot is synchronized to the boundary of the sending temporary time slot

Sending time slot

Send temporary time slot

Send temporary time slot number

Send synchronization to the temporary timing (send temporary slot is earlier than send slot, update label )

Send time slot number

30ms boundary of the sending time slot is synchronized to the boundary of the sending temporary time slot

Sending time slot

Send temporary time slot

Send temporary time slot number

Send synchronization to the temporary timing of sending (the temporary timing of sending is earlier than the sending timing, and the label is not updated )

Send time slot number

30ms boundary of the sending time slot is synchronized to the boundary of the sending temporary time slot

Sending time slot

Send temporary time slot

Send temporary time slot number

Send synchronization to the temporary timing (send temporary slot is later than send slot, update label )

Send time slot number

30ms boundary of the sending time slot is synchronized to the boundary of the sending temporary time slot

Sending time slot

Send temporary time slot

Send temporary time slot number

Send synchronization to the temporary timing of sending (the temporary timing of sending is later than the sending timing, and the label is not updated )

Figure 79 Timing diagram of sending time slot synchronization to sending temporary time slot

##### Send temporary time slot and receive time slot synchronization

Send temporary time slot synchronously update to the sending time slot, automatically update the time slot number: by configuring the register LAYER2_SLOT_UPDATE 0x808​ The corresponding reference timing diagram is shown in the figure below. The specific implementation is through a temporary time slot of 30ms The time slot boundary generates a synchronization pulse to the transmit time slot, and the transmit time slot count value is directly updated to the temporary time slot count value 0. The 30ms boundary update pulse condition for the transmit time slot is that its count value is greater than 257 when the temporary update pulse arrives . The transmit time slot number is automatically updated according to the setting.

Send temporary time slot synchronously update to the send time slot, do not update the time slot number: by configuring register LAYER2_SLOT_UPDATE 0x8​ The corresponding reference timing diagram is shown in the figure below. The specific implementation is through a temporary time slot of 30ms The time slot boundary generates a synchronization pulse to the sending time slot, and the sending time slot count value is directly updated to the temporary time slot count value 0. The 30ms of the sending time slot The boundary update pulse condition is that when the temporary update pulse arrives, its count value is greater than 257 , and the time slot number is sent without updating the time slot number.

Receive time slot number

30ms boundary of the receiving time slot is synchronized to the boundary of the sending temporary time slot

Receive time slot

Send temporary time slot

Send temporary time slot number

Receiving is synchronized to sending temporary timing (sending temporary slot is earlier than receiving slot, update label )

Receive time slot number

30ms boundary of the receiving time slot is synchronized to the boundary of the sending temporary time slot

Receive time slot

Send temporary time slot

Send temporary time slot number

Receiving is synchronized to sending temporary timing (sending temporary slot is earlier than receiving slot, and label is not updated )

Receive time slot number

30ms boundary of the receiving time slot is synchronized to the boundary of the sending temporary time slot

Receive time slot

Send temporary time slot

Send temporary time slot number

Receiving is synchronized to sending temporary timing (sending temporary slot is later than receiving slot, update label )

Receive time slot number

30ms boundary of the receiving time slot is synchronized to the boundary of the sending temporary time slot

Receive time slot

Send temporary time slot

Send temporary time slot number

Receiving is synchronized to sending temporary timing (sending temporary slot is later than receiving slot, so the label is not updated )

Figure 80 Timing diagram of receiving time slot synchronization to sending temporary time slot

#### Link layer transmission

###### Overview

Link layer transmission supports DMR Standard transmission of various types of frames, including voice frames, data frames and RC There are three types of frames.

###### Frame Type Description

##### Speech frame

| Voice(108) | SYNC(48) | Voice(108) |
| --- | --- | --- |

27.5ms

CC

PI  LCSS

Voice(108)

Voice(108)

Embeded signalling (32)

Figure 81 Voice frame with synchronous frame structure

27.5ms

EMB(8)

EMB(8)

Figure 82 Voice frame with EMB Data Structure

LC according to the superframe rule Or Null , build A(SYNC) , B(LC) , C(LC) , D(LC) , E(LC) , F(Null) superframes according to the standard, including:

- Support synchronization frame opt-in;

- Support EMB 7bit Join EMB Perform QR(16,7,6) encoding;

- Support LC 72bit Join, join 5bit CS Code, variable length BPTC Encoded, interleaved, and added to 4

timeslots (128 bits) ;

Table 54 Group call 72bit LC Information Table

| Information element | Length | Remark |
| --- | --- | --- |
| Protect Flag (PF) | 1 |  |
| Reserved | 1 | 0 |
| Full Link Control Opcode (FLCO) | 6 | 000000 |
| Feature set ID (FID) | 8 | 00000000 |
| Service Option | 8 |  |

| Group address | twenty four |  |
| --- | --- | --- |
| Source address | twenty four |  |

- Null support Time slot opt-in;

DataType FEC Parity

FEC Parity

CC

Info(98)

SYNC(48)

Info(98)

Data Control Frame

Reset_Info(82)

CC		DataType FEC Parity

FEC Parity

SYNC(48)

Reset_Info(82)

SlotType(10)

SlotType(10)

Figure 83 Data frame and control frame structure

RC(16)

SlotType(10)

SlotType(10)

RC(16)

Figure 84 Control Channel C_RC Frame structure (PDT)

The data frame structure mainly consists of three parts: control or data information, synchronization frame and slot Type Information, of which the control or data information part supports four coding methods ( Rate1 code , BPTC (196,96) , Rate3_4 Trellis code , BPTC ( 164 , 72 ) ) , S l ot Type information is provided by CC , Data Type Obtained by Golay ( 20,8 ) encoding.

1. PI Frame​

a ) Support voice PI Header , supports adding 80 bits PI , CRC16bits check and add CRC mask to perform

BPTC (196 , 96) encoding and interleaving; b ) Support SYNC joining;

c ) Supports adding Slot Type (20 bits) , including CC and DataType , and performing Golay (20 , 8) encoding; 2. LC Frame

- Support added 72bit LC , CRC24bit Check and add CRC Mask (Header and Terminator Difference ) , perform BPTC (196 , 96) encoding, and form voice Head and Terminator frames;

- SYNC support join in;

- Support adding Slot Type (20 bits) , including CC and DataType , is encoded using Golay (20 , 8) .

- Support 72bitLC Information structure 128 bits EMB Embedding information, used to embed into speech EMB area;

- Support LC Frame Information is dynamically updated and embedded in EMB area;

3. CSBK​ Frame

- Support for adding 80bits CSBK , CRC16 bits Check and add CRC Mask, perform BPTC (196 , 96) encoding and interleaving;

- SYNC support join in;

- Supports adding Slot Type (20bit) , including CC , DataType , and Golay (20 , 8) encoding ; 4. MBC Header Frame

- Support for adding 80bits MBC Header , CRC16 bits Check and add CRC Mask, perform BPTC (196 , 96) encoding and interleaving;

- SYNC support join in;

- Supports adding Slot Type (20bit) , including CC , DataType , and Golay (20 , 8) encoding ; 5. MBC Block Frame

- Support for adding 96bits MBC Block , BPTC (196 , 96) encoding and interleaving;

- Support 80bitMBC lastBlock , perform CRC16bit Check, perform BPTC (196 , 96) encoding and interleaving ;

- SYNC support join in;

- Supports adding Slot Type (20bit) , including CC , DataType , and Golay (20 , 8) encoding ; 6. Data Header Frame

- Support for adding 80bits Data Header , CRC16 bits Check and add CRC Mask, perform BPTC (196 , 96) encoding and interleaving;

- SYNC support join in;

- Supports adding Slot Type (20bit) , including CC , DataType , and Golay (20 , 8) encoding; 7 , Data Rate 1/2 Frame

- Support for adding 96bits Data Rate 1/2 , BPTC (196 , 96) encoding and interleaving; b ) Support SYNC joining;

c ) Supports Slot Type (20bit) , including CC , DataType , and Golay (20 , 8) encoding; 8 , Data Rate 3/4 Frame

a ) Support for 144 bits Data Rate 1/2 , Trelis (196 , 144) encoding and interleaving; b ) Support SYNC joining;

c ) Supports adding Slot Type (20bit) , including CC , DataType , and Golay (20 , 8) encoding ; 9. Idle Frame

a ) Support for 96bits Idle Frame , BPTC (196 , 96) encoding and interleaving; b ) Support SYNC joining;

c ) Supports adding Slot Type (20bit) , including CC , DataType , and Golay (20 , 8) encoding; 10 , Data Rate 1 Frame

a ) Support for 144 bits Data Rate 1. Perform Trelis (196 , 192) encoding and interleaving; b ) Support SYNC joining;

c ) Supports adding Slot Type (20bit) , including CC , DataType , and Golay (20 , 8) encoding ; 11. Reserved Frame

- Support Reserved Frame Four encoding methods are available;

- SYNC support join in;

- Support adding Slot Type (20 bits) , including CC and DataType , is encoded using Golay (20 , 8) .

      - RC frame

RC There are two types of frames: independent RC Control frames and embedded RC frame.

- Supports 7-bit EMB and QR (16,7,6) encoding;

- Support 11bit RC Signal, variable length BPTC , 32bit Interweave and join RC unit;

| CC | PI | LCSS |  |
| --- | --- | --- | --- |
| RC Info + FEC Parity |  |  |  |
| RC Info + FEC Parity |  |  |  |
| SYNC |  |  |  |
| SYNC |  |  |  |
| SYNC |  |  |  |
| SYNC |  |  |  |
| SYNC |  |  |  |
| SYNC |  |  |  |
| RC Info + FEC Parity |  |  |  |
| RC Info + FEC Parity |  |  |  |
| EMB Parity |  |  |  |

SYNC(48)

10ms

30ms

EMB(8)

RC(16)

RC(16)

EMB(8)

Figure 85 Independent RC Frame structure

###### Framing mode Continuous mode :

- Support voice superframe framing, LC Header and PI Header can be set or just the PI Header mode, ends the automatic formation of superframes and joins the LC Terminator ;

- Support data 4.8kbps Frame, add data Header , Data, and Data Terminator ;

- Support data 9.6kbps Frame, add data Header , Data, and Data Terminator ; Time slot mode:

- Support voice superframe framing, LC Header and PI Header can be set or just the PI Header mode, ends the automatic formation of superframes and joins the LC Terminator ;

- 4.8kbps for various data types Frame, add data Header , Data, and LC Terminator ; c)

Data Sync 	Voice Sync

Null

Data Sync

Header

A 	B 	C 	D 	E 	F

Terminator

Figure 86 Time slot speech timing

Data Sync 	Voice Sync

Null

Data Sync

Header

A 	B 	C 	D 	E 	F

Terminator

Figure 87 Continuous speech timing

Hdr

Data

Block

Data

Block

|  | Data Block |  | Last Block |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

Figure 88 Single slot data timing

Hdr		Data Block

| Data Block | Data Block | Data Block | Data Block | Data Block | Data Block | Last Block |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure 89 Dual-slot data timing

###### Frame type definition

The following table defines the list of frames supported for transmission in link layer mode, including voice superframes and frames defining various data types. The frame type transmission is configured by the register LAYER2_SEND_TYPE .

Table 55 List of link layer transmission frame types

| Timeslot frame type | LocalDataType | voice | Remark |
| --- | --- | --- | --- |
| Voice LC Header | 0001 | 0 |  |
| Voice PI Header | 0000 | 0 |  |
| Voice A | 0000 | 1 |  |
| Voice B | 0001 | 1 |  |
| Voice C | 0010 | 1 |  |
| Voice D | 0011 | 1 |  |
| Voice E | 0100 | 1 |  |
| Voice F | 0101 | 1 |  |
| RC frame | 0110 | 1 |  |
| Voice Terminator | 0010 | 0 |  |
| CSBK | 0011 | 0 |  |
| MBC Header | 0100 | 0 |  |
| MBC Intermedia | 0101 | 0 |  |
| MBC Last | 0101 | 0 |  |
| DataHeader | 0110 | 0 |  |
| DataRate1_2 | 0111 | 0 |  |
| DataRate3_4 | 1000 | 0 |  |
| DataRate1 | 1010 | 0 |  |

| Idle | 1001 | 0 | BS The data type of the IDLE frame type sent can be configured |
| --- | --- | --- | --- |
| Reserved | 1011 | 0 |  |
| Reserved | 1100 | 0 |  |
| Reserved | 1011 | 0 |  |
| Reserved | 1110 | 0 |  |
| Reserved | 1111 | 0 |  |

###### Sending Process

Transmit timeslot interrupt

二层发送处理中断

| 准备使和内容 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Sending energy

发送时隙

Figure 90 Link layer transmission timing diagram 1

The link layer sending steps are as follows:

1. Establish a transmit timeslot, generate a transmit timeslot interrupt and a Layer 2 transmit processing interrupt, refer to Section 10.5 ;

2. In the second layer of the transmission, the interrupt time slot switch is processed and the next time slot is ready to send enable through register

LAYER2_TXRX_CTRL Control implementation;

3. When the next time slot is enabled, configure the frame type and data content to be sent. The frame type is set by register LAYER2_SEND_TYPE Set up, send content through the second layer to send Ram Fill in the corresponding frame type address ( see section 10.4.1 for details ) .

The above steps refer to the Layer 2 transmission timing diagram above, using the Layer 2 transmission interrupt to prepare the content for transmission. Alternatively, the content transmission preparation can also be processed in the time slot interrupt as shown in the following figure.

发送准备使能和内容

发送时隙

Figure 91 Link layer transmission timing diagram 2

#### Link layer reception

###### Overview

The link layer supports sending and receiving all types of frames.

###### Functional Description

Receive system interrupts primarily consist of two types: deframing interrupts and post-access interrupts. Deframing interrupts are used for receiving and processing various data control frames and voice frames, while post-access interrupts are used for delayed voice access or the transmission of voice encryption key information. The following figure shows the receive interrupt status processing flow.

Idle

System outage detected

Deframing interruption

①

Voice access interruption

SYS_INTERP_LI ST==0x8

②

SYS_INTERP_LI ST==0x4

②

SYNC_TYPE==0 	SYNC_TYPE==1 	SYNC_TYPE==2

Cond1

③  Yes

Get LC Info

Cond2 	Cond2 	Cond1

Get Voice Info

③ Yes

③ Yes

Yes

Get Voice Info

Data Frame Rx Process

Yes

PI==0

Cond4

No

Cond2

① Read register SYS_INTERP_LIST to obtain interrupt type, write register

SYS_INTERP_CLEAR clears the interrupt

②Read register RX_TYPE_INFO to obtain receiving information

Cond3

No

Cond1

No

Yes

RX

VoiceA

Yes

No

Cond2

Yes

LC Update

③Read the receiving Ram address and obtain the de-framing information

Yes

VoiceRx Abnormal

RX Voice BCDEF

LC Update

Yes

Set Voice RxFlag

LC Update

Cond1: CRC/RS/CS check OK , CC match Cond2: Whether to start voice reception

Cond3: Whether the number of voice reception frame errors exceeds 1 superframe Cond4: Whether the address filter is relayed

Data Frame Rx Process

Yes

Cond8:数据包是否为最后一包

DATA_TYPE==0  DATA_TYPE==1  DATA_TYPE==2  DATA_TYPE==3  DATA_TYPE==4  DATA_TYPE==5  DATA_TYPE==6  DATA_TYPE==7  DATA_TYPE==8  DATA_TYPE==a  DATA_TYPE==

Reserved

③	③	③	③	③	③	③	③	③	③

No

No

No

Cond2

Cond4

Cond4

Cond4

Cond4

Cond4

No

Cond4

Cond6

No

No

Cond6

Cond6

Yes

Yes

Yes

Yes

Yes

Yes

Yes

Yes

Yes

Yes

RX	RX	RX

PI Header	CSBK	MBC Block

RX

Data Block

RX

Data Block

RX

Data Block

RX

LC Header

RX

LC Term

RX

MBC Header

No

Cond5

RX

Data Header

Cond7

No

Cond7

Cond7

No

Yes

Clear MBC RxFlag

Yes

Clear Data RxFlag

Yes

Clear Data RxFlag

Yes

Clear Data RxFlag

Data Frame Rx End

Set Data RxFlag、Num

Clear Voice RxFlag

Set Voice RxFlag

Get Data Block Info

Get Data Block Info

Get Data Block Info

Get Data Header Info

Get MBC Block Info

Get MBC Header Info

Get CSBK Info

Get LC Terminator Info

Get LC Header Info

Get LC Header Info

Cond5: Whether the MBC packet is started to be received Cond6: Whether the MBC packet is the last packet Cond7: Whether the data packet is started to be received

| Set MBC RxFlag 、 Num |  |
| --- | --- |
|  | Yes |

Figure 92 Link layer receiving state flow chart

###### Receiving Application

Please refer to the appendix for detailed application function examples.

## FM application

#### Overview

HR_C7000 FM​ Functions include analog sub-tone CTCSS , digital sub-tone CDCSS , dual-tone multi-frequency DTMF And signaling SELCALL , SQL In each mode, the standard mode is supported by default, and user-defined configurable modes are reserved to meet differentiated application needs.

#### register

###### Register Overview

Register Table 29 FM Register Overview ( Base Address: 0x1100_0000 )

| Offset address | name | describe | page number |
| --- | --- | --- | --- |
| 0x100 | WORK_MODE | Baseband working mode setting, set to FM model |  |
| 0x500 | FM_BANDWIDTH | FM Wide and narrow band settings |  |
| 0x504 | FM_DEV_COEF | FM Audio transmission limiting, and audio modulation frequency deviation coefficient |  |
| 0x508 | FM_SQL | Read-only, squelch output value in SQL mode |  |
| 0x50C | SUBVOICE_COEF | Sub-tone modulation frequency deviation, reception depth and reception threshold |  |
| 0x510 | FM_MOD | FM Working mode settings |  |
| 0x518 | CTCSS_ADDR | CTCSS Sub-audio sending and receiving addresses |  |
| 0x51C | USER_DEFINE_CTC | User-defined CTCSS Sending frequency |  |
| 0x520 | CDCSS_ADDR | CDCSS Sending and receiving addresses |  |
| 0x524 | SELCALL_TONE | Signaling sending frequency |  |
| 0x528 | TONE_SEND_CTRL | Signaling /DTMF Send Control |  |
| 0x530 | FM_FUNCTION_COEF1 | Receive demodulation coefficient 1 |  |
| 0x534 | FM_FUNCTION_COEF2 | Receive demodulation coefficient 2 |  |
| 0x538 | FM_FUNCTION_COEF3 | Receive demodulation coefficient 3 |  |
| 0x53C | FM_FUNCTION_COEF4 | Receive demodulation coefficient 4 |  |
| 0x540 | FM_FUNCTION_COEF5 | Receive demodulation coefficient 5 |  |
| 0x544 | FM_FUNCTION_COEF6 | Receive demodulation coefficient 6 |  |
| 0x548 | FM_FUNCTION_COEF7 | Receive demodulation coefficient 7 |  |
| 0x54C | FM_FUNCTION_COEF8 | Receive demodulation coefficient 8 |  |
| 0x550 | FM_TONE1 | DTMF Send frequency tone_1 and tone_2 |  |
| 0x554 | FM_TONE2 | DTMF Send frequency tone_3 and tone_4 |  |
| 0x558 | FM_TONE3 | DTMF Send frequency tone_5 and tone_6 |  |
| 0x55C | FM_TONE4 | DTMF Send frequency tone_7 and tone_8 |  |
| 0x560 | FM_PTT | FM Send PTT |  |
| 0x564 | FM_DECODE_FLAG1 | Signaling decoding output codeword |  |
| 0x568 | FM_DECODE_FLAG2 | DTMF Decode output code element, CDCSS , CTCSS Detection To the sign |  |
| 0x56C | FM_ADDR_SW | FM Address space selection control for sending and receiving audio data |  |

###### Register Description

      - WORK_MODE

Offset address: 0x100 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:9] | - | reserved |  |  |
| [8] | RW | digital_analog_en | 0x0 | The digital and analog simultaneous detection switch is required to be turned on in the default configuration digital mode. After receiving a digital signal or an analog signal, it switches to the digital mode or analog mode accordingly. 1 : Indicates that the simultaneous digital and analog reception control is enabled; 0 : Indicates that only digital or analog reception is enabled. According to Bit7 setting. |
| [7] | RW | modulator_mode | 0x0 | Working mode settings: 0 : Indicates DMR digital mode; 1 : Indicates FM analog mode. |
| [6] | RW | is_tier1_mode | 0x1 | 0 : indicates Tier I mode; 1 : Indicates Tier II mode. |
| [5] | RW | is_continuemode | 0x1 | 0 : Indicates Continue mode; 1 : Indicates TimeSlot mode. |
| [4:3] | RW | layermode | 0x1 | Hierarchy mode settings: 0 : indicates physical layer mode; 1 : indicates Layer 2 mode; 2 : Indicates three-layer mode. |
| [2] | RW | is_repeater | 0x0 | 0 : indicates non-relay mode; 1 : Indicates relay mode. |
| [1] | RW | is_aligned | 0x1 | 0 : indicates offset mode (offset in non-relay mode indicates single-frequency mode); 1 : Indicates alignment mode. |
| [0] | RW | work_slot_sel | 0x1 | The Layer 3 non-relay mode must be set to 1 ; 0 : indicates time slot 1 ; 1 : Indicates time slot 2 . |

      - FM_BANDWIDTH

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | - | reserved |  |  |
| [15:12] | - | reserved |  |  |
| [11:8] | - | reserved |  |  |
| [7] | - | reserved |  |  |
| [6] | RW | fmbpfon | 0x1 | FM带通滤波器开关：0表示关闭，1表示开启。 |

Offset address: 0x500 Reset mode: H

| [5] | - | reserved |  |  |
| --- | --- | --- | --- | --- |
| [4] | RW | fmpreemphasison | 0x1 | FM pre-emphasis switch: 0 means off, 1 means on. |
| [3] | RW | fmbandwidth | 0x1 | FM transmission bandwidth selection: 0 means 12.5K , 1 means 25K . |
| [2] | RW | fmbandwidth_adj | 0x1 | FM In this mode, when baseband IQ and IF, IF IQ are used for transmission, the bandwidth of the adjacent channel suppression filter is selected. 0 means 12.5KHz bandwidth, 1 means Indicates 25KHz bandwidth. |
| [1] | RW | fmbandwidth_r | 0x1 | FM receiving bandwidth selection: 0 means 12.5K , 1 means 25K . |
| [0] | RW | fmbandwidth_ctc | 0x1 | Sub-tone CTCSS receiving bandwidth selection: 0 means 12.5K , 1 means 25K . |

      - FM_DEV_COEF

Offset address: 0x504 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:16] | RW | tr_sig_lim | 0x0f | FM transmitter amplitude limiting modulation coefficient |
| [15:8] | RW | fm_dev_coef_r | 0x10 | FM receiving end modulation frequency deviation coefficient |
| [7:0] | RW | fm_dev_coef_t | 0x10 | FM transmitter modulation frequency deviation coefficient |

      - FM_SQL

Offset address: 0x508 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | - | reserved |  |  |
| [15:0] | RO | fm_sql | 0x0 | FM In this mode, the calculated value of the squelch signal |

      - SUBVOICE_COEF

Offset address: 0x50c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:28] | - | reserved |  |  |
| [27:16] | RW | sample_size | 0x7d0 | FM each functional module receives the sampling depth |
| [15:8] | RW | subvoice_dev_coef | 0x28 | Sub-audio frequency modulation deviation |
| [7:0] | RW | freq_amp_limit | 0x10 | FM each functional module receives the frequency energy demodulation threshold |

      - FM_MOD

Offset address: 0x510 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:16] | RW | inter_mask_limit | 0xa | Repeat symbol decision threshold. |

|  |  |  |  | - When the current code element is the same as the last detected code element and the update time interval is greater than the threshold, it means that the two are independent code element information, and the system is allowed to interrupt the output; - On the contrary, if the time interval is less than the threshold, the two will be regarded as the same code element and the system interruption will be masked. - If the user does not want to distinguish whether it is a continuous code element and outputs all decoded codes, the threshold can be set to 0. - The threshold step length is consistent with the DTMF decoding time (related to the sampling depth sample_size ). For example, when the threshold is set to 2 , the same code element update time interval is less than or equal to 2 times the unit detection time, which is considered The same code element. |
| --- | --- | --- | --- | --- |
| [15:14] | RW | xtcss_length | 0x0 | XTCSS Send signaling length : 00:40ms ; 01:60ms ; 10:80ms ; 11:100ms ; |
| [13:12] | RW | ctcss_phase_revers e | 0x1 | CTCSS tail tone flip mode: 00 : Tail sound squelch function off; 01 : Tail sound phase flip 240 °; 10 : Tail sound phase flip 180 °; 11 : reserved |
| [11] | RW | phase_f_en_t | 0x0 | CDCSS sends the positive and negative phase, 0: positive; 1 : negative |
| [10] | RW | phase_f_en_r | 0x0 | CDCSS receiving phase positive and negative, 0: positive; 1 : negative |
| [9] | RW | mude_code_en_in | 0x1 | CDCSS Enable sending tail tone close code, 0 is off, 1 is on. |
| [8] | RW | eom_en | 0x0 | Send XTCSS End message switch, 0 is off, 1 is on. |
| [7:5] | - | reserved |  |  |
| [4] | RW | selcall_tone_mode | 0x0 | Selcall mode switch configuration, high effective |
| [3] | RW | ctcss_mode | 0x0 | CTCSS mode switch configuration, high effective |
| [2] | RW | cdcss_mode | 0x0 | CDCSS mode switch configuration, high effective |
| [1] | RW | dtmf_mode | 0x0 | DTMF mode switch configuration, high effective |
| [0] | RW | xtcss_mode | 0x0 | XTCSS mode switch configuration, high effective |

      - CTCSS_ADDR

Offset address: 0x518 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | - | reserved |  |  |
| [15:8] | RW | ctcss_recv_addr | 0x0 | CTCSS sub-tone receiving address |
| [7:0] | RW | ctcss_send_addr | 0x0 | CTCSS sub-tone sending address |

      - USER_DEFINE_CTC

Offset address: 0x51c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31] | RW | user_define_en | 0x0 | User-defined function enable flag, “ 1 ”: enable, “ 0 ”: disable |
| [30:21] | - | reserved |  |  |

| [20:0] | RW | user_define_ctc_st ep | 0xe14 | CTCSS user-defined frequency setting is based on the formula: ( frequency to be sent /32000Hz sampling clock ) is obtained after quantization by 2^21 . The default is 55Hz |
| --- | --- | --- | --- | --- |

      - CDCSS_ADDR

Offset address: 0x520 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:25] | - | reserved |  |  |
| [24:16] | RW | cdcss_recv_addr | 0x0 | CDCSS sub-audio receiving address |
| [15:9] | - | reserved |  |  |
| [8:0] | RW | cdcss_send_addr | 0x0 | CDCSS sub-audio sending address |

      - SELCALL_TONE

Offset address: 0x524 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | - | reserved |  |  |
| [15:0] | RW | selcall_tone | 0x0fd9 | 2-Tone/5-Tone frequency to be sent |

      - TONE_SEND_CTRL

Offset address: 0x528 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:10] | - | reserved |  |  |
| [9] | RW | selcall_send | 0x0 | Signaling sending enable, " 1 ": module sends signaling, " 0 ": signaling sending interval or stop sending Note: This function can be used to control the side tone during signaling. " 1 " means playing the side tone; " 0 " means turning off the side tone. The switch sidevoice_en works in conjunction with AND logic. |
| [8] | RW | selcall_finish | 0x0 | The signaling sending end flag is given by the software : " 1 " means the sending is completed, and " 0 " means it is not completed. Note: When in the " 0 " state, it is used to shield the voice during the signaling process. Signal. |
| [7] | RW | dtmf_send | 0x0 | Code element sending enable, " 1 ": module sends code element, " 0 ": code element interval or stop sending; Note: This function can be used as the side tone playback control function during DTMF sequence transmission. “ 1 ” means playing the side tone; “ 0 ” means turning off the side tone. with the sidetone function switch sidevoice_en in “AND logic”. |
| [6] | RW | dtmf_finish | 0x0 | The code element sending end flag, for the "automatic code sending" mode, After the transmission is completed, the software will give the code element transmission end flag: " 1 " means the transmission is completed, and " 0 " means it is not completed. |

|  |  |  |  | Note: When in the " 0 " state, the screen is Mask voice signal. |
| --- | --- | --- | --- | --- |
| [5:4] | - | reserved |  |  |
| [3:0] | RW | dtmf_send_tone | 0x0 | The code element to be sent is configured by software before dtmf_send is valid |

      - XTCSS_ADDR

Offset address: 0x52c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:8] | RW | xtcss_tone | 0x0fd9 | XTCSS sending frequency configuration high 8bit , ( to be sent frequency / 32000Hz sampling Clock ) is obtained after 2^16 quantization |
| [7:0] | RW | xtcss_recv_addr | 0x0 | XTCSS receiving address |

      - FM_FUNCTION_COEF1

Offset address: 0x530 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:0] | RW | coef | 0x7ecd9d | FM sub-tone related functions receive demodulation coefficient 1 ( 697Hz ) |

      - FM_FUNCTION_COEF2

Offset address: 0x534 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:0] | RW | coef_2nd | 0x7e8a34 | FM sub-tone related functions receive demodulation coefficient 2 ( 770Hz ) |

      - FM_FUNCTION_COEF3

Offset address: 0x538 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:0] | RW | coef_3rd | 0x7e368c | FM sub-tone related functions receive demodulation coefficient 3 ( 852Hz ) |

      - FM_FUNCTION_COEF4

Offset address: 0x53c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |

| [31:24] | - | reserved |  |  |
| --- | --- | --- | --- | --- |
| [23:0] | RW | coef_4th | 0x7dd245 | FM sub-tone related functions receive demodulation coefficient 4 ( 941Hz ) |

      - FM_FUNCTION_COEF5

Offset address: 0x540 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:0] | RW | coef_5th | 0x7c690d | FM sub-tone related functions receive demodulation coefficient 5 ( 1209Hz ) |

      - FM_FUNCTION_COEF6

Offset address: 0x544 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:0] | RW | coef_6th | 0x7b9f03 | FM sub-tone related functions receive demodulation coefficient 6 ( 1336Hz ) |

      - FM_FUNCTION_COEF7

Offset address: 0x548 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:0] | RW | coef_7th | 0x7aa7a8 | FM sub-tone related functions receive demodulation coefficient 7 ( 1477Hz ) |

      - FM_FUNCTION_COEF8

Offset address: 0x54c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:24] | - | reserved |  |  |
| [23:0] | RW | coef_8th | 0x7979f5 | FM sub-tone related functions receive demodulation coefficient 8 ( 1633Hz ) |

      - FM_TONE1

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RW | tone_1 | 0x0593 | DTMF模式发送频率配置697Hz； |
| [15:0] | RW | tone_2 | 0x0629 | DTMF模式发送频率配置770Hz； |

Offset address: 0x550 Reset mode: H

      - FM_TONE2

Offset address: 0x554 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RW | tone_3 | 0x06d1 | DTMF mode is configured as 852Hz ; |
| [15:0] | RW | tone_4 | 0x0787 | DTMF mode is configured as 941Hz ; |

      - FM_TONE3

Offset address: 0x558 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RW | tone_5 | 0x09ac | DTMF mode is configured as 1209Hz ; |
| [15:0] | RW | tone_6 | 0x0ab0 | DTMF mode is configured as 1336Hz ; |

      - FM_TONE4

Offset address: 0x55c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:16] | RW | tone_7 | 0x0bd1 | DTMF mode is configured as 1477Hz ; |
| [15:0] | RW | tone_8 | 0x0d10 | DTMF mode is configured as 1633Hz ; |

      - FM_PTT

Offset address: 0x560 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:1] | - | reserved |  |  |
| [0] | RW | fm_ptt | 0x0 | Configure FM to send PTT command, high effective |

      - FM_DECODE_FLAG1

Offset address: 0x564 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:4] | RO |  | 0x0 |  |
| [3:0] | RO | selcall_tone_recv | 0x0 | Selcall detection symbol output |

      - FM_DECODE_FLAG2

Offset address: 0x568 Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |

| [31:26] | - | reserved |  |  |
| --- | --- | --- | --- | --- |
| [25] | RO | xtc_private_call_m ode | 0x0 | XTCSS individual call flag register |
| [twenty four] | RO | xtc_all_call_mode | 0x0 | XTCSS all-call flag register |
| [23:16] | RO | xtcss_type | 0x0 | XTCSS demodulation type flag register |
| [15:12] | - | reserved |  |  |
| [11:8] | RO | dtmf_recv_tone | 0x0 | DTMF decoding output |
| [7:2] | - | reserved |  |  |
| [1] | RO | cdcss_voice_en | 0x0 | CDCSS detects the flag and turns on the speaker |
| [0] | RO | ctcss_voice_en | 0x0 | CTCSS detects the flag and turns on the speaker |

      - FM_ADDR_SW

Offset address: 0x56c Reset mode: H

| Bits | Access | Name | Default | Description |
| --- | --- | --- | --- | --- |
| [31:2] | - | reserved |  |  |
| [1:0] | RO | fm_addr_sw | 0x0 | FM audio data address space selection control: 1 : Indicates that the CPU reads or writes low address data; 2 : Indicates that the CPU reads or writes high address data. |

#### Interrupt vector

FM Mode, define SYS_INTERP The interrupt types are shown in the following table.

Table 56 FM System interrupt sub-interrupt type list

| Serial number | name | type | describe |
| --- | --- | --- | --- |
| 1 | FM_TX_INTERP | Level trigger, high effective | FM Send data processing interrupt: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Via SYS_INTERP_CLEAR Register corresponding bit Bit Write 1 Clear. |
| 2 | FM_RX_INTERP | Level trigger, high effective | FM Receive data processing interrupt: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Via SYS_INTERP_CLEAR Register corresponding bit Bit Write 1 Clear. |
| 3 | FM_SIG_RX_INTERP | Level trigger, high effective | Simulate function receive detect interrupt: Via SYS_INTERP_MASK Corresponding bit of the register bit masking; Through SYS_INTERP_CLEA |

#### Memory Mapping

Baseband Ram Module is HR_C7000 The chip baseband digital function's main user data interaction storage space is divided into two RAMs, send and receive, according to different functional designs . The data length is 8 and the depth is 1200 .

###### FM Send Ram

FM Send Ram The base address is 0x16000000 , the depth is 1200 , and the following table defines the content allocation of each address segment.

Table 57 FM Send Ram distribute

| Offset Address | name | describe |
| --- | --- | --- |
| 0x030~0x22f | FM_VOICE_1 | 512 bytes in total Data, that is, 256 PCM​ Audio data. FM The voice data sent is stored in address 1 and address 2 Ping-pong is used, low address stores high 8-bit PCM Audio data |
| 0x230~0x42f | FM_VOICE_2 | 512 bytes in total Data, that is, 256 PCM​ Audio data. FM The voice data sent is stored in address 2 , and the address 1 Ping-pong is used, low address stores high 8-bit PCM Audio data |

###### FM Receive Ram

FM Receive Ram The base address is 0x160004b0 and the depth is 1200. The following table defines the content allocation of each address segment.

Table 58 FM Receive Ram distribute

| Offset Address | name | describe |
| --- | --- | --- |
| 0x030~0x22f | FM_VOICE_1 | 512 bytes in total Data, that is, 256 PCM audio data. The voice data received by FM is stored in address 1 and address 2 Ping-pong use, low address storage Amplify 8bits PCM Audio data |
| 0x230~0x42f | FM_VOICE_2 | 512 bytes in total Data, that is, 256 PCM audio data. The voice data received by FM is stored in address 2 , and the address 1 Ping-pong use, low address storage Amplify 8bits PCM Audio data |

### SQL

HR_C7000 FM mode supports SQL Squelch mode, FM mode system default support SQL The function is enabled, and the CPU obtains SQL by reading 0x508[15:0] Output value. A higher value indicates that the current reception process does not contain a valid signal in the audio band (or even if it exists, the signal is weak in the air ) . In this case, the user can set a threshold value to decide whether to shut down the audio playback path (above the threshold, it is shut down ) .

Through actual debugging, multiple groups of threshold parameters can be set. The high and low thresholds are SQL Squelch level.

### CTCSS

###### Overview

HR_C7000 FM Mode supports CTCSS Send and receive, support standard frequency and custom frequency sending and receiving functions.

###### Functional Description

CTCSS Features:

- Support standard frequency transmission, a total of 51 Group;

- Support custom frequency sending;

- Support standard frequency reception, a total of 51 Group;

- Support custom frequency reception;

- Support CTCSS Tail tone sending function;

      - CTCSS transmission

In standard mode, HR_C7000 Support 51 Group CTCSS The analog sub-audio sending frequency range is 62.5~254.1Hz . The mapping relationship between the sending address and the standard sub-audio frequency is shown in the following table.

Table 59 CTCSS List of mapping relationships between sending addresses and standard sub-tone frequencies

| CTCSS sending address | Sub-audio frequency (Hz) | Sine signal phase Step (decimal ) |
| --- | --- | --- |
| 0x01 | 67 | 4391 |
| 0x02 | 71.9 | 4712 |
| 0x03 | 74.4 | 4876 |
| 0x04 | 77 | 5046 |
| 0x05 | 79.7 | 5223 |
| 0x06 | 82.5 | 5407 |
| 0x07 | 85.4 | 5597 |
| 0x08 | 88.5 | 5800 |
| 0x09 | 91.5 | 5997 |
| 0x0a | 94.8 | 6213 |
| 0x0b | 97.4 | 6383 |
| 0x0c | 100 | 6554 |
| 0x0d | 103.5 | 6783 |
| 0x0e | 107.2 | 7025 |
| 0x0f | 110.9 | 7268 |
| 0x10 | 114.8 | 7524 |
| 0x11 | 118.8 | 7786 |
| 0x12 | 123 | 8061 |
| 0x13 | 127.3 | 8343 |
| 0x14 | 131.8 | 8638 |
| 0x15 | 136.5 | 8946 |
| 0x16 | 141.3 | 9260 |
| 0x17 | 146.2 | 9581 |
| 0x18 | 151.4 | 9922 |
| 0x19 | 156.7 | 10269 |
| 0x1a | 162.2 | 10630 |
| 0x1b | 167.9 | 11003 |
| 0x1c | 173.8 | 11390 |
| 0x1d | 179.9 | 11790 |
| 0x1e | 186.2 | 12203 |
| 0x1f | 192.8 | 12635 |
| 0x20 | 203.5 | 13337 |
| 0x21 | 210.7 | 13808 |
| 0x22 | 218.1 | 14293 |
| 0x23 | 225.7 | 14791 |

| 0x24 | 233.6 | 15309 |
| --- | --- | --- |
| 0x25 | 241.8 | 15847 |
| 0x26 | 250.3 | 16404 |
| 0x27 | 69.3 | 4542 |
| 0x28 | 62.5 | 4096 |
| 0x29 | 159.8 | 10473 |
| 0x2a | 165.5 | 10846 |
| 0x2b | 171.3 | 11226 |
| 0x2c | 177.3 | 11620 |
| 0x2d | 183.5 | 12026 |
| 0x2e | 189.9 | 12445 |
| 0x2f | 196.6 | 12884 |
| 0x30 | 199.5 | 13074 |
| 0x31 | 206.5 | 13533 |
| 0x32 | 229.1 | 15014 |
| 0x33 | 254.1 | 16653 |

the CTCSS corresponding to the sub-tone frequency they want to send. Sending address is realized through register CTCSS_ADDR configuration.

For non-standard application scenarios, when the user needs to customize the sending frequency, the software is required to calculate the phase of the sinusoidal signal of the corresponding frequency

Step 3 ( as shown in the table above ) Column) is configured to register USER_DEFINE_CTC , and its calculation formula is as follows:

user_define_ctc_step 	= 	round (send frequency * 65.536 )

It should be noted that the calculation result is a decimal number and needs to be converted to a binary number and configured in the register USER_DEFINE_CTC

Low 21 If the number of bits is less than 21 , the high bits are padded with zeros.

      - CTCSS reception

In standard mode, HR_C7000 Support 51 Group CTCSS The analog sub-audio receiving frequency range is 62.5~254.1Hz . The mapping relationship between the receiving address and the signal correlation detection coefficient coef is shown in the following table:

Table 60 CTCSS Receiving address and signal correlation detection coefficient coef List of mapping relationships

| CTCSS receiving address | Sub-audio frequency (Hz) | Detection coefficient coe f ( decimal) |
| --- | --- | --- |
| 0x01 | 67 | 8376996 |
| 0x02 | 71.9 | 8375236 |
| 0x03 | 74.4 | 8374291 |
| 0x04 | 77 | 8373273 |
| 0x05 | 79.7 | 8372179 |
| 0x06 | 82.5 | 8371005 |
| 0x07 | 85.4 | 8369746 |
| 0x08 | 88.5 | 8368352 |
| 0x09 | 91.5 | 8366956 |
| 0x0a | 94.8 | 8365367 |
| 0x0b | 97.4 | 8364075 |
| 0x0c | 100 | 8362749 |

| 0x0d | 103.5 | 8360908 |
| --- | --- | --- |
| 0x0e | 107.2 | 8358893 |
| 0x0f | 110.9 | 8356808 |
| 0x10 | 114.8 | 8354534 |
| 0x11 | 118.8 | 8352119 |
| 0x12 | 123 | 8349496 |
| 0x13 | 127.3 | 8346716 |
| 0x14 | 131.8 | 8343704 |
| 0x15 | 136.5 | 8340448 |
| 0x16 | 141.3 | 8337005 |
| 0x17 | 146.2 | 8333368 |
| 0x18 | 151.4 | 8329373 |
| 0x19 | 156.7 | 8325158 |
| 0x1a | 162.2 | 8320632 |
| 0x1b | 167.9 | 8315778 |
| 0x1c | 173.8 | 8310577 |
| 0x1d | 179.9 | 8305013 |
| 0x1e | 186.2 | 8299066 |
| 0x1f | 192.8 | 8292619 |
| 0x20 | 203.5 | 8281692 |
| 0x21 | 210.7 | 8274010 |
| 0x22 | 218.1 | 8265839 |
| 0x23 | 225.7 | 8257157 |
| 0x24 | 233.6 | 8247820 |
| 0x25 | 241.8 | 8237792 |
| 0x26 | 250.3 | 8227037 |
| 0x27 | 69.3 | 8376186 |
| 0x28 | 62.5 | 8378504 |
| 0x29 | 159.8 | 8322626 |
| 0x2a | 165.5 | 8317842 |
| 0x2b | 171.3 | 8312803 |
| 0x2c | 177.3 | 8307408 |
| 0x2d | 183.5 | 8301640 |
| 0x2e | 189.9 | 8295479 |
| 0x2f | 196.6 | 8288805 |
| 0x30 | 199.5 | 8285845 |
| 0x31 | 206.5 | 8278523 |
| 0x32 | 229.1 | 8253177 |
| 0x33 | 254.1 | 8222111 |

the CTCSS corresponding to the desired detection frequency. Receive address, which is achieved by configuring register CTCSS_ADDR .

For non-standard application scenarios, when the user needs to customize the receiving frequency, the software needs to calculate the detection coefficient Coef of the corresponding frequency

( As shown in Table 3 above Column) is configured to register FM_FUNCTION_COEF1 , and its calculation formula is as follows:

Coef 	= 	2*COS(2*π* frequency to be detected /8000)*2^22

It should be noted that when using a scientific calculator, cosine The object to be solved is radian unit, not degree unit. The result of the calculation is a decimal number, which needs to be converted to binary and configured into the lower 24 bits of the register FM_FUNCTION_COEF1. If the number of bits is less than 24 , the high bits are padded with zeros.

###### How it works

      - CTCSS transmission

The following figure describes CTCSS State flow diagram of sending.

user_define_en

N

Y

user_define_step

标准频率步进设置

相位累加

FM发送单元

CTCSS发送的其他参数配置

软件端

Figure 93 CTCSS Sending status flow chart

CTCSS The steps for sending configuration are as follows. You can choose between standard frequency and custom frequency:

- Configuration registers FM_MOD [3] = 1 	selection CTCSS model ;

- Configure register FM_MOD [13:11] to set CTCSS Tail flip mode;

- If the standard frequency is sent, configure the register CTCSS_ADDR [7:0] , and the corresponding address refers to CTCSS A list of mapping relationships between sending addresses and standard sub-audio frequencies;

- If sending a custom frequency, configure register USER_DEFINE_CTC[31] to “ 1 ” to enable CTCSS Custom configuration enabled;

- If a custom frequency is sent, calculate the phase Step of the sinusoidal signal The value is configured to register USER_DEFINE_CTC [20:0] ;

- Configuration registers SUBVOICE_COEF [15:8] 	adjusts the analog sub-audio transmission modulation frequency deviation (through comprehensive test

Usually the narrowband transmission is 350Hz and the broadband transmission is 600Hz . The adjustment accuracy is 10Hz about;

- ​

- Configuration register FM_PTT [0] = 1 , PTT Valid, start sending;

- Configuration register FM_PTT [0] = 0 , PTT Invalid, end sending, sending will be in CTCSS The process ends after the tail tone is sent .

      - CTCSS reception

CTCSS The receiving configuration steps are as follows, choose between standard frequency and custom frequency:

- Configuration registers FM_MOD [3] = 1 	selection CTCSS model ;

- If receiving standard frequency, configure register CTCSS_ADDR [15:8] , refer to CTCSS for the corresponding address A list of mapping relationships between receiving addresses and signal correlation detection coefficients coef ;

- If receiving a custom frequency, configure register USER_DEFINE_CTC[31] to “ 1 ” to enable CTCSS. Custom configuration enabled;

- If a custom frequency is received, calculate the detection coefficient Coef Configure the value to register FM_FUNCTION_COEF1[23:0] ;

- Configuration registers SUBVOICE_COEF [7:0] 	adjustment CTCSS Signal detection threshold ;

- If the received sub-tone frequency matches the frequency set for reception, the software will detect the FM sub-tone detection system interrupt and read the register FM_DECODE_FLAG2[0] , which is usually 1 Indicates CTCSS The sub-tone reception start flag, 0 represents the CTCSS sub-tone reception end flag.

### CDCSS

###### Overview

HR_C7000 FM Mode supports CDCSS Sending and receiving process.

###### Functional Description

CDCSS Features:

- Support standard code and non-standard code sending;

- Support standard code and non-standard code reception;

- Supports sending tail tone.

###### How it works

      - CDCSS send

- Configuration registers FM_MOD [2] = 1 	selection CDCSS model ;

- Supports standard CDCSS by default Sub-audio configuration, support forward and reverse settings, standard sending DCS The code is shown in the following table

- Configuration register CDCSS_ADDR[8:0] 	------ set up CDCSS Send codewords, such as CDCSS_ADDR [8:0] = “ 001001100 ” corresponds to the above table 114 ;

- Support non-standard CDCSS mode Symbols sent, for example, there is no 173 in the table above This option is not available, but it can still be sent by configuring CDCSS_ADDR[8:0]= “ 001111011 ”.

- Configuration registers SUBVOICE_COEF [15:8] 	adjusts the digital sub-tone transmission modulation frequency deviation (through comprehensive test

Observe with instruments and other means. Usually narrowband transmission is 350Hz and broadband is 600Hz about ) .

- 134.4Hz transmission Tail turns off single tone.

      - CDCSS take over

- Configuration registers FM_MOD [2] = 1 	selection CDCSS model ;

- Supports standard CDCSS by default Sub-audio configuration, supports forward and reverse settings.

- Configuration MODEM register CDCSS_ADDR[24:16] 	------ set up CDCSS Receive codewords , such as

CDCSS_ADDR[24:16]= “ 001001100 ” corresponds to 114 in the above table ;

- Support non-standard CDCSS mode Symbol reception, for example, there is no 173 in the table above This option is not available, but it can still be detected by configuring CDCSS_ADDR[24:16]= “ 001111011 ”.

- 134.4Hz is enabled by default Tail sound squelch function , By configuring FM_FUNCTION_COEF1[23:0] = 0x7f499d ( set the squelch detection frequency to 134.4Hz) , configuration register SUBVOICE_COEF [7:0] 	adjustment

CDCSS Tail tone detection threshold.

### DTMF

###### Overview

HR_C7000 FM Mode supports DTMF The process of sending and receiving signaling.

###### Functional Description

DTMF Features:

- Support standard and non-standard DTMF Signaling sent;

- Support standard and non-standard DTMF Signaling reception;

- Support DTMF Send sidetone playback.

DTMF The signal is 4 Group high frequency signal and 4 Group of low-frequency signals, a total of 16 The low frequency signal is 2.5dB lower than the high frequency signal . The high frequency signal and the low frequency signal are superimposed and weighted by the modulation frequency deviation coefficient . Modulated output.

调制系数

加权

FM调制

DTMF高频信号

DTMF低频信号

2.5dB信号衰减

Figure 94 DTMF Sending Block Diagram

Common standard DTMF The symbol frequency is shown in the figure below:

Figure 95 DTMF Symbol frequency correspondence

Usually DTMF The signal is sent before the start of the audio signal.

DTMF The demodulation process is to analyze the frequency distribution of the air signal and reversely decode it according to the DTMF combination method. The frequency response amplitude is set at a certain frequency, and the maximum amplitude in the high frequency part and the maximum amplitude in the low frequency part are selected. The combination of the two can determine the DTMF code.

###### How it works

      - DTMF send

- Configuration registers FM_MOD [1] = 1 	selection DTMF model ;

- Configuration registers SUBVOICE_COEF [15:8] 	adjustment DTMF Send modulation frequency deviation (through comprehensive tester

Observe by other means. Usually at 1.8KHz Left and right, select continuous DTMF Send code test ) .

- Configuring DTMF The frequency corresponding to the code element

- Configuration FM_TONE1[31:16] =16′h0593 	(Frequency 697Hz Corresponding phase value ) ;

- Configuration FM_TONE1[15:0] =16′h0629 	(Frequency 770Hz Corresponding phase value ) ;

- Configuration FM_TONE2[31:16] =16′h06D1 	(Frequency 852Hz Corresponding phase value ) ;

- Configuration FM_TONE2[15:0] =16′h0787 	(Frequency 941Hz Corresponding phase value ) ;

- Configuration FM_TONE3[31:16] =16′h09AC 	(Frequency 1209Hz Corresponding phase value ) ;

- Configuration FM_TONE3[15:0] =16′h0AB0 	(Frequency 1336Hz Corresponding phase value ) ;

- Configuration FM_TONE4[31:16] =16′h0BD1 	(Frequency 1477Hz Corresponding phase value ) ;

- Configuration FM_TONE4[15:0] =16′h0D10 	(Frequency 1633Hz corresponding phase value ) .

The calculation formula is: step 	= 	round (sending frequency * 2.048 )

DTMF The sending sequence diagram is as follows:

T0 	T1 	T2 T3

T4 	T5

PTT

Tone 2

Tone #

Tone *

Dtmf_send sends a signal

Dtmf_finish

Figure 96 DTMF Sending timing diagram

- Configure FM_PTT[0] = 1 , PTT Valid, establish carrier ( T0 in the figure moment ) .

- Configure TONE_SEND_CTRL[6] = 0 to ensure that DTMF is sent During the signaling process, the voice signal is shielded.

- Configure AUDIO_CONTROL[3] to enable or disable the sidetone playback function ( “ 1 ” means enable ) .

- Configure TONE_SEND_CTRL[3:0] to set DTMF Codewords to be sent.

- Configure TONE_SEND_CTRL[7]=1 to send the current symbol signal ( T1 in the figure moment ) .

- Configure TONE_SEND_CTRL[7]=0 to end the current symbol transmission ( T2 in the figure moment ) .

- Configure TONE_SEND_CTRL[3:0] to set the next set of DTMF Code element.

- Configure TONE_SEND_CTRL[7]=1 to send the second set of code element signals ( T3 in the figure moment ) .

- Configure TONE_SEND_CTRL[7]=0 to end the second group of code element transmission ( T4 in the figure moment ) . And so on.

- So after the code element is sent, configure TONE_SEND_CTRL[6]=1 to start the voice signal ( Dtmf_finish in the figure

rising edge moment ) .

      - DTMF take over

- Configuration registers FM_MOD [1] = 1 	selection DTMF model ;

- Configure SUBVOICE_COEF [ 27:16 ] and set DTMF​​​​​​​​ Demodulation time, standard mode is 12 ' d 1056 ( 3 2 K Hz , 33 ms , round ( 32000*0.033 ) , supports 30ms Non-standard mode ( fixed configuration is recommended, and the demodulation time is set to around 33ms . If the detection signal is much longer than 30ms , the DTMF detection module will output the detection code element at continuous equal intervals, or only output a single code element if the interrupt interval mask register is set ) .

- Configuration registers SUBVOICE_COEF [7:0] 	adjustment DTMF Signal detection threshold ;

- Configuration FM_FUNCTION_COEF1[23:0] = 24 h7ecd9d	 ( frequency 697Hz corresponds to the demodulation value ) ; configuration FM_FUNCTION_COEF2[23:0] = 24 h7e8a34	 ( frequency 770Hz corresponds to the demodulation value ) ; configuration FM_FUNCTION_COEF3[23:0] = 24 h7e368c	 ( frequency 852Hz corresponding demodulation value ) ; configuration FM_FUNCTION_COEF4[23:0] = 24′h7dd245 		( Frequency 941Hz corresponds to the demodulation value ) ; configuration FM_FUNCTION_COEF5[23:0] = 24 h7c690d	 ( frequency 1209Hz Corresponding demodulation value ) ; configuration FM_FUNCTION_COEF6[23:0] = 24′h7b9f03 	( Frequency 1336Hz Corresponding demodulation value ) ; configuration FM_FUNCTION_COEF7[23:0] = 24′h7aa7a8 	( Frequency 1477Hz Corresponding demodulation value ) ; configuration FM_FUNCTION_COEF8[23:0] = 24′h7979f5 	( frequency 1633Hz Corresponding demodulated value ) ;

Calculation formula: 2*COS(2*π* frequency to be detected /32000)*2^22 ;

- In the decoding results, if the adjacent detection results are the same codeword, the time interval between the two detections is used to determine whether they are the same codeword or independent codewords. Configure FM_MOD [23:16] to set the minimum interval threshold. For example , if FM_MOD [23:16] = 5 , it means that if the same codeword detection result occurs within 165ms ( 33ms*5 ),

If the interval between two adjacent codes is greater than this value, it is considered as a continuation of the previous code element, and the chip does not output an interrupt flag. On the contrary, if the interval between adjacent codes is greater than this value, it is considered as two independent decodings.

- Whenever interrupt information is received from the chip, read FM_DECODE_FLAG2[11:8] to obtain the code element information.

### SELCALL

###### Overview

HR_C7000 FM Mode supports 2-tone and 5-tone The process of sending and receiving signaling.

###### Functional Description

SELCALL Features:

- Support 2-tone Signaling sent;

- Support 5-tone Signaling sent;

- Support 2-tone Signaling reception;

- Support 5-tone Signaling reception;

- Support 2/5-tone Send sidetone playback.

###### How it works

      - SELCALL send

- Configuration registers FM_MOD[4] = 1 	selection SELCALL Signaling Mode ( 2/5 sound ) ;

- Configuration registers SUBVOICE_COEF [15:8] 	adjusts the signaling modulation frequency deviation (through a comprehensive tester, etc.

Means observation. Usually at 1.8KHz (Select continuous and uninterrupted single tone transmission code test ) .

T0 	T1 	T2 T3

PTT

Selcall_send

Tone B

Tone A

Sending a signal

Selcall_finish

Figure 97 SELCALL Sending timing diagram

- Configure FM_PTT[0] = 1 , PTT Valid, establish carrier ( T0 in the figure moment ) .

- Configure TONE_SEND_CTRL[8] = 0 to ensure that SELCALL is sent During the signaling process, the voice signal is shielded.

- Configure AUDIO_CONTROL[3] to enable or disable the sidetone playback function ( “ 1 ” means enable ) .

- Configure SELCALL_TONE [ 15:0] to set the signaling sending frequency; the calculation formula step = round ( sending frequency

*2.048 ) For example, 16′d4057 The corresponding frequency is 1981Hz Single tone.

- Configure TONE_SEND_CTRL[9]=1 to send the current SELCALL Signaling ( T1 in the figure moment ) .

- Configure TONE_SEND_CTRL[9]=0 to end the current signaling transmission ( T2 in the figure moment ) .

- Configure SELCALL_TONE [ 15:0] to set the frequency of the next set of signaling

- Configure TONE_SEND_CTRL[9]=1 to send the second set of signaling ( T3 in the figure moment ) .

- Configure TONE_SEND_CTRL[9]=0 to end the second group of signaling. And so on.

- So after the code element is sent, configure T ON E_ SEND_ CT R L [ 8 ] = 1 and start the voice signal ( Selcall_finish in the figure

rising edge moment ) .

      - SELCALL take over

- Configuration registers FM_MOD[4] = 1 	selection SELCALL Signaling Mode ( 2/5 sound ) ;

- Configure 0x50C[27:16] and set SELCALL Demodulation time, standard mode is 12'd1056 (32KHz , 33ms , round ( 32000*0.033 ) ) , supports compression to 20ms ( fixed configuration is recommended, demodulation time is set to 33ms Nearby, for detection signal much longer than 20ms In this case, the SELCALL detection module will output detection code elements at continuous and equal intervals ) .

- Configuration registers SUBVOICE_COEF[7:0] 	adjustment SELCALL Signal detection threshold .

- Configuration FM_FUNCTION_COEF1~8 	maximum support 8 Group frequency contrast test .

- Whenever an interrupt message is received from the chip, read FM_DECODE_FLAG1[3:0] to obtain the SELCALL signaling code element information.

## the term

| AHB | By British ARM The company develops and opens a high-performance bus architecture for connecting high-speed peripherals. |
| --- | --- |
| APB | By British ARM The company develops and opens a high-performance bus architecture for connecting low-speed peripherals. |
| DMR | A digital intercom standard protocol |
| FM | An analog intercom mode |
| CK803S | Hangzhou Zhongtian Processor |

## Abbreviations

| B |  |  |
| --- | --- | --- |
| BLC | Black Level Correction | Black level correction |
| D |  |  |
| DAC | Digital to Analog Converter | Digital-to-Analog Converter |
| DGC | Digital Gain Control | Digital gain control |
| G |  |  |
| GPIO | General Purpose Input Output | General Purpose Input and Output |
| I |  |  |
| I2C | The Inter-Integrated Circuit | A serial bus protocol standard |
| IRQ | Interrupt Request | Interrupt response |
| J |  |  |

| JTAG | Joint Test Action Group | Joint Test Group |
| --- | --- | --- |
| P |  |  |
| PLL | Phase-Loced Loop | Phase-Locked Loop |
| POR | Power On Reset | Power-on self-reset |
| PWM | Pulse Width Modulation | Pulse Width Modulation |
| R |  |  |
| RISC | Reduced Instruction Set Computer | RISC |
| UART | Universal Asynchronous Receiver Transmitter | Universal Asynchronous Receiver/Transmitter |
| S |  |  |
| SPI | Serial Peripheral Interface | Serial Peripheral Interface |

## Appendix A : Digital Group Call Application

#### Functional Description

The following figure defines the voice service timing diagram under the conventional system, including the voice frame header, voice superframe, and voice frame tail . BitLC​ Information ( refer to 9.6.2.1) , Voice EMB The embedded EMBLC in the four groups of BCDE and the 72- bit LC information defined at the end of the voice frame are exactly the same.

Data Sync	Voice Sync

Null

Data Sync

Checksum(5)

LC(72)

FEC Interleaver

BPTC

Header

A 	B 	C 	D 	E 	F

Terminator

Voice Superframe = 360ms

Figure 98 Voice service structure diagram

#### Group call sending

①	②	③	④	⑤	⑥	⑦	⑧	⑨	⑩	⑪	⑫	⑬	⑭	⑮	⑯	⑰	⑱

Transmit timeslot interruption 	Layer 2 transmit processing interruption

Term

F

E

D

C

B

A

LC

LC

Figure 99 Group call sending service sequence diagram 1

Transmit timeslot interrupt

①	②	③	④	⑤	⑥	⑦	⑧	⑨	⑩	⑪	⑫	⑬	⑭	⑮	⑯	⑰	⑱

Term

F

E

D

C

B

A

LC

LC

Figure 100 Group call sending service sequence diagram 2

Refer to group call sending sequence diagram 1 and 2 There are two ways to send a group call. The steps are as follows (the sending time slot is known ) : Step ①:

- Prepare voice LC72bits The group call service information of 72 bits Information write send Ram address

0x16000000~0x16000008 ;

- Configuration register LAYER2_SEND_TYPE 0x10​ Indicates sending voice frame header LC ;

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending; Step ③:

Repeat step ① to send the second voice frame header. You can send another voice frame header if needed. Step ⑤:

- Prepare voice LC72bits The group call service information of 72 bits Information write send Ram address

0x16000000~0x16000008 , EMB for voice superframe Area code filling;

- Configuration register LAYER2_SEND_TYPE 0x08​ Indicates sending voice frame A ;

- If the voice data is sent from Ram To obtain, you need to send Ram Address 0x16000030~0x1600004a

Write 27 bytes voice data stream.

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending; Step 7:

- Configuration register LAYER2_SEND_TYPE 0x19​ Indicates sending voice frame B ;

- If the voice data is sent from Ram To obtain, you need to send Ram Address 0x16000030~0x1600004a

Write 27 bytes voice data stream.

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending;

Step 9:

- Configuration register LAYER2_SEND_TYPE 0x2B​ Indicates sending voice frame C ;

- If the voice data is sent from Ram To obtain, you need to send Ram Address 0x16000030~0x1600004a

Write 27 bytes voice data stream.

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending; Step ⑪ :

- Configuration register LAYER2_SEND_TYPE 0x3B​ Indicates sending voice frame D ;

- If the voice data is sent from Ram To obtain, you need to send Ram Address 0x16000030~0x1600004a

Write 27 bytes voice data stream.

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending; Step ⑬ :

- Configuration register LAYER2_SEND_TYPE 0x4A​ Indicates sending voice frame E ;

- If the voice data is sent from Ram To obtain, you need to send Ram Address 0x16000030~0x1600004a

Write 27 bytes voice data stream.

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending; Step ⑮ :

- Configuration register LAYER2_SEND_TYPE 0x50​ Indicates sending voice frame F ;

- Voice F Frame EMB The filling of the area can be controlled by register VOICE_EMB_CTRL Control, the corresponding content is stored in the sending Ram address range 0x16000029~0x1600002C ;

- If the voice data is sent from Ram To obtain, you need to send Ram Address 0x16000030~0x1600004a

Write 27 bytes Voice data stream;

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending; Step ⑰ :

- Prepare voice LC72bits The group call service information of 72 bits Information write send Ram address

0x16000000~0x16000008 ;

- Configuration register LAYER2_SEND_TYPE 0x20​ Indicates sending the voice frame tail LC ;

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending;

The remaining steps ②④⑥⑧⑩ ⑫⑭⑯⑱ all configure the register LAYER2_TXRX_CTRL to 0x0 or 0x40 , indicating that the transmission of the next time slot is turned off and the reception is turned on or off.

#### Group call reception

The group call reception service is started by detecting the system interruption of the voice frame header or the system interruption of the voice access after the voice, and the timeslot for receiving the voice service is opened when both meet the group call address matching. The corresponding design timing is shown in the figure below.

系统中断

接收时隙中断

发送的语

音业务

Term

F

E

D

C

B

A

LC

LC

sys_interp1 sys_interp1 sys_interp2

sys_interp3

sys_interp4

Post-access

语音数据接收开关

Figure 101 Group call receiving service sequence diagram

SYS_INTERP_CLEAR清除中断

②读取寄存器RX_TYPE_INFO，获取接收信息

③读取接收Ram地址0x160004b0~0x160004bb，获取LC信息

④读取接收Ram地址，根据所在接收时隙标号选取27bytes语音数据

Idle

检测到系统中断

Cond1:CRC/RS/CS等校验OK、CC匹配

Cond2:地址匹配（ID Match） Cond3:是否已经开始语音接收

Cond4:语音接收帧错误个数超过1个超帧

二层解帧中断

①

二层语音后接入中断

SYS_INTERP_LI ST==0x8

②

SYS_INTERP_LI ST==0x4

②

Cond1

No

SYNC_TYPE==0

SYNC_TYPE==2

SYNC_TYPE==1

③ Yes

Cond1

④

Voice 27Bytes &&

VoiceBurstErrorNum=0

LC 72Bits

No

Cond1

No

Yes	④

Yes

③

No

Cond4

Idle

PI==0

Yes

Voice 27Bytes &&

VoiceBurstErrorNum=0

Yes

DATA_TYPE==1

VoiceBurstError Num++

语音呼叫

异常结束

DATA_TYPE==2

&& Voice Start

Voice Key Update

Cond2

No

No	Yes

Cond2

Cond2

Idle

Yes

Yes

Yes

Cond3

Cond3

语音呼叫开始

语音呼叫结束

语音呼叫

异常结束

No

No

语音呼叫开始

语音呼叫中

语音呼叫中

语音呼叫开始

① Read register SYS_INTERP_LIST to obtain interrupt type, write register

Figure 102 Group call receiving state flow chart

The steps for receiving group call service are as follows:

1. The receiver detects the interruption of the de-framing system during blind reception and reads the register RX_TYPE_INFO to determine whether the correct voice frame header LC information is received. Then, it reads the receive RAM address 0x160004b0~0x160004bb to determine whether the received group address matches.

2. If the address matches, start the timeslot to receive voice service and start the timeslot switch register.

LAYER2_TXRX_CTRL register;

3. When the voice frame A information is received at the time slot interruption , the time slot is started to receive the voice data content.

Register LAYER2_TXRX_CTRL control;

4. When the voice access system is interrupted, the receiving RAM address 0x160004b0~0x160004bb is read to determine whether the received group address matches. If there is no voice service, the access information is correct and the address matches, the time slot for receiving voice service can be started (note: the reception of voice data needs to wait until the start of frame A of the next superframe ) . If a reception error occurs, the voice reception can be turned off and a voice abnormal end prompt will be issued if there is voice service.

5. When the system interrupt is detected at the end of the voice frame, read the register RX_TYPE_INFO to determine whether the correct voice frame header LC information is received, and then read the receive Ram address 0x160004b0~0x160004bb to determine whether the received group address matches;

## Appendix B : Digital SMS Application

#### Functional Description

Conventional SMS services primarily include two types of short message functions: unconfirmed and confirmed . SMS messages with group addresses default to the unconfirmed SMS data frame format, while messages with individual addresses can be configured by the CPS as either confirmed or unconfirmed. The following describes these two SMS service applications under conventional mode. The following figure defines the IP -based SMS service block diagram for a conventional digital intercom system .

IPv4 Packet

User Data

Response Header

Yes

Confirmed

No

Private Flg

Yes

Response Data (33bytes)

Response Header

Send RspData

IPv4 UnPacket

IPv4 Packet

Received Response Data

User Data

Send Data

Received Data

MS0 	MS1

Figure 103 SMS service application flow chart

The SMS service of the conventional system uses the format of the IP data service, and the corresponding protocol-defined IP non-confirmation frame header and IP confirmation frame header formats. The actual SMS content to be sent needs to be encapsulated in IPV4 before being sent. When displaying the SMS, the receiving end needs to first decapsulate the IPV4 encapsulated data and then display and save the actual received SMS content.

The figure above defines the workflow of the IP SMS service. The general process is described as follows:

1. After the user edits the content of the SMS to be sent, the software begins to encapsulate the user data in IPv4 format to prepare for sending, and uses different values to define IP non-confirmation and confirmation encapsulation data; the receiving MS receives the SMS data completely and correctly. Under the premise, if the current SMS type is a confirmation SMS, you need to send a Response first Header to the sender indicates receipt of ACK ;

2. Then, regardless of whether the IP is confirmed or not , the receiving MS begins to parse the IPv4 structure data content and saves the actual valid received short message content after meeting the relevant verification results of the IPv4 frame format;

3. If the current address is a single-address SMS, a 33 -byte Response needs to be sent as a single-address SMS. Data , which also satisfies the IPv4 structure;

4. For the MS that actively sends the SMS content of the IP address , it needs to receive the 33-Byte Response Data from the receiving MS to completely complete the SMS .

#### SMS service timing

Conventional SMS services can be divided into single-frame header and double-frame header data sequences based on whether the second data header is used, as shown in the following figure. The second data frame header carries custom control information, such as encryption applications.

Hdr

Data

Block

Data

Block

Data

Block

Last

Block

Single frame header

Last Block

Hdr

Hdr

Data

Block

Data

Block

Data

Block

Double frame header

Figure 104 SMS service timing

The default definition of group address sending SMS type is non-confirmed SMS format, using the data packet body frame format of Rate1/2 Rate data, no confirmation response is required.

The following figure defines the number of data packets actually required to send a blank SMS message, a non-confirmation SMS message.

Hdr

Data

Block 1

Data

Block 2

Data

Block 3

Last

Block 4

DataType=6 	DataType=7

Figure 105 Non-confirmed SMS service structure diagram

The address SMS type is defined to have two modes: unconfirmed SMS format and confirmed SMS format. Confirmed SMS uses the data packet body frame format of Rate 3/4 Rate, the timing definition of non-confirmed SMS is consistent with the process of group address, but in the entire process of SMS sending and receiving, the difference between individual address and group address mode is that for SMS sent by individual address, no matter whether it is confirmed mode or non-confirmed mode, after correctly processing the current SMS content, the receiver will send a feedback SMS to the sender after receiving the SMS. The sending mode of this SMS is determined by CPS. Whether the pre-set is confirmation or non-confirmation

Decide.

The following figure defines the timing relationship diagram for confirming SMS sending. In the figure, the receiver needs to provide feedback on the data frame header to determine whether the current reception is correct. The gray data packet body is only activated when an error frame is currently received and is used to indicate the corresponding error frame sequence number.

After correctly completing the reception of the SMS, the receiver will start to send an SMS defined as feedback information.

Hdr

Data

Block 1

Data

Block 2

Data

Block 3

Last

Block 4

Term

TX

DataType=6 DataType=8 	DataType=2

Rsp

Hdr

Rsp

Block

RX

DataType=6 DataType=7

Figure 106 Confirm SMS service structure diagram

#### SMS data frame structure

The SMS service consists of several types of packets, including a data frame header, a data frame body, an acknowledgment data frame trailer, a response data frame header, and a response data feedback packet body .

###### Data frame header structure

PDU is defined as follows Structural diagram.

| G/I | A | HC | POC | DPF |
| --- | --- | --- | --- | --- |
| SAP |  |  |  | POC |
| Destination logical link ID |  |  |  |  |
| Source logical link ID |  |  |  |  |
| F | Blocks to Follow |  |  |  |
| 0 | 0 | 0 | 0 | FSN |
| Header CRC |  |  |  |  |

7 	6 	5 	4 	3 	2 	1 	0

0

1

2

3

4

5

6

7

8

9

10

11

7 	6 	5 	4 	3 	2 	1 	0

| G/I | A | HC | POC | DPF |
| --- | --- | --- | --- | --- |
| SAP |  |  |  | POC |
| Destination logical link ID |  |  |  |  |
| Source logical link ID |  |  |  |  |
| F | Blocks to Follow |  |  |  |
| S | N(S) |  |  | FSN |
| Header CRC |  |  |  |  |

0

1

2

3

4

5

6

7

8

9

10

11

Non-confirmation data header PDU structure 	Confirmation data header PDU structure

Figure 107 Data frame header PDU structure

Table 61 Unacknowledged data frame header PDU definition

| Information element | Length | Value | Remark |
| --- | --- | --- | --- |
| G/I | 1 | 1 | Is the destination address an address or a group address? |
| A | 1 | 0 |  |
| HC | 1 | 0 | Full data header usage |
| POC | 1 |  | The highest bit of the number of padding bytes ( in actual calculated ) |
| DPF | 4 | 0010 | IP UnConfirmed Data |
| SAP | 4 | 0100 | SAP points to the data type after the data frame header , the default is 0100 , if there is a data header Here it is 1001 |
| POC | 4 |  | 4 bytes of the number of padding bytes Bit ( in actual calculated ) |
| Destination Address | twenty four |  | Destination Address |
| Source Address | twenty four |  | Source Address |
| F | 1 |  | 1 Indicates the first time sending, 0 Indicates retransmission |
| Blocks to Follow | 7 |  | Total number of packets sent |
| Reserved | 4 | 0000 |  |
| FSN | 4 | 0000 |  |
| Header CRC | 16 |  | Frame header CRC16 Verification results |

Table 62 Confirm data frame header PDU definition

| Information element | Length | Value | Remark |
| --- | --- | --- | --- |
| G/I | 1 | 0 | Is the destination address an address or a group address? |
| A | 1 | 1 | Response confirmation is required, 1 Indicates need, 0 Indicates no need |
| HC | 1 | 0 | Full data header usage |
| POC | 1 |  | The highest bit of the number of padding bytes (obtained from actual calculation ) |
| DPF | 4 | 0011 | IP Confirmed Data |
| SAP | 4 | 0100 | SAP Points to the data type after the data frame header, the default is 0100 , If there is a data header, this is 1001 |
| POC | 4 |  | 4 bytes of the number of padding bytes Bits (actually calculated ) |
| Destination Address | twenty four |  | Destination Address |
| Source Address | twenty four |  | Source Address |
| F | 1 |  | 1 Indicates the first time sending, 0 Indicates retransmission |
| Blocks to Follow | 7 |  | Total number of packets sent (actually calculated ) |
| S | 1 | 1 | Default |
| N(S) | 3 |  | Send sequence 0~7 |
| FSN | 4 | 1000 | Single frame confirmation frame sequence value 1000 |
| Header CRC | 16 |  | Frame header CRC16 Verification results |

###### Data frame structure

The following table defines the number of bytes of data content carried by the data frame body occupied by various rate types . Unconfirmed Rate ½ is used in the format messaging application and Confirmed Rate ¾ .

Table 63 Data frame body byte count table

| Confirmation / non-confirmation type | Coding rate | Bytes per packet |
| --- | --- | --- |
| Confirmed | Rate ½ | 10 |
| Confirmed | Rate ¾ | 16 |
| Confirmed | Rate 1 | twenty two |
| Unconfirmed | Rate ½ | 12 |
| Unconfirmed | Rate ¾ | 18 |
| Unconfirmed | Rate 1 | twenty four |

The rate 1/2 data block (R_1_2_DATA) PDU length is 96 bits and contains 96 bits of user data when used for unacknowledged services. Its contents are shown in the following table.

Table 64 1/2 Rate non-acknowledged data block ( R_1_2_DATA ) PDU content

| information | Length (Bit) | annotation |
| --- | --- | --- |
| User data | 96 | User data field |

Table 65 1/2 Rate non-acknowledged last data block ( R_1_2_LDATA ) PDU content

| information | Length (Bit) | annotation |
| --- | --- | --- |
| User data | 64 | User data field |
| Entire data verification | 32 | CRC32 checksum |

Non-last data block 	Last data block

User data

(the second last block may contain pad octets

If pad octets > 8)

7 	6 	5 	4 	3 	2 	1 	0

0

1

2

3

4

5

6

7

8

9

10

11

0

1

2

3

4

5

6

7

8

9

10

11

7 	6 	5 	4 	3 	2 	1 	0

Fragment CRC

User data followed by pad octets

Figure 108 Unconfirmed Rate 1/2 Data block structure diagram

Rate 3/4 Data Block (R_3_4_DATA) PDU The length is 144 bits, including 128 when used for services with confirmation Bit User

Data and 7 bit data block sequence number, and 9 CRC of bits Verify the data, the contents of which are shown in the following table.

Table 66 3/4 Rate with acknowledgement data block ( R_3_4_DATA ) PDU content

| information | Length (Bit) | annotation |
| --- | --- | --- |
| Data Block Sequence Number (DBSN) | 7 |  |
| Data block checksum | 9 | CRC9 is used for data block sequence number and user data |
| User data | 128 | User data field |

Table 67 3/4 Rate with confirmation of the last data block ( R_3_4_LDATA ) PDU content

| information | Length (Bit) | annotation |
| --- | --- | --- |
| Data Block Sequence Number (DBSN) | 7 |  |
| Data block checksum | 9 | CRC9 is used for data block sequence number and user data |
| User data | 96 | User data field |
| Entire data verification | 32 | CRC32 checksum |

###### Response data suspension time

The receiving end needs to send a response to confirm receipt of the data. To ensure the fast transmission of the response, the system needs to reserve a channel for the response . This step is called the "data response hang time."

In direct mode, the MS Need to end LC by transmitting "data Signaling" to indicate the start of the data response suspension time, the receiver should send a response in a polite manner. To distinguish the data end LC For signaling and other bursts, you need to set the data type information in SLOT Type to " LC end flag".

In the transfer mode, the base station needs to send "data end LC Signaling" starts with the data response suspension time. To avoid conflicts, the repeater should set CACH during the data response suspension period. AT​ The mobile station shall return a response in an impolite manner within its "data response hang-up time".

|  | 7 | 6 | 5 | 4 | 3 | twenty one | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | PF | R | FLCO |  |  |  |  |
| 1 | FID |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 3 |  | De | stina | tion | Logic | al Link ID |  |
| 4 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 6 |  |  | Source Logical |  |  | Link ID |  |
| 7 |  |  |  |  |  |  |  |
| 8 | G/ I | A | F | 0 | S | N(S) |  |
| 9 |  |  |  |  |  |  |  |
| 10 |  |  |  | RS |  |  |  |
| 11 |  |  |  |  |  |  |  |

Figure 109 Data frame tail PDU Structure diagram

Table 68 Data frame tail PDU definition

| Information element | Length | Value | Remark |
| --- | --- | --- | --- |
| PF | 1 | 0 | reserve |
| R | 1 | 0 | reserve |
| FLCO | 6 | 110000 | Data frame tail |
| FID | 8 | 00000000 |  |
| Destination Address | twenty four |  | Destination Address |
| Source Address | twenty four |  | Source Address |
| G/I | 1 |  | The destination address is an address |
| A | 1 | 0 |  |
| F | 1 | 0 | 1 Indicates the first time sending, 0 Indicates retransmission |
| 0 | 1 |  |  |
| S | 1 | 1 |  |
| N(S) |  |  | Send sequence 0~7 |

###### Response data frame

The SMS confirmation process requires sending an acknowledgment feedback frame. The following figure describes the SMS confirmation acknowledgment feedback frame information. It mainly consists of a feedback data frame header followed by a data frame. The feedback frame header structure information is consistent with the data frame header structure information, with only the control field having different defined values. The following table describes the control information of the feedback data frame header.

The feedback data packet body only exists when an error packet occurs during the actual reception process, and it uses Rate 1/2 Rate data frame structure , which can carry up to 64 blocks​ Error packet sequence number information, the corresponding position value is 0 Indicates that the data packet with this sequence number is received incorrectly . bits​ Therefore, in actual application, at most 2 data packet bodies can be sent after the feedback data frame header .

Response Data Header 	Response Packet Format

7 	6 	5 	4 	3 	2 	1 	0 	7 	6 	5 	4 	3 	2 	1 	0

7	6	5	4	3	2	1	0

8

16

24

32

40

48

63

56

CRC

| G/ I | A | 0 | 0 | DPF |  |
| --- | --- | --- | --- | --- | --- |
| SAP |  |  |  | 0000 |  |
| Destination Logical Link ID |  |  |  |  |  |
| Source Logical Link ID |  |  |  |  |  |
| 0 | Blocks to follow |  |  |  |  |
| Class |  | Type |  |  | Status |
| Header CRC |  |  |  |  |  |

0 	0

1 	1

twenty 	two

3 	3

4 	4

5 	5

6 	6

7 	7

8 	8

9 	9

10 	10

11 	11

Figure 110 Response data frame PDU Structure diagram

Table 69 Response data frame header PDU definition

| Information element | Length | Value | Remark |
| --- | --- | --- | --- |
| G/I | 1 | 0 | The destination address is an address |
| A | 1 | 0 |  |
| Reserved | 1 | 0 | reserve |
| Reserved | 1 | 0 | reserve |
| DPF | 4 | 0001 | Response Packet |
| SAP | 4 | 0100 |  |
| Reserved | 4 | 0000 | reserve |
| Destination Address | twenty four |  | Destination Address |
| Source Address | twenty four |  | Source Address |
| Reserved | 1 | 0 | reserve |
| Blocks to Follow | 7 |  | Total number of sent feedback packets |
| Class | 2 | 0 | 00 : ACK ; 01 : NACK ; 10 : SACK |
| Type | 3 |  | Range: 000~110 , 000 is usually used for SACK , 001 ACK​ |
| Status | 3 |  | Send sequence 0~7 |

The feedback packet body primarily uses the retransmission mechanism for confirming SMS messages. Retransmission is completed by simply retransmitting the corresponding error packet. The following figure illustrates the error pattern update process after the error packet reflected in the feedback packet body is re-received.

First Time Received 	Retry Time Received 	Update

7  6  5

| 1 | 0 | 1 | 0 | 1 | 1 | 0 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 |
| x | x | x | x | x | x | 1 | 0 |
| x | x | x | x | x | x | x | x |
| x | x | x | x | x | x | x | x |
| x | x | x | x | x | x | x | x |
| x | x | x | x | x | x | x | x |
| x | x | x | x | x | x | x | x |

7

15

twenty three

31

39

47

55

63

4  3  2  1  0

0

8

16

twenty four

32

40

48

56

7  6  5  4

| 1 | 0 | 1 | 1 | 1 | 1 | 0 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| x | x | x | x | x | x | 1 | 0 |
| x | x | x | x | x | x | x | x |
| x | x | x | x | x | x | x | x |
| x | x | x | x | x | x | x | x |
| x | x | x | x | x | x | x | x |
| x | x | x | x | x | x | x | x |

7

15

twenty three

31

39

47

55

63

3  2  1  0

0

8

16

twenty four

32

40

48

56

7  6  5  4

| 1 | 0 | 1 | 1 | 1 | 1 | 0 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| x | x | x | x | x | x | 1 | 0 |
| x | x | x | x | x | x | x | x |
| x | x | x | x | x | x | x | x |
| x | x | x | x | x | x | x | x |
| x | x | x | x | x | x | x | x |
| x | x | x | x | x | x | x | x |

7

15

twenty three

31

39

47

55

63

3  2  1  0

0

8

16

twenty four

32

40

48

56

Figure 111 Feedback data package error pattern update process

#### IPv4 Structure Description

IPv4 is defined as follows Structural diagram.

0 	15 16 	31

| 4 -digit version | 4 -bit header length | 8 -bit service type | 16 -bit total length (bytes ) |  |
| --- | --- | --- | --- | --- |
| 16 -bit identifier |  |  | 3 is the symbol | 13 -bit slice offset |
| 8 -bit lifetime |  | 8 -bit protocol | 16 -bit header checksum |  |
| 32 -bit source IP address |  |  |  |  |
| 32 -bit destination IP address |  |  |  |  |
| Options |  |  |  |  |
| data |  |  |  |  |

Figure 112 IPv4 The structure diagram below defines the format of each field definition of UDP checksum calculation.

0 	15 16 	31

| 32 -bit source IP address |  |  |  |
| --- | --- | --- | --- |
| 32 -bit destination IP address |  |  |  |
| 0 | 8 -bit protocol ( 17 ) | 16 -bit UDP length |  |
| 16 -bit source port number |  | 16 -bit destination port number |  |
| 16 -bit UDP length |  | 16 -bit UDP checksum |  |
| Options |  |  |  |
| data |  |  | Padding Bytes ( 0 ) |

Figure 113 UDP Verify and calculate each field definition

###### IPv4 Packet Data

Reference IPv4 Structural diagram of IP During the data encapsulation process, 34 bytes are added before the original data content. IPv4​

The frame structure content is described in detail in the following table.

Table 70 IPv4 Packet Data

| Field | length | value | describe |
| --- | --- | --- | --- |
| Version | 4 | 0100 |  |
| Header length | 4 | 0101 |  |
| Service Type | 8 | 00000000 |  |
| Total length | 16 |  | Its value is 34 + actual data content length. |
| Logo | 16 |  | After the intercom is turned on, each time a text message is sent, |

|  |  |  | Index+ 1 . |
| --- | --- | --- | --- |
| Logo | 3 | 000 |  |
| Offset | 13 | 0000000000000 |  |
| Survival time | 8 | 01000000 |  |
| protocol | 8 | 00010001 |  |
| Header checksum | 16 |  | IPv4 Header First 20 bytes CheckSum​ Calculated. |
| Source Address | 32 |  | The source address consists of 12 + the address of the intercom itself. |
| Destination Address | 32 |  | If the target address is a group address, it is composed of 225 + target group address; if the target address is an individual address, of 12+ target addresses. |
| Source port number | 16 | 0x0FA7 | 16 Base |
| Destination port number | 16 | 0x0FA7 | 16 Base |
| UDP length | 16 |  | Its value is 14 + actual data content length. |
| UDP Checksum | 16 |  |  |
| Option + data length | 16 |  | Its value is 4 + actual data content length. |
| Option 1 | 8 |  | 0xe0 Represents confirmation; 0xa0 Stands for non-confirmation. |
| Option 2 | 16 |  | 0x0081~0x009f cycle |
| Option 3 | 8 | 0x04 |  |
| . . . |  |  | Actual data content |

###### IPv4 Response Packet Data

Referring to the IPv4 structure diagram, during the IP Response data encapsulation process, it is a fixed 33-byte IPv4 frame structure content, and its detailed information is described in the following table.

Table 71 IPv4 Response Packet Data

| Field | length | value | describe |
| --- | --- | --- | --- |
| Version | 4 | 0100 |  |
| Header length | 4 | 0101 |  |
| Service Type | 8 | 00000000 |  |
| Total length | 16 | 33 | Its value is 33 . |
| Logo | 16 |  | Its value is determined by the above table in the received confirmation message. ( Option 2 - 0x0081 ) . |
| Logo | 3 | 000 |  |
| Offset | 13 | 0000000000000 |  |
| Survival time | 8 | 01000000 |  |
| protocol | 8 | 00010001 |  |
| Header checksum | 16 |  | IPv4 Header First 20 bytes CheckSum​ Calculated. |
| Source Address | 32 |  | The source address consists of 12 + the address of the intercom itself. |
| Destination Address | 32 |  | The destination address consists of 12+ target addresses. |
| Source port number | 16 | 0x0FA7 | 16 Base |
| Destination port number | 16 | 0x0FA7 | 16 Base |

| UDP length | 16 | 14 |  |
| --- | --- | --- | --- |
| UDP Checksum | 16 |  |  |
| Option + data length | 16 | 3 |  |
| Option 1 | 8 | 0xBF |  |
| Option 2 | 16 |  | Its value is determined by the above table in the received confirmation message. ( Option 2 - 0x0080 ) . |

#### Non-confirmation SMS

###### Sending non-confirmed SMS

①	②	③	④	⑤	⑥	⑦	⑧	⑨	⑩

|  | HDR |  | Data Block 1 |  | Data Block 2 |  | Data Block 3 |  | Last Block 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure 114 Sequence diagram of non-confirmed SMS sending service

Refer to the sequence diagram for sending non-confirmed SMS messages. The sending steps are as follows (given that the sending time slot is established ) : Step 1:

- Based on the data rate (see the structure of the non-acknowledged data packet at each rate ) and the data length, calculate the number of data packets to be sent, the number of padding bytes (fill with 0 ) , calculate the CRC32 of the data content , and divide the data content to be carried by each data packet .

- According to the data packet number, padding byte number and SMS sending method, the frame size is 80 bits. The data frame header information is written into the sending Ram address 0x16000000~0x16000009 ;

- Configuration register LAYER2_SEND_TYPE 0x60​ Indicates sending data frame header;

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending; Step ③:

- The first Package 96/144/192 bits Three rates ( select one ) information write and send Ram address

0x16000000~0x16000017 ;

- Configuration register LAYER2_SEND_TYPE 0x70 /0x80/0xa0 Indicates sending data packet body;

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending; Step ⑤:

- The 2nd Package 96/144/192 bits Three rates ( select one ) information write and send Ram address

0x16000000~0x16000017 ;

- Configuration register LAYER2_SEND_TYPE 0x70 /0x80/0xa0 Indicates sending data packet body;

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending; Step 7:

- The 3rd Package 96/144/192 bits Three rates ( select one ) information write and send Ram address

0x16000000~0x16000017 ;

- Configuration register LAYER2_SEND_TYPE 0x70 /0x80/0xa0 Indicates sending data packet body;

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending; Step 9:

- The 4th Package 96/144/192 bits Three rates (select one) information including CRC32 The verification result is written to the sent Ram

Address 0x16000000~0x16000017 ;

- Configuration register LAYER2_SEND_TYPE 0x70 /0x80/0xa0 Indicates sending data packet body;

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending;

The remaining steps ②④⑥⑧⑩ all configure the register LAYER2_TXRX_CTRL to 0x0 or 0x40 , indicating that the transmission of the next time slot is turned off and the reception is turned on or off.

###### Unconfirmed SMS reception

The unconfirmed SMS reception service is started by detecting the system interrupt of the data frame header and opening the time slot to receive the SMS service when the address match is met. The corresponding design timing is shown in the figure below.

|  | Hdr |  | Data Block 1 |  | Data Block 2 |  | Data Block 3 |  | Data Block 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

System interruption

Figure 115 Sequence diagram of non-confirmed SMS reception service

Receive slot interrupt

Yes

DATA_TYPE==6  DATA_TYPE==7  DATA_TYPE==8  DATA_TYPE==a  DATA_TYPE==2

③

③

③

DataHeader Process

Rate½ DataProcess

③

DataBlock RxProcess

③

DataBlock RxProcess

DataTerm Process

Cond2

Cond2

Yes

Yes

Confirmed==1 &&

A==1

DPF==0010 DPF==0011

DPF==1111

DPF==0001

Cond5

Yes

No

Cond3

Ready to Rx Data Block

Cond4

Yes

No

No

Yes

Ready to Rx Proprietary DataHeader

Ready to Tx ACK Rsp

Ready to Rx Data Block

ConfirmedData Send Success

Ready to Rx Response packet

Ready to Tx SACK Rsp

Block_sum、

Confirmed 、

A、F

Rate½ DataProcess

DataBlock RxProcess

Yes

Cond6

No

Cond8

No

Yes

Cond7

Cond7

No

No

DataBlock RxProcess

Yes

Yes

CRC9 CALC CRC32 CALC

CRC32 CALC

CRC9 CALC

Ready To Retry Error Block

Block Error Index

Error Block Num

Block Data/CRC9 Result

BlockCRC9/DataC RC32 Result

Block Data

Block_to_follows

Idle

① 读取寄存器SYS_INTERP_LIST获取中断类型，写寄存器

SYS_INTERP_CLEAR清除中断

②读取寄存器RX_TYPE_INFO，获取接收信息

③读取接收Ram地址0x160004b0~0x160004c7，获取解帧信息

检测到系统中断

①

SYS_INTERP_LI ST==0x8

②

Cond1

Cond1:CRC/RS/CS等校验OK、CC匹配

Cond2:地址匹配（ID Match） Cond3:SAP==1001

Cond4:确认短信反馈ACK OK Cond5:确认短信CRC9和CRC32 OK Cond6:最后一个数据包

Cond7:确认短信接收

Cond8:确认短信反馈数据包体接收

RxUnconfirmed DataEnd

Data CRC32 Result

Figure 116 SMS receiving flow chart

The steps for receiving non-confirmed SMS services are as follows:

1. The receiver detects the interruption of the de-framing system during blind reception and reads the register RX_TYPE_INFO to determine whether the correct data frame header information is received. Then, it reads the receive RAM address 0x160004b0~0x160004bc to determine whether the received address matches.

2. If the address matches, start the time slot to receive non-confirmed SMS service and start the time slot switch register

LAYER2_TXRX_CTRL register;

3. According to the interrupt of the de-framing system, read the register RX_TYPE_INFO , and read the corresponding data packet body information from the receiving Ram in turn until the last packet is received;

4. Perform CRC32 check calculation on all received data packets in sequence to determine whether the received information is completely correct.

#### Confirm SMS

###### Confirm SMS sending

Transmit timeslot interrupt

①	②	③	④	⑤	⑥	⑦	⑧	⑨	⑩	⑪	⑫

|  | HDR |  | Data Block 1 |  | Data Block 2 |  | Data Block 3 |  | Last Block 4 |  | Data Term |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure 117 Confirm SMS sending service sequence diagram

Refer to the SMS confirmation sending sequence diagram, the sending steps are as follows (the sending time slot is known to be established ) : Step 1:

- Based on the data rate (refer to the data packet structure diagram for each rate ) and the data length, calculate the number of data packets to be sent , the number of padding bytes (fill with 0 ) , calculate the CRC32 of the data content , and divide the data content to be carried by each data packet .

- According to the data packet number, padding byte number and SMS sending method, the frame size is 80 bits. The data frame header information ( refer to section 6.1 ) is written into the sending RAM address 0x16000000 ~0x16000009 ;

- Configuration register LAYER2_SEND_TYPE 0x60​ Indicates sending data frame header;

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending; Step ③:

- The first The packet is sent at 96/144/192 bits (select one of the three rates). The CRC9 check result of the packet data is written to the send RAM address 0x16000000~0x16000017 .

- Configuration register LAYER2_SEND_TYPE 0x70 /0x80/0xa0 Indicates sending data packet body;

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending; Step ⑤:

- The 2nd The packet is sent at 96/144/192 bits (select one of the three rates). The CRC9 check result of the packet data is written to the send RAM address 0x16000000~0x16000017 .

- Configuration register LAYER2_SEND_TYPE 0x70 /0x80/0xa0 Indicates sending data packet body;

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending; Step 7:

- The 3rd The packet is sent at 96/144/192 bits (select one of the three rates). The CRC9 check result of the packet data is written to the send RAM address 0x16000000~0x16000017 .

- Configuration register LAYER2_SEND_TYPE 0x70 /0x80/0xa0 Indicates sending data packet body;

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending; Step 9:

- The 4th The packet is sent at 96/144/192 bits (select one of the three rates). The information includes the CRC9 check result of the data content of this packet and the CRC32 check result of all data contents, and is written to the send RAM address 0x16000000~0x16000017 ;

- Configuration register LAYER2_SEND_TYPE 0x70 /0x80/0xa0 Indicates sending data packet body;

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending; Step ⑪ :

- Prepare voice LC72bits The confirmation message frame tail information will be 72 bits Information write send Ram address

0x16000000~0x16000008 ;

- Configuration register LAYER2_SEND_TYPE 0x20​ Indicates sending the frame tail LC ;

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending;

The remaining steps ②④⑥⑧⑩⑫ all configure the register LAYER2_TXRX_CTRL to 0x0 or 0x40 , indicating that the transmission is closed in the next time slot and the reception is opened or closed. After the end of the frame is sent, the reception is required to be opened to receive the confirmation SMS feedback.

###### Confirm SMS receipt

Confirm the start of the SMS reception service. By detecting the system interrupt of the data frame header and satisfying the address match, the time slot for receiving SMS service is started . After receiving the end frame of the data , an ACK needs to be fed back to the other party. The following figure reflects the timing diagram of correctly receiving SMS.

System interrupt 	send time slot interrupt

|  | HDR |  | Data Block 1 |  | Data Block 2 |  | Data Block 3 |  | Last Block 4 |  | Data Term |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

时隙接收使能

时隙发送使能

Hdr Rsp

Figure 118 Confirm SMS reception service sequence diagram

The steps to confirm SMS service reception are as follows (refer to SMS reception flow chart ) :

1. The receiver detects the interruption of the de-framing system and reads the register RX_TYPE_INFO to determine whether the correct data frame header information is received. Then, it reads the receiving Ram address 0x160004b0~0x160004bb to determine whether the received address is

No match;

2. If the address matches, start the time slot reception confirmation SMS service and start the time slot switch register

LAYER2_TXRX_CTRL register;

3. According to the interrupt of the de-framing system, read the register RX_TYPE_INFO , read the corresponding data packet body information from the receiving Ram in turn, and calculate the CRC9 checksum of each packet data accordingly until the last packet is received;

4. Perform CRC32 check calculation on all received data packets in sequence to determine whether the received information is completely correct.

5. If the CRC9 of all data packets is correct and the CRC32 of the data content is correct, the receiving end needs to feedback ACK information to the sending end.

## Appendix C : Digital Signaling Applications

#### Functional Description

The following figure defines CSBK under the conventional system Signaling service timing diagram, including preamble CSBK control packet, function CSBK control packet and feedback CSBK control packet.

ACKCSBK/ NACKCSBK

FuncCSBK

PreCSBK1

PreCSBK2

Figure 119 CSBK Signaling service structure diagram

#### Signaling

发送时隙中断

①	②	③	④	⑤	⑥

|  | Pre CSBK1 |  | Pre CSBK2 |  | Func CSBK |  |
| --- | --- | --- | --- | --- | --- | --- |

Figure 120 CSBK Signaling service sending sequence diagram

Reference CSBK The signaling service sending sequence diagram has the following steps (given that the sending time slot is established ) :

Step 1:

- Prepare the first leading CSBK80 bits The control information is written to the send Ram Address 0x16000000~0x16000009 ;

- Configuration register LAYER2_SEND_TYPE 0x30​ Indicates sending data frame header;

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending; Step ③:

- Prepare the second leading CSBK80 bits The control information is written to the send Ram Address 0x16000000~0x16000009 ;

- Configuration register LAYER2_SEND_TYPE 0x30​ Indicates sending data frame header;

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending; Step ⑤:

- Prepare function CSBK80bits The control information is written to the send Ram Address 0x16000000~0x16000009 ;

- Configuration register LAYER2_SEND_TYPE 0x30​ Indicates sending data frame header;

- Configuration register LAYER2_TXRX_CTRL 0x80​ Indicates that the next time slot starts sending;

The remaining steps ②④⑥ all configure the register LAYER2_TXRX_CTRL to 0x0 or 0x40 to indicate that the next time slot transmission is closed and reception is opened or closed. After the CSBK packet is sent, the reception is required to be opened to receive the CSBK response feedback.

#### Signaling reception

System interruption Transmit timeslot interrupt

|  | Pre CSBK1 |  | Pre CSBK2 |  | Func CSBK |  |
| --- | --- | --- | --- | --- | --- | --- |

时隙发送使能

ACK CSBK

Figure 121 CSBK Signaling service reception timing diagram

Idle

检测到系统中断

①

SYS_INTERP_LI ST==0x8

②

Cond1

DATA_TYPE==3

Cond2

④

CSBKO

SendRspCSBK

① Read register SYS_INTERP_LIST to obtain interrupt type, write register

SYS_INTERP_CLEAR clears the interrupt

②Read register RX_TYPE_INFO to obtain receiving information

③Read the receiving Ram address 0x160004b0~0x160004bb to obtain the de-framing information

④ Confirm signaling function services according to CSBKO type

Cond1: CRC/RS/CS check OK , CC match Cond2: Address match ( ID Match​

Figure 122 CSBK Signaling service receiving flow chart

CSBK signaling service are as follows:

1. The receiver detects the interruption of the de-framing system during blind reception and reads the register RX_TYPE_INFO to determine whether the correct CSBK control information is received. Then, it reads the receive RAM address 0x160004b0~0x160004bb to determine whether the received address matches.

2. If the addresses match and the CSBK signaling function defined in the function is met , the receiving end needs to feedback the CSBK signaling to the sending end.

## Appendix D : Digital Encryption Applications

#### Overview

HR_C7000 digital intercom system mainly provides a way to keep the communication business confidential. The communication business includes voice and data . Voice services support both static basic encryption and dynamic enhanced encryption. Only radio users with the same encryption enabled can receive text messages and voice services. Any other radio users with unmatched encryption keys will be unable to access the communication service.

Digital encryption only protects voice and data traffic. Layer 2 voice and data frame headers, data feedback packets, and link control data are not encrypted. This means that source and destination individual and group addresses are not encrypted. Control signaling such as remote kill, remote monitoring, intercom detection, call notification, and embedded RC signaling are also not encrypted.

The digital encryption function is applied to all digital working modes and runs through the communication between the sending intercom and the destination intercom. The digital encryption function is applied to protect individual voice calls, group voice calls, all calls, emergency calls and all group calls.

Data services include individual, group, unconfirmed and confirmed.

#### Voice encryption

HR_C7000 supports voice encryption. Based on different encryption requirements, users can choose to configure static basic encryption mode or dynamic enhanced encryption mode.

###### Basic encryption at rest

Voice

Voice

Static basic encryption uses configuration encryption register values. HR_C7000 generates corresponding encryption and decryption keys based on the register settings for the sending and receiving processes. The process diagram is shown in the figure below, and the configuration requirements are shown in the table below.

语音加密

TX

RX

语音解密

解帧

解密密钥寄存器

组帧

声码器

加密密钥寄存器

声码器

Figure 123 Schematic diagram of the static encryption process

Table 72 Static basic encryption configuration table

| Serial number | Register /Ram address | Configuration Values | describe |
| --- | --- | --- | --- |
| 1 | LOCAL_CC | 0x1 | Encryption mode selection |
| 2 | SCRAMBLE_REG |  | Configure transmit and receive key register values |

###### Dynamic Enhanced Encryption

The following figure defines the process of voice encryption and decryption. The key consists of the key value stored locally.

( by KeyID Get) and initial vector value, corresponding to the algorithm ID Generate the corresponding key. The encryption parameters only transmit the algorithm serial number, the ID value of the local key , and the initial vector. The local key value is not transmitted over the air interface.

密钥

序号

密钥

序号

加密密

钥生成

算法

序号

算法

序号

解密密

钥生成

初始

密钥

初始

密钥

初始密钥

算法序号密钥序号

明文

语音

密文

语音

密文

TX

RX

语音

声码器

组帧

解帧

语音解密

明文

语音

语音加密

初始密钥算法序号密钥序号

声码器

Figure 124 Schematic diagram of the dynamic encryption process

Table 73 Dynamic Enhanced Encryption Configuration Table

| Serial number | Register /Ram address | Configuration Values | describe |
| --- | --- | --- | --- |
| 1 | LOCAL_CC | 0x10001 | Encryption mode selection |
| 2 | VOICE_EMB_CTRL |  | PI flag control |
| 3 | 0x16000029 |  | Send and pass KeyID Value, F Frame EMB Zone transfer usage |
| 4 | 0x1600002a |  | High 3 Bit represents the sending and transmitting ALOG ID Value, F Frame EMB Zone transfer usage |
| 5 | 0x16000000~0x16000009 |  | PI Frame transfer encryption parameters |
| 6 | 0x16000000~0x16000008 |  | EMBLC Passing encrypted parameters |
| 7 | 0x16000495~0x160004af |  | Send 216 bits Key, used for sending voice encryption |
| 8 | 0x16000945~0x1600095f |  | Send 216 bits Key, used for decrypting received voice |

##### Encryption parameter definition

The encryption parameters mainly consist of three parts: initial vector, encryption identifier ( KeyID ) and encryption algorithm . The frame content will include the above three parts, and the voice superframe will be connected to the EMB Region transfer initial vector, speech frame F EMB​ The zone will pass two pieces of information: KeyID and encryption algorithm.

| FID(用户定义) |  |
| --- | --- |
| 初始密钥 |  |
| KeyID |  |
| Voice Key Flag | 算法序号 |
| 保留(可以扩展为初始密钥) |  |
| Header CRC(16) |  |

PI is defined as follows The structural content of the frame.

|  | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |  |

Figure 125 PI Frame PDU Structure diagram

        - Initialization Vector

Initial vector definition 48 bits Information that can be obtained through PI Frame delivery or voice superframe delivery.

PI Frame transmission method see the PI above Frame PDU Structural diagram.

The following figure defines the voice superframe transmission initial vector IV A method (user-defined ) mainly generates a 72- bit changed initial vector value through CRC8 and CRC16 .

将CRC8结果与IV级联

针对IV计算CRC8

将级联的结果计算CRC16

将CRC16结果与上一步级联结果级联得到改变的IV

将改变的IV经过CHECKSUM、 BPTC编码、交织发送

Figure 126 The initial vector generation process is shown in the figure below, which defines the structure of the initial vector actually transmitted within the superframe.

Voice Sync

KeyID

ALGOID

FEC Interleaver

BPTC

Checksum(5)

CRC16

CRC8

IV(48)

A 	B 	C 	D 	E 	F

Voice Superframe = 360ms

Figure 127 Initial vector generation EMB Transmission method

        - KeyID and ALGOID

KeyID Corresponding to the locally saved key ID Value index, used to find the locally saved key value.

Algorithm ID is the method of key calculation, which can usually be implemented by encryption algorithms such as ARC4 , AES , and DES . The above two encryption parameters can have PI Frames are passed directly, or in the form of voice F Frame delivery, voice F Frame delivery implementation

As shown in the figure below.

LCSS

PI

CC

Known KeyID 8​ Bit, algorithm ID 3​ Bit, 11 The information of each bit is passed through FEC Produces 32 bits of embedded information . This embedded information is in the voice F EMB of frame Zone transfer. FEC The encoding method of BPTC ( 32,11 ) specified in the protocol was used .

| Key ID (8) | Algorithm ID | FEC (21) |
| --- | --- | --- |
|  | (3) |  |

| Voice(108) | EMB(8) | Embeded signalling (32) | EMB(8) | Voice(108) |
| --- | --- | --- | --- | --- |

27.5ms

VoiceBurstF

Figure 128 KeyID and algorithm ID Voice F Frame transmission mode

##### Encrypted parameter passing

...

The following figure defines a schematic diagram of the encryption parameter transmission and update process, which chooses to transmit the key and voice access information in a 1:1 manner .

...

...

Voice superframe 0

| 帧头 |  | PI头 | 帧A |  | 帧B |
| --- | --- | --- | --- | --- | --- |

| 帧E |  | 帧F | 帧A |  | 帧B |
| --- | --- | --- | --- | --- | --- |

Later access to the EMB area

Voice superframe 1 	Voice superframe 2

| 帧E |  | 帧F | 帧A |  | 帧B |
| --- | --- | --- | --- | --- | --- |

| 帧E |  | 帧F |  |
| --- | --- | --- | --- |

encrypting and decrypting the EMB area , 	access the EMB area

发送更新	发送更新

加密参数	加密参数

发送更新

发送更新72bit	72bit后接入

后接入信息	信息

发送更新216

bit加密序列

发送更新216

bit加密序列

EMB区域PI标志

接收更新加密

参数

接收更新

加密参数

接收更新72

bit后接入信息

接收更新72

bit后接入信息

接收更新216	接收更新216

bit加密序列	bit加密序列

The blue dotted line connection shows that the encryption parameter information at the sending and receiving ends is the same; the red dotted line connection shows that the 216-bit encryption sequence information at the sending and receiving ends is the same;

Figure 129 Schematic diagram of dynamic key transmission update

        - Initial key transfer and encryption process

During the key update process, the first voice superframe number is defined as 0 and the numbers are accumulated sequentially.

The hyperframe numbers are 0 , 2 , 4 , 6 , ..., and the EMB of these frames is The information that needs to be loaded in the area is the voice access information, which is a total of 72 bits. , requires that in frame A After voice data transmission begins, this information is written to the sender's Ram 0x16000000~0x16000008 address space and the PI flag bit of the VOICE_EMB_CTRL register is set to 0 .

The superframes need to be marked as 1 , 3 , 5 , 7 , ..., and the EMB of these frames The information that needs to be loaded in the area is the initial key information, which is a total of 72 bits. , requires that in frame A After voice data transmission begins, this information is written to the sender's Ram 0x16000000~0x16000008 address space and the PI flag bit of the VOICE_EMB_CTRL register is set to 1 .

In encrypted voice communication, after the voice frame header, the voice frame A Before sending, you need to send 1 One or more voice PIs The frame header contains the initial encryption key, encryption algorithm sequence number and key ID . Complete PI After the frame is sent, it needs to be The information sent by the frame generates 216 bits The voice encryption sequence and the generated 216bit The key information is stored in the RAM of the sender The address 0x16000495~0x160004af has a total of 27 bytes of space.

Completed the first 216bit After the voice encryption sequence is generated, the next new encryption sequence is generated in the super frame number 1 Superframe F The time slot before sending is completed, so that in the 3rd , 5th , 7th , ..., frame F of the voice superframe Previously 216bit Encryption sequence updated .

In preparation for sending F In the idle time slot before the frame, the current superframe sequence number information is written into the sending end RAM 0x16000029​ Address , algorithm ID is written to the upper 3 bits of 0x1600002A .

The following figure defines the flow chart of the transfer state of dynamic key parameters.

PTT

语音发送

发送30ms

时隙中断

奇数次中断

写72bit帧头信息进TxRAM

待机

偶数次中断

发送30ms

时隙中断

奇数次中断

准备PI头初始密钥、密钥ID和算法序号，更新216bit加密密钥

超帧序号=0

是

超帧序号为

偶数

否

依次发送完

成语音帧A-F

发送30ms

时隙中断

发送30ms

时隙中断

写72bit初始加密信息进TxRAM

发送30ms

时隙中断

超帧序号+1

否

发送结束

发送完成

语音帧F

是

否

待机

发送结束

是

待机

根据目前的加密初始密

钥、算法ID和密钥ID生成216bit加密密钥

依次发送完成语音帧A-E

超帧序号+1

写72bit后接入信息进TxRAM

Even interrupts

Figure 130 Dynamic key parameter encryption transfer flow chart

        - Initial key reception and decryption process

The voice decryption process steps are as follows:

1. When the receiving end receives the voice frame header and the encryption bit in the voice frame header control field is 1 , the receiving end starts the voice encryption mode.

2. Receive PI Frame information, by receiving Ram Obtaining PI Frame control information, get key ID , algorithm ID And the initial key vector, calculate the voice key 216 bits , and write it into the decryption key storage address of the receiving Ram ;

3. Steps 1 and 2 This describes the first time that PI is passed normally. Frame method for obtaining voice key, if PI In the case of frame loss, the PI will be judged by the subsequent access interrupt. The flag bit is 1 Get the encryption initial vector parameters in the case of , get the key ID and algorithm ID parameters through the F frame EMB area, calculate the voice key 216 bits , and write it into the decryption key storage address of the receiving Ram ;

4. After each access interrupt, according to PI The flag bit determines whether it is voice post-access information or encryption parameter information , and obtains the key ID and algorithm ID in the EMB area of the voice F frame ;

5. Each superframe PI The flag bit is 1 When completing the voice F Update the voice key of the receiving end after the frame reception is interrupted.

待机

语音帧头

信息匹配

语音迟后接

入信息匹配

正常语音

接收

迟后接入

语音接收

PI帧接收

中断

F帧超帧序

号解析中断

初始密钥、

密钥ID和算法序号

解析当前帧F

嵌入式超帧序号信息

接收语音

超帧A-E

语音迟后接入中断匹配

否

是

PI ==1

更新后接入

信息

解析超帧中B-

E更新初始密钥和算法序 号，并更新超帧序号，计算新解密密钥

接收结束

中断

否

是

待机

计算解密密钥存入RAM

Figure 131 Dynamic key parameter decryption and transfer flow chart

##### Encrypted control frames

        - Voice frame header

The following table defines the voice frame header control field. The difference from the regular voice frame header is the FID And the corresponding service option Options

Privacy is turned on in The field is 1 .

Table 74 Voice frame header definition

| Information Unit |  | length | describe |
| --- | --- | --- | --- |
| PF |  | 1 | 0 |
| Reserved |  | 1 | 0 |
| FLCO |  | 6 | Group call 000000 , individual call 000011 |
| FID |  | 8 | 00010000 |
| Service Options | Emergency | 1 | 1 For emergency calls |
|  | Privacy | 1 | 1 Encryption mode |
|  | Reserved | 2 |  |
|  | Broadcast | 1 | 1 All Call |
|  | OVCM | 1 | 0 |
|  | Priority level | 2 | 00 |
| Target addr |  | twenty four |  |
| Source addr |  | twenty four |  |
| RS24 |  | twenty four |  |

        - Voice PI frame

Table 75 Voice PI Frame Definition

| Information Unit | length | describe |
| --- | --- | --- |
| FID | 8 | 00010000 |
| Initialization Vector IV | 48 |  |
| KeyID | 8 | Key ID value |
| Voice Key | 5 | 00100 , voice encryption mode flag |
| ALGOID | 3 | encryption algorithm |
| Reserved | 8 |  |
| CRC16 | 16 |  |

        - Voice F Frame EMB

Voice F Frame EMB The area filled is 32 bits of information. This part is represented by KeyID 8 bits and ALGOID 3 bits are BPTC ( 32,11 ) encoded and then interleaved and filled into F EMB of frame During voice communication on the selected channel , the F frame EMB area information remains unchanged once the CPS setting is confirmed .

#### Data encryption

Data encryption uses enhanced encryption. The encryption parameter definition method is consistent with voice encryption, but the encryption parameter transmission method is different from voice encryption.

###### SMS encryption timing

The following figure defines the timing diagram of SMS services under a conventional system. Compared with conventional SMS, there is an additional data header to transmit encryption parameters , including the initialization vector, KeyID , and ALGOID .

Hdr

Hdr

Last

Block

DataType=6

DataType=6

DataType=7

Figure 132 Unconfirmed SMS encryption sequence diagram

Hdr

Hdr

Last

Block 1

Term

TX

DataType=6 DataType=6 	DataType=2

Rsp

Hdr

Rsp

Block

RX

DataType=6 DataType=7

Figure 133 Confirm SMS encryption sequence diagram

###### SMS encryption parameter transmission

The SMS encryption parameter transmission uses the custom SMS frame header, and its structure is shown in the figure below.

Proprietary Data Header

| SAP | DPF |
| --- | --- |
| MFID |  |
| The structure of three 8 octets is defined by the manufacture |  |
| Header CRC |  |

7 	6 	5 	4 	3 	2 	1 	0

0

1

2

3

4

5

6

7

8

9

10

11

Figure 134 Custom data frame header PDU Structure diagram 76 Privacy Data Header

| Information element | Length | Value | Remark |
| --- | --- | --- | --- |
| SAP | 4 | 0100 | Regular SMS is 0100 |

| DPF | 4 | 1111 |  |
| --- | --- | --- | --- |
| MFID | 8 | 00010000 |  |
| Initialization Vector IV | 48 |  |  |
| KeyID | 8 |  |  |
| Data Privacy | 5 | 00010 |  |
| ALGOID | 3 | 001 |  |
| Reserved | 16 | 0000000000000000 |  |
| Header CRC | 16 |  |  |

## Appendix E : Digital DCDM application

#### Overview

The DCDM mode is used to realize the dual time slot call function when the intercom works in direct mode. Normally, in order to transmit two time slots simultaneously and independently, a repeater is generally required to provide a time reference. In this mode, the two-way radios can automatically synchronize and cooperate, and no longer need a repeater to provide a time reference. Therefore, two time slots can be used simultaneously, doubling the capacity and improving frequency efficiency without having to purchase repeaters and related auxiliary equipment.

#### Basic principles

DCDM mode using independent TDMA To share a channel, one thing that must be considered is to keep the time slot boundaries of the two MSs aligned during transmission to avoid interference between time slots. In order to reduce the probability of interference between time slots, all MSs in the wide area system It is important that all cells use the same channel timeslot definition. It is designated as the wide area slot leader and sets the time slot of the wide area system channel. Units share TDMA This extends the time slot information from the leading MS to all MS units in the system .

###### State machine type

DCDM Leader is used in this mode MS The implementation involves four high-level states: leader and slot unknown, leader unknown, leader and slot known, and leader state. Each of these is described below .

##### Leader and slot unknown status

In the leader and slot unknown state, the MS does not know the slot status of the current channel. This state usually occurs when the MS Startup or MS Change different working frequencies . Change the channel time slot from time slot 1 at the same operating frequency In the case of time slot 2 , it will maintain the current time slot state. In addition, the MS does not know the leader of the current channel time slot. In this state, the MS There are three ways to proceed: obtaining a leader and channel slot, designating a leader, and being designated as a leader.

##### Leader unknown state

In the leader unknown state, MS The time slot information is obtained through the previous acquisition. However, MS The leader of the current channel slot is unknown , and it can no longer be assumed that the previous slot leader receives the current slot leader. With

3.1.1 The same treatment is applied to the status . When transmission is required, MS The last known channel slot is sent to minimize interference between slots.

##### Leader and slot known status

When the leader and slot are known, the MS Know the current channel time slot and channel time slot leader. In this state, the MS has the following five strategies: designate a leader, be designated as a leader, accept a new leader, send leader time slot push, and send leader time slot correction.

##### Leader Status

In the leadership state, MS Is the slot leader and sets the channel slot. In this state, the MS There are five strategies : appoint a new leader, accept a new leader, resolve leader ID Collision, send beacon signal, send leader time slot correction.

###### Data frame definition

##### Synchronization frame definition

When transmitting wide area timing information, the MS will use the synchronization word of the specified time slot to communicate with other MSs. The following table defines TDMA Direct Mode Synchronous field definitions under schema.

Table 77 Sync frame field definition

| TDMA direct mode time slot 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Voice | Hex | 5 | D | 5 | 7 | 7 | F | 7 | 7 | 5 | 7 | F | F |
|  | Binary | 0101 | 1101 | 0101 | 0111 | 0111 | 1111 | 0111 | 0111 | 0101 | 0111 | 1111 | 1111 |
| Data | Hex | F | 7 | F | D | D | 5 | D | D | F | D | 5 | 5 |
|  | Binary | 1111 | 0111 | 1111 | 1101 | 1101 | 0101 | 1101 | 1101 | 1111 | 1101 | 0101 | 0101 |
| TDMA direct mode time slot 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Voice | Hex | 7 | D | F | F | D | 5 | F | 5 | 5 | D | 5 | F |
|  | Binary | 0111 | 1101 | 1111 | 1111 | 1101 | 0101 | 1111 | 0101 | 0101 | 1101 | 0101 | 1111 |
| Data | Hex | D | 7 | 5 | 5 | 7 | F | 5 | F | F | 7 | F | 5 |
|  | Binary | 1101 | 0111 | 0101 | 0101 | 0111 | 1111 | 0101 | 1111 | 1111 | 0111 | 1111 | 0101 |

##### Color code definition

With DMR Same as other working modes, DCDM The color code is also defined in the mode. The color code settings of the two time slots can be the same or different. In addition, an " All Site " color code ( F ) is defined for use in DCDM In the DCDM system , a color code value of F is received . is considered a qualified color code . Site " color code is only used for transmission channel timing CSBK When the control packet is in time slot 1 , the packet may be Or time slot 2. MS The global color code can be received in both specified and unspecified time slots. This reduces the number of CSBK packets required to support wide area time slot transmission.

      - Channel Timing CSBK

Channel timing information is obtained through a Channel Timing CSBK ( CT_CSBK ) packet transmission. When a CT_CSBK is received When MS CT_CSBK will be evaluated The information elements include leader ID , leader dynamic identifier, generation, synchronization period, channel timing operation code, new leader flag, source dynamic identifier, source ID . However, since all transmissions may not start on time slot boundaries, this requires the MS to decode frames transmitted in a non-aligned time slot manner.

If MS If the received channel timing information is better than the known timing information, the MS The newly received CT_CSBK will be used Information to set the channel timing information. If the MS The timing information of several channels obtained is not the same as the known timing information.

If the information is good, then MS The current channel slot timing will continue to be used. CT_CSBK May be received in MS The time slot specified by the unit may also be in the non-specified time slot . Will receive CT_CSBK in any time slot This reduces the number of CT_CSBK packets that need to be transmitted . The following lists five different types of CT_CSBK packets.

        - CT_CSBK_Beacon

The channel timing CT_CSBK periodically transmitted by the leader MS is called a beacon CT_CSBK ( CT_CSBK_Beacon ) .​​ CT_CSBK_Beacon uses the appropriate slot synchronization words and global color code in the time slot designated by the leader, and attempts to send a polite strategy in the direct mode at 12.5KHz . If there is RF activity in any time slot of the current channel, CT_CSBK_Beacon Will not be sent immediately. This ensures that CT_CSBK_Beacon It will be sent with the correct channel slot timing . If the MS receives a CT_CSBK_Beacon , it will wait in line and retry the transmission after a delay. If the MS receives a CT_CSBK_Beacon and it is a wide area timing leader, it will use it to set its channel timeslot. The interval between two CT_CSBK_Beacon transmissions is called the beacon interval.

        - CT_CSBK_Prop

CT_CSBK_Prop can be considered as a mandatory propagation message. CT_CSBK_Prop packet is mainly used by non-leader MS to send a message after receiving CT_CSBK_Beacon. or CT_CSBK_Prop After that, through CT_CSBK_Prop Packet flooding transmits accurate wide-area timing information to all MSs within geographical range Unit. CT_CSBK_Prop Use the appropriate slot synchronization words and global color code on the specified time slot and try to send it in a polite way in 12.5KHz direct mode. If there is RF on any time slot of the current channel Active, CT_CSBK_Prop It will not be sent immediately. This ensures that CT_CSBK_Prop will be sent with the correct channel slot timing . If the MS receives CT_CSBK_Prop , it will set its channel timeslot accordingly if it receives the wide area timing leader information .

CT_CSBK packets transmitted on the channel , CT_CSBK_Prop will be canceled in some specific cases . A sliding window CT_RHOT strategy is necessary to spread the transmission of CT_CSBK_Prop messages from different MS units , so that the channel time slot timing information will be more likely to reach a wide area. An MS After sending CT_CSBK_Prop , the time to send the next CT_CSBK_Prop packet is distributed with a lower limit of 2.16 seconds to an upper limit of 3.24 seconds using 60ms increments using the specified CT_RHOT value. If the MS cancels the transmission of CT_CSBK_Prop at some point , in this case the MS will reduce the CT_RHOT The upper and lower limits of the table are 120ms each . This can improve the transmission CT_CSBK_Prop The probability is higher than the previous time. This 120ms The decrement is used until the predetermined CT_CSBK_Prop When a packet is sent or the lower range value reaches 0 , the CT_RHOT range will be set back to 2.16s to 3.24 seconds.

        - CT_CSBK_Term

CT_CSBK_Term This character can be considered a message termination character. It is transmitted as a frame end after a specific transmission service. An MS will immediately follow a voice transmission, data transmission, or CSBK transmission, except in the following two cases: CT_CSBK_Term will not be sent after the header of an acknowledgment data frame or after an acknowledgment CSBK packet, such as the UU_Ans_Rsp for an OACSU call.

The following figure defines the use of channel timing termination frames in basic services, where CT_CSBK_Term It will replace the original confirmation message frame tail. In addition, the response CSBK and confirmation response do not need to add CT_CSBK_Term .

C

D

E

F

Term

LC

CT

CSBK

Term

Voice Call

Data

Data

Data

Data

Data

Last Block

CT

CSBK

Term

Unconfirmed Data

Data

Data

Data

Data

Last Block

CT

CSBK

Term

Confirmed Data

C_R HEAD

CSBK

CT

CSBK

Term

Resp

CSBK

Figure 135 Channel timing CSBK termination

        - CT_CSBK_Req

CT_CSBK_Req The request packet is used to send a request to obtain the current leader and channel timing information. Before sending, the MS A random waiting time CT_RHOT is used . CT_CSBK_Req is sent only when the channel is idle.

        - CT_CSBK_Resp

CT_CSBK_Resp is used to be transmitted as a response to the current leader and channel timing information request, and is also used as an error correction information frame when the MS When incorrect channel timing information is observed, the processing mechanism adopted by the sender is also the random wait timer CT_RHOT .

###### DCDM State Flow

This chapter will introduce DCDM The state flow under the mode is described, mainly involving 4 The process of confirming the master state about the leader and timing information.

##### Leader and slot unknown status

The following figure describes the start-up of the intercom or switching from other channels to DCDM In the mode channel, the walkie-talkie needs to handle the process. Need to initialize its own wide area timing ID ( MS_WATID ) , which is generated by MS_DI and MS_ID It is composed of two parts. At the same time, it is also necessary to set the generation (MS_Gen ) , synchronization period ( MS_SA ) and its leader WATID . The initial values of these three variables are all set to 0. After the initial value setting is completed, MS It is necessary to start the leader search timer and enter the state where the leader and timer are unknown.

MS

开机启动、

信道改变

MS_Gen= 0,

MS_SA= 0,

MS_LWATID= 0

初始MS_DI、

MS_ID

Leader_and_Ti ming_Unknown

设置定时器

NoLeaderTimer

Figure 136 Power on and channel change state flow chart

##### Leader and slot unknown state process

The following figure describes the state flow chart when the leader and time slot are unknown. It mainly involves three state branch processes.

1. When MS In this state, a sending request is made, and the MS The sending process will be established, but due to the MS If it is sent without knowing the leader , the end CT_CSBK_Term frame sent by the service will use an SDI value of 0 .

2. MS​ Receive CT_CSBK frame, it will enter CT_CSBK Frame evaluation analysis status;

3. After the MS times out in its Leader search, it will actively send a CT_CSBK_Req to obtain the required information. The MS will first start the random hang-up wait timer CT_RHOT . In this state, there are two branches. First, if a CT_CSBK packet is received during the waiting process , the information in the CT_CSBK packet is judged. If the received LWATID is equal to 0, or LWATID Not 0 And LDI Smaller than MS MS_DI​ MS​ Only the receiving time slot information is obtained and the system will continue to wait in the sending CT_CSBK_req state. If the received LWATID is not 0 and LDI Greater than or equal to MS MS_DI​ MS​ Will release CT_RHOT Timer enters CCE Second, when CT_RHOT After the timer expires, if the current channel is idle, the MS CT_CSBK will be sent Then it returns to the leader and time slot unknown state waiting, where the CT_CSBK packet sets Gen , SA , LWATID , NL , CTO All are 0 , SDI is set to MS_DI , and SID is set to MS_ID . If the current channel is busy, the transmission is rejected and the MS resets CT_RHOT to try to send. The upper limit of the sending attempt is 2 minutes, after which the sending will be canceled and return to the original state.

| SDI:=MS_DI |  |
| --- | --- |
| Set (CT_RHOT) (Total_Timer) |  |

TX_CT_CSBK

Leader_and_ Timing_Unknown

RX_CT_CSBK

Total_Timer

CT_RHOT

else

LWATID

=0

Channel_	Yes

Busy

no

LDI

<MS_DI

Leader_and_ Timing_Unknown

TX_Denied

Leader_and_ Timing_Unknown

TX_CT_ CSBK

Set (CT_RHOT)

CT_CSBK_Req Gen=0 SA=0

LWATID=0 SDI=MS_DI SID=MS_ID NL=0 CTO=00

TX_CT_CSBK

Cancel Tx_CT_CSBK

Leader_and_ Timing_Unknown

NoLeaderTimer

RX_CT_CSBK

TX_Request

CCE

Cancel (CTRHOT)

CCE

SDI:=0

CT_CSBK_Term

Transmit

TX_CT_CSBK

Slot_Timing:= RX_Slot_Timing

Figure 137 Leader and slot unknown state flow chart

##### Leader unknown state process

The following figure defines the state flow diagram for the case where the time slot is known but the leader is unknown. The processing flow is similar to the case where both the leader and time slot are unknown. It mainly involves three state branch processes.

1. When MS In this state, a sending request is made, and the MS A sending process will be established.

2. MS​ Receive CT_CSBK frame, it will enter CT_CSBK Frame evaluation analysis status;

3. After the MS times out in its Leader search, it will actively send a CT_CSBK_Req to obtain the required information. The MS will first start the random hang-up wait timer CT_RHOT . In this state, there are two branches. First, if a CT_CSBK packet is received during the waiting process , the information in the CT_CSBK packet is judged. If the received LWATID is equal to 0, or LWATID Not 0 And LDI Smaller than MS MS_DI​ MS​ Only the receiving time slot information is obtained and the system will continue to wait in the sending CT_CSBK_req state. If the received LWATID is not 0 and LDI Greater than or equal to MS MS_DI​ MS​ Will release CT_RHOT Timer enters CCE Second, when CT_RHOT After the timer expires, if the current channel is idle, the MS CT_CSBK will be sent Then it returns to the leader and time slot unknown state waiting, where the CT_CSBK packet sets Gen , SA , LWATID , NL , CTO All are 0 , SDI is set to MS_DI , and SID is set to MS_ID . If the current channel is busy, the transmission is rejected and the MS resets CT_RHOT to try to send. The upper limit of the sending attempt is 2 minutes, after which the sending will be canceled and return to the original state.

Yes

LDI

<MS_DI

Leader_Unknown

TX_Denied

Leader_Unknown

CT_CSBK_Req Gen=0 SA=0

LWATID=0 SDI=MS_DI SID=MS_ID NL=0 CTO=00

Leader_Unknown

Transmit

Cancel Tx_CT_CSBK

Set (CT_RHOT)

(Total_Timer)

NoLeaderTimer

TX_CT_CSBK

RX_CT_CSBK

TX_Request

CCE

Cancel (CTRHOT)

CCE

TX_CT_CSBK

Leader_Unknown

RX_CT_CSBK

Total_Timer

Slot_Timing:= RX_Slot_Timing

TX_CT_CSBK

CT_RHOT

else

LWATID

=0

Channel_ Busy

no

Set (CT_RHOT)

TX_CT_ CSBK

Figure 138 Leader unknown state flow chart

##### Leader and slot known state process

The following figure describes MS When the leader and time slot are known, the state flow chart mainly has four branches.

1. When MS In this state, a sending request is made, and the MS A sending process will be established;

2. When MS Receive CT_CSBK frame, it will enter CT_CSBK Frame evaluation analysis status;

3. When SyncAgeWarning ( value is 9 before losing synchronization information) After the timer expires, MS Will actively send CT_CSBK Request to obtain updated synchronization information, MS The sending waiting timer CT_RHOT will be started . If the MS Receive CT_CSBK Package, judge CT_CSBK Package information, a case CTO 10 or 11​ And LWATID Greater than MS_LAWTID , another case CTO is 10 or 11 and LWATID Equal to MS_LWATID And SA Less than MS_SA , in the above two cases MS Will release CT_RHOT Entering the CCE state, the remaining received CT_CSBK Case MS Will keep the current CT_CSBK The sending status remains unchanged. After CT_RHOT is satisfied, MS The decision of whether to send will be based on the current channel idle status. The maximum time the MS attempts to send is 2 minutes.

4. When SyncAge Satisfied is, MS Will release known status information and reset NoLeaderTimer Entering leader unknown state.

| MS_Gen:= 0, MS_SA:= 0, MS_LWATID:= 0 |  | Set (NoLeaderTimer) |
| --- | --- | --- |

CCE

Leader_Unknown

TX_CT_CSBK

Leader_and_ Timing_Known

RX_CT_CSBK

Total_Timer

CT_RHOT

10,11

CTO

>MS_LWATID

00,01

Channel_ Busy

no

Yes

LWATID

<MS_LWATID

Leader_and_ Timing_Known

TX_Denied

=MS_LWATID

<MS_SA

SA

else

Leader_and_ Timing_Known

TX_CT_CSBK

TX_CT_ CSBK

Set (CT_RHOT)

CT_CSBK_Req Gen=MS_Gen SA=MS_SA LWATID=MS_LWATID SDI=0

SID=MS_ID NL=0 CTO=10

TX_CT_CSBK

Transmit

Cancel Tx_CT_CSBK

Set (CT_RHOT)

(Total_Timer)

Leader_and_ Timing_Known

SyncAge

SyncAgeWarning

RX_CT_CSBK

TX_Request

Cancel (CTRHOT)

CCE

Figure 139 Leader and slot known state flow chart

##### Leader State Process

The following figure describes MS The work process in the leader state mainly involves three branches: receiving CT_CSBK processing, sending requests, and sending beacon interval CT_CSBK . Mainly about the received CT_CSBK The processing process is described as follows: First, the received LWATID is compared with the MS 's MS_LWATID . There are three branches:

1. When the received LWATID Greater than MS_LWATID When​ Receive access receive leader processing flow;

2. When the received LWATID Equal to MS_LWATID When​ Enter Leader ID conflict resolution process;

3. When the received LWATID Less than MS_LWATID When the received SDI With MS_SDI , if SDI Greater than MS_SDI , then MS Enter the process of accepting the new leader, otherwise enter the process of sending CT_CSBK error correction information status.

This process also applies to LDR Status judgment process, that is, leader CT_CSBK Information evaluation and analysis.

else

Total_Timer

CT_RHOT

else

SDI

>MS_DI

SC

ANL

IC

AL

Channel_	Yes

Busy

no

TX_Denied

Leader

CT_CSBK_Beacon Gen=0

SA=MS_SA LWATID=MS_WATID SDI=MS_DI SID=MS_ID

NL=0 CTO=11

TX_CT_CSBK

Set (CT_RHOT)

(Total_Timer)

Set (Beacon_Interval)

Transmit

Leader

RX_CT_CSBK

Tx_Request

TX_Beacon

Leader

Cancel TX_CT_CSBK

BeaconInterval

LDR

Leader

=MS_WATID >MS_WATID

LWATID

Set (CT_RHOT)

TX_CT_ CSBK

Figure 140 Leader State Flowchart

      - Leader ID Conflict State Process

The leader ID is described in the figure below Conflict Situation Leader MS Processing flow chart of Leader ID A conflict occurs when the DI and ID information of the CT_CSBK received by the leader are the same as those of the MS . The main judgment process for handling it is as follows:

1. If the received SA With MS_SA The difference is +-5 Seconds range, then MS will still be considered a leader;

2. If the received SA With MS_SA The difference is 5 Multiples of seconds, then MS It will still be considered as the leader and enter the send error correction timing CSBK state;

3. In addition to the above two situations, MS A new ID will be generated With the current MS_ID Comparing the received LIDs under different conditions With NewMSID , if NewMSID Greater than LID MS​ Still the leader, otherwise it can only receive the leader.

IC

SAmod BeaconInterval)

=MS_SA

else

SA

else

SC

=MS_ID

New_MS_ID

else

>LID

MS_ID

else

Leader

AL

MS_ID:=

New_MS_ID

Generate_New_ MS_ID

Figure 141 Leader ID Conflict State Flowchart

CCE

=MS_WATID

else

LWATID

=1

=0

NL

SID

else

=0

MS_LWATID

else

else

01

CTO

else

<MS_LWATID

LWATID

-

SC

<MS_LWATID

>MS_LWATID

LWATID

=MS_LWATID

10

>MS_SA

CTO

SA

<MS_SA

11

=MS_SA

SC

11

CTO

LWATID

=MS_Gen

>MS_Gen

10

Gen

0

<MS_Gen

TP

AL

ANL

Leader_and_ Timing_Known

Slot_Timing:= RX_Slot_Timing

MS_Gen:=Gen+1

Slot_Timing:= RX_Slot_Timing

Slot_Timing:= RX_Slot_Timing

Slot_Timing:= RX_Slot_Timing

Generate_New_ MS_WATID

CT_CSBK Information Assessment Status Process

| Slot_Timing:= RX_Slot_Timing |  |  |
| --- | --- | --- |
| MS_Gen:=0, MS_SA:=SA, LWATID:= MS_WATID |  |  |
| Set (BeaconInterval) |  |  |

TX_Beacon To Leader

Leader

Figure 142 CT_CSBK Information Assessment Status Flowchart

      - Send correction CT_CSBK State Flow

The following figure describes the sending of the corrected CT_CSBK state flow chart.

<MS_LWATID

SWATID

else

-

Currently_

no	Leader	Yes

CCE

LDR

CT_CSBK_Resp Gen=0 SA=MS_SA

LWATID=MS_WATID SDI=MS_DI SID=MS_ID

NL=0 CTO=10

CT_CSBK_Resp Gen=MS_Gen SA=MS_SA LWATID=MS_LWATID SDI=MS_DI SID=MS_ID

NL=0 CTO=10

LWATID

LWATID

else

<MS_LWATID

TX_Denied

Channel_ Busy

no

CTO

else

Set (CT_RHOT)

(Total_Timer)

TX_CT_CSBK

TX_CT_CSBK

Leader_and_ Timing_Known

Leader

else

SC

TX_CT_CSBK

RX_CT_CSBK

-

Set (CT_RHOT)

Total_Timer

CT_RHOT

00,01

Yes

-

Cancel Tx_CT_CSBK

Figure 143 Send correction CT_CSBK State Flowchart

##### Accepting the leader status process

The following figure describes the state flow chart of accepting the leader . By receiving CT_CSBK CTO​ The field determines whether to accept the leader MS .

If you receive the CTO 00​ Or 01 , then MS will not accept leaders;

If you receive the CTO is 10 or 11 , MS Will receive channel timing information and channel timing parameters, MS Set

SyncAge and SyncAgeWarning The initial value of the timer is the received SA value.

AL

00,01

CTO

10,11

-

| MS_Gen:=Gen+1, MS_SA:=SA, MS_LWATID:= LWATID |  |
| --- | --- |
| Set (SyncAgeWarning) |  |
| Set (SyncAge) |  |
| Slot_Timing:= RX_Slot_Timing |  |

Leader_and_ Timing_Known

Figure 144 Accepting Leader State Flowchart

##### Designate a new leader state process

The following diagram describes the state flow of appointing a new leader.

1. First set Gen When it is 1 , MS_LWATID is set to the received SWATID , and the MS starts CT_RHOT and enters the waiting state for sending CT_CSBK ;

2. MS​ When receiving CT_CSBK during the random wait state , the timing information will be judged and processed as follows: If LWATID is received Greater than MS_LWATID Or received SWATID If it is greater than MS_LWATID , the MS will release CT_RHOT and cancel the transmission; in other cases, the MS will maintain the original state;

3 . When CT_RHOT When satisfied, MS Determine the channel idle state and send, if the channel is busy MS Will reset

CT_RHOT : Start retrying sending, the maximum attempt time is 2 minute.

Note: This is where the Leader is assigned. Previously added a level about SDI The priority judgment of the protocol is not mentioned in the protocol standard document , but it was found that there are limitations of this function during the actual testing process.

LWATID

else

Channel_ Busy

no

Yes

SWATID

else

>New_Leader

-

TX_Denied

-

Leader_and_ Timing_Known

CT_CSBK_Req Gen=MS_Gen SA=MS_SA LWATID=MS_L WATID SDI=MS_DI SID=MS_ID NL=1

CTO=10

TX_CT_CSBK

Cancel Tx_CT_CSBK

Set (CT_RHOT)

(Total_Timer)

RX_CT_CSBK

ANL

TX_CT_CSBK

Cancel (CTRHOT)

CCE

SDI>=MS_ DI>00

Yes

MS_Gen:= 1, MS_LWATID:= SWATID

Leader_and_ Timing_Known

Transmit_ CT_CSBK

Total_Timer

CT_RHOT

<New_Leader

Set (CT_RHOT)

-

Figure 145 Designate New Leader State Flowchart

##### Scheduled push status process

The following figure describes the state flow chart of channel scheduled push sending.

1. First, MS From the received CT_CSBK Get timing parameters, Gen , SA etc., where Gen+1 ;

2. MS​ CT_RHOT will be started The timer starts and attempts to send CT_CSBK ;

3. MS​ Enter CT_CSBK After sending the status, in CT_RHOT If CT_CSBK is received before it is satisfied Package, then

MS CT_CSBK will be analyzed The packet information is processed as follows: If the received LWATID Greater than

MS_LWATID or the received SWATID is greater than MS_WATID , the MS will cancel sending the push CT_CSBK and enter the CCE state ; if the received LWATID is equal to MS_LWATID , and CTO is equal to 11 , SA is equal to MS_SA MS​ The CT_RHOT will be released , the push will be canceled and the state will return to the leader and time slot known state; in other cases , the MS will maintain the original state.

4. When CT_RHOT After meeting the MS The idle time slot will be judged. If the channel is idle, the MS Will send scheduled push

TP

CT_CSBK , if the channel is busy MS Will send rejection and restart send wait, the total maximum attempt time is 2 minute.

| MS_Gen:= Gen+1, MS_SA:=SA MS_LWATID:= LWATID |  |
| --- | --- |
| Set (CT_RHOT) (Total_Timer) |  |

>MS_LWATID

SWATID

TX_Denied

else

<MS_LWATID

LWATID

Leader_and_ Timing_Known

=MS_LWATID

CTO

else

11

CCE

else

SA

=MS_SA

-

TX_CT_CSBK

Cancel (CTRHOT)

Cancel (CTRHOT)

CT_CSBK_Prop Gen=MS_Gen SA=MS_SA LWATID=MS_LW ATID SDI=MS_DI SID=MS_ID NL=0

CTO=11

TX_CT_CSBK

Leader_and_ Timing_Known

RX_CT_CSBK

Total_Timer

CT_RHOT

-

Set (CT_RHOT)

>MS_LWATID

LWATID

else

Channel_ Busy

no

Yes

-

Cancel Tx_CT_CSBK

Figure 146 Scheduled push status flow chart

#### Application Examples

DCDM The key point of the mode implementation is the setting and acquisition of channel timing information, namely Channel Timing CSBK The corresponding sending type is described in the previous chapter. There are five types of CT_CSBK Transmission, where CT_CSBK_Beacon For Leader MS sends periodic intervals, CT_CSBK_Prop is used for non-leader MS channel timing push, CT_CSBK_Term End terminator for regular services, CT_CSBK_Req Used to obtain timing information sending request , CT_CSBK_Resp is used to send correction channel timing information.

DCDM MS in mode It will mainly go through the four working states described above, which will not be described in detail in this chapter.

###### Basic Unit

In DCDM In the functional mode, several important basic units are mainly involved. The information comes from CT_CSBK This section explains some of the most basic unit concepts in signaling .

##### Time slot unit

DCDM Mode is TDMA work in a manner that defines TDMA Slot1 And TDMA Slot2 , usually when set by the frequency writing software, one channel can only set one working time slot. After the corresponding working time slot is set, the walkie-talkie will send and receive the corresponding time slot synchronization frame.

The definition and related description of time slots are as follows:

1 . DCDM In this mode, after the time slot is set, the corresponding transmission synchronization frame type is distinguished by the time slot definition. The reception can receive the synchronization frame type of the two time slots respectively . However, for the reception of voice, data and other functional services, only the set working time slot can be processed. The acquisition of the time slot is still CT_CSBK The acquisition of information can be received and judged in another time slot ;

2 . For the channel switching process between two time slots at the same frequency, DCDM The corresponding channel timing related information does not change in this mode;

3 . When switching between two time slots at the same frequency, the radio will send a TP (channel timing push information ) on the other time slot when switching to the other time slot .

4. When switching between two channels at different frequencies or starting up, the walkie-talkie will obtain channel-related information from the initial state ;

5. The color code settings of two time slot channels at the same frequency can be different, but the color code F must be used for the CT_CSBK frame .

##### Leader

DCDM In the model system, Leader MS It is the key to the operation of the entire system. Whether it is a newly powered on or working walkie-talkie, the first step in its working process is to obtain channel timing information through various means, including the boundaries and leaders used to establish time slots. MS Each intercom in the system can designate and accept Leader according to its corresponding priority and system working status. The entire system may be affected by the Leader in the system during operation. Lost and re-specified.

As DCDM The Leader in the system mainly needs to send beacon messages at intervals to maintain the system's channel timing information (for details, please refer to the Leader State Process ) .

##### generations

The following table shows CT_CSBK The definition of generation in the , generation is 0 It is the leader of DCDM system and the only generation when DCDM system is working.

Table 78 Generation ( Gen ) Definition

| Information Unit | length | value | describe |
| --- | --- | --- | --- |

| Generation | 5 | 0 | MS Set when the leader is unknown |
| --- | --- | --- | --- |
|  |  | Other values |  |

The following figure simply illustrates the range coverage corresponding to the generation, where the leader's 0 The generation is unique. The general process is as follows:

1 , No. 0 Sending beacon messages on behalf of radio equipment;

2. No. 1 After receiving the beacon message, the generation radio equipment sends a mandatory broadcast message to spread the channel timing information;

3. 2nd​ The first generation of radio equipment received After the mandatory propagation message is sent, the mandatory propagation message is sent;

4. Expand the coverage in the same way.

It should be noted that the n+1 The radio equipment of the nth generation can only be Generations of radio equipment receive channel timing information, and if cross- generation reception occurs, the information is considered incorrect.

Figure 147 DCDM Multiple radio coverage diagram in mode In addition, two points need to be explained:

1. As shown in the figure above, when the 0th After the beacon message is sent, according to the normal mechanism, the corresponding 1st Generation devices G1-N , G1-S , G1-W , G1-E will be required to send a mandatory propagation message. How to solve 4 No.1​ To solve the transmission conflict problem of the next-generation devices , a concept called random delay suspension is proposed. In the system, it is assumed that G1-N Once the CT_CSBK message is sent , the corresponding other devices will automatically cancel the sending after receiving the message. This will be explained in detail in the timer definition section.

2. The 	second point is about The corresponding relationship between the sending times of generations 0 , 1 , 2 , 3... is that they are sent in sequence. As shown in the figure below, the beacon interval can be divided into the sending times of several generations.

Figure 148 Channel timing diffusion timing diagram

##### Synchronous period

The following table shows CT_CSBK Sync​ Age ) .

Table 79 Synchronization Period ( SA ) Definition

| Information Unit | length | value | describe |
| --- | --- | --- | --- |
| Sync Age | 11 | 0 | MS Set when the leader is unknown |
|  |  | Other values | Sync in Beacon Age Incremental, increments are 500ms |

0 in the table The description of the value is in accordance with the protocol standard, but usually , the SA value corresponding to the leader when sending a beacon message is not necessarily 0. The reason is the existence of the CT_RHOT timer.

SA The use of SyncAge and SyncAgeWarning Two timers.

##### Identifier

DCDM The identifiers in the system mainly include ID Identifier, Dynamic ( DI ) Identifier and WATID composed of both , as described below.

At the same time, in order to distinguish the leader from other devices, the DCDM system defines a leader identifier and a source identifier. The variables defined in the previous section 2.3 are explained as follows:

LDI : Leader dynamic identifier; SDI : MS source dynamic identifier; MS_DI : MS initial dynamic identifier; LWATID : Received leader WATID MS_WATID : MS source WATID MS_LWATID : MS WATID​

WATID By DI and ID It consists of two parts and is defined as a wide area timing ID , usually for MS For example, there is the MS_LWATID of your system The important basis for evaluating channel timing in the system is the received LWATID and SWATID .

Table 80 WATID definition

| Information Unit | length | value | describe |
| --- | --- | --- | --- |
| Wide | 2 |  | DI value |
| Area | 20 |  | ID value |
| Timing |  |  |  |
| Identifier |  |  |  |

The following table defines DCDM Mode CT_CSBK The definition of the ID ( Identifier ) field . Distinguished as Leader ID and Source ID , for Leader MS For its LID and SID is the same as for non-leader MS Its LID It is the Leader ID in the system . If the leader is unknown, it is set to 0 .

Table 81 Identifier ( ID ) definition

| Information Unit | length | value | describe |
| --- | --- | --- | --- |
| Identifier | 20 | 0 | ID when power on or channel changed Set to 0 . Leader ID when the leader is unknown The value is set to 0 . |
|  |  | Other values | MS ID​ instruct |

Additional information about the device ID The value will be in DCDM There are changes in the dynamic system, when there is an ID In case of value conflict, please refer to the state flow chart in Section 2.3 for details.

The following table defines Dynamic Identity In the actual system CT_CSBK During the receiving process, DI is an important basis for determining timing information.

Table 82 DI definition

| Information Unit | length | value | describe |
| --- | --- | --- | --- |
| Dynamic Identifier | 2 | 00 | SDI when power on or channel changed value LDI when the leader is unknown value |
|  |  | 01 | Leaders have low priority |
|  |  | 10 | Leader Priority Medium |
|  |  | 11 | Leaders have high priority |

About DI The definition of each intercom device's initial DI The values may be different, but one thing that must be determined is DI The value is 00 cannot be confirmed as the initial leader of the system. When the leader of the system is confirmed, the non-leader MS Send CT_CSBK Information corresponding to the SDI The value is 00. When the leader is lost in the system, it will return to the initial value at startup. May change its MS_DI The value is 00 To give up its leadership role. Actual test found that during the long test process, when the leader MS​ The leader in the system sends the change when the battery is low.

##### Channel Timing Opcode

Channel Timing Operation Code Timing Opcod e ) , usually used to indicate CT_CSBK Type, distinguish the CT_CSBK type in different states.

Table 83 CTO definition

| Information Unit | length | value | describe |
| --- | --- | --- | --- |
| CT Opcode | 2 | 00 | Misaligned Request |
|  |  | 01 | Misaligned terminator |
|  |  | 10 | Channel timing status |
|  |  | 11 | Channel scheduled push |

It should be noted that in CT_CSBK_Term When sending, CTO The value should be set according to the following principles:

1. If MS If there is no leader information in the system, its value will be set to 00 ;

2. If MS There is leader information in the system and there is no DMR in another time slot of the current channel When the business signal

Its value will be set to 10 ;

3. If MS There is leader information in the system and a DMR exists in another time slot of the current channel When the service signal is sent , it is set to 10 for the leader MS , but needs to be set to 01 for the non-leader MS .

##### New leader logo

Table 84 New Leader definition

| Information Unit | length | value | describe |
| --- | --- | --- | --- |
| NL | 1 | 0 | MS Receive the current leader |
|  |  | 1 | When sending, MS Appoint a new leader; When receiving, the target MS Appointed as the new leader |

###### CT_CSBK PDU

      - CT_CSBK structure

| LB | PF | CSBKO |  |  |  |
| --- | --- | --- | --- | --- | --- |
| FID |  |  |  |  |  |
| Sync Age |  |  |  |  |  |
|  |  |  | Generation |  |  |
| Leader Identifier |  |  |  |  | NL |
|  |  |  |  | Ldr DI | Op1 |
| Source Identifier |  |  |  |  | R |
|  |  |  |  | Src DI | Op0 |
| CSBK CRC |  |  |  |  |  |

CT_CSBK PDU

|  | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |  |

Figure 149 CT_CSBK Structure diagram

      - CT_CSBK Field Definition

The following table defines CT_CSBK PDU The following is a list of definitions of several key fields. Table 85 CT_CSBK PDU Definition

| Information Unit | length | describe |
| --- | --- | --- |
| LB | 1 | Set to 1 |
| PF | 1 | Set to 0 |
| CSBKO | 6 | Set to 000111 |
| FID | 8 | Set to 00000000 |
| SyncAge | 11 |  |
| Generation | 5 |  |
| Leader Identifier | 20 |  |
| New Leader | 1 |  |

| Leader Dynamic Identifier | 2 |  |
| --- | --- | --- |
| Channel Timing Op1 | 1 |  |
| Source Identifier | 20 |  |
| Reserved | 1 |  |
| Source Dynamic Identifier | 20 |  |
| Channel Timing Op0 | 1 |  |
| CRC | 16 | CRC16 Verification result information |

###### Timer definition

The following table defines DCDM Related timers used in the system . The definition of is relatively special and will be described in detail below.

Table 86 Timer constant definition

| name | value | describe |
| --- | --- | --- |
| CT_RHOT | 0~3.24 Second | CT_CSBK The transmission process randomly suspends the waiting timer, which is mainly used to reduce CT_CSBK Transmission conflicts. 0~3.24 is defined when the leader and time slot are unknown seconds, in increments of 60ms . When the leader and time slot are known, the range is 2.16 seconds to 3.24 seconds, in increments of 60ms . CT_CSBK appears in the middle Transmission canceled, reducing increment range by 120ms |
| NoLeader | 4.5 minute | the MS is powered on or the channel is changed, When sending CT_CSBK Wait for search before requesting Leader's Time |
| SyncAge | 10 minute | MS The synchronization information needs to be updated regularly, with an increment of 500ms |
| SyncAgeWarning | 9 minute | MS Reminder before updating synchronization information |
| BeaconDuration | Minimum 600ms | CT_CSBK_Beacon and CT_CSBK_Prop Transfer duration |
| BeaconInterval | 4.5 minute | Beacon interval transmission time |
| CTDuration | Minimum 180ms | CT_CSBK Request and CT_CSBK Response transmission time |
| TxTimer | 2 minute | Send CT_CSBK Maximum time to try |

      - CT_RHOT illustrate

CT_RHOT , also known as CT_CSBK Transmission Random HoldOff Timer , which can be understood as CT_CSBK Transmission random hang-up wait timer. It is mainly used to reduce the probability of CT_CSBK transmission conflicts in the system.

the CT_RHOT mentioned in the protocol standard The timer range is 0~3.24 Seconds, the increment is 60ms . This article uses the sliding window uniform distribution method with 1.08 The window size of seconds spans 3.24 The sliding window of seconds is evenly distributed and is selected for CT_RHOT The specific distribution table is shown in the following table, and the uniform distribution function U (lower limit, upper limit ) is defined for each row .

Table 87 CT_RHOT Value distribution table

|  |  | Uniform sliding window (unit: seconds ) |  |
| --- | --- | --- | --- |
| N | M | Lower limit | Upper limit |
| 0 | 18 | 2.16 | 3.24 |
| 1 | 17 | 2.04 | 3.12 |

| 2 | 16 | 1.92 | 3.00 |
| --- | --- | --- | --- |
| 3 | 15 | 1.80 | 2.88 |
| 4 | 14 | 1.68 | 2.76 |
| 5 | 13 | 1.56 | 2.64 |
| 6 | 12 | 1.44 | 2.52 |
| 7 | 11 | 1.32 | 2.40 |
| 8 | 10 | 1.20 | 2.28 |
| 9 | 9 | 1.08 | 2.16 |
| 10 | 8 | 0.96 | 2.04 |
| 11 | 7 | 0.84 | 1.92 |
| 12 | 6 | 0.72 | 1.80 |
| 13 | 5 | 0.60 | 1.68 |
| 14 | 4 | 0.48 | 1.56 |
| 15 | 3 | 0.36 | 1.44 |
| 16 | 2 | 0.24 | 1.32 |
| 17 | 1 | 0.12 | 1.20 |
| 18 | 0 | 0.00 | 1.08 |

开始

否

信道定时领导信息是否已知

是

是否已经成功地发送信道定时消息

否

是

从U（下限，上限）提取随机值，其中下限

=M*0.12秒，上限至下限+1.08秒

是

是否发送成功

否

结束

设置M=18

一旦RHOT期满，则发送信道定时消息

启动具有随机超时的RHOT

如果M>0，则设置M=M-1

从U（0,3.24）提取随机值并且设置 M=18

Figure 150 CT_RHOT Using Flowcharts

CT_RHOT is defined as shown above. The specific process used is as follows:

1. The first step is to determine whether the channel timing leader information is known. If it is unknown, a random value is selected from U ( 0, 3.24 ) to set CT_RHOT , and M is set to the initial value of 18 .

2. The second step is to determine whether the channel timing message has been sent, that is, various CT_CSBK If the type frame is missing, the method in the first step in the unknown state is used ;

3. Based on the known M value, take a random value from the corresponding U (lower limit, upper limit) uniform distribution function, and then add the known M

Decrement value;

4. Set CT_RHOT After the value is set, the timer starts;

5. CT_RHOT​ After the count is full, the channel timing message is started to be sent; based on whether the message is sent successfully, if it is successful , the M value is set to 18 , if it is unsuccessful, the current M value is retained.

###### Minimized system example

This section briefly introduces a minimized DCDM System workflow based on DCDM A brief introduction to the process of building a system from scratch.

##### Standalone startup

开始

MS1开机

MS1等待 CT_CSBK_Rsp

MS1发送

CT_CSBK_Req

（3帧）

TotalTimer

结束

MS1重复发送

CT_CSBK_Req

（16帧）

Figure 151 Standalone startup flowchart

The figure above describes a single MS under normal circumstances. The post-boot process, i.e., how to obtain effective DCDM system timing information, can be found in 17.2.3.1 for its main status flow. Here is a brief overview of the process, which is mainly divided into the following

step:

1. Start NoleaderTimer and start sending CT_CSBK_Req after timeout . Request packet, the request packet starts with 3 Frame CT_CSBK

For the corresponding frame information settings, refer to 17.2.3.1 ;

2. After sending, wait for the response from the system. The timeout period of the entire sending is set to 2 Minutes later;

3 , 2 If no feedback signal is received after 1 minute, the MS The request information will be resent, and the number of data frames measured is 16

frame, and then ends the send request operation.

Note: The entire request process will only be executed once, and the subsequent system will be in an idle receiving state.

CTO 00​ The operation code is sent as follows.

##### Boot leader confirmation

The following figure defines and describes DCDM under normal circumstances The leadership establishment process after the system is turned on or after switching between different frequency channels. The corresponding main steps are described as follows:

1. MS1​ The first one to start up first satisfies NoleaderTimer , MS1 Send CT_CSBK_Req ;

2. MS2​ Received from MS1 Request packet to determine the current CT_CSBK Frame information, mainly in LWATID 0​ Determine its SDI at zero time Value to determine whether there is a Leader Qualifications, one thing must be explained about SDI 00​ Those who cannot be designated as leaders ;

3. If SDI If the value is non-zero, MS2 MS1​ Can serve as system leader, MS2 CT_CSBK_Rsp will be sent Used to designate MS1 as the leader of the DCDM system;

4. MS1​ After receiving the accurate After the information, mainly NL=1 As a sign, establish yourself as the leader of the system .

5. Supplementary explanation: Among the two machines with minimized system, if the first machine to start is MS1 If you do not have the qualifications to be a leader, then MS2 will then start with MS1 The same request sending process, if MS2 If it does not have it, the system will continue to remain in a leaderless state until a new machine enters the system.

是

开始

MS1发送CT_CSBK请求包

MS1先开机，MS2后开机

MS2 CCE

是

MS2指定MS1为领导者

MS2发送CT_CSBK包

MS1 CCE

MS1确立领导者地位

MS1准备发送信标消息

Figure 152 Start-up leader confirmation flow chart

##### Beacon message sending

To establish MS1 Leader​ The following is an example of a beacon message.

First of all, it should be noted that the beacon message is sent by the Leader , with a corresponding sending interval of every 4.5 minutes and a corresponding sending time length of 600ms ( 10 frames ) .

When MS1 When switching time slots at the same frequency, the corresponding MS1 After switching to another working time slot, a beacon message will be sent immediately in the other time slot.

MS2 in the system After receiving the beacon message, after CT_CSBK CCE After that, a forced propagation message is sent, which can also be considered as a scheduled push message. For the corresponding reference, refer to 17.2.3.11 .

MS2发送强制传播消息

MS2 CCE

是

MS2接收信标消息

MS1发送信标消息

Figure 153 Beacon message sending flow chart

##### Business sending process

The following figure describes the service delivery process in the DCDM system, which mainly involves the definition and delivery of CT_CSBK_Term and the definition of CTO .

Begin

Channel_Access

TX_Granted

TX_Denied

MS_LWAT ID

=0

else

TS1,TS2,Both

Channel Activity

End

CTO=00

CTO=10

CTO=01

Transmit_CT_ CSBK_Term

Transmit

Figure 154 Business sending flow chart

      - Leader Offline

Taking the above system as an example, MS1 After shutdown, the system will be in Leader Offline status, MS Last time Leader was received SyncAgeWarning is started after the beacon message and SyncAge Two timers, the corresponding times are 9 minutes and 10 minutes.

Timer 9 When the specified time is reached, the mandatory broadcast message is sent, similar to the one that needs to be sent after receiving a beacon message. This frame still contains timing information with MS1 as the leader . For details, refer to the processing flow in 2.3.4 .

Timer 10 Minutes meet MS2 Enter LeaderUnknown State, start a new Leader For details on the acquisition process, please refer to 17.2.3.3 .

MS2发送CT_CSBK_Req（以 MS1为Leader的信息）

MS2定时器 SyncAgeWarning满足时

MS1关机后

MS2进入领导者未知状态

MS2定时器SyncAge满足时

MS2重新尝试获取领导者

Figure 155 Leader Offline flow chart

###### HR_C7000 application

HR_C7000 Support DCDM Function development, and HR_C7000 Working on digital DMR The DCDM -related functions are realized in the Layer 2 mode . The conventional service frame sending and receiving mechanism is referred to the Layer 2 application manual and will not be described in detail in this article . In time slot DCDM Special application scenarios required by the functions include time slot management, color code, synchronization field control, dual receiving channel settings, etc.

##### Synchronous field application

DCDM Mode TDMA1 SYNC Corresponding to TDMA1 SLOT , TDMA2 SYNC Corresponding to TDMA2 SLOT ,

DCDM Mode Slot The definition needs to rely on the synchronization frame field for division.

The application of synchronization field mainly includes sending and receiving, among which sending requires time slot alternating sending TDMA1 and TDMA2 SYNC , send synchronization field information through the configuration register SEND_DATA_SYNC_H and SEND_DATA_SYNC_L , the configuration values are detailed in Table 1 .

DCDM The application does not support intercommunication with conventional handheld or transfer services, so for DCDM The receiver only supports TDMA1 and TDMA2 SYNC The time slot receiving detection does not support MS and BS SYNC Field reception detection. At the same time, you need to pay attention to TDMA1 and TDMA2 SYNC It is an independent time slot reception synchronization, so two sets of different time slots are required for synchronous reception

Mechanism to ensure TDMA1 and TDMA2 Normal reception of business .

DCDM Synchronous reception requires HR_C7000 Make the following settings:

1. Turn off MS , BS SYNC Receive, by configuring register RECV_MS_SYNC_H[31]=0 ,

RECV_BS_SYNC_H[31]=0 ;

2. Enable TDMA1 、 TDMA2 SYNC Detection, configuration register RECV_TDMA1_SYNC_H 、

RECV_TDMA1_SYNC_L , RECV_TDMA2_SYNC_H , RECV_TDMA2_SYNC_L ;

3. Enable dual-channel receiver and set register RF_MODE[11]=1 .

4. Receive enable needs to be always open.

##### Color code application

Since the CT_CSBK package uses ALL The color code of Site ( 0xF ) is different from the color code of other voice or data services. This requires the receiver to support different color code reception. Therefore, it is necessary to Set to color code without matching to decode the frame , the color code matching process needs to be processed by software, corresponding to the setting register LOCAL_CC[9:8]=1 . CT_CSBK Set register LOCAL_CC[3:0]=1111 when sending a packet .

##### Time slot management

DCDM mode differs from other modes in the way it manages time slots between radios to prevent conflicts. The service flow is essentially the same as other modes, except for the addition of message terminators. Within a given area, time slots are established by the leader (active establishment ) and remain consistent unless the leader becomes a non-leader. The leader periodically sends beacons to inform non-leaders of their time slot timings. Upon receiving the leader's time slot information (passive establishment ) , non-leaders do not close their time slots; instead, they synchronize their time slots based on the received beacon . If, after the time slot synchronization update timeout, no valid time slot beacon frame is received, the non-leader returns to the leader-unknown state. However, the established wide-area time slot remains open, and the next transmission will use the most recently synchronized time slot to minimize time slot conflicts. True dual-slot mode and time slot closure are not achieved until the non-leader powers off or switches channels.

In true dual-slot mode, the time of the relevant timers is fixed by default, as specified in the standard protocol. However, the maximum timeout for leader information and slot synchronization is 10 Minutes, so it is necessary to ensure that Even if no time slot beacon frame or time slot push frame is received within 2 minutes, it can still be guaranteed that the signals of the two time slots do not overlap or conflict. Therefore, it is necessary to ensure the consistency of time accuracy between different walkie-talkies.

In true dual-slot mode, the wide area identifiers are used to distinguish different handsets, and the relationship between wide area identifiers is used to establish a leader, where the leader's DI The value must be greater than or equal to the DI of the leader of the time slot , and the wide area identifier of the leader must be greater than the wide area identifier of the non-leader.

The following is the process control processing of network access status in true dual-slot mode.

开始

状态判断

初始化

领导者未知

领导者已知

领导者身份

设置相关参数为初

始值0

设置随机退避发送

定时器

设置同步更新超时

定时器

设置搜寻领导者的

超时定时器4.5分钟

设置同步更新超时

警告定时器

结束

设置为领导者未知状态

设置信标发送定时器

Figure 156 True dual-slot network access status management flow chart

DCDM In this mode, time slot management has the following application scenarios:

1. Receive information sent by other devices that have not obtained wide-area time slots before obtaining wide-area time slots;

2. Before obtaining the wide area time slot, the whole machine receives the information sent by the known leader synchronization time slot. At this time, the whole machine can obtain the wide area time slot of true dual time slot;

3. After obtaining the wide area time slot of the true dual time slot, receive the information sent by the unknown time slot whole device;

4. After obtaining the wide area time slot of the true dual time slot, receive the information sent by the whole device with the known time slot, which involves time slot update or correction;

5. If the whole machine needs to send information in an unknown time slot, it will directly initiate the sending process;

6. If the time slot is known and the whole machine needs to send information, it needs to use the obtained wide area time slot axis to establish the time slot and send information;

Based on the above time slot application scenarios, the HR_C7000 time slot management mechanism must meet the requirements of independent processing of transmit and receive time slots, as well as the independence of transmit time slots and wide area establishment time slots, so that signals sent at any time slot boundary can be received during idle time to realize the time slot management mode.

HR_C7000 For details on Layer 2 timeslot management, refer to HR_C7000. Application Manual ( DMR Digital Layer 2 applications ) .

We can define DCDM Wide area time slot using HR_C7000 Defined temporary time slot for sending, DCDM The sending and receiving time slots use the sending and receiving time slots defined by HR_C7000 , which can meet the requirements of DCDM time slot management.